| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0102-v1 |
| --- | --- |

| Title: | CE4-related: Interweaved Prediction for Affine Motion Compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Hongbin Liu, Yue Wang, Pengwei Zhao, Dingkun Hong 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

With the affine motion compensation (AMC) in the benchmark set (BMS), a coding-block is divided into sub-blocks as small as 4×4, each of which is assigned with an individual motion vector derived by the affine model. In this contribution, an interweaved prediction approach is proposed for AMC. With the interweaved prediction, a coding block is divided into sub-blocks with two different dividing patterns. Then two auxiliary predictions are generated by AMC with the two dividing patterns. The final prediction is calculated as a weighted-sum of the two auxiliary predictions. The interweaved prediction is only applied on the luma-component for affine-coded blocks with uni-prediction. Simulation results reportedly show 0.29% BD-rate savings under BMS Random Access (RA) configurations.

# Introduction

Fig. 1. 4×4 sub-blocks in a 16×16 AMC-coded block.

With affine motion compensation (AMC) in the benchmark set (BMS), a coding-block is divided into sub-blocks as small as 4×4, each of which is assigned with an individual motion vector (MV) derived by the affine model, as shown in Fig. 1. With the affine model, a MV is derived from mv0 and mv1 at the two control points as formulated as below.

| . | (1) |
| --- | --- |

AMC faces a dilemma on the size of sub-blocks. With smaller sub-blocks, AMC can achieve a better coding performance but suffers a higher complexity burden.

# Interweaved Prediction

To address the dilemma of AMC, interweaved prediction is proposed to achieve a finer granularity of MVs without increasing the complexity too much.

First, a coding block is divided into sub-blocks with two different dividing patterns. The first dividing pattern is the same as that in BMS-1.1 as shown in Fig. 2(a), while the second dividing pattern also divides the coding-block into 4×4 sub-blocks but with a 2×2 offset as shown in Fig. 2(b).

Second, two auxiliary predictions are generated by AMC with the two dividing patterns. The MV for each sub-block in a dividing pattern is derived from mv0 and mv1 by eq. (1).

The final prediction is calculated as a weighted-sum of the two auxiliary predictions, formulated as:

| . | (2) |
| --- | --- |

As shown in Fig. 3, an auxiliary prediction sample located at the center of a sub-block is associated with a weighting value 3, while an auxiliary prediction sample located at the boundary of a sub-block is associated with a weighting value 1.

In this contribution, the proposed interweaved prediction is only applied on the luma-component for affine-coded blocks with uni-prediction, so the bandwidth in the worst case is not changed.

| (a) Divided by 4×4 sub-blocks (as in BMS) | (b) Divided by 4×4 sub-blocks with an offset |
| --- | --- |

Fig. 2. Two dividing patterns.

Fig. 3. Weighting values in a sub-block

# Experimental Results

The proposed enhanced interweaved prediction is implemented on BMS-1.1, and results of Random Access (RA), Low-delay B (LB) and Low-delay P (LP) configuration is summarized in Table 1, Table 2 and Table 3 respectively. The simulations were performed following the JVET CTC [1]. More detail results can be found in the attached spreadsheets. Running time will be updated later.

Table 1: Compared with BMS-1.1 (RA)

|  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.01% | 0.08% |  |  |
| Class A2 | -0.44% | -0.01% | -0.09% |  |  |
| Class B | -0.46% | -0.29% | -0.19% |  |  |
| Class C | -0.15% | -0.10% | -0.03% |  |  |
| Class E |  |  |  |  |  |
| Overall | -0.29% | -0.12% | -0.07% |  |  |
| Class D | -0.31% | -0.15% | -0.18% |  |  |
| Class F (optional) |  |  |  |  |  |

Table 2: Compared with BMS-1.1 (LB)

|  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.18% | -0.08% | -0.10% |  |  |
| Class E | -0.42% | 0.31% | 0.31% |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.17% | -0.25% | -0.22% |  |  |
| Class F (optional) |  |  |  |  |  |

Table 3: Compared with BMS-1.1 (LP)

|  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.24% | -0.10% | 0.13% |  |  |
| Class E | -0.26% | -0.14% | -0.08% |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.41% | -0.26% | -0.35% |  |  |
| Class F (optional) |  |  |  |  |  |

# References

- J. Boyce, K. Suehring, X. Li, V. Seregin, JVET-J1010, “JVET common test conditions and software reference configurations”, Apr. 2018.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).