| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0103-v1 |
| --- | --- |

| Title: | CE4-related: Simplified Affine Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Hongbin Liu, Yue Wang, Pengwei Zhao, Dingkun Hong 8910 University Center Lane, San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In this contribution, affine prediction in the benchmark set (BMS) is modified in three aspects. First, the sub-block size is fixed to be 4×4, instead of being calculated by an equation with division operations. Second, the constraints of block size for affine merge mode and affine AMVP mode are unified. Both affine merge mode and the affine AMVP mode are applicable when the width and height of a block is greater than or equal to 8. Third, the sub-block size for chroma components is expanded from 2×2 to 4×4. Simulation results reportedly show 0.03% BD-rate increase on the Y component for Random Access (RA) configuration in average compared with BMS-1.1.

# Introduction

Fig. 1. 4×4 sub-blocks in a 16×16 block coded with AMC.

With the affine motion compensation (AMC) in the benchmark set (BMS), a coding block is divided into sub-blocks, each of which is assigned with an individual motion vector (MV) derived by the affine model, as shown in Fig. 1. With the affine model, a MV is derived from mv0 and mv1 at the two control points as formulated as below.

| . | (1) |
| --- | --- |

The sub-block size w×h is derived as in eq. (2), where W×H is the size of the current block.

| . | (2) |
| --- | --- |

After eq. (2), w and h are then adjusted downward until it is a divisor of W and H, respectively. Finally, w and h are set to be at least 4.

| . | (3) |
| --- | --- |

The minimum size of sub-block is 4×4 for the luma-component, while it is 2×2 for chroma-components if the colour format if 4:2:0.

In addition, two different constrains on the usage of affine prediction for merge mode and AMVP mode are applied. For the merge mode, affine prediction is applicable for a block with W×H >= 64, while for the AMVP mode, it is applicable for a block with W>8 and H >8.

There are several problems in the current design in BMS.

- The calculation of the sub-block size is complicated with division operations, but it cannot decrease the bandwidth in the worst case of affine prediction;

- The constrains for affine merge mode and affine AMVP mode are not the same;

- The minimum sub-block size of chroma-components is 2×2.

# Simplifications on Affine Prediction

To address the problems of affine prediction, three simplified methods are proposed.

- The sub-block size is fixed to be 4×4. Comparing to the current design, a bundle of complicated operations is removed without increasing the bandwidth in the worst case.

- The constrains on affine prediction are unified. Both affine merge mode and affine AMVP mode are applicable for a block with W >= 8 and H >= 8. To avoid necessary affine motion estimation for small blocks at encoder, affine AMVP mode is skipped for a block with W equal to 8 or H equal to8 if its parent block does not choose the affine mode.

- The minimum sub-block size of chroma-components is set to be 4×4. Based on the simplification 2), it is guaranteed that W or H for chroma components is 4 at least. Therefore, a block can be partitioned into 4×4 sub-blocks for chroma components. With the expanded sub-block size, the bandwidth in the worst case is reduced.

# Experimental Results

The proposed three simplifications are implemented on BMS-1.1. The simulations were performed following the JVET CTC [1] and the anchor data are generated with the BMS settings with BMS-1.1. Three tests are conducted incrementally:

Test 1: Simplification 1)

Test 2: Simplification 1) + 2)

Test 3: Simplification 1) + 2) + 3)

Results of Random Access (RA) configuration for the three tests are summarized in the Table 1, Table 2 and Table 3 respectively. More detail results can be found in the attached spreadsheets. Running time will be updated later.

Table 1: Test 1 compared with BMS-1.1 (RA)

|  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.06% | -0.02% |  |  |
| Class A2 | 0.00% | 0.04% | -0.08% |  |  |
| Class B | 0.00% | -0.04% | -0.04% |  |  |
| Class C | 0.00% | 0.00% | -0.04% |  |  |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.02% | -0.05% |  |  |

Table 2: Test 2 compared with BMS-1.1 (RA)

|  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | -0.07% |  |  |
| Class A2 | 0.02% | 0.04% | 0.01% |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.02% | -0.10% | 0.02% |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |

Table 3: Test 3 compared with BMS-1.1 (RA)

|  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.12% | 0.02% |  |  |
| Class A2 | 0.07% | 0.41% | 0.39% |  |  |
| Class B | 0.01% | 0.18% | 0.25% |  |  |
| Class C | 0.04% | 0.10% | 0.09% |  |  |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.14% | 0.19% |  |  |

# References

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, JVET-J1010, Apr. 2018.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).