| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0104-v5 |
| --- | --- |

| Title: | CE4-related: History-based Motion Vector Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang, Kai Zhang, Hongbin Liu Yue Wang, Pengwei Zhao, Dingkun Hong 8910 University Center Ln, San Diego, CA, U.S.A. 92122 | Tel: Email: | 1-858-999-7093 lizhang.idm@bytedance.com zhangkai.video@bytedance.com liuhongbin.v@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution presents a History-based Motion Vector Prediction (HMVP) method for inter coding. In HMVP, a table of HMVP candidates is maintained and updated on-the-fly. After decoding a non-affine inter-coded block, the table is updated by adding the associated motion information as a new HMVP candidate to the last entry of the table. A First-In-First-Out (FIFO) or constraint FIFO rule is applied to remove and add entries to the table. The HMVP candidates could be applied to either merge candidate list or AMVP candidate list. It is asserted that the line buffer size is kept unchanged compared to VTM. When the merge candidate list size is extended by 10, compared with VTM-1.0, simulation results reportedly show that HMVP with FIFO and 16 entries of the table achieves 1.00%, 0.51% and 1.04% BD rate reduction for RA Main10, LDB Main10, and LDP Main10 configurations respectively. Compared with BMS-1.0, simulation results reportedly show that HMVP achieves 0.81%, 0.42% and 0.44% BD rate reduction for RA Main10, LDB Main10, and LDP Main10 configurations respectively. In addition, when the merge candidate list size is kept unchanged, compared with VTM-1.0, 0.82% BD rate reduction for RA Main10 configurations are reported by applying HMVP with constraint FIFO and only 8 entries of the table.

# Introduction

In Versatile Video Coding (VVC), motion vectors of inter-coded blocks can be signaled in two ways: Advanced motion vector prediction (AMVP) mode or merge mode. With AVMP mode, a difference between the real motion vector and a motion vector prediction (MVP), a reference index and a MVP index referring to an AMVP candidate list are signaled. For the merge mode, a merge index referring to a merge candidate list is signaled and all the motion information associated with the merge candidate is inherited.

For both the AMVP candidate list and the merge candidate list, they are derived from temporally or spatially neighboring coded blocks. More specifically, the merge candidate list is constructed by checking the following four types of merge MVP candidates in order:

  - Spatial merge candidates (from five spatial neighbouring blocks, as depicted in Figure 1)

  - Temporal MVP (TMVP) merge candidate

  - Combined bi-predictive merging candidates

  - Zero motion vector merging candidates

Once the number of available merge candidates reaches the signaled maximally allowed merge candidates (e.g., 5 in common test conditions), the merge candidate list construction process is terminated.

Similarly, for the AMVP candidate list, three types of MVP candidates are checked in order:

- Up to two spatial MVP candidates (one from B0, B1, and B2, and the other from A0 and A1,  as depicted in Figure 1)

- Temporal MVP (TMVP)

- Zero MVP candidates

Figure 1. Positions of spatial neighbouring blocks used in merge and AMVP candidate list construction

# Proposed method

A history-based MVP (HMVP) method is proposed wherein a HMVP candidate is defined as the motion information of a previously coded block. A table with multiple HMVP candidates is maintained during the encoding/decoding process. The table is emptied when a new slice is encountered. Whenever there is an inter-coded block, the associated motion information is added to the last entry of the table as a new HMVP candidate. The overall coding flow is depicted in Figure 2.

Figure 2. Decoding flow char with the proposed HMVP method

- FIFO

- Constraint FIFO

Figure 3. Example of updating the table in the proposed HMVP method

In this contribution, the table size S is set to be 16, which indicates up to 16 HMVP candidates may be added to the table. If there are more than 16 HMVP candidates from the previously coded blocks, a First-In-First-Out (FIFO) rule is applied so that the table always contains the latest previously coded 16 motion candidates. Figure 3 (a) depicts an example wherein the FIFO rule is applied to remove a HMVP candidate and add a new one to the table used in the proposed method.

To further improve the coding efficiency, a constraint FIFO rule is introduced wherein when inserting a HMVP to the table, redundancy check is firstly applied to find whether there is an identical HMVP in the table. If found, the identical HMVP is removed from the table and all the HMVP candidates afterwards are moved, i.e., with indices reduced by 1.

HMVP candidates could be used in the merge candidate list construction process. All HMVP candidates from the last entry to the first entry in the table are inserted after the TMVP candidate. Pruning is applied on the HMVP candidates. Once the total number of available merge candidates reaches the signaled maximally allowed merge candidates, the merge candidate list construction process is terminated.

Similarly, HMVP candidates could also be used in the AMVP candidate list construction process. The motion vectors of the last K HMVP candidates in the table are inserted after the TMVP candidate. Only HMVP candidates with the same reference picture as the AMVP target reference picture are used to construct the AMVP candidate list. Pruning is applied on the HMVP candidates. In this contribution, K is set to 4.

In addition, when the total merge candidate number is larger than or equal to 15, a truncated unary plus fixed length (with 3 bits) binarization methods is applied to code an merge index. With the total number of merge candidates denoted as Nmrg, the binarization method is tabulated in Table 1.

Table 1 – Bin string of the merge index (assume Nmrg is 15)

| merge index | Bin String |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 0 |  |  |  |  |  |  |  |  |
| 2 | 1 | 1 | 0 |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |
| 5 | 1 | 1 | 1 | 1 | 1 | 0 |  |  |  |  |
| 6 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |  |  |  |
| 7 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 |
| 8 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 |
| … |  |  |  |  |  |  |  |  |  |  |
| 14 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Bin index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |

# Simulation results

The tables below show the BD-rate gain and processing time impact of the proposed approach. Three sets of simulations are conducted, following the common test conditions [1] on top of reference software BMS-1.0 [2] .

- VTM simulations

  - Anchor: VTM (by disabling all BMS tools in BMS-1.0)

  - Test: Anchor with Proposal

- BMS simulations

  - Anchor: BMS

  - Test: Anchor with Proposal

## Simulation results with 10 additional merge candidates

This sub-section reports the coding performance of HMVP with Nmrg equal to 15 and 16 for VTM and BMS, respectively. In this test, the table size S is set to 16 and FIFO rule is applied.

Table 2. Performance of HMVP under Random Access Main 10

|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.76% | -0.99% | -1.04% | 106% | 101% | -0.46% | -0.51% | -0.52% | 102% | 103% |
| Class A2 | -0.92% | -1.38% | -1.29% | 107% | 104% | -0.91% | -1.14% | -1.13% | 103% | 100% |
| Class B | -1.39% | -1.47% | -1.19% | 106% | 102% | -1.15% | -1.17% | -1.12% | 104% | 102% |
| Class C | -0.74% | -0.94% | -1.07% | 108% | 102% | -0.57% | -0.77% | -0.88% | 106% | 103% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.00% | -1.22% | -1.15% | 107% | 102% | -0.81% | -0.92% | -0.94% | 104% | 102% |
| Class D | -0.54% | -0.74% | -0.81% | 111% | 104% | -0.32% | -0.53% | -0.41% | 112% | 103% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table 3. Performance of HMVP under Low Delay B Main 10

|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.85% | -0.62% | -0.40% | 109% | 101% | -0.61% | -0.83% | -0.24% | 96% | 98% |
| Class C | -0.23% | -0.29% | -0.21% | 111% | 104% | -0.21% | -0.42% | -0.39% | 102% | 102% |
| Class E | -0.32% | -0.36% | -0.33% | 111% | 105% | -0.37% | 0.25% | 0.12% | 102% | 101% |
| Overall | -0.51% | -0.44% | -0.32% | 110% | 103% | -0.42% | -0.42% | -0.20% | 99% | 100% |
| Class D | -0.12% | -0.28% | -0.41% | 112% | 104% | 0.05% | 0.63% | -0.02% | 105% | 100% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table 4. Performance of HMVP under Low Delay P Main 10

|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.39% | -1.01% | -0.76% | 111% | 103% | -0.68% | -0.66% | -0.70% | 99% | 90% |
| Class C | -0.85% | -0.80% | -1.07% | 112% | 108% | -0.32% | -0.22% | -0.52% | 104% | 102% |
| Class E | -0.72% | -0.74% | -0.27% | 110% | 101% | -0.18% | 0.27% | -0.36% | 105% | 103% |
| Overall | -1.04% | -0.87% | -0.74% | 111% | 104% | -0.44% | -0.28% | -0.55% | 102% | 97% |
| Class D | -0.71% | -0.63% | -0.76% | 114% | 105% | -0.19% | 0.29% | 0.30% | 107% | 104% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

## Simulation results with same merge list size

This sub-section reports the coding performance of HMVP with Nmrg equal to 5 and 6 for VTM and BMS (i.e., unchanged), respectively. In this test, the table size S is set to 8 and constraint FIFO rule is applied.

Table 5. Performance of HMVP under Random Access Main 10

|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.61% | -0.70% | -0.73% | 96% | 100% | -0.28% | -0.26% | -0.40% | 75% | 83% |
| Class A2 | -0.74% | -1.03% | -0.99% | 95% | 98% | -0.50% | -0.61% | -0.59% | 77% | 84% |
| Class B | -1.13% | -1.17% | -0.98% | 99% | 103% | -0.73% | -0.76% | -0.68% | 91% | 97% |
| Class C | -0.66% | -0.90% | -0.99% | 99% | 103% | -0.38% | -0.55% | -0.53% | 98% | 102% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.82% | -0.98% | -0.93% | 98% | 101% | -0.50% | -0.58% | -0.57% | 86% | 93% |
| Class D | -0.40% | -0.40% | -0.52% | 99% | 103% | -0.20% | -0.31% | -0.30% | 98% | 100% |
| Class F (optional) | -0.63% | -0.68% | -0.70% | 97% | 101% | -0.28% | -0.26% | -0.40% | 75% | 83% |

Table 6. Performance of HMVP under Low Delay B Main 10

|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -0.26% | -0.15% | 0.01% | 79% | 87% | -0.16% | -0.17% | -0.11% | 89% | 98% |
| Class E | -0.26% | -0.44% | -0.31% | 93% | 91% | -0.16% | 0.63% | 0.38% | 90% | 99% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.21% | -0.35% | -0.18% | 101% | 99% |  |  |  |  |  |
| Class F (optional) | -1.03% | -1.17% | -0.85% | 89% | 95% |  |  |  |  |  |

Table 7. Performance of HMVP under Low Delay P Main 10

|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.08% | -0.42% | -0.45% | 92% | 96% |  |  |  |  |  |
| Class C | -0.66% | -0.49% | -0.66% | 87% | 91% | -0.26% | -0.05% | -0.23% | 92% | 96% |
| Class E | -0.39% | -0.55% | -0.81% | 100% | 108% | -0.26% | 0.31% | -0.49% | 95% | 95% |
| Overall | -0.76% | -0.48% | -0.61% | 92% | 97% |  |  |  |  |  |
| Class D | -0.55% | -0.35% | -0.41% | 102% | 102% |  |  |  |  |  |
| Class F (optional) | -1.17% | -1.56% | -1.16% | 91% | 98% |  |  |  |  |  |

## Simulation results with same merge list size

This sub-section reports the coding performance of HMVP with Nmrg equal to 5 and 6 for VTM and BMS (i.e., unchanged), respectively. In this test, the table size S is set to 8 and FIFO rule is applied.

Table 5. Performance of HMVP over VTM1.0

|  | Random Access Main 10 |  |  |  |  | LDB Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.64% | -0.79% | -0.80% | 97% | 99% |  |  |  |  |  |
| Class A2 | -0.73% | -1.07% | -0.96% | 96% | 101% |  |  |  |  |  |
| Class B | -1.07% | -1.15% | -0.91% | 98% | 102% | -0.56% | -0.13% | -0.29% | 87% | 85% |
| Class C | -0.64% | -0.77% | -0.88% | 97% | 101% | -0.26% | -0.12% | -0.10% | 79% | 88% |
| Class E |  |  |  |  |  | -0.14% | -0.30% | -0.40% | 92% | 94% |
| Overall | -0.80% | -0.96% | -0.89% | 97% | 101% | -0.35% | -0.17% | -0.26% | 85% | 88% |
| Class D | -0.41% | -0.38% | -0.36% | 100% | 103% | -0.18% | -0.01% | -0.75% | 99% | 100% |
| Class F (optional) | -0.56% | -0.57% | -0.61% | 98% | 100% | -0.80% | -1.25% | -0.66% | 86% | 97% |

### Additional simulation results with same merge list size

In this section, the results of HMVP only applied to the merge mode are provided (i.e., the usage of HMVP to the AMVP mode is prohibited). Simulation results are provided in the following tables:

Table 6. Performance of HMVP only applied to merge mode

|  | Random Access Main 10 (over VTM, merge list size: 5) |  |  |  |  | Random Access Main 10 (over BMS, merge list size: 6) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.53% | -0.53% | -0.59% | 94% | 95% | -0.25% | -0.28% | -0.23% | 66% | 76% |
| Class A2 | -0.54% | -0.67% | -0.65% | 94% | 94% | -0.31% | -0.46% | -0.39% |  |  |
| Class B | -0.83% | -0.68% | -0.62% | 99% | 103% | -0.51% | -0.53% | -0.44% | 63% | 82% |
| Class C | -0.52% | -0.56% | -0.63% | 100% | 102% | -0.25% | -0.28% | -0.32% | 59% | 60% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.63% | -0.61% | -0.62% | 97% | 99% | -0.35% | -0.40% | -0.36% |  |  |

# Conclusions

In this contribution, the HMVP method is proposed to improve the coding efficiency of MVP while there is no line buffer increased. In addition, the computational complexity is also kept very low since the total number of merge candidate list is kept unchanged and only up to 8 HPMV candidates may be checked in the merge candidate list construction process. It is recommended to consider adoption of the second test (i.e., HMVP with constraint FIFO, table size S set to 8 and merge candidate list size kept unchanged) to the next version of VTM/BMS.

# Reference

- “JVET common test conditions and software reference configurations”, J. Boyce, K. Suehring, X. Li, V. Seregin, doc. JVET-J1010, April 2018.

- BMS-1.0, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).