| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0105-v1 |
| --- | --- |

| Title: | CE9-related: Simplification of Decoder Side Motion Vector Derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu, Li Zhang, Kai Zhang Yue Wang, Pengwei Zhao, Dingkun Hong Yingdu Mansion F11, 48 Zhichun Road, Building B, Haidian District, 100086 Beijing, China | Tel: Email: | +86 13911971613 liuhongbin.01@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution presents two modifications on Decoder Side Motion Vector Derivation (DMVR) in the benchmark set (BMS): 1) DMVR for 4x4 blocks is disabled; 2) Partial Sum of Absolute Differences (SAD) is calculated instead of full SAD. Simulation results reportedly show 0.05%/0.07% BD-rate increase for BMS-1.0/VTM-1.0 under BMS Random Access (RA) configurations.

# Introduction

The benchmark set (BMS) [1] employs a decoder-side motion vector refinement (DMVR) method. In DMVR, the two motion vectors of bi-prediction are further refined by a bilateral template matching process. Nine MV candidates are searched for each list. The nine MV candidates include the original MV and eight surrounding MVs with one luma sample offset to the original MV in either the horizontal or the vertical direction, or both. Finally, the two new MVs, i.e., MV0′ and MV1′ as shown in Figure 1, are used for generating the final bi-prediction results. The sum of absolute differences (SAD) is used as the cost measure.

Figure 1. DMVR based on bilateral template matching

For a block with the size W×H, there are two ways to implement the DMVR process: 1) for each reference list, apply motion compensation (MC) 9 times with the same block size W×H but with refined MVs; 2) directly perform MC on a region with the size (W + 2) × (H + 2) to cover all possible refined MVs. Apparently, the second method will benefit for the hardware implementation. In the following discussion, our analysis is based on the second implementation method.

With DMVR in BMS, more samples are fetched to perform MV refinement and the worst-case bandwidth is increased largely. As can be seen in Table 1, the worst-case bandwidth is increased by around 40% (from 15.125 to 21.125 for 4x4 bi-predicted blocks) when DMVR is enabled.

Table 1: bandwidth comparison between DMVR on and DMVR off.

|  | 4x4 bi-predicted block | 4x8/8x4 bi-predicted block |
| --- | --- | --- |
| DMVR off | 2 × (4 + 7) × (4 + 7) / 16 = 15.125 | 2 × (4 + 7) × (8 + 7) / 32 = 10.3125 |
| DMVR on | 2 × (4 + 9) × (4 + 9) / 16 = 21.125 | 2 × (4 + 9) × (8 + 9) / 32 = 13.8125 |
| Ratio | 140% | 134% |

Moreover, full SAD is calculated for DMVR to select the best among several candidate blocks, which increases the decoding computational complexity.

# Proposed method

To address the problems mentioned above. We propose two simplifications on DMVR.

- Disable DMVR for 4x4 blocks.

- Partial SAD calculation: SAD is calculated for every 2 rows.

With proposed simplifications, when DMVR is enabled, the worst-case bandwidth is not increased anymore. Meanwhile, the computational complexity of SAD cost calculation is reduced by 50%.

# Experimental results

The proposed method was tested on top of reference software BMS-1.0 [1], and simulations were performed following the common test conditions [2]. Since DMVR is only enabled for the random access (RA) configuration, all simulation results are based on RA. The proposed method was evaluated under two configurations:

- All BMS tools are enabled.

- All BMS tools except DMVR are disabled, denoted as VTM-1.0 + DMVR on.

Two sets of simulations were conducted:

- Test #1: Partial SAD calculation.

- Test #2: #1 + Disable DMVR for 4x4 blocks.

The results are shown in Table 2 and Table 3. Note that the encoding time is not reliable because the encoding tasks may be run on computers with different cores or CPUs. To get reliable decoding time, we run the decoding tasks on one same computer for all test cases.

Table 2: Coding performance of Test #1

|  | Over VTM-1.0 + DMVR on |  |  |  |  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.08% | 0.08% | 100% | 99% | 0.03% |  | -0.01% |  | 0.01% |  | 81% | 100% |
| Class A2 | 0.08% | 0.11% | 0.10% | 100% | 99% | 0.05% |  | 0.02% |  | 0.04% |  | 83% | 100% |
| Class B | 0.06% | 0.02% | 0.06% | 97% | 99% | 0.04% |  | -0.01% |  | 0.03% |  | 111% | 100% |
| Class C | 0.10% | 0.03% | 0.02% | 105% | 100% | 0.08% |  | 0.12% |  | 0.06% |  | 132% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.07% | 0.05% | 0.06% | 100% | 99% | 0.05% |  | 0.03% |  | 0.04% |  | 103% | 100% |
| Class D | 0.09% | 0.12% | 0.11% | 125% | 100% | 0.11% |  | 0.03% |  | 0.13% |  | 124% | 100% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table 3: Coding performance of Test #2

|  | Over VTM-1.0 + DMVR on |  |  |  |  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.09% | 0.06% | 122% | 98% | 0.03% |  | 0.00% |  | 0.00% |  | 122% | 100% |  |
| Class A2 | 0.12% | 0.09% | 0.12% | 123% | 98% | 0.05% |  | 0.01% |  | 0.01% |  | 127% | 100% |  |
| Class B | 0.06% | 0.05% | 0.10% | 116% | 98% | 0.03% |  | 0.00% |  | 0.05% |  | 159% | 99% |  |
| Class C | 0.08% | 0.04% | 0.05% | 117% | 99% | 0.09% |  | 0.10% |  | 0.06% |  | 145% | 99% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.07% | 0.06% | 0.08% | 119% | 98% | 0.05% |  | 0.03% |  | 0.03% |  | 141% | 99% |  |
| Class D | 0.11% | 0.24% | 0.24% | 131% | 99% | 0.10% |  | -0.04% |  | 0.03% |  | 133% | 100% |  |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

# References

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

- J. Boyce, K. Suehring, X. Li, V. Seregin, JVET-J1010, “JVET common test conditions and software reference configurations”, Apr. 2018.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).