| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0106-v3 |
| --- | --- |

| Title: | Energy Assessment for Video Decoding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christian Herglotz André Kaup | Tel: Email: | +49 9131 85 27101 christian.herglotz@fau.de andre.kaup@fau.de |
| Source: | Multimedia Communications and Signal Processing, Friedrich-Alexander University Erlangen-Nürnberg (FAU) |  |  |

_____________________________

# Abstract

This proposal describes a contribution on the assessment of processing energy for video decoding. It is stated that the processing energy is a relevant metric especially for portable devices like smartphones or tablet PCs, because small energy requirements help to increase the operating time until the battery is empty. It is proposed to use energy measurements to assess the energy efficiency of the decoding process.

In this proposal, a general method for energy measurements is presented that can reportedly be used for various video coding platforms. The results can be used to evaluate a decoder in terms of processing energy.  Finally, it is proposed to use the Bjøntegaard-Delta Metric to assess the energy efficiency of the complete decoding process.

# Contents

Abstract	i

Contents	i

1	Introduction	1

2	Energy Measurements	1

2.1	Relevant Hardware Modules	1

2.2	Static and Dynamic Power Consumption	2

2.3	Measurement Setups	3

2.4	Statistical Test	3

3	Energy Assessment	4

4	Closing remarks	7

5	References	7

6	Patent rights declaration(s)	7

# Introduction

The latest developments indicate that new coding tools may benefit from the use of modern processing chips like multi-core CPUs, GPUs, DSPs, or dedicated hardware chips. For example, FRUC-techniques [1] or in-loop filters based on neural networks [2] can benefit from parallel processing because processing times can be decreased. However, at the same time, it is not clear whether the processing energy also decreases because multiple cores are running at the same time. As a consequence, parallel processing is not necessarily beneficial for portable devices when real-time capabilities can be ensured using single-thread processing.

This contribution suggests using the measured processing energy as an additional metric because of the following reasons:

- The energy can be measured for any decoding hardware and software.

- When real-time capabilities can be assured, it is stated that the energy is especially relevant for battery-driven portable devices.

This document is structured as follows: First, Chapter 2 shows a general measurement setup that can potentially be used for any decoder hardware and software. Chapter 3 shows the traditional rate-distortion diagrams and suggests to additionally use distortion-energy diagrams for assessment. As a test case, decoding energy is measured for a RaspberryPi that has a similar setup like modern smartphones. Then, the well-known Bjøntegaard-Delta method is applied to the energy-distortion curve. The results can be used to assess codecs and coding tools in terms of rate-distortion performance and processing energy.

# Energy Measurements

In this chapter, the authors explain how energy measurements for decoding processes can be performed. First, relevant hardware modules for the processes are reviewed. Second, the general power consumption is introduced which can be split up into dynamic and static power. The energy is then calculated by integrating over time. Afterwards, various measurement setups are shown allowing the measurement for heterogeneous chips like Intel CPUs, ARM CPUs, FPGAs, or dedicated hardware chips. Finally, a statistical test is introduced that is supposed to ensure the validity of the measurements.

## Relevant Hardware Modules

All the hardware modules that are needed in practical encoding or decoding applications are depicted in Fig. 1. Please note that in this contribution, the main focus is on the decoding process. First of all, as shown on the left side, a source sequence (compressed or raw) must be loaded from the memory, received from a network, or recorded with a camera. As the type of the source can be highly variable and is independent from the coding standard, it is not part of the energy measurement. Likewise, the output of the encoder/decoder, which can be sent to an HDMI output, displayed, saved, or sent via a network, is also neglected (right side).

Fig.  1 Hardware modules needed for practical encoder or decoder applications. For energy measurements, only the parts in the red box are considered.

Hence, only the modules which are needed for processing the video are considered, as shown in the red box in the middle. Generally, two main components are needed: a processing chip (top) and the memory (bottom). These two components can be separate entities (like CPU and RAM) or integrated in a single chip (modern SoCs). Additionally, a high variety of implementations can be considered: On the processor side, the chips can be categorized by traditional CPUs (Intel, AMD, ARM), GPUs (NVIDIA, ATI), GPGPUs, DSPs, VPUs, ASICs, FPGAs, etc. On the memory side, typical categories are RAM (DDR, LPDDR) and caches (cache size, replacement strategy, hierarchy, etc.).

## Static and Dynamic Power Consumption

For any hardware implementation mentioned in the last section, the energy needed during the decoding of a video can be split into two parts: the static and the dynamic energy. The static energy is always needed when the device is switched on. It is, e.g., needed to keep up the operating system, keep modules in a standby state, or it is leakage power. Generally, this energy cannot be controlled by users or software developers because it is an inherent property of the device.

The dynamic energy is directly caused by the decoding process itself and is hence influenced by the complexity of the process. In this contribution, the measurement setup is designed to only target the dynamic energy. Fig. 2 visualizes this distinction.

Fig.  2: Power Consumption during decoding for a showcase decoder setup. The blue curve shows the power during the decoding process on an ARM-based platform (dynamic plus static power). The red curve is the power consumption in idle mode (static power).

The figure shows the processing power of a showcase decoding device. The static case is depicted by the red curve. Here, the device is in idle mode, meaning that it is waiting for user commands and that it is not performing any user processes. Integrating this idle power over the time, the consumed static energy can be calculated for a predefined time interval.

The blue curve depicts the case of software decoding. At approximately 1 s, the process is started and it finishes at 9 s. It can be seen that the power rises to a higher level. Integrating this curve, the energy needed for processing (i.e. the dynamic energy) including the static energy can be calculated.

For evaluation purposes in this work, only the dynamic energy is of interest because the static energy cannot be controlled. In addition, for fair comparisons, the static energy would have to be considered only for the decoder processing time which is technically difficult to achieve. Synchronization between the process and the energy measurement is error-prone in real measurements when an external power meter is used with triggering over a delay-prone interface.

As a consequence, it is suggested to perform two measurements to obtain the dynamic energy: First, a reference measurement is performed to obtain the idle energy. The time interval is set to the decoding time plus an additional offset. For the second measurement, the interval is the same and the decoding process shall be performed in this interval. The dynamic decoding energy is then calculated by subtracting the former reference measurement from the latter measurement.

## Measurement Setups

For power and energy measurements, two main setups can be considered. In the first setup, an external power meter is used. In the second setup, internal power measuring circuitry, which is included in many modern processor SoCs, can be used.

In the first setup, the device under test (DUT), which performs the decoding, is powered from an external power source. The current and voltage from this source can be used for power measurements as shown in Fig. 3. This setup is useful for small devices like handhelds, smartphones, or evaluation boards. If the device is powered through data cables (like USB or power over Ethernet), the corresponding cables can be shortcut and used for power measurements.

Fig.  3 Measurement setup for device-under-test (DUT) to determine the energy consumption with a power meter.

In the second setup, internal performance counters of SoCs can be used. The authors state that for some processors, this method is simple to implement and to use for true energy measurements. Modern Intel CPUs provide the feature “running average power limit” (RAPL) [3], which monitors the energy consumed by the processor core, the memory, and the complete package [4]. The corresponding energies are continuously updated and saved in the following system files that can be read at any time instance:

/sys/class/powercap/intel-rapl/intel-rapl:0/energy_uj (complete package)
/sys/class/powercap/intel-rapl/intel-rapl:0/intel-rapl:0:0/energy_uj (CPU)
/sys/class/powercap/intel-rapl/intel-rapl:0/intel-rapl:0:1/energy_uj (RAM)

Similar to the measurement method using the power meter, the energy values can be read at the beginning and at the end of a certain time interval such that the dynamic power plus the static power is obtained or the pure static power can be obtained.

## Statistical Test

As the decoding process is usually performed on devices driven by an operating system (OS), background system processes are likely to disturb the measurements. Other sources of measurement noise like temperature variations can also occur. The authors claim that as a consequence, a single measurement for one process is not sufficient. As a solution, a series of measurements should be performed and a statistical test should ensure the validity of the measured energy.

Assume that the processing energy for decoding a sequence shall be measured. The process is measured m times. A confidence interval test is conducted with [5]

|  |  | (2.1) |
| --- | --- | --- |

where cleft and cright are the lower and the upper bound of the confidence interval, x is the mean of the measured energies, σ the standard deviation, and tα is the critical t-value for m samples of Student’s t-distribution. The size of the confidence interval can be chosen as

|  |  | (2.2) |
| --- | --- | --- |

where β reflects the relative size of the allowed interval. In the literature [6], α=0.99% and β=0.02 have been chosen regularly. In this case, if the confidence interval condition is kept, with a probability of 99%, the true energy μ is located in the range (0.99x;1.01x). In the following, the mean energy x is used as the processing energy E.

# Energy Assessment

In this chapter, results of true energy measurements for the decoding energy of HM-16.18, JEM-7.0, and VTM-1.0 are given. It is shown how the energy can be used as an additional evaluation criterion.

The processing energies are measured for the decoding process of three sequences: BasketballDrive, BQTerrace, and Cactus. The sequences are encoded with the four QPs 22, 27, 32, 37 and the standard randomaccess encoder configurations for HM-16.18, JEM-7.0, and the VTM-1.0. The decoding device is a Raspberry Pi 3B+ equipped with an ARM Cortex-A53 quadcore processor. The operating system is Raspbian 4.14.34-v7+ and the gcc compiler version is 6.3.0. For stable measurements, dynamic voltage and frequency scaling (DVFS) is switched off such that a constant clock frequency of 1.4 GHz is set. Three software decoders are used: JEM-7.0, VTM-1.0, and HM-16.18 as a reference decoder. The power is measured through the USB cable and the power meter is a ZES Zimmers LMG95. The rate-distortion values and the measured energies are listed in Tables 1 to 3.

Table 1 Rate-distortion performance and decoding energy of the BQTerrace sequence (40 frames).

| Sequence | Software | QP | YUV-PSNR | File size | Decoding Energy |
| --- | --- | --- | --- | --- | --- |
| BQTerrace | HM-16.18 | 22 | 39.32 dB | 3388451 Bytes | 17.58 J |
|  |  | 27 | 37.08 dB | 934539 Bytes | 11.59 J |
|  |  | 32 | 35.49 dB | 380594 Bytes | 9.23 J |
|  |  | 37 | 33.79 dB | 190716 Bytes | 8.64 J |
|  | JEM-7.0 | 22 | 39.45 dB | 3158360 Bytes | 104.85 J |
|  |  | 27 | 37.37 dB | 852039 Bytes | 73.32 J |
|  |  | 32 | 35.99 dB | 362688 Bytes | 69.95 J |
|  |  | 37 | 34.42 dB | 183538 Bytes | 77.49 J |
|  | VTM-1.0 | 22 | 39.36 dB | 3371320 Bytes | 27.78 J |
|  |  | 27 | 37.15 dB | 927732 Bytes | 14.14 J |
|  |  | 32 | 35.64 dB | 382417 Bytes | 10.42 J |
|  |  | 37 | 33.99 dB | 189886 Bytes | 9.25 J |

Table 2 Rate-distortion performance and decoding energy of the BasketballDrive sequence (40 frames).

| Sequence | Software | QP | YUV-PSNR | File size | Decoding Energy |
| --- | --- | --- | --- | --- | --- |
| BasketballDrive | HM-16.18 | 22 | 41.05 dB | 1465150 Bytes | 14.20 J |
|  |  | 27 | 39.67 dB | 465347 Bytes | 11.52 J |
|  |  | 32 | 38.14 dB | 230400 Bytes | 10.82 J |
|  |  | 37 | 36.32 dB | 121969 Bytes | 9.94 J |
|  | JEM-7.0 | 22 | 41.31 dB | 1292581 Bytes | 91.56 J |
|  |  | 27 | 40.04 dB | 403096 Bytes | 89.76 J |
|  |  | 32 | 38.69 dB | 198311 Bytes | 93.80 J |
|  |  | 37 | 37.01 dB | 105290 Bytes | 105.35 J |
|  | VTM-1.0 | 22 | 41.19 dB | 1405018 Bytes | 17.17 J |
|  |  | 27 | 39.88 dB | 447320 Bytes | 12.94 J |
|  |  | 32 | 38.43 dB | 219234 Bytes | 11.15 J |
|  |  | 37 | 36.70 dB | 115404 Bytes | 9.58 J |

Table 3 Rate-distortion performance and decoding energy of the Cactus sequence (40 frames).

| Sequence | Software | QP | YUV-PSNR | File size | Decoding Energy |
| --- | --- | --- | --- | --- | --- |
| Cactus | HM-16.18 | 22 | 39.15 dB | 2106639 Bytes | 14.52 J |
|  |  | 27 | 37.67 dB | 718929 Bytes | 10.25 J |
|  |  | 32 | 36.03 dB | 348281 Bytes | 9.00 J |
|  |  | 37 | 34.09 dB | 179822 Bytes | 8.11 J |
|  | JEM-7.0 | 22 | 39.36 dB | 1887502 Bytes | 87.27 J |
|  |  | 27 | 37.94 dB | 618702 Bytes | 87.63 J |
|  |  | 32 | 36.49 dB | 298655 Bytes | 95.52 J |
|  |  | 37 | 34.74 dB | 154394 Bytes | 98.65 J |
|  | VTM-1.0 | 22 | 39.24 dB | 2095966 Bytes | 17.09 J |
|  |  | 27 | 37.78 dB | 711967 Bytes | 11.13 J |
|  |  | 32 | 36.19 dB | 342753 Bytes | 8.98 J |
|  |  | 37 | 34.32 dB | 176199 Bytes | 7.75 J |

For visualization, Fig. 4 shows the classic rate-distortion diagram (left) and the energy-distortion diagram (right) for the BQTerrace sequence. The blue lines correspond to HEVC (HM-16.18), the green lines to JEM-7.0, and the red lines to VTM-1.0.

Fig.  4 Rate-distortion diagram (left) and decoding-energy-distortion diagram (right) for the BQTerrace sequence. The blue line corresponds to HM-16.18, the green line to JEM-7.0, and the red line to VTM-1.0.

First of all, the left diagram shows the superior compression performance of JEM-7.0 over the standard HEVC coded sequences. Furthermore, due to the more sophisticated block partitioning scheme, the VTM-1.0 encoder shows a slightly higher compression performance than HM-16.18. In the right diagram, the rate on the x-axis is replaced with the measured decoding energy. It can be seen that JEM-7.0 requires a significantly larger amount of decoding energy.

From the rate-distortion diagram, the Bjøntegaard-Delta rate (BDR) can be calculated which, in this case, yields -22.79% for JEM-7.0 in comparison to HM-16.18 and -5.36% for VTM-1.0 in comparison to HM-16.18. In the same manner, the Bjøntegaard-Delta metric can also be used to calculate the relative difference in terms of decoding energy. This is done by replacing the rate with the decoding energy in the BD-calculus. The resulting value (the Bjøntegaard-Delta energy (BDE)) reflects the mean relative difference in decoding energy needed to obtain the same visual objective quality. Here, the BDE yields 562% for JEM-7.0 in comparison to HM-16.18, which means that the tested JEM-7.0 decoder requires more than five times more energy than the HM-16.18 decoder to achieve the same visual objective quality. The VTM-1.0 decoder needs 20.68% more energy. All Bjøntegaard-Delta values for all tested sequences are summarized in Table 4.

Table 4: Bjøntegaard-Delta values for the three tested sequences from the common test conditions. The reference is the HM-16.18 encoder for BDR and the HM-16.18 decoder for BDE.

| Sequence | Software | BDR | BDE |
| --- | --- | --- | --- |
| BQTerrace | JEM-7.0 | -22.79% | 561.76% |
|  | VTM-1.0 | -5.36% | 20.68% |
| BasketballDrive | JEM-7.0 | -31.75% | 721.84% |
|  | VTM-1.0 | -15.84% | 4.57% |
| Cactus | JEM-7.0 | -28.01% | 826.65% |
|  | VTM-1.0 | -7.70% | 2.07% |

For visualization, the Bjøntegaard-Delta values can be plot into a rate-energy diagram as shown in Fig.  5.

Fig.  5: Rate-energy diagram for three test sequences from the common test conditions. The reference is HM-16.18 decoding (blue markers). The green markers represent JEM-7.0 decoding. It can be seen that the rate decreases by 20% to 35%, in contrast, the energy is five to nine times higher. The VTM-1.0 software (red) shows lower rate savings but relatively small energy increases (below 25%).

The authors claim that instead of using the Bjøntegaard-Delta-metric, the mean values could also be calculated, as regularly used for decoding time measurements. The drawback is that PSNR differences between bit streams (which can occur although the same QP is chosen) are not taken into account such that the BD-calculus is preferred.

For practical evaluation of new coding tools, a measurement of the processing energy with and without the new tool can be performed and an evaluation with BDR and BDE can be made.  As a consequence, another evaluation criterion based on the true processing energy can be considered.

# Closing remarks

Energy measurements for decoders were proposed. It is suggested to consider the energy as another criterion for assessing the performance of new coding tools in VVC. Please note that the presented approach is not restricted to the decoding energy. The encoding energy could also be assessed in the same manner.

# References

[1] Seo, Lee, Kim, Jang, Lim, “Simplification and improvements on FRUC”, JVET-G0065, July 2017.

[2] Zhou, Song, Yao, Wang, Chen, “Convolutional Neural Network Filter (CNNF) for intra frame”, JVET-I0022, Jan. 2018.

[3] https://01.org/rapl-power-meter

[4] Hackenberg, Schöne, Ilsche, Molka, Schuchart, and Geyer,  „An energy efficiency feature survey of the Intel Haswell processor”, Proc. IEEE International Parallel and Distributed Processing Symposium Workshop (IPDPSW), (pp. 896-904), May 2015.

[5] J.S. Bendat and A.G. Piersol, “Random Data: Analysis and Measurement Procedures”, John Wiley & Sons, Inc., 1971.

[6] C. Herglotz, D. Springer, M. Reichenbach, B. Stabernack, and A. Kaup, „Modeling the energy consumption of the HEVC decoding process”, IEEE Transactions on Circuits and Systems for Video Technology, 28:217–229, Jan 2018.

# Patent rights declaration(s)

The Friedrich-Alexander University Erlangen-Nürnberg (FAU) does not have any current or pending patent rights relating to the technology described in this contribution.