| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0107-v1 |
| --- | --- |

| Title: | Energy Modeling for Video Decoding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christian Herglotz Matthias Kränzler André Kaup | Tel: Email: | +49 9131 85 27101 christian.herglotz@fau.de matthias.kraenzler@fau.de andre.kaup@fau.de |
| Source: | Multimedia Communications and Signal Processing, Friedrich-Alexander University Erlangen-Nürnberg (FAU) |  |  |

_____________________________

# Abstract

In this contribution, a recently published energy model is presented which can be used for video decoding energy estimations. It is reported that the model is able to estimate the energy of both the processor and the memory access. Furthermore, it is stated that using sophisticated training methods, the trained model values can be interpreted as coding tool specific energies, and that the model can be used for decoder energy optimization.

# Contents

Abstract	i

Contents	i

1	Introduction	1

2	Energy Modeling	1

2.1	Bit Stream Features	1

2.2	Practical Application: Energy Modeling	2

2.3	Feature Set for JEM	3

3	Specific Energy Training	3

3.1	Curve Fitting	3

3.2	Training Sets	4

3.2.1	Conventional Bit Streams	4

3.2.2	Modified Bit Streams	4

3.2.3	Reference Bit Streams	5

3.2.4	Energy Optimized Bit Streams	5

4	Analysis	5

5	Conclusion	6

6	References	6

7	Patent rights declaration	7

# Introduction

In the development of the next generation video coding standard VVC, the energy consumption of the decoder is a crucial problem. Especially for modern portable devices like smartphones or tablet PCs, a low energy consumption of the decoding process is desired to extend the operating time until the battery is empty. To obtain a deeper understanding of the energy consumption during decoding, a so-called feature based energy model was proposed for various decoders [1]. Related work showed that this model can, e.g., be used to construct energy saving bit streams [2]. In this contribution, such a model is proposed for intra-coding in the JEM-5.1 software.

In the practical application of this model, real energy measurements are necessary to train the energy parameters. Potential measurement procedures are described in [3] and they are used for the evaluation given in this document.

This work is structured as follows. At first, the linear decoder energy model is introduced. An example is given on how the corresponding features of the model can be interpreted. Furthermore, it is shown how such a model can be applied in practice. Afterwards, methods for determining feature specific energies are presented and problems and limitations to the method are described. Especially one method is shown in detail which is then used to derive energy values for comparison. In the last section, explicit energy values are listed and evaluated.

# Energy Modeling

The model used for estimating the energy is given by

|  | E=f ∈ Fnf∙ef, | (2.1) |
| --- | --- | --- |

where E is the estimated energy, F the total amount of considered features, f the feature index, nf the number of occurences of a feature, and  ef  the feature specific energy which describes the energy that is consumed each time the feature occurs.

## Bit Stream Features

To give an example, a feature can be the intra prediction of a certain block (cf. Fig.  1). The process itself is defined by the standard and the same calculations are performed each time the feature occurs. As a consequence, it is likely that the energy needed for each execution is rather constant and can be determined by suitable measurement procedures.

Fig.  1: Intra prediction feature for angular prediction.

Other features can be the inter prediction of a block, the fractional pel filtering of a pixel (horizontal and vertical), the deblocking filtering of a block boundary, and many more (cf. [1]). Additionally, different block sizes can be considered because larger blocks usually require more energy for processing than smaller blocks. When defining these features, it is important to ensure that the feature can be counted, that the feature is defined unambiguously, and that the corresponding process has a predefined and homogeneous processing flow. Furthermore, it is convenient if the feature is defined for an explicit coding tool. Examples for feature sets used for decoder energy modeling are given in [1, 4]. In these publications, it was shown that this kind of model achieves mean estimation errors below 10%.

## Practical Application: Energy Modeling

In various publications, the validity of the feature based models was proven by considering the decoding process of true bit streams [1, 4]. This was done as shown in Fig.  2: For a certain codec, a high amount of bit streams was encoded with a certain encoder (various sequences, QPs, and encoder configurations).

Fig.  2: Training and validation of energy models.

Then, the decoding energy of these bit streams was measured with the setup presented in [2]. In addition, the bit streams were analyzed for the features using a bit stream analyzer. As a result, the true decoding energies E and the true feature numbers nf (the sequence properties) were obtained for the selected set of test sequences. This information was then used to train the feature based energy model using least-squares curve fitting or linear regression inside a 10-fold cross-validation loop. As a result, values for the specific energies ef  (the trained parameters) and estimated decoding energies E for each bit stream are obtained. In the publications, the validity of the model was verified by assessing the mean relative estimation error.

A possible result of the training process is shown in Fig.  3.

Fig.  3: Energy evaluation for HEVC coded bit stream (Kimono). Note that not all features are displayed.

The figure shows the modeled distribution of the energy for the Kimono sequence (HEVC). Corresponding decoding energies are visualized by bars, the complete energy is obtained adding up all bars. Fig.  3 indicates that in this case, most energy is needed for skipped blocks and for fractional pel interpolation.

Note that the information given by this model must be handled with care. There are two main effects that can cause inaccurate energy values. The first effect is an inherent property of the decoding process. Although the computations are deterministic for a given coding tool, the processing energy is subject to variations because of, e.g., the memory management. As an example, consider inter prediction. If a block of 8x8 pixels is predicted and the corresponding reference block is located in the cache, the process will be lightweight and use little energy. In contrast, if the block must be loaded from the RAM, more energy will be needed. As this effect highly depends on the used hardware and further non-deterministic effects like the initial state of the cache, it is not modeled and hence a source of inaccuracies.

The second effect is called overfitting. The more features are used for modeling, the more likely it is that overfitting occurs. Overfitting may occur if, e.g., the occurrence of two different features is highly correlated. For example, the more blocks of size 8x8 are intra predicted, the more 8x8 transforms are usually applied. If the training set includes no bit streams handling this correlation explicitly, the corresponding trained energies may also be inaccurate. Section 3.2 will present a method that suppresses this effect.

## Feature Set for JEM

In this contribution, the selected set of features is constructed to describe intra prediction coding tools. In [5] it was shown that in principle, the set can be extended to inter prediction. The complete list of 16 features is given in Table 1.

Table 1: Features for modeling JEM-intra prediction.

| Feature | Explanation | Block sizes |
| --- | --- | --- |
| OFFSET | Offset energy | - |
| FRAME | Frame-wise energy | - |
| INTRA_PRED | Intra predicted blocks | 4x4 up to 128x128 |
| COEFF | Nonzero coefficients | - |
| VAL | Coefficient value | - |
| TRANS | Inverse transforms | 4x4 up to 128x128 |

From all available block sizes, 6 representative block sizes are chosen which shall represent blocks with similar sizes. For example, the feature representing 8x8 pixel blocks includes all blocks containing 8x8=64 pixels and all blocks containing 32 pixels (which is, e.g., made up by a 4x8 or a 8x4 block). Please note that for more sophisticated models, a higher number of block sizes could be considered.

# Specific Energy Training

Usually, the goal of training is to show that a given model is able to accurately estimate the target parameter, which is the complete decoding energy in this case. In the work presented here, furthermore it is desired to obtain meaningful values for the specific energies ef. To ensure this, two requirements have to be kept: First, a suitable curve fitting algorithm must be used, second, the training set must be representative for the considered features. Otherwise, overfitting may occur which does not affect the estimation accuracy of complete bit streams, but which can strongly deter the validity of the trained specific energies. These two problems are discussed in the following subsections.

## Curve Fitting

Chapter 2 explained that to train the specific energies, a high amount of bit streams with corresponding feature numbers nf  and measured decoding energies E has to be available (at least as many bit streams as features). Then, the specific energies are trained using the minimization function

|  | minefe2(E, E)=l=1LEl-ElEl2 | (3.1) |
| --- | --- | --- |

with l the bit stream index, L the total amount of bit streams, and El the estimated energies as defined by (3.1). Please note that in this case, the relative error is minimized (in the literature, the absolute error is regularly used). The authors state that this is performed to assign the same importance to each bit stream during training. Otherwise, single bit streams having large measured energies would have more influence on the training process than bit streams showing small processing energies.

In practice, such training functions are readily available for various programming languages (e.g. lsqcurvefit() in MatLab or scipy.optimize.curve_fit in Python).

## Training Sets

The next problem is the choice of the training set. For this choice, it must be made sure that the features of the model are well represented by the training set. For example, if a feature does not occur at all in the training set (because it is unlikely that it is chosen by the encoder), the training process cannot determine a corresponding specific energy. Furthermore, it must be made sure that there are no linear dependencies. Further information can be found in [6]. For the training of the specific energies in this work, three kinds of different bit streams are used as explained in the next subsections.

### Conventional Bit Streams

The first set of bit streams consists of conventionally coded bit streams. Therefore, the JEM-5.1 encoder is used to compress sequences with four QPs (10, 20, 30, 40) using the intra encoder configuration. The following input sequences were used:

BQMall, BQSquare, BQTerrace, BasketballDrillText, BasketballDrill, BasketballDrive, BasketballPass, BlowingBubbles, Cactus, ChinaSpeed, FourPeople, Johnny, Kimono, Kristen&Sarah, ParkScene, PartyScene, PeopleOnStreet, RaceHorses (ClassC), RaceHorses, SlideEditing, SlideShow, Traffic, Akiyo, bus, car_phone, coastguard, crew, foreman, miss_america, news, suzie, tennis, vidyo1, vidyo3, vidyo4.

The sequences were coded with 8 to 40 frames, all in all the set consists of 35*4=140 bit streams. Please note that in terms of energy modeling, these bit streams are additionally used for model validation because they are most representative for realistic bit streams used in practical applications.

### Modified Bit Streams

For the modified bit streams, the standard RDO process in the JEM-5.1 encoder was extended to prefer or reject specific features. The goal is to construct bit streams in which certain features occur more or less often such that they represent the feature more accurately. Therefore, during encoding, the coding costs are modified by

|  | minJ=D+λR+10λPf, | (3.2) |
| --- | --- | --- |

where Pf is a penalty value for a target feature f. The factor 10λ is chosen to ensure that the target feature has a higher impact on the coding costs than a single bit coded to the bit stream. For example, one modified bit stream is coded in which CUs of size 8x8 shall occur more frequently. In this case, each time the block size 8x8 is tested, a penalty value of Pf=-1 is used. As a consequence, the coding costs are smaller and it is chosen more often.

Likewise, a positive penalty value (e.g., Pf=1) can be chosen such that the corresponding feature is chosen less often. The output bit streams can then be considered to be representative for the corresponding feature and are well suited for training.

In addition, to take into account that various features might have a high correlation, further bit streams are encoded that are supposed to break up this correlation. For example, intra prediction on a certain block size is highly correlated with transformation on the same block size. This observation can be confirmed by calculating the features’ correlation coefficients using the conventional bit stream set.

To break up this correlation, the following steps are taken: First, for the intra prediction feature, a penalty value of Pintra=-1 is chosen. Second, another penalty value of Ptrans=1 is used. In this case, the intra mode is chosen more often, but only if the transform is skipped. As a result, a bit stream with more intra prediction but fewer transforms is created.

Please note that the authors are aware that this procedure does not lead to efficient bit streams in terms of the rate-distortion performance. However, tests showed that such bit streams are helpful for reliable training of specific energies.

These bit streams were constructed for all features shown in Table 1 (16 features with positive and negative penalty). Furthermore, additional bit streams were constructed for the features showing a high correlation (additional 8 bit streams). As input sequences, Kimono and vidyo3 were used with the two QPs 22 and 37. As a result, the training set consists of 160 sequences.

### Reference Bit Streams

As the energy model includes offset energies (OFFSET and FRAME from Table 1), additional short bit streams were encoded. In this case, the set consists of eight sequences. They are coded from the first frame of the SlideEditing sequence using the four QPs 22, 27, 32, and 37. In the first case, only a single CTU is coded (resolution of 128x128 pixels) and in the second case, the complete first frame is coded.

### Energy Optimized Bit Streams

Furthermore, a third type of bit stream is used which is coded with the goal of energy efficient decoding. The exact method and the encoder implementation used for coding them is presented in [7]. To obtain these bit streams, the following steps must be taken:

- Measure the decoding energy for all bit streams introduced in Sections 3.2.1 to 3.2.3.

- Train an initial set of specific energies ef using the resulting decoding energies and the frequency of occurrence of the features nf.

- Code new bit streams with this set of energies using DERDO [7].

- Measure the decoding energy of these new bit streams.

- Merge the first training set and the new bit streams to a final training set.

- Train the specific energies again using the final training set.

Studies showed that this procedure helps in returning more accurate and reliable specific energy values that are additionally useful to encode decoding energy saving bit streams [6].

Using this method, 96 bit streams were coded with the following configurations:

- QP ∈{22, 27, 32, 37}

- τ∈{0, 0.25, 0.5, 0.75, 1}

- Sequences BasketballDrive, BasketballPass, Johnny, PeopleOnStreet, RaceHorses (class C), Kimono

# Analysis

The measurements are performed with the JEM-5.1 decoder on a pandaboard (TI OMAP-4430 SoC with an ARM-Cortex A9 dual core processor). The measurement method is explained in [3]. For the tested bit stream set, the mean relative estimation error is smaller than 10%. The resulting specific energy values are listed in Table 2.

Table 2: Trained specific energy values.

| Feature | Trained specific energy ef [µJ] |
| --- | --- |
| OFFSET | 25808 |
| FRAME | 30497 |
| INTRA_PRED_4x4 | 1.833 |
| INTRA_PRED_8x8 | 3.283 |
| INTRA_PRED_16x16 | 9.141 |
| INTRA_PRED_32x32 | 22.81 |
| INTRA_PRED_64x64 | 80.28 |
| INTRA_PRED_128x128 | 99.42 |
| COEFF | 0.1155 |
| VAL | -0.09176 |
| TRANS_4x4 | 0.005478 |
| TRANS_8x8 | 1.627 |
| TRANS_16x16 | 2.892 |
| TRANS_32x32 | 9.941 |
| TRANS_64x64 | 25.94 |
| TRANS_128x128 | 285.0 |

The table indicates that, as expected, processing larger block sizes requires more energy than processing smaller block sizes. It is interesting to note that the energy for the feature VAL is negative which is not expected. This observation indicates that either overfitting occurs, or the model is not well suited for the estimation of the processing energy related to coefficient value coding. Further work could improve this model. However, please note that such a value was also observed in the literature [6].

# Conclusion

This contribution presented an energy model that is able to accurately estimate the decoding energy of the JEM.-5.1 decoder. Constructing a suitable training set, realistic specific energies can be derived which can be used to compare the processing energy of various coding tools. As will be shown in another contribution, these energies can be used to construct decoding energy saving bit streams [7].

# References

[1] C. Herglotz, Y. Wen, B. Dai, M. Kränzler, and A. Kaup. A bitstream feature based model for video decoding energy estimation. In Proc. Picture Coding Symposium (PCS), Nürnberg, Germany, Dec 2016.

[2] C. Herglotz, A. Heindel, and A. Kaup. Decoding energy distortion optimization for video coding. IEEE Transactions on Circuits and Systems for Video Technology, preprint, 2017, DOI: 10.1109/TCSVT.2017.2771819.

[3] C. Herglotz, A. Kaup, “Energy Assessment for Video Decoding”, JVET-K0106, July 2018.

[4] C. Herglotz, D. Springer, M. Reichenbach, B. Stabernack, and A. Kaup, „Modeling the energy consumption of the HEVC decoding process”, IEEE Transactions on Circuits and Systems for Video Technology, 28:217–229, Jan 2018.

[5] C. Herglotz, M. Kränzler, and A. Kaup. Decoding Energy Modeling for the Next Generation Video Codec Based on JEM. In Proc. Picture Coding Symposium (PCS), San Francisco, USA, Jun 2018.

[6] C. Herglotz, “Energy Efficient Video Decoding”, Verlag Dr, Hut, ISBN 3843934614, München 2018.

[7] C. Herglotz, M. Kränzler, A. Kaup, “Decoding-Energy-Rate-Distortion Optimization”, JVET-K0108, July 2018.

# Patent rights declaration

The Friedrich-Alexander University Erlangen-Nürnberg (FAU) does not have any current or pending patent rights relating to the technology described in this contribution.