| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0108-v1 |
| --- | --- |

| Title: | Decoding-Energy-Rate-Distortion Optimization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christian Herglotz Matthias Kränzler André Kaup | Tel: Email: | +49 9131 85 27101 christian.herglotz@fau.de matthias.kraenzler@fau.de andre.kaup@fau.de |
| Source: | Multimedia Communications and Signal Processing, Friedrich-Alexander University Erlangen-Nürnberg (FAU) |  |  |

_____________________________

# Abstract

This contribution proposes to include Decoding-Energy-Rate-Distortion Optimization (DERDO) into the encoder reference software. It is reported that DERDO is an extension of RDO which can be used to control and minimize the decoding energy on the receiver side which is interesting for portable, battery driven devices. As the complexity of the next coding standard is expected to increase significantly, this tool may help in decreasing the complexity requirements of decoders. In this work, the potential savings are analyzed for intra only coding and reach 5.74% of energy savings at the expense of a bitrate increase of 7.57% or 1.1% energy savings at 0.13% bitrate increase. The authors claim that due to the high amount of new coding tools proposed for VVC, energy savings of a higher magnitude at lower rate increases can be expected for inter prediction coding tools.

# Contents

Abstract	i

Contents	i

1	Introduction	1

2	Calculation of the Lagrange Multipliers	1

2.1	General encoder algorithm description	2

2.1.1	Calculation of the Lagrange Multipliers	2

2.1.2	Decoding Energy Estimation	2

2.1.3	Cost calculations	4

2.2	Values for specific energies	4

3	Evaluation Results	4

3.1	Multi QP optimization	4

3.2	Single QP Optimization	5

4	Closing remarks	7

5	References	8

6	Patent rights declaration(s)	8

# Introduction

Complexity assessments of the JEM-reference software indicate that the complexity of the encoder and the decoder will increase dramatically. Important reasons are the use of neural networks and motion vector refinement on the decoder side. As a consequence, the requirements for portable, battery driven devices will also increase. To leverage the complexity and thus the energy consumption on the receiver side, it could be helpful to use compression tools that consume less energy than others.

This contribution presents decoding-energy-rate-distortion optimization (DERDO) which is an extension of classic RDO. In addition to optimizing for rate and distortion, the estimated decoding energy is used as a third optimization criterion. Hence, the standard minimization function is extended to

|  | minJ=D+λRR+λEE, | (1.1) |
| --- | --- | --- |

where J are the coding costs, D is the distortion, R the rate, E the estimated decoding energy, and λR and λE two Lagrange multipliers. The decoding energy is estimated using the energy model presented in [1]. To control the trade-off between the distortion, the rate, and the energy, a rate-energy trade-off parameter τ is introduced.

This contribution is structured as follows. At first, the control parameter τ is introduced theoretically. At the same time, the calculation of the Lagrange multipliers is motivated. Afterwards, the modifications in the encoder are shown. Finally, in the analysis section, an explicit implementation is presented in which decoding-energy saving bit streams are constructed for intra-only coded sequences of the JEM-5.1 reference software.

# Calculation of the Lagrange Multipliers

In classic RDO, a theoretic relationship between the Lagrange multiplier and the QP is derived using source models from the literature. Namely, these are the rate-distortion function for a memoryless Gaussian source [2]

|  | RD=12max0,logσ²D, | (2.1) |
| --- | --- | --- |

the high-rate approximation [2]

|  | D≈2q212, | (2.2) |
| --- | --- | --- |

and the relation between the Quantization q and the QP [3]

|  | q=2QP-46. | (2.3) |
| --- | --- | --- |

Setting the first derivative of (2.1) to zero to find the minimum and inserting (2.2) and (2.3), an analytic relation between the Lagrange multiplier and the QP is obtained. To verify this relationship, a multi QP optimization can be performed with fixed values of the Lagrange multiplier to ensure that the models from literature are valid [3]. From multi QP optimization, optimal parameter values are derived for the λ-QP relation.

For DERDO, a similar approach is proposed (cf. [4]). To include the decoding energy into the calculus, another model is used which links the decoding energy to the rate. The model reads [5]

|  | E=c1pIR+c2pI+c3R+c4, | (2.4) |
| --- | --- | --- |

where the parameters ci are constants and pI is the fraction of intra coded frames of all frames of the considered sequence.  Inserting this relation into (1.1) and setting the partial derivative with respect to D to zero, the following relation between the Lagrange multipliers and the QP can be obtained:

|  | λR=eQPαeρ1+1τ-1-13 | (2.5) |
| --- | --- | --- |

and

|  | λE=eQPαeε1+1τ-113. | (2.6) |
| --- | --- | --- |

In these equations, the QP is the classic quantization parameter. A new parameter τ∈[0,1] is introduced that can be chosen by the user. τ=0 corresponds to classic RDO. Increasing τ effects that λR decreases while λE increases. Hence, the importance of the rate decreases and the importance of the energy increases in the optimization process. For τ=1, λR=0 such that the rate is not considered for cost calculation at all. In this case, a pure decoding-energy-distortion optimization is performed. The other parameters αe,  ε, and ρ can be determined by multi QP optimization, similar to the case of RDO.

## General encoder algorithm description

To include DERDO into the code, only the encoder software needs to be extended. Generally, the functionality can be added to all cost calculations on all levels like block level, prediction mode level, or fast intra-mode decision level. Depending on this level, the calculation of the two Lagrange multipliers can differ. In this work, as only intra coding is considered, the DERD-costs are only calculated on prediction mode level. Further levels can be added straightforwardly.

Three modules have to be added. The first module is the calculation of the Lagrange multipliers, the second module is the application of the energy model, and the third module is the calculation of the coding costs. These modules are described in the next sections.

### Calculation of the Lagrange Multipliers

The Lagrange multipliers are calculated on slice level. To this end, (2.5) and (2.6) are evaluated depending on the slice-dependent QP and the user-defined rate-energy trade-off parameter τ. The resulting values are then used throughout a single slice. As in the used model, different prediction modes are assumed to consume the same amount of energy, no energy optimization is performed on the level of fast intra mode decisions. The authors claim that this functionality can be added later.

### Decoding Energy Estimation

Generally, the decoding energy is estimated before the cost function is evaluated. As a basis, the energy model for the complete decoding energy, which was proposed in [1], is used which reads

|  | E=f ∈ Fnf∙ef. | (2.7) |
| --- | --- | --- |

In this model, the variable f describes the feature which can, e.g., be intra prediction for a certain block size. n describes the number of occurences of the feature and e is the specific energy that relates to the execution of the feature. The considered features are listed in Table 1. Please note that in this document, only intra prediction is considered. The extension to inter prediction is straight-forward and can be included in future implementations.

Table 1: Features for energy modeling, definition and value.

| Feature f | Explanation | Normalized value |
| --- | --- | --- |
| E0 | Offset energy | 25808 |
| SLICE | Energy to decode one slice | 30497 |
| INTRA_L0 | Intra prediction (4x4 pixels) | 1.833 |
| INTRA_L1 | Intra prediction (8x8 pixels and smaller) | 3.283 |
| INTRA_L2 | Intra prediction (16x16 pixels and smaller) | 9.141 |
| INTRA_L3 | Intra prediction (32x32 pixels and smaller) | 22.81 |
| INTRA_L4 | Intra prediction (64x64 pixels and smaller) | 80.28 |
| INTRA_L5 | Intra prediction (128x128 pixels and smaller) | 99.42 |
| COEFF | Decoding a single nonzero coefficient | 0.1155 |
| VAL | Decoding the remaining value of coefficients | -0.09176 |
| TRANS_L0 | Transform (4x4 pixels) | 0.005478 |
| TRANS_L1 | Transform (8x8 pixels and smaller) | 1.627 |
| TRANS_L2 | Transform (16x16 pixels and smaller) | 2.892 |
| TRANS_L3 | Transform (32x32 pixels and smaller) | 9.941 |
| TRANS_L4 | Transform (64x64 pixels and smaller) | 25.94 |
| TRANS_L5 | Transform (128x128 pixels and smaller) | 285.0 |

For the calculation of the energy in DERDO, only the features with variable specific energies are considered. On block level, these are all features but E0 and SLICE because the block size, the use of the transformation, and the residual coefficients can be chosen. On transform level, the latter eight features are considered including the coefficients (COEFF and VAL) and the application of the transformation (TRANS_LX).

An example is given for prediction mode energy estimation. After prediction, transform, and quantization of the residual coefficients, a 4x4-block could be coded with the two alternatives (a) and (b) as shown in Fig.  1.

Fig.  1: Examples for residual coefficients of different intra prediction modes.

The calculation of the rate and the distortion is not changed. The calculation of the decoding energy is performed as follows:

|  | E(a)=eINTRA_L0+eTRANS_L0 +7∙eCOEFF+(ld(19)+ld(6)+ld(2)+ld(3))∙eVAL | (2.8) |
| --- | --- | --- |
|  | E(b)=eINTRA_L0 | (2.9) |

Here, ld corresponds to the logarithm to the basis 2. The factor 7 is the number of nonzero coefficients in alternative (a), the values in the logarithms correspond to the absolute coefficients greater one, decremented by one. The employed model for the values of the coefficients is presented in [6].

### Cost calculations

For the cost calculations, the traditional RD-costs are extended by additionally adding the estimated energy from Section 2.1.2 multiplied with the Lagrange multiplier from Section 2.1.1. The decision process is then unchanged which means that the coding mode showing the smallest costs is selected.

## Values for specific energies

The authors claim that generally, it is crucial that the values for the specific energies (cf. Table 1) are well chosen. If the values are not sufficiently representative for the true decoding energy, the rate-distortion performance as well as the true energetic needs of the coded sequences can suffer significantly. To achieve this, training methods as presented in [6] can be used. For the evaluation performed in this document, the values trained with a dedicated training set presented in [1] are used.

# Evaluation Results

As a basis for the proposed DERDO, JEM-5.1 was chosen. The functionality explained above is implemented for all-intra coding. Other configurations are not yet tested but are planned to be tested in the future. The energy values from Table 1 are used for energy estimation. Note that the values are normalized after training as

|  | ef,norm=efc1pI+c3, | (3.1) |
| --- | --- | --- |

with c1, pI, and c3 from (2.4). Normalization makes sure that the Lagrange multipliers do not depend on the absolute energy consumption of a decoder.

Before performing traditional single QP encoding, multi QP encoding is performed for a single input sequence to obtain rough estimates for the parameters αe,  ε, and ρ (see Section 3.1). Afterwards, the results for single QP encoding are shown in Section 3.2.

## Multi QP optimization

As initial values for αe,  ε, and ρ, theoretic values resulting from (2.1), (2.2), and (2.3) are chosen. Then, multi QP optimization on CTU level is enabled by setting the encoder input flag “MaxDeltaQP”  to 5. The Kimono sequence was chosen as an input sequence. QPs were set to 22, 27, 32, and 37. The finally chosen QPs are recorded for each CTU and used for training new values of αe,  ε, and ρ. The theoretic and the trained values are listed in Table 2.

Table 2: Theoretic and trained values to calculate Lagrange multipliers.

| Parameter | Theoretic value | Trained value |
| --- | --- | --- |
| αe | 4.3281 | 4.4394 |
| ε | 24 | 6.2092 |
| ρ | 24 | 25.1182 |

Please note that the value for αe and  ε are close to the values reported for HEVC [6]. In contrast, the value for ρ is much higher (ρ≈9 for HEVC).

## Single QP Optimization

For this evaluation, the Cactus and the Kimono sequences were chosen. The encoder configuration was the intra profile, QPs were set to 22, 27, 32, and 37. The decoding energy was measured with the method shown in [7] for a pandaboard (TI OMAP-4430 SoC with an ARM Cortex-A9 dual core processor). The rate-distortion and the energy performance are listed in Table 3 and Table 4.

Table 3: Rate, distortion and decoding energy performance of the proposed encoding method (Cactus sequence).

| Sequence | τ | QP | YUV-PSNR [dB] | File size [Kb] | Decoding Energy [J] |
| --- | --- | --- | --- | --- | --- |
| Cactus | 0 | 22 | 41.93 | 72.03 | 8.945 |
|  |  | 27 | 39.09 | 31.23 | 8.299 |
|  |  | 32 | 37.13 | 16.76 | 8.324 |
|  |  | 37 | 35.07 | 9.37 | 7.036 |
|  | 0.25 | 22 | 41.95 | 72.04 | 8.956 |
|  |  | 27 | 39.10 | 31.33 | 8.057 |
|  |  | 32 | 37.14 | 16.80 | 8.124 |
|  |  | 37 | 35.07 | 9.39 | 7.054 |
|  | 0.5 | 22 | 42.08 | 77.64 | 8.245 |
|  |  | 27 | 39.14 | 34.17 | 7.820 |
|  |  | 32 | 37.12 | 18.13 | 7.929 |
|  |  | 37 | 35.03 | 10.24 | 6.828 |
|  | 0.75 | 22 | 42.11 | 90.80 | 8.492 |
|  |  | 27 | 39.16 | 40.31 | 7.796 |
|  |  | 32 | 37.11 | 21.01 | 7.313 |
|  |  | 37 | 35.00 | 11.93 | 7.070 |
|  | 1 | 22 | 42.11 | 94.08 | 8.722 |
|  |  | 27 | 39.16 | 41.27 | 7.787 |
|  |  | 32 | 37.10 | 21.39 | 7.329 |
|  |  | 37 | 34.98 | 12.20 | 7.059 |

The tables show that with an increasing τ, the PSNR remains constant, the bitrate rises, and the measured decoding energy decreases. For visualization, the results for the Cactus sequence are plotted in Fig.  2.

The left plot shows the classic rate-distortion curves for different values of τ. The blue line corresponds to τ=0 which is classic RDO. Increasing τ now leads to a decrease in rate-distortion performance which is indicated by the other curves that tend to the right.

The right plot shows the decoding-energy-distortion performance. One can see that the blue curve (τ=0) is located rightmost. Increasing τ now leads to a decrease in decoding energy because the curves tend to the left. Please note that for the Kimono sequence, a similar observation can be made.

Table 4: Rate, distortion and decoding energy performance of the proposed encoding method (Kimono sequence).

| Sequence | Τ | QP | YUV-PSNR [dB] | File size [Kb] | Decoding Energy [J] |
| --- | --- | --- | --- | --- | --- |
| Kimono | 0 | 22 | 43.87 | 23.17 | 8.945 |
|  |  | 27 | 42.42 | 12.49 | 8.299 |
|  |  | 32 | 40.72 | 7.536 | 8.324 |
|  |  | 37 | 38.71 | 4.610 | 7.037 |
|  | 0.25 | 22 | 43.87 | 23.20 | 8.956 |
|  |  | 27 | 42.41 | 12.49 | 8.057 |
|  |  | 32 | 40.71 | 7.528 | 8.124 |
|  |  | 37 | 38.70 | 4.603 | 7.054 |
|  | 0.5 | 22 | 43.89 | 24.45 | 8.245 |
|  |  | 27 | 42.38 | 13.19 | 7.820 |
|  |  | 32 | 40.63 | 7.939 | 7.929 |
|  |  | 37 | 38.56 | 4.903 | 6.828 |
|  | 0.75 | 22 | 43.99 | 27.93 | 8.492 |
|  |  | 27 | 42.40 | 14.34 | 7.796 |
|  |  | 32 | 40.65 | 8.710 | 7.313 |
|  |  | 37 | 38.58 | 5.545 | 7.070 |
|  | 1 | 22 | 44.00 | 28.57 | 8.722 |
|  |  | 27 | 42.40 | 14.49 | 7.787 |
|  |  | 32 | 40.65 | 8.827 | 7.329 |
|  |  | 37 | 38.57 | 5.701 | 7.059 |

Fig.  2: Rate-distortion diagram (left) and decoding energy distortion diagram (right) of the Cactus sequence. The colors correspond to different values of the parameter τ.

To give some numbers, the Bjøntegaard-Delta rate (BDR) and the Bjøntegaard-Delta decoding energies (BDDE) [1] are calculated for both sequences and all values of τ. The results are given in Table 5.

Table 5: BDR and BDE for the Cactus and the Kimono sequence using the proposed encoder method.

| Sequence | τ | BDR (%) | BDE (%) |
| --- | --- | --- | --- |
| Cactus | 0 | 0 | 0 |
|  | 0.25 | 0.02 | -0.34 |
|  | 0.5 | 7.62 | -6.69 |
|  | 0.75 | 25.64 | -6.67 |
|  | 1 | 28.80 | -6.35 |
| Kimono | 0 | 0 | 0 |
|  | 0.25 | 0.24 | -1.86 |
|  | 0.5 | 7.52 | -4.79 |
|  | 0.75 | 17.39 | -7.73 |
|  | 1 | 19.17 | -7.48 |

The table confirms the observation from the rate-distortion and the decoding-energy-distortion diagram. Generally, for an increasing τ, the rate rises and the energy decreases. For the presented approach, decoding energy savings up to 7.73% can be achieved while the bitrate increases by almost 17.39%. For smaller values of τ, energy savings around 6% can be achieved sacrificing 7-8% of bitrate. The values are visualized in the rate-energy diagram in Fig.  3.

Fig.  3: Rate-energy diagram for the Cactus and the Kimono sequence. τ=0 is located top left in the origin, τ=1 is located bottom right.

The diagram shows that especially for small values of τ, energy savings can be obtained at marginal increases of the rate. As a consequence, it makes sense to test further values of τ. In addition, higher energy savings are expected when the model is extended with further features.

# Closing remarks

This contribution presented an encoder method to construct decoding energy saving bit streams. The method is based on an extension of the classic RDO-approach and makes use of a model for estimating the decoding energy. For intra coding, energy savings of up to 8% at the expense of 17% bitrate were achieved. It is expected that the performance can be increased when further features are included in the energy model. In the literature it is shown that for HEVC, energy savings of more than 10% can be achieved at relatively small bitrate increases of less than 2% [6] which is also expected for the next generation video codec.

# References

[1] C. Herglotz, M. Kränzler, A. Kaup, “Energy Modeling for Video Decoding”, JVET-K0107, July 2018.

[2] G.J. Sullivan and T.Wiegand. Rate-distortion optimization for video compression. IEEE Signal Processing Magazine, 15(6):74 –90, Nov 1998.

[3] B. Li, J. Xu, D. Zhang, and H. Li. QP refinement according to Lagrange multiplier for high efficiency video coding. In Proc. IEEE International Symposium on Circuits and Systems (ISCAS), pages 477–480, May 2013.

[4] C. Herglotz, A. Heindel, and A. Kaup. Decoding energy distortion optimization for video coding. IEEE Transactions on Circuits and Systems for Video Technology, preprint, 2017, DOI: 10.1109/TCSVT.2017.2771819.

[5] P. Raoufi and J. Peters. Energy-efficient wireless video streaming with H.264 coding. In Proc. IEEE International Conference on Multimedia and Expo Workshops (ICMEW), pages 1–6, Jul 2013.

[6] C. Herglotz, “Energy Efficient Video Decoding”, Verlag Dr, Hut, ISBN 3843934614, München 2018.

[7] C. Herglotz, A. Kaup, “Energy Assessment for Video Decoding”, JVET-K0106, July 2018.

# Patent rights declaration(s)

The Friedrich-Alexander University Erlangen-Nürnberg (FAU) does not have any current or pending patent rights relating to the technology described in this contribution.