| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0109-v1 |
| --- | --- |

| Title: | CE1: Partitioning Structure in JVET-J0024 (Tests 1.0.12, 1.0.13, 1.0.14, and 1.0.15) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Minsoo Park Chanyul Kim Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | m.w.park@samsung.com mss.park@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

- Abstract

This contribution presents a partitioning structure proposed in JVET-J0024. The proposed structure allows binary and triple trees (BTT) for block partitioning, and introduces restrictions of ratio between width and height, and size of a block to reduce search space in encoder. Based on the partitioning structure proposed in JVET-J0024, in this contribution four tests are conducted; (1.0.12) BTT with quad tree split on the top level of CTU, (1.0.13) BTT without quad tree split on the top level of CTU, (1.0.14) VVC with block ratio restriction, and (1.0.15) VVC with block ratio and size restrictions. When the simulation for four tests is conducted based on VTM, results reportedly show –0.88% BD-rate with 146% and 97% encoding and decoding time under AI, 0.68% BD-rate with 74% and 100% encoding and decoding time under RA, and 0.38% BD-rate with 85% and 99% encoding and decoding time under LD for test (1.0.12), –0.87% BD-rate with 159% and 98% encoding and decoding time under AI, 0.04% BD-rate with 85% and 99% encoding and decoding time under RA, and 0.08% BD-rate with 98% and 98% encoding and decoding time under LD for test (1.0.13), no performance change under AI, 0.10% BD-rate with 85% and 97% encoding and decoding time under RA, and 0.17% BD-rate with 92% and 98% encoding and decoding time under LD for test (1.0.14), and no performance change under AI, 0.41% BD-rate with 74% and 97% encoding and decoding time under RA, and 0.31% BD-rate with 80% and 98% encoding and decoding time under LD for test (1.0.15).

Proposed Methods

In this contribution, a partitioning structure proposed in JVECT-J0024 [1] is presented. The proposed partitioning structure allows binary and triple trees (BTT) for recursive partitioning of a block, and introduces restrictions to reduce search space in encoder.

CU shapes can be described by the ratio between width and height of a block. For instance, if width and height of a block are the same it can be represented as 1:1 ratio CU or a square CU, and if width is equal to 64 and height is equal to 16 it can be represented as 1:4 ratio CU. CU partitioning is conducted based on the allowed CU shapes and their allowed maximum and minimum sizes.

Figure 1-1. BTT split modes

Depending on the shape of the current CU, allowed split modes can be varied by sequence level signaling. In this proposal, for each shape of the CU the minimum and the maximum sizes are defined in the sequence header as below. (All the sizes are in log2 scale and minus2 implies original size in log2 scale minus two)

- log2_cu_11_ratio_max_minus2: the maximum allowed size of 1:1 ratio CU

- log2_cu_11_ratio_min_minus2: the minimum allowed size of 1:1 ratio CU

- log2_cu_12_ratio_max_minus2: the maximum allowed size of 1:2 or 2:1 ratio CU (for longer side)

- log2_cu_12_ratio_min_minus2: the minimum allowed size of 1:2 or 2:1 ratio CU (for longer side)

- log2_cu_14_ratio_max_minus2: the maximum allowed size of 1:4 or 4:1 ratio CU (for longer side)

- log2_cu_14_ratio_min_minus2: the minimum allowed size of 1:4 or 4:1 ratio CU (for longer side)

In addition to this, triple split modes can be configured by signaling the minimum and the maximum allowed CU size as below.

- log2_tri_split_max_ minus2: the maximum allowed size of tri-split (for longer side)

- log2_tri_split_min_ minus2: the minimum allowed size of tri-split (for longer side)

For example, if CU size is 64x64 and log2_cu_12_ratio_max_minus2 is larger than 4 (real size is 64), bi-vertical and bi-horizontal split modes can be applied. And if log2_tri_split_max_minus2 is larger than 4 (real size is 64) but log2_cu_14_ratio_max_minus2 is smaller than 4 (real size is 64), tri-splits are not allowed. Because tri-sub CU of the CU has shape of 64x16 or 16x64 CU but these shape are not allowed by log2_cu_14_ratio_max_minus2. This is shown in the following figure.

Figure 1-2. Example of allowed split modes

Based on the proposed partitioning structure, four tests are conducted in CE1 [2] as follows:

- Test 1 (Sub-CE 1.0.12)

  - Configuration: BTT with quad tree on the top level of CTU

  - Max BT depth: 8

  - Max TT depth: 6

  - Allowed CU ratio between width and height: 1:1, 1:2, and 1:4

  - Max CU size for 1:4 ratio CU and triple tree split: 64

  - Number of CABAC context for split information: 12 (VTM: 17  12)

  - Encoder optimization technique: caching split cost for each split mode and use it next visit

- Test 2 (Sub-CE 1.0.13)

  - Configuration: BTT without quad tree on the top level of CTU

  - Min QT size: 128

  - Max BT depth: 10

  - Max TT depth: 8

  - Allowed CU ratio between width and height: 1:1, 1:2, and 1:4

  - Max CU size for 1:4 ratio CU and triple tree split: 64

  - Number of CABAC context for split information: 12 (VTM: 17  12)

  - Encoder optimization technique: caching split cost for each split mode and use it next visit

- Test 3 (Sub-CE 1.0.14)

  - Configuration: QT/BT/TT (VTM)

  - Allowed CU ratio between width and height: 1:1, 1:2, 1:4, and 1:8

- Test 4 (Sub-CE 1.0.15)

  - Configuration: QT/BT/TT (VTM)

  - Allowed CU ratio between width and height: 1:1, 1:2, 1:4, and 1:8

  - Max CU size for 1:4 ratio CU and triple tree split: 64

Experimental Results

The simulation was carried out under the JVET common test condition [4]. The software was compiled with GCC version 4.8.3 and the AVX SIMD instructions. Table 2-1 shows results summary for all tests.

Table 2-1. Summary of results for all tests

|  |  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1 (1.0.12) |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
|  | AI | -0.88% | -1.25% | -1.40% | 146% | 97% | -0.62% | -1.59% | -1.78% | 144% | 99% |
|  | RA | 0.68% | 0.43% | 0.27% | 74% | 100% | 1.00% | 0.25% | -0.09% | 81% | 99% |
|  | LD | 0.38% | 1.25% | 1.00% | 85% | 99% | 0.69% | 0.71% | 0.64% | 79% | 101% |
| Test 2 (1.0.13) |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
|  | AI | -0.87% | -1.28% | -1.45% | 159% | 98% | -0.61% | -1.67% | -1.85% | 159% | 101% |
|  | RA | 0.04% | -0.06% | -0.27% | 85% | 99% | 0.21% | -0.48% | -0.84% | 91% | 100% |
|  | LD | 0.08% | 1.00% | 0.48% | 98% | 98% | 0.14% | 0.18% | -0.01% | 89% | 102% |
| Test 3 (1.0.14) |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
|  | AI | 0.00% | 0.00% | 0.00% | 99% | 98% | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | RA | 0.10% | 0.19% | 0.22% | 88% | 100% | 0.11% | 0.16% | 0.16% | 93% | 100% |
|  | LD | 0.17% | 0.33% | 0.38% | 92% | 98% | 0.08% | 0.27% | 0.39% | 93% | 99% |
| Test 4 (1.0.15) |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
|  | AI | 0.00% | 0.00% | 0.00% | 99% | 98% | 0.00% | 0.00% | 0.00% | 100% | 99% |
|  | RA | 0.41% | 0.43% | 0.43% | 76% | 100% | 0.46% | 0.44% | 0.45% | 81% | 100% |
|  | LD | 0.31% | 0.55% | 0.74% | 80% | 98% | 0.26% | 0.48% | 0.58% | 80% | 99% |

Table 2-2. Results for Test 1 (Sub-CE 1.0.12)

Table 2-3. Results for Test 2 (Sub-CE 1.0.13)

Table 2-4. Results for Test 3 (Sub-CE 1.0.14)

Table 2-5. Results for Test 4 (Sub-CE 1.0.15)

The fundamental information for each test is as follows:

- Test 1 (Sub-CE 1.0.12)

  - Minimum and maximum CTU size: 4 and 64 (VTM: 4 and 128)

  - Minimum and maximum TU size: 4 and 64

  - Minimum and maximum PU size: 4 and 64 (VTM: 4 and 128)

  - Additional transform types and sizes: None

  - Number of CABAC contexts for split information: 12 (VTM: 17)

- Test 2 (Sub-CE 1.0.13)

  - Minimum and maximum CTU size: 4 and 128

  - Minimum and maximum TU size: 4 and 64

  - Minimum and maximum PU size: 4 and 128

  - Additional transform types and sizes: None

  - Number of CABAC contexts for split information: 12 (VTM: 17)

- Test 3 (Sub-CE 1.0.14)

  - Minimum and maximum CTU size: 4 and 128

  - Minimum and maximum TU size: 4 and 64

  - Minimum and maximum PU size: 4 and 128

  - Additional transform types and sizes: None

- Test 4 (Sub-CE 1.0.15)

  - Minimum and maximum CTU size: 4 and 128

  - Minimum and maximum TU size: 4 and 64

  - Minimum and maximum PU size: 4 and 128

  - Additional transform types and sizes: None

Conclusion

In this contribution, the partitioning structure using binary and triple trees is proposed, and some restrictions are also proposed to reduce search space in encoder. Using binary and triple trees without quad tree can be an alternative of the current partitioning structure of VVC. The restriction of ratio between width and height would be appropriate to reduce encoder complexity on the current partitioning method in VTM. It shows about 10% of complexity reduction with 0.1% BD-rate increase for both VTM and BMS software.

References

[1] “Description of SDR, HDR and 360° video coding technology proposal considering mobile application scenario by Samsung, Huawei, GoPro, and HiSilicon,” JVET-J0024, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, San Diego, US, Apr. 2018.

[2] “Description of Core Experiment 1 (CE 1): Partitioning,” JVET-J1021, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, San Diego, US, Apr. 2018.

[3] “Draft 1 of Versatile Video Coding,” JVET-J1001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, San Diego, US, Apr. 2018.

[4] “JVET common test conditions and software reference configuration,” JVET-J1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, San Diego, US, Apr. 2018.

Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).