| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0110-v2 |
| --- | --- |

| Title: | CE6-related: NSST restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne, A. Dorrell, Building A, 5 Talavera Rd, Macquarie Park, 2113, NSW, Australia | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
| Source: | Canon Information Systems Research Australia Pty Ltd. |  |  |

_____________________________

# Abstract

Non-secondary separable transforms (NSST) provides a compression advantage to the Benchmark Set (BMS) software from JVET, at a cost of increased complexity both in the support of the set of secondary transforms and in the encoder search to select a transform out of the candidate transforms for a given block. This contribution presents a restriction on the use of NSST to prohibit use on highly elongated blocks, that is, non-square blocks having an elongation (aspect ratio) exceeding a threshold amount. This restriction removes the encoder search from these cases.  Two thresholds are tested: Prohibition on blocks exceeding 2:1 and prohibition on blocks exceeding 4:1.  BD-rate results resulting from these are to be provided.

# Introduction

In BMS, NSST is applied to blocks as shown in Figure 1, and 64x64 transform blocks (not shown in the figure).

Figure 1. NSST usage in BMS-1.0.

For an intra-predicted coding unit, 35×3 transform matrices are defined.  One of 35 is selected according to the intra-prediction mode.  One of three, or NSST not applied (i.e. bypassed) is selected based on a search. NSST has been developed in prior JVET work, e.g. as described in [1], and is present in the BMS.

# Description of the proposed method

The searching and signalling of NSST is restricted to blocks that are not highly elongated.  Two definitions of highly elongated are made, i.e. blocks exceeding a 2:1 aspect ratio and blocks exceeding a 4:1 aspect ratio (with aspect ratio being defined as longer side length to shorter or equal side length).  See Figure 2 for an illustration of the impact on block sizes (64x64 is not affected, and not shown in the figure).

Figure 2. NSST restriction in place for blocks exceeding 2:1 and 4:1 aspect ratios.

# Experimental results

Simulation results under the JVET common test conditions [2] are as follows, for a restriction on application to blocks exceeding 2:1 aspect ratio:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0_BMS_jvetj1010.tlist |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.14% | 0.31% | 0.28% | 115% | 133% |
| Class C | 0.10% | 0.14% | 0.34% | 137% | 170% |
| Class E | 0.28% | 0.29% | 0.04% | 129% | 131% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.10% | 0.34% | 0.34% | 146% | 196% |
| Class F (optional) | 0.07% | 0.12% | -0.20% | 134% | 157% |

# Conclusion

This input document presents a method to reduce the complexity of NSST, with a view to substantially preserving the gain of this coding tool while reducing the complexity to a level suitable for adoption into the VTM.

# References

- J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce, “Algorithm Description of Joint Exploration Test Model 7 (JEM 7)”, JVET-G1001, Torino, IT, July 2017.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, JVET-J1010, San Diego, US, April 2018.

# Patent rights declaration(s)

Canon Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).