| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0111_WD |
| --- | --- |

| Title: | Proposed WD for CE1: Picture Boundary Split in JVET-J0024 (Test 2.0.12) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Minsoo Park Chanyul Kim Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | m.w.park@samsung.com mss.park@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

- Proposed Working Draft Changes

This document provides the proposed text of VVC Working Draft. The changes are highlighted in green

      - Multi-type tree syntax

| multi_type_tree( x0, y0, cbWidth, cbHeight, mttDepth, partIdx ) { | Descriptor |
| --- | --- |
| if( (allowSplitBtVer ¦ ¦ allowSplitBtHor ¦ ¦ allowSplitTtVer ¦ ¦ allowSplitTtHor) && x0 + ( 1 << log2CbSize ) <= pic_width_in_luma_samples && y0 + ( 1 << log2CbSize ) <= pic_height_in_luma_samples) |  |
| mtt_split_cu_flag | ae(v) |
| if( mtt_split_cu_flag ) { |  |
| if( ( allowSplitBtHor ¦ ¦ allowSplitTtHor ) && ( allowSplitBtVer ¦ ¦ allowSplitTtVer ) ) |  |
| mtt_split_cu_vertical_flag | ae(v) |
| if( ( allowSplitBtVer && allowSplitTtVer && mtt_split_cu_vertical_flag ) ¦ ¦ ( allowSplitBtHor && allowSplitTtHor && !mtt_split_cu_vertical_flag ) ) |  |
| mtt_split_cu_binary_flag | ae(v) |
| if( MttSplitMode[ x0 ][ y0 ][ mttDepth ] = = SPLIT_BT_VER ) { |  |
| multi_type_tree( x0, y0, cbWidth / 2, cbHeight, mttDepth + 1, 0 ) |  |
| multi_type_tree( x0 + ( cbWidth / 2 ), y0, cbWidth / 2, cbHeightY, mttDepth + 1, 1 ) |  |
| } else if( MttSplitMode[ x0 ][ y0 ][ mttDepth ] = = SPLIT_BT_HOR ) { |  |
| multi_type_tree( x0, y0, cbWidth, cbHeight / 2, mttDepth + 1, 0 ) |  |
| multi_type_tree( x0, y0 + ( cbHeight / 2 ), cbWidth, cbHeight / 2, mttDepth + 1, 1 ) |  |
| } else if( MttSplitMode[ x0 ][ y0 ][ mttDepth ] = = SPLIT_TT_VER ) { |  |
| multi_type_tree( x0, y0, cbWidth / 4, cbHeight, mttDepth + 1, 0 ) |  |
| multi_type_tree( x0 + ( cbWidth / 4 ), y0, cbWidth / 2, cbHeight, mttDepth + 1, 1 ) |  |
| multi_type_tree( x0 + ( 3 * cbWidth / 4 ), y0, cbWidth / 4, cbHeight, mttDepth + 1, 2 ) |  |
| } else { /* SPLIT_TT_HOR */ |  |
| multi_type_tree( x0, y0, cbWidth, cbHeight / 4, mttDepth + 1, 0 ) |  |
| multi_type_tree( x0, y0 + ( cbHeight / 4 ), cbWidth, cbHeight / 2, mttDepth + 1, 1 ) |  |
| multi_type_tree( x0, y0 + ( 3 * cbHeight / 4 ), cbWidth, cbHeight / 4, mttDepth + 1, 2 ) |  |
| } |  |
| } else |  |
| coding_unit( x0, y0, cbWidth, cbHeight ) |  |
| } |  |

      - Coding quadtree semantics

qt_split_cu_flag[ x0 ][ y0 ] specifies whether a coding unit is split into coding units with half horizontal and vertical size. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When qt_split_cu_flag[ x0 ][ y0 ] is not present, the following applies:

- If one or more of the following conditions are true, the value of qt_split_cu_flag[ x0 ][ y0 ] is inferred to be equal to 1:

- x0 + ( 1  <<  log2CbSize ) is greater than pic_width_in_luma_samples and (log2CbSize - MinCbLog2SizeY) > MaxMttDepth.

- y0 + ( 1  <<  log2CbSize ) is greater than pic_height_in_luma_samples and (log2CbSize - MinCbLog2SizeY) > MaxMttDepth.

- x0 + ( 1  <<  log2CbSize ) is greater than pic_width_in_luma_samples and y0 + ( 1  <<  log2CbSize ) is greater than pic_height_in_luma_samples.

–	Otherwise, the value of qt_split_cu_flag[ x0 ][ y0 ] is inferred to be equal to 0.

      - Multi-type tree semantics

The variables allowSplitBtVer, allowSplitBtHor, allowSplitTtVer allowSplitTtHor are derived as follows:

- The allowed binary split process as specified in clause 6.4.1 is invoked with the binary split mode SPLIT_BT_VER, the coding block width cbWidth, the coding block height cbHeight, the location ( x0, y0 ), the current multi-type tree depth mttDepth, the current partition index partIdx as input, and the output is assigned to allowSplitBtVer.

- The allowed binary split process as specified in clause 6.4.1 is invoked with the binary split mode SPLIT_BT_HOR, the coding block height cbHeight, the coding block width cbWidth, the location ( x0, y0 ), the current multi-type tree depth mttDepth, the current partition index partIdx as input, and the output is assigned to allowSplitBtHor.

- The allowed ternary split process as specified in clause 6.4.2 is invoked with the ternary split mode SPLIT_TT_VER, the coding block width cbWidth, the coding block height cbHeight, the location ( x0, y0 ), the current multi-type tree depth mttDepth, the current partition index partIdx as input, and the output is assigned to allowSplitTtVer.

- The allowed ternary split process as specified in clause 6.4.2 is invoked with the ternary split mode SPLIT_TT_HOR, the coding block height cbHeight, the coding block width cbWidth, the location ( x0, y0 ), the current multi-type tree depth mttDepth, the current partition index partIdx as input, and the output is assigned to allowSplitTtHor.

mtt_split_cu_flag equal to 0 specifies that a coding unit is not split. mtt_split_cu_flag equal to 1 specifies that a coding unit is split into two coding units using a binary split or into three coding units using a ternary split as indicated by the syntax element mtt_split_cu_binary_flag. The binary or ternary split can be either vertical or horizontal as indicated by the syntax element mtt_split_cu_vertical_flag.

When mtt_split_cu_flag is not present, the following applies:

- If one or more of the following conditions are true, the value of mtt_split_cu_flag is inferred to be equal to 1:

  - x0 + ( 1  <<  log2CbSize ) is greater than pic_width_in_luma_samples

  - y0 + ( 1  <<  log2CbSize ) is greater than pic_height_in_luma_samples

- Otherwise, the value of mtt_split_cu_flag is inferred to be equal to 0.

mtt_split_cu_vertical_flag equal to 0 specifies that a coding unit is split horizontally. mtt_split_cu_vertical_flag equal to 1 specifies that a coding unit is split vertically

When mtt_split_cu_vertical_flag is not present, it is inferred as follows:

- If allowSplitBtHor is equal to TRUE or allowSplitTtHor is equal to TRUE or x0 + ( 1  <<  log2CbSize ) is greater than pic_width_in_luma_samples, the value of mtt_split_cu_vertical_flag is inferred to be equal to 0.

- Otherwise, the value of mtt_split_cu_vertical_flag is inferred to be equal to 1

mtt_split_cu_binary_flag equal to 0 specifies that a coding unit is split into three coding units using a ternary split. mtt_split_cu_binary_flag equal to 1 specifies that a coding unit is split into two coding units using a binary split.

When mtt_split_cu_binary_flag is not present, it is inferred as follows:

- If x0 + ( 1  <<  log2CbSize ) is greater than pic_width_in_luma_samples or y0 + ( 1  <<  log2CbSize ) is greater than pic_height_in_luma_samples, the value of mtt_split_cu_binary_flag is inferred to be equal to 1.

- Othterwise if allowSplitBtVer is equal to FALSE and allowSplitBtHor is equal to FALSE, the value of mtt_split_cu_binary_flag is inferred to be equal to 0.

- Otherwise if allowSplitTtVer is equal to FALSE and allowSplitTtHor is equal to FALSE, the value of mtt_split_cu_binary_flag is inferred as to be equal to 1.

- Otherwise if allowSplitBtHor is equal to TRUE and allowSplitTtVer is equal to TRUE, the value of mtt_split_cu_binary_flag is inferred to be equal to !mtt_split_cu_vertical_flag.

- Otherwise (allowSplitBtVer is equal to TRUE and allowSplitTtHor is equal to TRUE), the value of mtt_split_cu_binary_flag is inferred to be equal to mtt_split_cu_vertical_flag.

The variable MttSplitMode[ x ][ y ][ mttDepth ] is derived from the value of mtt_split_cu_vertical_flag and from the value of mtt_split_cu_binary_flag as defined in Table 7-4 for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1.

…

- Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).