| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0112-v1 |
| --- | --- |

| Title: | CE2: long-tap deblocking filter (Test 2.2.1.5) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woong Il Choi Chanyul Kim 1803-101 110, Naesammi-ro, Osan-si Gyeonggi-do, Republic of Korea | Tel: Email: | +82 10 2452 0977 woongil.choi@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics |  |  |

_____________________________

- Abstract

This contribution is corresponding to CE2 test 2.2.1.5 and proposes changes on filtering process of deblocking filter on large block boundary. To improve visual quality by removing blocking artifact on large and smooth block boundary, it is proposed to selectively apply an additional longer tap strong filter. The proposed strong filtering process is different according to block edge direction. The number of filtered pixels are reduced in horizontal edge than vertical in order to reduce line memory for longer tap filter. The test results show that the proposed method achieves 0.03%, -0.02% and 0.10% BDBR for AI, RA, LDP compared to VTM1.0 and 0.03%, 0.03% and 0.07% BDBR for AI, RA, LDP compared to BMS, respectively.

Introduction

The deblocking filter has been employed to reduce visible artifacts at block boundary in video coding standards which utilize block-based coding structure. In the proposal for a response of Call for Proposals, JVET-J0024 [1], we applied extended deblocking process based to HEVC by employing the additional longer-tap strong filter because it is observed that the blocking artifacts on the boundaries of large and smooth block area is not effectively removed by the conventional strong filter. This extension of deblocking filter is introduced in JVET-D0047 [2] and we improved it further by applying different filtering process. This contribution reports coding performance of the proposed long tap deblocking filter studied in CE2 [3] test 2.2.1.5.

Proposed method

In the proposed scheme, filtering process is identical to HEVC except strong filtering process. If it is decided to apply the strong deblocking filter by the conventional filtering condition in the given block boundary, the following condition is additionally checked to decide type of strong filter. If the following conditions are satisfied, the proposed strong filter with longer-tap is applied instead of the conventional strong filter. The corresponding pixels on block boundary for filter condition are illustrated in Figure 1.

- Condition 1: block width of P and Q are greater than 8 in horizontal direction filtering or block height of P and Q are greater than 8 in vertical direction filtering

- Condition 2:

Figure 1 Pixel position to decide the type of strong filter

The filter coefficients of the proposed strong filter are shown in Table 1 and Table 2. A total of 14 pixels (p6 ~ q6) are filtered for horizontal direction filtering while a total of 10 pixels (p2 ~ q6) are filtered for vertical direction filtering due to memory bandwidth efficiency.

Table 1 New strong filter coefficients for horizontal direction

| Output pixel | Filter coefficients | Input pixels |
| --- | --- | --- |
| p6 | {3, 5, 2, 1, 1, 1, 1, 1, 1} | p7 ~ q0 |
| p5 | {1, 2, 5, 2, 1, 1, 1, 1, 1, 1} | p7 ~ q1 |
| p4 | {1, 3, 3, 3, 1, 1, 1, 1, 1, 1} | p6 ~ q2 |
| p3 | {1, 2, 2, 2, 2, 2, 1, 1, 1, 1, 1} | p6 ~ q3 |
| p2 | {1, 2, 2, 2, 2, 2, 2, 1, 1, 1} | p5 ~ q3 |
| p1 | {1, 1, 2, 2, 2, 2, 2, 2, 1, 1} | p5 ~ q3 |
| P0 | {1, 2, 2, 2, 2, 2, 2, 2, 1} | p4 ~ q3 |
| q0 | {1, 2, 2, 2, 2, 2, 2, 2, 1} | p3 ~ q4 |
| q1 | {1, 1, 2, 2, 2, 2, 2, 2, 1, 1} | p3 ~ q5 |
| q2 | {1, 1, 1, 2, 2, 2, 2, 2, 2, 1} | p3 ~ q5 |
| q3 | {1, 1, 1, 1, 1, 2, 2, 2, 2, 2, 1} | p3 ~ q6 |
| q4 | {1, 1, 1, 1, 1, 1, 3, 3, 3, 1} | p2 ~ q6 |
| q5 | {1, 1, 1, 1, 1, 1, 2, 5, 2, 1} | p1 ~ q7 |
| q6 | {1, 1, 1, 1, 1, 1, 2, 5, 3} | p0 ~ q7 |

Table 2 New strong filter coefficients for vertical direction

| Output pixel | Filter coefficients | Input pixels |
| --- | --- | --- |
| p2 | {3, 5, 2, 2, 1, 1, 1, 1} | p3 ~ q3 |
| p1 | {2, 2, 4, 2, 2, 2, 1, 1} | p3 ~ q3 |
| p0 | {2, 2, 2, 2, 2, 2, 2, 2} | p3 ~ q3 |
| q0 | {1, 2, 2, 2, 2, 2, 2, 2, 1} | p3 ~ q4 |
| q1 | {1, 1, 2, 2, 2, 2, 2, 2, 1, 1} | p3 ~ q5 |
| q2 | {1, 1, 1, 2, 2, 2, 2, 2, 2, 1} | p3 ~ q5 |
| q3 | {1, 1, 1, 1, 1, 2, 2, 2, 2, 2, 1} | p3 ~ q6 |
| q4 | {1, 1, 1, 1, 1, 1, 3, 3, 3, 1} | p2 ~ q6 |
| q5 | {1, 1, 1, 1, 1, 1, 2, 5, 2, 1} | p1 ~ q7 |
| q6 | {1, 1, 1, 1, 1, 1, 2, 5, 3} | p0 ~ q7 |

Experimental results

The proposed method is implemented based on BMS-1.0 software. VTM simulation result is obtained by running BMS-1.0 software with VTM configuration. The test condition is followed by CTC [4] and the simulation results compared to VTM anchor and BMS anchor are illustrated table 3 and 4, respectively. In both VTM and BMS configurations, the simulation results show the BDBR gain is only observed in 4K sequences, so that overall objective quality change is very minor. The complexity increment is very minor as well except RA cases.

Table 3 Simulation results with VTM configuration

|  | All Intra Main10 (VTM) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.00% | 0.00% | 99% | 101% |
| Class A2 | 0.03% | -0.01% | 0.00% | 99% | 102% |
| Class B | 0.04% | 0.00% | 0.00% | 98% | 101% |
| Class C | 0.01% | 0.00% | 0.01% | 98% | 100% |
| Class E | -0.02% | -0.01% | -0.01% | 98% | 99% |
| Overall | 0.03% | 0.00% | 0.00% | 98% | 100% |
| Class D | 0.01% | -0.01% | 0.00% | 99% | 101% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 99% | 103% |

|  | Random Access Main 10 (VTM) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.03% | 0.03% | 98% | 106% |
| Class A2 | -0.06% | -0.07% | -0.02% | 98% | 103% |
| Class B | 0.05% | -0.06% | 0.07% | 100% | 105% |
| Class C | 0.00% | 0.05% | 0.04% | 100% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.03% | 0.03% | 99% | 105% |
| Class D | 0.02% | 0.09% | -0.03% | 101% | 105% |
| Class F (optional) | 0.06% | 0.07% | 0.05% | 101% | 106% |

|  | Low delay B Main10 (VTM) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.10% | 0.19% | 0.23% | 99% | 98% |
| Class C | 0.01% | 0.00% | 0.01% | 100% | 96% |
| Class E | 0.22% | 0.18% | 0.30% | 100% | 102% |
| Overall | 0.10% | 0.12% | 0.17% | 100% | 98% |
| Class D | 0.01% | 0.07% | 0.06% | 100% | 100% |
| Class F (optional) | -0.06% | -0.09% | 0.15% | 100% | 102% |

Table 4 Simulation results with BMS configuration

|  | All Intra Main10 (BMS) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.00% | 0.01% | 99% | 101% |
| Class A2 | 0.04% | -0.01% | 0.00% | 99% | 100% |
| Class B | 0.04% | 0.01% | 0.00% | 99% | 98% |
| Class C | 0.01% | 0.00% | 0.01% | 99% | 99% |
| Class E | 0.00% | -0.01% | -0.02% | 99% | 98% |
| Overall | 0.03% | 0.00% | 0.00% | 99% | 99% |
| Class D | 0.01% | 0.00% | 0.00% | 99% | 99% |
| Class F (optional) | 0.01% | 0.02% | 0.01% | 99% | 99% |

|  | Random Access Main 10 (BMS) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.05% | 0.02% | 104% | 104% |
| Class A2 | 0.03% | 0.06% | 0.02% | 105% | 105% |
| Class B | 0.06% | 0.04% | 0.04% | 109% | 113% |
| Class C | 0.03% | 0.00% | -0.02% | 114% | 110% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.02% | 0.01% | 109% | 109% |
| Class D | 0.10% | 0.07% | 0.00% | 103% | 108% |
| Class F (optional) | 0.10% | 0.07% | 0.03% | 111% | 110% |

|  | Low delay B Main10 (BMS) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.10% | -0.06% | 0.26% | 98% | 99% |
| Class C | 0.02% | -0.06% | -0.19% | 99% | 94% |
| Class E | 0.08% | -0.03% | 0.55% | 99% | 99% |
| Overall | 0.07% | -0.05% | 0.18% | 99% | 97% |
| Class D | 0.01% | 0.35% | -0.46% | 100% | 98% |
| Class F (optional) | 0.05% | 0.14% | 0.45% | 99% | 99% |

In case of the deblocking filter, the subjective quality result is more important than objective one. Figure 2 shows the example of subjective quality comparison between VTM anchor and the proposed one. It is shown that the blocking artifact on large block boundary is still visible on VTM anchor while it is significantly reduced on the proposed scheme.

- VTM

- The proposed

Figure 2 The subjective quality comparison between VTM anchor and the proposed method (FoodMarket, QP=37, #28)

Conclusion

This contribution proposes additional strong filter to reduce blocking artifact on large and smooth block boundary. When the strong filter is decided to be applied to a given block boundary, the additional condition is checked to select the type of strong filter. In order to avoid additional line memory requirement, the number of filtered pixels is limited on horizontal edge boundary. In simulation result, coding gain is observed in 4K sequences in terms of BDBR and it is shown that blocking artifact is significantly removed in reconstructed image.

References

- “Description of SDR, HDR and 360° video coding technology proposal considering mobile application scenario by Samsung, Huawei, GoPro, and HiSilicon”, JVET-J0024, April 2018.

- K. Kawamura, and S. Naito, "Extended deblocking-filter decision for large block boundary", JVET-D0047, October 2016.

- L. Zhang and K. Andersson, “Description of Core Experiment 2 (CE2) on In-Loop Filters,” JVET-J1022, April 2018.

- Jill Boyce, K. Suehring, X. Li, and Vadim Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, April 2018.

Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).