| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0113-v1 |
| --- | --- |

| Title: | CE6-related: EMT signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne, A. Dorrell, Building A, 5 Talavera Rd, Macquarie Park, 2113, NSW, Australia | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
| Source: | Canon Information Systems Research Australia Pty Ltd. |  |  |

_____________________________

# Abstract

Explicit Multiple Transforms (EMT) provides a compression advantage to the Benchmark Set (BMS) software from JVET, in particular due to the provision of transforms with different basis functions and independent control of the applied primary transform horizontally and vertically.  This contribution presents a method for jointly signalling the selection of the horizontal and vertical transform index using a truncated unary binarisation.  BD-rate results resulting from this are to be provided.

# Introduction

In BMS-1.0, the EMT tool provides a compression advantage by enabling selection of the (separable) primary transform in the horizontal and vertical directions.  Firstly, from the intra-prediction mode two transform sets are selected, one for the horizontal transform and the other for the vertical transform.  Each transform set consists of two transforms.  Then, an index into each transform set is signalled in the bitstream, each index being signalled with one context-coded bin.  For each index a separate context is used.

The transform sets are defined as shown in Table 1.

Table 1. Transform set defintion in BMS.

|  | Transform candidates |  |
| --- | --- | --- |
| Transform set | Index 0 | Index 1 |
| 0 | DST-VII | DCT-VIII |
| 1 | DST-VII | DST-I |
| 2 | DST-VII | DCT-V |

The transform set is defined according to the intra-prediction mode, or a specific set is used for inter prediction. EMT is described in prior work, such as in [1].

# Description of the proposed method

This contribution proposed to replace the independent coding of each index, via the present 2-bit fixed-length code (FLC), with a truncated unary binarisation of cMax=3 to jointly code both indices. The proposed binarisation is shown in Table 2. This binarisation was chosen based on analysis of the statistics of selection of the pairs of indices in the BMS software, under common test conditions. Compared to the current BMS software, there is no change in the number of contexts.

Table 2. Proposed binarisation of EMT indices.

| Current (H, V – consecutive coding) |  | New (joint coding of H and V). | Binarisation |
| --- | --- | --- | --- |
| 00 |  | 0 | Ctx0 |
| 01 |  | 110 | Ctx0 + ctx1 + byp |
| 10 |  | 111 | Ctx0 + ctx1 + byp |
| 11 |  | 10 | Ctx0 + ctx1 |

# Experimental results

The experimental results under JVET common test conditions [2] are to be provided.

# Conclusion

This input document presents a method to jointly code the horizontal and vertical EMT indices with a truncated unary binarisation, with no change in the number of contexts in the design, for consideration for alteration of the current BMS design, prior to incorporation into VTM.

# References

- J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce, “Algorithm Description of Joint Exploration Test Model 7 (JEM 7)”, JVET-G1001, Torino, IT, July 2017.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, JVET-J1010, San Diego, US, April 2018.

# Patent rights declaration(s)

Canon Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).