| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0116-v1 |
| --- | --- |

| Title: | [CE4.3.5 – Adaptive Motion Vector Resolution in JVET-J0024] |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Anish Tamse Seungsoo Jeong Min Woo Park Chanyul Kim |  | tamse.anish@samsung.com ss00.jeong@samsung.com m.w.park@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics |  |  |

_____________________________

# Abstract

This contribution proposes the Adaptive Motion Vector Resolution tool proposed in J0024. The supported motion vector resolutions in the proposed method are 1/4–pel, 1/2–pel, 1–pel, 2–pel and 4–pel. The difference of this tool with the existing tool (IMV) in BMS are additional resolutions (1/2–pel and 2–pel) and coupling of MVR index and MVP index.

This tool shows BD-Rate improvement of -1.16% compared to VTM anchor and -0.59% for BMS anchor under RA conditions for JEM CTC.

# Introduction

Adaptive Motion Vector Resolution (AMVR) is a tool to reduce the bits spent in coding the MVD. Usually the MVD is coded with 1/4-pel precision. AMVR allows other resolutions for coding the MVD too, more specifically 1/4–pel, 1/2–pel, 1–pel, 2–pel and 4–pel resolution are allowed. The AMVR flag is signaled at CU level and the AMVR index is sent indicating which resolution should be used. Another component of this proposal is the coupling of MVR index and MVP index. The MVP index is not sent separately, the MVP is derived based on MVR index itself.

# Tool Description

Supported motion vector resolutions in proposed method range from 1/4–pel to 4–pel (1/4–pel, 1/2–pel, 1–pel, 2–pel and 4–pel). Information about the motion vector resolution is signaled at the CU level when MVD information is signaled.

Depending on the resolution of CU, both motion vector (MV) and motion vector predictor (MVP) of the CU are adjusted. If applied motion vector resolution is denoted as R, MV (MVx, MVy) and MVP (MVPx, MVPy) are represented as follows:

(MVx, MVy) = (Round(MVx / (R * 4)) * (R * 4), Round(MVy / (R * 4)) * (R * 4))

(MVPx, MVPy) = (Round(MVPx / (R * 4)) * (R * 4), Round(MVPy / (R * 4)) * (R * 4))

Since the motion vector predictor and MV are both adjusted by an adaptive resolution, MVD (MVDx, MVDy) is also aligned to the resolution and, is signaled according to the resolution as follows:

(MVDx, MVDy) = ((MVx – MVPx) / (R * 4), (MVy – MVPy) / R * 4))

In HEVC, AMVP mode uses MVP index for selecting the motion vector predictor among those in the MVP candidate list. In this proposal, motion vector resolution index (MVR index) indicates MVP index as well as the motion vector resolution. As result, the proposed method has no MVP index signaling. Table 1 shows what each value of MVR index represents.

Table 1 - MVR Index

| MVR Index Coded | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| Resolution (R) in pel | 1/4 | 1/2 | 1 | 2 | 4 |
| Nth MVP | 1st MVP | 2nd MVP | 3rd MVP | 4th MVP | 5th MVP |

Figure 1 - AMVR inter prediction

The MVR index codewords are as shown in Table 2. Four context models are used, one for each position of the bin.

Table 2 - MVR Index

| MVD Resolution | 1/4 | 1/2 | 1 | 2 | 4 |
| --- | --- | --- | --- | --- | --- |
| Codeword | 0 | 10 | 110 | 1110 | 1111 |

# Experimental Results

The experiments were performed on JVET CTC. The software was compiled using GCC version 4.8.3. SIMD optimization level used was up to AVX.

There are four sub-tests for this CE. The common difference between the JEM IMV tool and all the sub-tests of the proposed AMVR is the flag position, way the AMVR index is coded and the number of context models used. Furthermore the four subtests are as follows:

a) AMVR in J0024 with three MV resolutions (1/4 or 1 or 4)

b) AMVR in J0024 with five MV resolutions (1/4 or ½ or 1 or 2 or 4)

c) Combined signaling of MVR index and MVP index with three MV resolutions ( ¼ or 1 or 4) in J0024

d) Combined signaling of MVR index and MVP index with five MV resolutions (1/4 or ½ or 1 or 2 or 4) in J0024

Test a)

Resolution: ¼, 1, 4

MVR MVP coupling: Off

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.95% | -1.90% | -2.41% | 81% | 98% | -0.70% | -1.63% | -1.78% | 90% | 92% |
| Class A2 | -1.93% | -2.36% | -2.28% | 85% | 99% | -0.87% | -1.30% | -1.27% | 90% | 94% |
| Class B | -1.31% | -1.94% | -1.70% | 92% | 98% | -0.75% | -1.35% | -1.21% | 90% | 90% |
| Class C | -0.94% | -1.53% | -1.79% | 103% | 99% | -0.65% | -1.19% | -1.37% | 93% | 81% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.46% | -1.91% | -1.98% | 91% | 98% | -0.74% | -1.35% | -1.38% | 91% | 89% |
| Class D | -0.55% | -1.20% | -1.49% | 113% | 100% | -0.47% | -0.91% | -1.20% | 98% | 85% |
| Class F (optional) | -1.34% | -1.33% | -1.52% | 103% | 99% | -1.06% | -1.02% | -1.08% | 93% | 82% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.90% | -1.53% | -1.58% | 135% | 104% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.59% | -1.21% | -1.34% | 126% | 95% | -0.55% | -1.14% | -1.57% | 103% | 80% |
| Class E | -0.31% | -0.83% | -0.91% | 127% | 93% | -0.25% | -0.81% | -0.59% | 97% | 78% |
| Overall | -0.65% | -1.25% | -1.33% | 130% | 98% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.30% | -1.11% | -1.08% | 127% | 86% | -0.26% | -1.25% | -0.74% | 100% | 71% |
| Class F (optional) | -2.18% | -2.43% | -2.41% | 125% | 99% | -1.75% | -1.87% | -2.51% | 96% | 82% |

Test b)

Resolution: ¼, ½, 1, 2, 4

MVR MVP coupling: Off

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.19% | -1.96% | -2.77% | 96% | 99% | -0.78% | -1.67% | -1.89% | 94% | 93% |
| Class A2 | -2.07% | -2.80% | -2.68% | 104% | 99% | -0.97% | -1.52% | -1.47% | 97% | 94% |
| Class B | -1.48% | -2.28% | -2.05% | 113% | 98% | -0.84% | -1.55% | -1.39% | 96% | 90% |
| Class C | -0.97% | -1.73% | -2.16% | 126% | 99% | -0.69% | -1.27% | -1.62% | 99% | 82% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.60% | -2.17% | -2.35% | 111% | 99% | -0.81% | -1.49% | -1.56% | 97% | 89% |
| Class D | -0.53% | -1.48% | -1.78% | 142% | 99% | -0.47% | -0.95% | -1.20% | 106% | 86% |
| Class F (optional) | -1.45% | -1.50% | -1.73% | 122% | 100% | -1.21% | -1.32% | -1.41% | 98% | 83% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.97% | -1.80% | -1.72% | 172% | 101% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.62% | -1.61% | -1.89% | 164% | 101% | -0.55% | -1.23% | -1.54% | 112% | 81% |
| Class E | -0.26% | -1.06% | -1.24% | 163% | 95% | -0.29% | -0.34% | 0.19% | 109% | 80% |
| Overall | -0.67% | -1.55% | -1.66% | 167% | 99% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.26% | -1.33% | -1.40% | 167% | 87% | -0.19% | -1.01% | -1.06% | 115% | 72% |
| Class F (optional) | -2.21% | -2.61% | -2.48% | 160% | 102% | -1.89% | -1.64% | -2.72% | 105% | 82% |

Test c)

Resolution: ¼, 1, 4

MVR MVP coupling: On

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.44% | -1.34% | -1.92% | 83% | 100% | -0.51% | -1.24% | -1.56% | 91% | 93% |
| Class A2 | -1.38% | -1.82% | -1.70% | 87% | 99% | -0.63% | -1.10% | -1.04% | 92% | 94% |
| Class B | -0.98% | -1.61% | -1.46% | 94% | 99% | -0.55% | -1.13% | -0.92% | 91% | 90% |
| Class C | -0.55% | -1.16% | -1.36% | 105% | 100% | -0.42% | -0.89% | -1.06% | 94% | 82% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.04% | -1.48% | -1.57% | 93% | 99% | -0.52% | -1.08% | -1.11% | 92% | 89% |
| Class D | -0.35% | -1.13% | -1.31% | 114% | 100% | -0.29% | -0.73% | -0.91% | 99% | 85% |
| Class F (optional) | -0.96% | -1.17% | -1.32% | 104% | 100% | -0.80% | -0.87% | -0.89% | 95% | 83% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.76% | -1.43% | -1.52% | 138% | 102% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.31% | -0.92% | -1.20% | 129% | 96% | -0.27% | -0.49% | -0.99% | 100% | 80% |
| Class E | -0.44% | -1.24% | -1.23% | 128% | 93% | -0.29% | -0.60% | 0.56% | 100% | 80% |
| Overall | -0.53% | -1.21% | -1.34% | 132% | 98% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.05% | -1.29% | -1.22% | 128% | 87% | 0.07% | -0.72% | -0.71% | 101% | 72% |
| Class F (optional) | -1.84% | -2.13% | -2.05% | 138% | 99% | -1.63% | -1.81% | -2.19% | 97% | 82% |

Test d)

Resolution: ¼, ½, 1, 2, 4

MVR MVP coupling: On

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.70% | -1.64% | -2.35% | 99% | 99% | -0.61% | -1.39% | -1.79% | 95% | 92% |
| Class A2 | -1.51% | -2.21% | -2.06% | 108% | 100% | -0.69% | -1.21% | -1.22% | 99% | 94% |
| Class B | -1.09% | -1.87% | -1.65% | 116% | 99% | -0.64% | -1.28% | -1.12% | 97% | 90% |
| Class C | -0.60% | -1.36% | -1.64% | 130% | 99% | -0.45% | -1.11% | -1.31% | 101% | 82% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.16% | -1.75% | -1.87% | 114% | 99% | -0.59% | -1.24% | -1.33% | 98% | 89% |
| Class D | -0.31% | -1.35% | -1.57% | 145% | 100% | -0.28% | -0.75% | -1.08% | 107% | 86% |
| Class F (optional) | -1.12% | -1.43% | -1.62% | 125% | 100% | -0.95% | -1.19% | -1.28% | 100% | 83% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.80% | -1.53% | -1.73% | 164% | 56% | -0.69% | -1.35% | -1.65% | 116% | 96% |
| Class C | -0.33% | -1.34% | -1.74% | 167% | 100% | -0.31% | -0.87% | -1.38% | 116% | 89% |
| Class E | -0.46% | -1.03% | -1.15% | 167% | 97% | -0.33% | -0.65% | -0.26% | 116% | 86% |
| Overall | -0.56% | -1.34% | -1.59% | 166% | 78% | -0.47% | -1.02% | -1.21% | 116% | 91% |
| Class D | -0.02% | -1.93% | -1.31% | 168% | 87% | 0.11% | -1.29% | -0.96% | 119% | 74% |
| Class F (optional) | -2.23% | -2.68% | -2.64% | 174% | 101% | -1.62% | -1.85% | -2.56% | 110% | 90% |

# Conclusion

Test (a) provides the best performance speed trade off among the four sub-tests. The encoder is faster than the reference software (91%) while providing a performance improvement of -1.46% and -0.74% under RA conditions for VTM and BMS anchor respectively. The position of the AMVR flag signaling is more intuitive and easier to understand compared to the existing implementation, because when the MVD is decoded, its resolution is already known with the proposed implementation. This is not the case in the existing method as IMV index is decoded in the end after decoding the MVD. Also, the number of contexts is reduced to 2 as compared to 4 context models of the existing method. It is proposed that the modifications of the test (a) be adopted into the software.

# Patent rights declaration(s)

Samsung Electronics Co. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).