| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0117v1 |
| --- | --- |

| Title: | CE4: Results on Reference picture boundary padding in J0025 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minsoo Park Min Woo Park Woongil Choi Chanyul Kim Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | mss.park@samsung.com m.w.park@samsung.com woongil.choi@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

- Abstract

This contribution presents a reference picture boundary padding technique proposed in JVET-J0025. The proposed method is that if an edge pixel of a picture is coded with motion compensation, outside region of picture corresponding to the edge pixel is padded by motion compensation using the motion information of the edge pixel. Experimental results reportedly show 0.18% BD-Rate gain with 100% and 103% encoding and decoding time under RA for BMS.

Proposed Methods

In VVC, outside areas of a reference picture are padded by extrapolating edge pixel of a picture. This contribution proposes a padding method for padding outside areas of a picture with motion compensation with motion information of edge pixel of the picture. It is shown in Figure 1.

Figure 1: Boundary pixel padding using motion compensation

To use the motion information of the edge pixels, each 4x4 block is checked at the boundary of the picture. If there is motion information in the block, the location of the block is checked in the reference picture of the block. If the location is located in the image area, check whether the neighboring area of the reference area is available.

The location of the neighboring area may be located in four directions, up, down, left, right. The orientation of the adjacent area is the same as the location of the padding area. For example, if the padding area is located on the left side of the picture, then the inspection area is also on the left side of the pixel. In addition, the length of the side that does not face the picture of the padding area is determined by the distance between the position of the pixel of the reference picture and the position of the edge pixel or by the size of the padding area. The shorter of them is decided. If the predetermined length is shorter than the size of the padding area, the rest of the area is filled with extrapolated edge pixels of the external picture.

The available adjacent area is derived by motion compensation. However, a conventional padding method is performed when an adjacent area is unavailable or there is no information about a motion in a boundary block. The block can have two pieces of information about movement. In this case, each information is used to create a padding image and integrate two images into one. In addition, the last pixel of each position is extrapolated to induce a left upper portion, a right upper portion, a left lower portion, and a right padding area.

Experimental Results

The simulation was conducted on JVET common test condition. The software was compiled with GCC version 4.8.3 and the AVX SIMD instructions. Table 2 shows results of the proposed method.

Table 2. Results for Proposed methods

Conclusion

In this proposal, we propose a boundary pixel padding method using motion compensation. The proposed method can improve the coding performance. Experimental results show that both VTM and BMS have 0.18 BD-rate gain under RA.

References

[1] “Description of SDR, HDR and 360° video coding technology proposal by Huawei, GoPro, HiSilicon, and Samsung,” JVET-J0025, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, San Diego, US, Apr. 2018.

[2] “Description of CE4: Inter prediction and motion vector coding,” JVET-J1024, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, San Diego, US, Apr. 2018.

[3] “Draft 1 of Versatile Video Coding,” JVET-J1001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, San Diego, US, Apr. 2018.

[4] “JVET common test conditions and software reference configuration,” JVET-J1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, San Diego, US, Apr. 2018.

Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).