| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0120-v1 |
| --- | --- |

| Title: | CE1: Processing to support large CUs (Test 6.2.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo, Masaru Ikeda, Teruhiko Suzuki 2-10-1 Osaki, Shinagawa-ku, Tokyo Japan | Tel: Email: | +81-50-3750-2744 kenji.kondo@sony.com, teruhiko.s@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes a technique to avoid a large transform usage. The techniques are studied in CE1 (SubCE6.2). This contribution reports the test results as follow the test condition of CE1. With the Benchmark set (BMS) tools as reference, simulation results reportedly show that the proposed technique achieves 0.00%, 0.16% and -0.03% BD-rate, 101%, 97%, 97% encoding time, 99%, 99% and 101% decoding time for all intra (AI), random access (RA) and low delay B (LB) configurations respectively. Without BMS tool, simulation results reportedly show that the proposed technique achieves 0.00%, 0.08% and 0.06% BD-rate, 101%, 98%, 95% encoding time, 99%, 98% and 101% decoding time for AI, RA and LB.

Introduction

This technique was proposed in JVET Call for Proposal [1] and it was planned to study in CE1 [2]. In the CE1, the proposed techniques are categorized and tested in SubCE6.1.2 to study large CTU. This contribution reports the test results.

Proposed method

On the very low bitrate application, the impact of overhead is larger than when higher bitrate. To allow large transform helps to decrease the overhead. But, a large transform makes complexity high. To avoid the large transform, this method proposes to implicitly set 0 for CBF.

In this method, if the width or height size of the CU is larger than 64 for an inter prediction mode, the residual coding is implicitly disabled to avoid usage of large transforms beyond 64. The skip mode is used or the CBF values are inferred to be equal to 0.

Experimental results

The software is implemented based on BMS-1.0. And it is committed in SVN server [3].

Each testing item is evaluated using BD rates, encoding time, decoding time according to Common Test Conditions (CTC) in [4]. As following a test condition of CE, two test conditions are used as below.

- Configuration 1: VVC software with Benchmark set (BMS) configuration as provided in the repository.

- Configuration 2: Same software as in Configuration 1 but with disabling the BMS tools at config file level.

For below fundamental setting, it is same as anchor in this implementation and configuration. For also the maximum TU size, the proposed method is same as VTM. In the VTM implementation, TU is automatically split if the CU is larger than 64. Consequently the maximum TU size is 64.

         Minimum and maximum CTU size

         Minimum and maximum TU size

         Minimum and maximum PU size

         Additional transform types and sizes

Simulation environment

For the simulations, same platform machines as below in PC cluster were used.

- Encode

  - OS: CentOS 6.9

  - Compiler: g++ (GCC) 6.3.1

  - CPU: Intel Core i7-7700K 4.2GHz

    - SIMD: AVX2 instruction is enabled.

  - Memory: 32 GB

- Decode

  - OS: CentOS 6.9

  - Compiler: g++ (GCC) 6.3.1

  - CPU: Intel Core i5-2500K 3.3 GHz

    - SIMD: AVX instruction is enabled.

  - Memory: 24 GB

For a runtime measurement, the user time in the output log was used. For a random access case and an all intra case, a parallel encoding is used. To measure encoding time in parallel encoding, the total of all user time were calculated. Because the encoding time for each frame was elapsed time (not user time) and it is not reliable, the encode time for first I-frame was not subtracted. So the encoding time for first I frame is duplicated. For decoding time, it is reliable.

Configuration 1

The results of configuration-1 are summarized in the Table 1.

Table 1 : Over BMS-1.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 97% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.41% | 0.50% | 0.73% | 97% | 99% |
| Class A2 | 0.17% | 0.15% | 0.20% | 97% | 100% |
| Class B | 0.10% | 0.12% | 0.13% | 97% | 98% |
| Class C | 0.04% | 0.00% | -0.05% | 98% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.16% | 0.17% | 0.22% | 97% | 99% |
| Class D | 0.02% | -0.11% | -0.05% | 98% | 107% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | -0.06% | 0.29% | 97% | 99% |
| Class C | -0.03% | -0.11% | -0.04% | 97% | 103% |
| Class E | -0.20% | 0.85% | 0.40% | 95% | 102% |
| Overall | -0.03% | 0.15% | 0.21% | 97% | 101% |
| Class D | 0.10% | 0.28% | -0.06% | 99% | 106% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Configuration 2

The results of configuration-2 are summarized in the Table 3.

Table 3 : Over VTM-1.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 97% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 98% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.24% | 0.39% | 0.56% | 97% | 97% |
| Class A2 | 0.04% | 0.08% | 0.16% | 98% | 95% |
| Class B | 0.07% | 0.04% | 0.19% | 97% | 97% |
| Class C | 0.00% | 0.03% | 0.00% | 98% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | 0.12% | 0.21% | 98% | 98% |
| Class D | -0.02% | -0.03% | 0.03% | 97% | 105% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | 0.11% | 0.20% | 96% | 98% |
| Class C | 0.08% | -0.13% | 0.01% | 97% | 104% |
| Class E | 0.05% | 0.32% | 0.48% | 92% | 101% |
| Overall | 0.06% | 0.09% | 0.21% | 95% | 101% |
| Class D | 0.03% | -0.32% | 0.33% | 97% | 108% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Conclusion

This contribution reports simulation results of CE test 6.2, which evaluates to avoid usage of a large transform by implicitly setting CBF zero. And the impact was checked that it is reported, with BMS 0.00%, 0.16% and -0.03% BD-rate, 101%, 97%, 97% encoding time, 99%, 99% and 101% decoding time for AI, RA and LB configurations respectively. Without BMS tool, simulation results reportedly show that the proposed technique achieves 0.00%, 0.08% and 0.06% BD-rate, 101%, 98%, 95% encoding time, 99%, 98% and 101% decoding time for AI, RA and LB.

References

[1] T. Suzuki, M. Ikeda and K. Sharman, “Description of SDR and HDR video coding technology proposal by Sony” JVET-J0028, JVET 10th Meeting: San Diego, April 2018.

[2] Jackie Ma, Fabrice Le Léannec, Min Woo Park, “Description of Core Experiment: Partitioning” JVET-J1021, JVET 10th Meeting: San Diego, April 2018.

[3] https://jvet.hhi.fraunhofer.de/svn/svn_VCC_Software_BMS/branches/CE/J_SanDiego/CE1/CE1-6.2.1

[4] Jill Boyce, K. Suehring, X. Li, and Vadim Seregin, “JVET common test conditions and software reference configurations” JVET-J1010, April 2018.

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).