| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K00121-v1 |
| --- | --- |

| Title: | CE6: Chroma AMT (CE6.1.7.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Masaru Ikeda Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-2779 takeshi.tsukuba@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution reports the results of CE6.1.7.1 Chroma AMT, which extends Adaptive Multiple Core Transforms (AMT) on top of BMS to chroma transform block (TB) with additional flag (chromaEmtFlag) signaling whether to use DCT-2 or that of the collocated luma TB’s as chroma transform.

The experiments show that

- average BD-rate gains of each Y, U and V over VTM are 3.42%, 2.34% and 2.34% for AI, 2.13%,  1.19% and 1.33% for RA, 1.54%, 0.16% and -0.01% for LB with approximately 170~214% ENC runtime and 115~132% DEC runtime, respectively.

- average BD-rate gains of each Y, U and V over VTM with AMT enabled (--EMT=3 --EMTFast=3) are -0.04%, 0.81% and 0.82% for AI, -0.04%, 1.15% and 1.21% for RA, 1.54%, 0.16% and -0.01% for LB, with approximately 1~2% runtime increase for encoder and decoder , respectively.

- average BD-rate gains of each Y, U and V over BMS are -0.05%, 0.43% and 0.38% for AI, -0.08%, 0.77% and -0.90% for RA, -0.05%, 1.31% and 1.10% for LB, with approximately 1% runtime increase for encoder and decoder, respectively.

Introduction

This technology is, originally proposed in JVET-J0028 [1] as Sony’s response to Call for Proposals at the last meeting, as a part of changes to Adaptive Multiple core Transforms (AMT). AMT uses totally five types of trigonometric transforms: DCT-2, DCT-5, DCT-8, DST-1 and DST-7, which is used for luma transform blocks (TB) while DCT-2 is used for chroma TBs.

Table 1-1 shows the coding performance of AMT (--EMT=3 –EMTFast=3) over VTM-1.1 (BMS-1.1 with vtm config). It can be seen that AMT achieves luma BD-rate reduction by 3.46%, 2.17% and 1.57% for AI, RA and LB, respectively although there are some chroma BD-rate losses for RA and LB.

In the same way, it is expected that applying AMT for chroma TB improves chroma coding efficiency.

For CE6.1.7.1, the use of AMT for chroma is tested with additional flag (chromaEmtFlag) signaling whether to use DCT-2 or that of the collocated luma TB’s as chroma transform.

Figure 1-1 Coding performance of AMT (--EMT=3 --EMTFast=3) over VTM-1.0

|  | All Intra Main10 |  |  | Random Access Main 10 |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  | Over VTM-1.1 |  |  | Over VTM-1.1 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A1 | -3.29% | -1.32% | -1.22% | -1.91% | 0.08% | 0.15% |  |  |  |
| Class A2 | -3.16% | -1.65% | -1.43% | -2.05% | 0.14% | 0.47% |  |  |  |
| Class B | -3.46% | -1.44% | -1.42% | -2.49% | -0.27% | -0.58% | -1.92% | 0.83% | 0.37% |
| Class C | -3.12% | -0.90% | -1.16% | -2.06% | 0.03% | -0.20% | -1.30% | 0.95% | 0.93% |
| Class E | -4.40% | -2.74% | -2.65% |  |  |  | -1.34% | 2.05% | 1.92% |
| Overall | -3.46% | -1.55% | -1.53% | -2.17% | -0.04% | -0.12% | -1.57% | 1.18% | 0.94% |
| Class D | -3.00% | -0.66% | -0.74% | -1.82% | 0.51% | -0.15% | -1.24% | 0.71% | 0.86% |

Proposed Technology

With a basic design of AMT on top of BMS kept unchanged, the proposed method is to extend AMT to chroma transform blocks as follows:

- signal one bit flag (chromaEmtFlag) for each non-zero chroma TB (cbf>0) based on EmtFlag for luma and chroma TB size, which indicates whether to use default transform type (DCT-2) or the collocated luma TB’s as chroma transform type.

- If chromaEmtFlag is equal to 0, set chroma transform type to DCT-2.

- Otherwise (chromaEmtFlag is equal to1), set chroma transform type to the collocated luma TB’s.

Table 2-1 shows the transform type decision table for chroma inter predicted blocks. In the same way, a transform type for chroma intra predicted blocks is determined based on chroma TB size, chromaEmtFlag and the transform type of the collocated luma TB.

Table 2-1: Proposed decision table on the core transform type for inter predicted blocks

| Chroma TB Size | Chroma EmtFlag (Cb/Cr) | EmtFlag of the collocated luma TB | EmtIdx of the collocated luma TB | Luma |  | Chroma (Cb/Cr) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Hor. Tr. | Ver. Tr. | Hor. Tr. | Ver. Tr. |
| - | - | 0 | - | DCT-2 | DCT-2 | DCT-2 | DCT-2 |
| < 8x8 or > 32x32 | Not signal | 1 | 0 | DCT-8 | DCT-8 | DCT-2 | DCT-2 |
|  |  |  | 1 | DST-7 | DCT-8 |  |  |
|  |  |  | 2 | DCT-8 | DST-7 |  |  |
|  |  |  | 3 | DST-7 | DST-7 |  |  |
| >= 8x8 and <= 32x32 | 0 (signal) |  | 0 | DCT-8 | DCT-8 | DCT-2 | DCT-2 |
|  |  |  | 1 | DST-7 | DCT-8 |  |  |
|  |  |  | 2 | DCT-8 | DST-7 |  |  |
|  |  |  | 3 | DST-7 | DST-7 |  |  |
|  | 1 (signal) |  | 0 | DCT-8 | DCT-8 | DCT-8 | DCT-8 |
|  |  |  | 1 | DST-7 | DCT-8 | DST-7 | DCT-8 |
|  |  |  | 2 | DCT-8 | DST-7 | DCT-8 | DST-7 |
|  |  |  | 3 | DST-7 | DST-7 | DST-7 | DST-7 |

For chormaEmtFlag coding, four additional contexts are assigned based on a CU coding type (intra CU or Inter CU) and component ID (Cb or Cr) as below:

ctxInc = 0 for intra CU and compID==Cb

ctxInc = 1 for intra CU and compID==Cr

ctxInc = 2 for inter CU and compID==Cb

ctxInc = 3 for inter CU and compID==Cr

The initial cabac value is set to 154 without tuning.

In addition, the best chromaEmtFlag for each Cb and Cr is determined based on RDCost.

The main difference relative to JVET-J0028 is that chroma transform type signaling is unified between chroma intra predicted blocks and chroma inter predicted blocks. Note that signaling and transform type selection for Luma AMT is the same as BMS.

Complexity metrics

- Operation counts for individual transforms.

  - Same as for BMS.

- Memory requirements for storing transform cores

  - Same as for BMS.

- Specify the minimum bit-precision required by the transforms

  - Same as for BMS.

- Other operations and memory requirements relevant to the proposed tool.

  - No other operations and memory requirements.

- Provide analysis of implementation and arithmetic commonalities of proposed transforms.

  - Same as for BMS.

- List of all combinations of transform types and transforms block sizes used for primary transformation and secondary transformation.

  - Table 3-1 shows the list of all combinations for chroma AMT.

  - On AMT for Luma, same as for BMS.

  - On NSST, same as for BMS.

Table 3-1 List of all combinations for primary transform for chroma AMT

| Block size | primary transform |  |
| --- | --- | --- |
|  | horizontal | vertical |
| < 8x8 or > 32x32 | DCT-2 | DCT-2 |
| >=8x8 and <=32x32 | DCT-2 | DCT-2 |
|  | DCT-5/DCT-8/DST-1/DST-7 | DCT-5/DCT-8/DST-1/DST-7 |

Simulation Results

The proposed method was integrated on top of BMS-1.1 [2], and four simulations were conducted according to test condition described in CE6 description [3] with following settings:

- Macro for CE6.1.7.1:

  - SO_CHROMA_AMT        1

  - SO_CHROMA_AMT_TEST  0

- VTM vs Tested

  - anchor: BMS-1.1 s/w, vtm config

  - additional anchor: BMS-1.1 s/w, vtm config, option: --EMT=3 --EMTFast=3

  - tested: CE6.1.7 s/w, vtm config, option: --EMT=3 --EMTFast=3

- BMS vs Tested

  - anchor: BMS-1.1 s/w, bms config

  - tested: CE6.1.7 s/w, bms config

- VTM with NSST enabled vs Tested

  - anchor: BMS-1.1 s/w, vtm config, option: --NSST=1

  - tested: CE6.1.7 s/w, vtm config, option: --EMT=3 --EMTFast=3 --NSST=1

- BMS with NSST disabled vs Tested

  - anchor: BMS-1.1 s/w, bms config, option: --NSST=0

  - tested: CE6.1.7 s/w, bms config, option: --NSST=0

CE6.1.7 s/w is available at 
https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE6/CE6-1.7/

VTM vs Tested

The coding performance over VTM and VTM with AMT enabled is summarized in Table 3-1.

Over VTM, it is observed that:

- average BD-rate gains of each Y, U and V over VTM are 3.42%, 2.34% and 2.34% for AI, 2.13%,  1.19% and 1.33% for RA, 1.54%, 0.16% and -0.01% for LB, respectively.

- average ENC runtime are 214% for AI, 170% for RA and 167% for LB, respectively.

- average DEC runtime are 132% for AI, 116% for RA and 115% for LB, respectively.

Over VTM with AMT enabled, it is observed that:

- average BD-rate gains of each Y, U and V over VTM with AMT enabled (--EMT=3 --EMTFast=3) are -0.04%,  0.81% and 0.82% for AI, -0.04%, 1.15% and 1.21% for RA, -0.03%, -1.32% and -0.92% for LB , respectively.

- average ENC runtime are 101% for AI, 102% for RA and 101% for LB, respectively.

- average DEC runtime are 102% for AI, 102% for RA and 100% for LB, respectively.

It seems that chroma AMT improves chroma coding efficiency with negligible luma coding loss (<0.05%) and has small impacts to runtime increase by roughly 1~2% for encoder and decoder.

Table 4-1 Results over VTM and VTM with AMT enabled (--EMT=3 –EMTFast=3)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  | Over VTM-1.1 --EMT=3 --EMTFast=3 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.32% | -2.76% | -2.79% | 205% | 157% | -0.03% | -1.47% | -1.59% | 101% | 105% |
| Class A2 | -3.10% | -2.69% | -2.35% | 215% | 138% | 0.06% | -1.05% | -0.93% | 101% | 102% |
| Class B | -3.40% | -2.25% | -2.21% | 222% | 136% | 0.06% | -0.82% | -0.80% | 101% | 102% |
| Class C | -3.08% | -1.18% | -1.47% | 222% | 115% | 0.04% | -0.29% | -0.32% | 101% | 100% |
| Class E | -4.32% | -3.29% | -3.27% | 201% | 123% | 0.08% | -0.56% | -0.64% | 100% | 101% |
| Overall | -3.42% | -2.34% | -2.34% | 214% | 132% | 0.04% | -0.81% | -0.82% | 101% | 102% |
| Class D | -2.97% | -0.82% | -0.86% | 233% | 111% | 0.03% | -0.16% | -0.12% | 100% | 100% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  | Over VTM-1.1 --EMT=3 --EMTFast=3 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.94% | -2.02% | -2.29% | 174% | 128% | -0.03% | -2.10% | -2.44% | 102% | 106% |
| Class A2 | -1.97% | -0.96% | -0.67% | 164% | 115% | 0.09% | -1.10% | -1.13% | 102% | 101% |
| Class B | -2.44% | -1.55% | -1.73% | 170% | 116% | 0.05% | -1.29% | -1.14% | 102% | 101% |
| Class C | -2.01% | -0.27% | -0.62% | 171% | 109% | 0.06% | -0.30% | -0.42% | 102% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -2.13% | -1.19% | -1.33% | 170% | 116% | 0.04% | -1.15% | -1.21% | 102% | 102% |
| Class D | -1.80% | 0.01% | -0.29% | 167% | 104% | 0.02% | -0.50% | -0.13% | 103% | 100% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  | Over VTM-1.1 --EMT=3 --EMTFast=3 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.87% | -0.84% | -0.53% | 177% | 122% | 0.05% | -1.65% | -0.88% | 102% | 101% |
| Class C | -1.28% | 0.54% | 0.34% | 167% | 112% | 0.03% | -0.41% | -0.58% | 101% | 99% |
| Class E | -1.35% | 0.02% | 0.44% | 151% | 110% | -0.01% | -1.98% | -1.45% | 99% | 100% |
| Overall | -1.54% | -0.16% | 0.01% | 167% | 115% | 0.03% | -1.32% | -0.92% | 101% | 100% |
| Class D | -1.13% | 0.61% | 0.02% | 154% | 104% | 0.11% | -0.11% | -0.83% | 101% | 99% |

BMS vs Tested

The coding performance over BMS is summarized in Table 3-2.

It is observed that:

- average BD-rate gains of each Y, U and V are -0.05%, 0.43% and 0.38% for AI, -0.08%, 0.77% and -0.90% for RA, -0.05%, 1.31% and 1.10% for LB, respectively.

- average ENC runtime are 101% for AI, 101% for RA and 101% for LB, respectively.

- average DEC runtime are 101% for AI, 100% for RA and 101% for LB, respectively.

Comparing VTM with AMT enabled vs Tested case, it is also observed that

- average chroma BD-rate gains decrease by 0.4% for AI and 0.3% for RA with similar chroma BD-rate gains for LB.

- average luma BD-rate losses slightly increase by 0.01% for AI, 0.04% for RA and 0.02% for LB.

Table 4-2 Results over BMS

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | -0.75% | -0.80% | 101% | 101% |
| Class A2 | 0.05% | -0.66% | -0.64% | 102% | 101% |
| Class B | 0.05% | -0.43% | -0.36% | 101% | 100% |
| Class C | 0.02% | -0.09% | -0.04% | 101% | 101% |
| Class E | 0.04% | -0.32% | -0.18% | 100% | 100% |
| Overall | 0.05% | -0.43% | -0.38% | 101% | 101% |
| Class D | 0.06% | -0.12% | -0.01% | 101% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -1.00% | -1.42% | 101% | 101% |
| Class A2 | 0.09% | -0.64% | -0.83% | 102% | 100% |
| Class B | 0.05% | -1.01% | -0.87% | 102% | 100% |
| Class C | 0.11% | -0.40% | -0.60% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | -0.77% | -0.90% | 101% | 100% |
| Class D | 0.10% | 0.04% | -0.11% | 101% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | -1.58% | -1.44% | 102% | 101% |
| Class C | 0.02% | -0.43% | -0.60% | 101% | 100% |
| Class E | 0.05% | -2.02% | -1.17% | 100% | 101% |
| Overall | 0.05% | -1.31% | -1.10% | 101% | 101% |
| Class D | 0.07% | -0.28% | 0.08% | 101% | 100% |

VTM with NSST enabled vs Tested

The coding performance over VTM with NSST enabled is summarized in Table 3-3.

It is observed that:

- average BD-rate gains of each Y, U and V are 2.33%, 1.97% and 1.96% for AI, 1.67%, 1.13% and 1.20% for RA, 1.35%, 0.57% and 0.53% for LB, respectively.

- average ENC runtime are 244% for AI, 207% for RA and 185% for LB, respectively.

- average DEC runtime are 126% for AI, 115% for RA and 115% for LB, respectively.

Table 3-3 Results over VTM with NSST enabled

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 --NSST=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.20% | -2.41% | -2.35% | 226% | 147% |
| Class A2 | -2.30% | -2.38% | -1.78% | 246% | 131% |
| Class B | -2.43% | -1.90% | -2.04% | 254% | 130% |
| Class C | -1.94% | -0.74% | -1.05% | 254% | 111% |
| Class E | -2.86% | -2.84% | -2.98% | 234% | 119% |
| Overall | -2.33% | -1.97% | -1.98% | 244% | 126% |
| Class D | -1.81% | -0.55% | -0.73% | 264% | 108% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 --NSST=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.43% | -1.73% | -2.01% | 211% | 125% |
| Class A2 | -1.64% | -0.94% | -0.55% | 198% | 114% |
| Class B | -1.98% | -1.58% | -1.54% | 209% | 114% |
| Class C | -1.47% | -0.26% | -0.64% | 207% | 108% |
| Class E |  |  |  |  |  |
| Overall | -1.67% | -1.13% | -1.20% | 207% | 115% |
| Class D | -1.31% | -0.26% | -0.06% | 204% | 104% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 --NSST=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.67% | -1.07% | -0.93% | 201% | 121% |
| Class C | -1.14% | 0.18% | 0.06% | 187% | 112% |
| Class E | -1.09% | -0.76% | -0.66% | 158% | 110% |
| Overall | -1.35% | -0.57% | -0.53% | 185% | 115% |
| Class D | -1.06% | 0.52% | 0.71% | 167% | 103% |

BMS with NSST disabled vs Tested

The coding performance over BMS with NSST disabled is summarized in Table 3-4.

It is observed that:

- average BD-rate gains of each Y, U and V over BMS with NSST disabled are -0.05%, 0.53% and 0.55% for AI, -0.06%, 0.90% and 1.00% for RA, -0.05%, 1.22% and 1.06% for LB, respectively.

- average ENC runtime are 101% for AI, 101% for RA and 101% for LB, respectively

- average DEC runtime are 101% for AI, 100% for RA and 100% for LB, respectively.

Table 3-4 Results over BMS with NSST disabled

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.1--NSST=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.85% | -0.87% | 101% | 101% |
| Class A2 | 0.05% | -0.73% | -0.74% | 101% | 102% |
| Class B | 0.07% | -0.65% | -0.61% | 101% | 101% |
| Class C | 0.03% | -0.19% | -0.15% | 101% | 101% |
| Class E | 0.06% | -0.25% | -0.49% | 101% | 101% |
| Overall | 0.05% | -0.53% | -0.55% | 101% | 101% |
| Class D | 0.04% | -0.20% | -0.09% | 100% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.1--NSST=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -1.28% | -1.61% | 101% | 100% |
| Class A2 | 0.11% | -0.81% | -1.03% | 102% | 100% |
| Class B | 0.06% | -1.15% | -1.05% | 101% | 100% |
| Class C | 0.05% | -0.38% | -0.47% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | -0.90% | -1.00% | 101% | 100% |
| Class D | 0.01% | -0.40% | 0.09% | 101% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.1--NSST=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | -1.79% | -1.12% | 102% | 100% |
| Class C | 0.07% | -0.89% | -0.72% | 101% | 101% |
| Class E | 0.01% | -0.70% | -1.42% | 98% | 100% |
| Overall | 0.05% | -1.22% | -1.06% | 101% | 100% |
| Class D | -0.10% | -0.61% | -1.04% | 101% | 101% |

Conclusion

In this contribution, it is proposed to extend AMT to chroma transform block with additional flag (chromaEmtFlag) signaling whether to use DCT-2 or that of the collocated luma TB’s as chroma transform.

From the results over VTM with AMT enabled, the proposed method can achieve chroma BDrate reduction by roughly 0.8% for AI, 1.2% for RA and 1.1% for LB with negligible luma coding loss (<0.05%). It also has small runtime increase of roughly 1~2% for encoder and decoder.

It is recommended to further study on the use of chroma AMT in CE.

References

[1] T. Suzuki, M. Ikeda, K. Sharman, “Description of SDR and HDR video coding technology proposal by Sony”, JVET-J0028

[2] BMS, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.1

[3] A. Said, X. Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signalling”, JVET-J1026

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).