| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0122-v1 |
| --- | --- |

| Title: | CE3-related: Alternative techniques for DC mode without division |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy, Jianle Chen | Tel: Email: | +7-495-660-44-59 alexey.filippov@huawei.com vasily.rufitskiy@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

In test CE3-1.2.1 [1], a mechanism of calculating approximations of DC values for generating DC-based intra-predictors was proposed. This contribution presents 2 techniques alternative to CE3-1.2.1. The 1st technique uses multiplication operation and an additional LUT to avoid division. In the 2nd technique, reference samples only along with a longer side of a rectangular block are taken to calculate a DC value. In both cases, the conventional technique of calculating DC values is used for square blocks as a division operation is not required for them. On top of VTM, the simulation results for the 1st technique reportedly show 0.00% of luma BD-rate savings for All Intra (AI) and -0.01% of luma BD-rate savings for Random Access (RA) configuration. On top of BMS, the simulation results for the 1st technique reportedly show -0.01% luma BD-rate savings for All Intra (AI) configuration, and -0.02% of luma BD-rate savings for Random Access (RA) configuration.

On top of VTM, the simulation results for the 2nd technique reportedly show -0.02% luma BD-rate savings for All Intra (AI) configuration, and -0.01% luma BD-rate savings for Random Access (RA) configuration. On top of BMS, the simulation results for the 2nd technique reportedly show -0.02% luma BD-rate savings for All Intra (AI) configuration, and -0.02% luma BD-rate savings for Random Access (RA) configuration.

# Introduction

In the HEVC/H.265 standards, every intra-coded block has a square shape and length of its each side is a power of 2. Thus, no division operations are required to generate an intra-predictor using DC mode. After introducing the QTBT and MTT frameworks, blocks can have rectangular shapes that causes the necessity of using a division operation per block in the general case. An approach to avoiding division operations is studied in test CE3-1.2.1 where only add and shift operations are used. This contribution presents 2 alternative techniques for generating a DC-based intra-predictor without using division operations.

# Proposed techniques

The 2 proposed techniques have different emphases. The 1st technique that uses one multiplication per block is aimed at approximating the conventional DC intra-predictor as close to as possible. In the 2nd technique, we try to take advantage of new partitioning framework due to further reducing computational complexity of DC mode that can be important for commercial implementations of video codecs.

## Multiplication-based DC mode without division

The proposed mechanism requires only one multiplication per block, some other elementary arithmetic operations and a lookup table (LUT). Multiplication operation and the LUT are not required for square blocks that can be important for the smallest blocks (4x4 for luma and 2x2 for chroma) to avoid such operations as multiplication for the worst case when a picture or its part is subdivided into the smallest blocks. The LUT size (in records) is computed as follows:

Fig. 1 Reference samples used for calculating DC value.

Each record comprises a pair , since the value of  can be different for different values of  to maximize the accuracy of calculations. Here,  is an aspect ratio of a block.

In this technique DC value is calculated as follows:

,

,

,

are width and height (respectively) of the predicted block.

## DC mode using reference samples only along with a longer side

This technique is close to the one being studied in CE3.1.2.1 [1]. The difference with this technique is that only longest side component is considered when calculating DC value. Specifically:

,

if  and

,

if .

Denotations used here are the same as used previously:,  are width and height (respectively) of the predicted block.

Fig. 2 Reference samples used for calculating DC value.

# Experimental Results

Both proposed techniques are implemented on top of VTM-1.0 and BMS-1.0. The experimental results of All Intra and Random Access configurations for both techniques are summarized in the Tables 1-2 respectively. The simulations were performed following the JVET CTC [2]. In addition, the current results have been estimated on top of CE3.1.2.1 [1] (see Tables 3-4).

More detail results can be found in the accompanying Excel tables.

## Table 1. Experimental results for multiplication-based DC mode without division (CTC)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.01% | 101% | 102% | 0.00% | 0.09% | -0.07% | 101% | 101% |
| Class A2 | 0.00% | 0.01% | -0.01% | 102% | 101% | 0.00% | 0.06% | -0.01% | 100% | 101% |
| Class B | -0.01% | -0.03% | 0.04% | 101% | 102% | -0.01% | 0.06% | 0.11% | 101% | 101% |
| Class C | 0.00% | -0.06% | -0.07% | 101% | 101% | 0.00% | 0.01% | 0.08% | 101% | 100% |
| Class E | 0.00% | -0.08% | -0.15% | 101% | 102% | -0.01% | 0.00% | -0.11% | 101% | 101% |
| Overall | 0.00% | -0.03% | -0.03% | 101% | 102% | -0.01% | 0.04% | 0.02% | 101% | 101% |
| Class D | -0.01% | 0.05% | 0.06% | 100% | 101% | 0.01% | 0.00% | 0.00% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.06% | -0.06% | 102% | 102% | 0.02% | 0.11% | 0.02% | 97% | 100% |
| Class A2 | -0.01% | -0.02% | -0.03% | 101% | 102% | 0.01% | 0.00% | 0.04% | 97% | 100% |
| Class B | 0.00% | 0.00% | 0.08% | 101% | 102% | 0.01% | -0.03% | -0.19% | 97% | 99% |
| Class C | -0.03% | -0.07% | -0.10% | 101% | 101% | 0.05% | 0.18% | -0.14% | 98% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.01% | -0.03% | -0.02% | 101% | 102% | 0.02% | 0.06% | -0.09% | 98% | 100% |
| Class D | 0.01% | -0.02% | -0.10% | 100% | 101% | 0.05% | -0.02% | -0.18% | 99% | 100% |

## Table 2. Experimental results for DC mode using reference samples only along with a longer side (CTC)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | -0.01% | 101% | 102% | 0.01% | 0.02% | -0.08% | 100% | 101% |
| Class A2 | -0.02% | -0.03% | 0.00% | 101% | 102% | 0.00% | -0.03% | -0.02% | 101% | 101% |
| Class B | -0.02% | -0.03% | 0.00% | 101% | 102% | -0.01% | 0.06% | 0.02% | 100% | 101% |
| Class C | -0.03% | -0.13% | -0.09% | 101% | 101% | -0.05% | -0.04% | -0.05% | 101% | 100% |
| Class E | -0.02% | 0.03% | -0.06% | 100% | 102% | -0.03% | 0.00% | -0.13% | 101% | 101% |
| Overall | -0.02% | -0.03% | -0.03% | 101% | 102% | -0.02% | 0.01% | -0.05% | 101% | 101% |
| Class D | -0.03% | 0.05% | -0.08% | 100% | 100% | -0.04% | 0.02% | 0.05% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.02% | 0.07% | 102% | 102% | 0.02% | 0.06% | 0.09% | 97% | 100% |
| Class A2 | -0.01% | 0.09% | 0.08% | 101% | 102% | 0.02% | -0.04% | 0.00% | 97% | 100% |
| Class B | -0.01% | 0.07% | -0.15% | 101% | 102% | 0.00% | 0.01% | 0.02% | 97% | 99% |
| Class C | -0.02% | 0.00% | -0.09% | 100% | 101% | 0.05% | 0.10% | -0.13% | 98% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.01% | 0.04% | -0.04% | 101% | 102% | 0.02% | 0.03% | -0.01% | 98% | 100% |
| Class D | 0.03% | -0.25% | -0.25% | 100% | 100% | 0.02% | -0.06% | 0.13% | 99% | 100% |

## Table 3. Experimental results for multiplication-based DC mode without division (over CE3.1.2.1)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over CE3.1.2.1-VTM-1.0 |  |  |  |  | Over CE3.1.2.1-BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | 0.03% | 100% | 100% | 0.00% | -0.02% | -0.04% | 100% | 100% |
| Class A2 | -0.01% | -0.07% | -0.04% | 100% | 100% | -0.03% | 0.02% | 0.04% | 100% | 100% |
| Class B | -0.02% | -0.04% | 0.03% | 100% | 100% | -0.02% | -0.01% | 0.08% | 100% | 100% |
| Class C | -0.02% | -0.08% | -0.06% | 100% | 100% | -0.01% | -0.12% | -0.01% | 100% | 100% |
| Class E | -0.01% | -0.16% | -0.20% | 100% | 100% | 0.00% | -0.07% | -0.14% | 101% | 100% |
| Overall | -0.01% | -0.07% | -0.04% | 100% | 100% | -0.01% | -0.04% | 0.00% | 100% | 100% |
| Class D | -0.03% | 0.04% | -0.15% | 100% | 100% | -0.01% | -0.24% | -0.09% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over CE3.1.2.1-VTM-1.0 |  |  |  |  | Over CE3.1.2.1-BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.17% | -0.11% | 100% | 100% | -0.02% | 0.07% | 0.02% | 100% | 100% |
| Class A2 | -0.01% | -0.09% | 0.01% | 100% | 100% | -0.01% | -0.06% | 0.04% | 100% | 100% |
| Class B | -0.02% | -0.05% | 0.14% | 100% | 100% | 0.01% | -0.23% | -0.26% | 100% | 100% |
| Class C | -0.02% | -0.18% | -0.11% | 100% | 100% | 0.01% | 0.04% | 0.14% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.02% | -0.12% | -0.01% | 100% | 100% | 0.00% | -0.06% | -0.04% | 100% | 100% |
| Class D | -0.05% | -0.15% | 0.25% | 100% | 100% | 0.03% | 0.07% | -0.22% | 100% | 100% |

## Table 4. Experimental results for DC mode using reference samples only along with a longer side (over CE3.1.2.1)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over CE3.1.2.1-VTM-1.0 |  |  |  |  | Over CE3.1.2.1-BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.04% | 0.01% | 100% | 100% | 0.00% | -0.09% | -0.06% | 99% | 100% |
| Class A2 | -0.03% | -0.11% | -0.03% | 100% | 100% | -0.02% | -0.07% | 0.03% | 100% | 100% |
| Class B | -0.03% | -0.05% | 0.00% | 100% | 100% | -0.03% | -0.01% | -0.01% | 99% | 100% |
| Class C | -0.05% | -0.14% | -0.07% | 100% | 100% | -0.06% | -0.17% | -0.15% | 100% | 100% |
| Class E | -0.03% | -0.05% | -0.10% | 100% | 100% | -0.03% | -0.07% | -0.16% | 100% | 100% |
| Overall | -0.03% | -0.08% | -0.04% | 100% | 100% | -0.03% | -0.08% | -0.07% | 100% | 100% |
| Class D | -0.05% | 0.03% | -0.29% | 100% | 99% | -0.05% | -0.21% | -0.04% | 99% | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over CE3.1.2.1-VTM-1.0 |  |  |  |  | Over CE3.1.2.1-BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.13% | 0.02% | 100% | 100% | -0.02% | 0.02% | 0.09% | 100% | 100% |
| Class A2 | -0.01% | 0.02% | 0.12% | 100% | 100% | -0.01% | -0.10% | 0.00% | 100% | 100% |
| Class B | -0.03% | 0.01% | -0.10% | 100% | 100% | 0.00% | -0.19% | -0.05% | 100% | 100% |
| Class C | -0.01% | -0.10% | -0.10% | 100% | 99% | 0.01% | -0.04% | 0.15% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.02% | -0.04% | -0.03% | 100% | 100% | 0.00% | -0.09% | 0.04% | 100% | 100% |
| Class D | -0.03% | -0.38% | 0.10% | 100% | 99% | 0.00% | 0.03% | 0.09% | 100% | 101% |

# References

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” April 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, JVET-B1010, “JVET common test conditions and software reference configurations”, April 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).