| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0124-v1 |
| --- | --- |

| Title: | CE4: Adaptive three and four parameter motion model (Test 4.1.8) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo, Masaru Ikeda, Teruhiko Suzuki 2-10-1 Osaki, Shinagawa-ku, Tokyo Japan | Tel: Email: | +81-50-3750-2744 kenji.kondo@sony.com, teruhiko.s@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes to add motion models that are a rotation, scaling and Affine for an inter prediction. The techniques are studied as test number 1.8 in CE4. This contribution reports the test results as follow the test condition of CE4. In tool on case 1 (VTM as reference), simulation results reportedly show that the proposed technique achieves 3.19% and 2.15% BD rate reduction for random access (RA) main 10 and low delay B (LB) Main10 configurations respectively. In tool off case 2 (BMS-Affine as reference), simulation results reportedly show that the techniques achieves 2.06% and 2.14% for RA and LB Main 10.

Introduction

This technique was proposed in JVET Call for Proposal [1] and it was planned to study in CE4 [2]. In the CE4, the proposed techniques are categorized and tested in CE4.1 that is Affine motion compensation. This contribution reports test results.

Proposed method

In JEM software, the simplified affine transform is used in motion compensation. It can improve the quality of inter prediction but it increases the overhead as a motion information. To reduce the overhead and find better trade-off, this technique adds two motion modes which are a rotation and a scaling.

An encoder can choose four modes which are a conventional translate, scaling, rotation and Affine. A number of parameters are 2 (one motion vector), 3, 3 and 4 (two MVs) for translate mode, scaling mode, rotation mode and Affine mode.

As shown in the below figure, the motion model index is defined and it is sent to the decoder. The number in parenthesizes shows the number of parameters of differential motion vector that are sent to a decoder.

Scaling mode

As mentioned above, a scaling mode is added in this technique. If the test sequence has zoom in or zoom out, the inter prediction needs a scaling translate. This scaling mode can predict the zoom in/out and it can reduce the number of parameters of Affine mode.

When the scaling mode is chosen, three parameters are used to predict as shown in the below figure. It is one motion vector (v0x, v0y) and one element (v1x). For the prediction process, the frame work is used the same as for Affine transform in JEM software. The parameter (v1y) is implicitly assumed as the same as (v0y).

Rotation mode

As mentioned above, a rotation mode is added in this technique. If the motion in a test sequence has rotation, the inter prediction can use a rotational translate. This rotation mode can predict the rotation and it can reduce the number of parameters than affine mode.

When the rotation mode is chosen, three parameters are used to predict as shown in the below figure. It is one motion vector (v0x, v0y) and one element (v1y). For the prediction process, the frame work is used the same as for Affine transform in JEM software. The parameter (v1x) is implicitly calculated by below equation. Here, W is width in CU size.
v1x=v0x-v1y-v0y22W

Experimental results

The software is implemented based on BMS-1.0. And it is committed in SVN server [3]. Three tests are prepared to separately report as below.

- Test –A: switch a scaling mode (3-paramers) or Affine mode (4-parameters)

- Test –B: switch a rotation mode (3-parameters) or Affine mode

- Test –C: combine above A and B, switch a scaling mode, a rotation mode or Affine mode.

Each testing item is evaluated using BD rates, encoding time, decoding time according to Common Test Conditions (CTC) in [4]. As following a test condition of CE4, two test conditions are used that is Tool on case 1 (VTM as reference) and Tool off case 2 (BMS-Affine as reference).

Simulation environment

For the simulations, same platform machines as below in PC cluster were used.

- Encode

  - OS: CentOS 6.9

  - Compiler: g++ (GCC) 6.3.1

  - CPU: Intel Core i7-7700K 4.2GHz

    - SIMD: AVX2 instruction is enabled.

  - Memory: 32 GB

- Decode

  - OS: CentOS 6.9

  - Compiler: g++ (GCC) 6.3.1

  - CPU: Intel Core i5-2500K 3.3 GHz

    - SIMD: AVX instruction is enabled.

  - Memory: 24 GB

For a runtime measurement, the user time in the output log was used. For a random access case and an all intra case, a parallel encoding is used. To measure encoding time in parallel encoding, the total of all user time were calculated. Because the encoding time for each frame was elapsed time (not user time) and it is not reliable, the encode time for first I-frame was not subtracted. So the encoding time for first I frame is duplicated. For decoding time, it is reliable.

Test 1.8-A

The results of test 1.8-A are summarized in the Table 1 and Table 2.

Table 1 : Over VTM-1.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.93% | -1.57% | -1.73% | 175% | 108% |
| Class A2 | -6.28% | -5.05% | -4.79% | 185% | 126% |
| Class B | -3.31% | -2.20% | -2.32% | 185% | 113% |
| Class C | -1.32% | -0.79% | -0.79% | 179% | 110% |
| Class E |  |  |  |  |  |
| Overall | -3.10% | -2.27% | -2.29% | 181% | 114% |
| Class D | -1.93% | -1.29% | -1.45% | 199% | 110% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.53% | -1.71% | -1.74% | 238% | 114% |
| Class C | -1.52% | -0.71% | -0.70% | 208% | 111% |
| Class E | -2.23% | -1.43% | -1.71% | 359% | 115% |
| Overall | -2.12% | -1.31% | -1.39% | 252% | 113% |
| Class D | -3.19% | -2.23% | -2.18% | 217% | 117% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 2 : Over BMS-1.0 - Affine

|  | Over BMS-1.0 - Affine |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.11% | -0.84% | -0.86% | 118% | 101% |
| Class A2 | -3.65% | -2.60% | -2.56% | 121% | 104% |
| Class B | -2.20% | -1.55% | -1.56% | 119% | 101% |
| Class C | -1.13% | -0.74% | -0.82% | 115% | 100% |
| Class E |  |  |  |  |  |
| Overall | -1.99% | -1.40% | -1.42% | 118% | 101% |
| Class D | -1.71% | -1.54% | -1.46% | 113% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Over BMS-1.0 - Affine |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.58% | -1.73% | -1.96% | 144% | 105% |
| Class C | -1.57% | -1.19% | -1.28% | 136% | 104% |
| Class E | -2.01% | -1.76% | -1.75% | 193% | 104% |
| Overall | -2.10% | -1.56% | -1.68% | 152% | 104% |
| Class D | -3.21% | -3.21% | -3.19% | 134% | 105% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Test 1.8-B

The results of test 1.8-B are summarized in the Table 3 and Table 4.

Table 3 : Over VTM-1.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.04% | -1.74% | -1.72% | 175% | 108% |
| Class A2 | -5.94% | -4.79% | -4.54% | 185% | 122% |
| Class B | -3.38% | -2.25% | -2.45% | 185% | 112% |
| Class C | -1.30% | -0.76% | -0.79% | 178% | 110% |
| Class E |  |  |  |  |  |
| Overall | -3.07% | -2.26% | -2.28% | 181% | 112% |
| Class D | -1.81% | -1.17% | -1.30% | 198% | 108% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.56% | -1.61% | -1.47% | 238% | 113% |
| Class C | -1.46% | -0.57% | -0.67% | 206% | 111% |
| Class E | -2.28% | -1.66% | -2.01% | 363% | 114% |
| Overall | -2.12% | -1.28% | -1.34% | 252% | 113% |
| Class D | -2.91% | -1.96% | -1.64% | 216% | 116% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 4 : Over BMS-1.0 - Affine

|  | Over BMS-1.0 - Affine |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.14% | -0.89% | -0.93% | 118% | 100% |
| Class A2 | -3.47% | -2.53% | -2.52% | 121% | 104% |
| Class B | -2.26% | -1.57% | -1.62% | 119% | 101% |
| Class C | -1.10% | -0.75% | -0.81% | 115% | 100% |
| Class E |  |  |  |  |  |
| Overall | -1.97% | -1.41% | -1.44% | 118% | 101% |
| Class D | -1.52% | -1.19% | -1.14% | 113% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Over BMS-1.0 - Affine |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.61% | -1.76% | -1.88% | 144% | 104% |
| Class C | -1.44% | -1.19% | -1.23% | 136% | 103% |
| Class E | -2.05% | -2.36% | -1.77% | 192% | 104% |
| Overall | -2.08% | -1.72% | -1.64% | 152% | 104% |
| Class D | -2.93% | -2.40% | -2.35% | 134% | 104% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Test 1.8-C

The results of test 1.8-C are summarized in the Table 5 and Table 6.

Table 5 : Over VTM-1.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.03% | -1.62% | -1.70% | 212% | 110% |
| Class A2 | -6.35% | -4.98% | -4.78% | 227% | 128% |
| Class B | -3.44% | -2.31% | -2.42% | 227% | 113% |
| Class C | -1.37% | -0.87% | -0.79% | 215% | 109% |
| Class E |  |  |  |  |  |
| Overall | -3.19% | -2.32% | -2.31% | 221% | 114% |
| Class D | -1.92% | -1.22% | -1.30% | 245% | 110% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.60% | -1.71% | -2.03% | 304% | 114% |
| Class C | -1.52% | -0.66% | -0.52% | 257% | 111% |
| Class E | -2.26% | -1.68% | -2.14% | 492% | 115% |
| Overall | -2.15% | -1.35% | -1.55% | 324% | 114% |
| Class D | -3.14% | -1.72% | -1.97% | 271% | 117% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 6 : Over BMS-1.0 - Affine

|  | Over BMS-1.0 - Affine |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.16% | -0.77% | -0.94% | 127% | 101% |
| Class A2 | -3.72% | -2.69% | -2.64% | 132% | 104% |
| Class B | -2.30% | -1.62% | -1.65% | 129% | 101% |
| Class C | -1.17% | -0.79% | -0.88% | 123% | 100% |
| Class E |  |  |  |  |  |
| Overall | -2.06% | -1.44% | -1.50% | 127% | 102% |
| Class D | -1.71% | -1.46% | -1.31% | 120% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Over BMS-1.0 - Affine |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.66% | -1.72% | -1.98% | 165% | 105% |
| Class C | -1.54% | -1.08% | -1.10% | 152% | 103% |
| Class E | -2.09% | -2.51% | -2.33% | 238% | 104% |
| Overall | -2.14% | -1.71% | -1.77% | 176% | 104% |
| Class D | -3.29% | -2.54% | -3.27% | 149% | 105% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Conclusion

This contribution reports simulation result of CE4 test 1.8, which evaluates 3 / 4 parameter switching.  To reduce the parameter, this proposed technique adds two motion models which are scaling and rotation. Three test cases were tested that a 3-para (scaling) / 4-para (Affine), 3-para (rotation) / 4-para and 3-para (scaling and rotation) / 4 para switching as test names A, B and C. Test-C results show that the proposed method can provide 3.19% and 2.15% BD rate reduction, 221% and 324% encoding time, 114% and 114% decoding time for RA and LB in tool on case 1.

References

[1] T. Suzuki, M. Ikeda and K. Sharman, “Description of SDR and HDR video coding technology proposal by Sony” JVET-J0028, JVET 10th Meeting: San Diego, US, April 2018.

[2] Haitao Yang, Shan Liu and Kai Zhang, “Description of CE4: Inter prediction and motion vector coding” JVET-J1024, JVET 10th Meeting: San Diego, US, April 2018.

[3] https://jvet.hhi.fraunhofer.de/svn/svn_VCC_Software_BMS/branches/CE/J_SanDiego/CE4/CE4-1.8

[4] Jill Boyce, K. Suehring, X. Li, and Vadim Seregin, “JVET common test conditions and software reference configurations” JVET-J1010, April 2018.

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).