| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0131-v1 |
| --- | --- |

| Title: | CE13: Modified Cubemap Projection in JVET-J0019 (Test 5) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ya-Hsuan Lee, Jian-Liang Lin, Shen-Kai Chang, Chi-Cheng Ju No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Jian-Liang Lin +886-3-5670766 ext. 25250 {louise.lee, jl.lin, shenkai.chang, cc.ju} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the experiment results of test 5 in CE13: the Modified Cubemap Projection (MCP) with and without padding. Compared to the VTM anchor, the simulation results reportedly show that, in terms of end-to-end WS-PSNR-Y, WS-PSNR-U, and WS-PSNR-V, MCP without padding achieves the average -11.73%, -7.42%, and -7.72% BD-rate reduction, respectively. For MCP with padding, the results report the average -11.56%, -7.58%, and -8.01% BD-rate reduction in terms of end-to-end WS-PSNR-Y, WS-PSNR-U, and WS-PSNR-V, respectively, while the seam artifact is significantly reduced.

# Introduction

In the last meeting, several new projection formats were proposed in response to the “Joint Call for Proposals on Video Compression with Capability beyond HEVC” [1] for the 360° video category. To investigate the performance of these projections, a CE [2] is held with several tests including MCP [3] with and without padding.

MCP is based on the cubemap projection format adopting the compact 3x2 layout with a discontinuous edge in the middle as shown in Fig. 1. MCP introduces a coordinate mapping function to improve the sampling distribution compared to the conventional cubemap projection (CMP). For the Front, Back, Right, and Left faces, MCP applies the equal-angular mapping function on u-axis while on v-axis, the proposed mapping function is applied as follows:

| 2D-to-3D: | fv=v1+0.4(1-u2)(1-v2) | (1) |
| --- | --- | --- |
| 3D-to-2D: | gv=v , if t=01-1-4t(v-t)2t, otherwise , t=0.4v(gu2-1) | (2) |

For the Top and Bottom faces, the equal-angular mapping function is applied on both u-axis and v-axis to keep continuity with other faces as shown in Fig. 2. By this modification, MCP has more uniform sampling distribution, and can achieve better compression efficiency.

Fig. 1 An image of MCP in 3x2 layout

Fig. 2 Mapping function on u-axis and v-axis for each face of MCP

# Padding on Modified Cubemap Projection

To reduce the seam artifact caused by the discontinuous edge and frame boundary in the cubemap layout, padding is generated by the geometry mapping and applied on the boundary of the two 3x1 face rows, [Right, Front, Left] and [Top, Back, Bottom], as shown in Fig. 3. Each face with padding region is then shrunk by a scaling factor, as listed in Table I, to make the frame size remained the same.

Fig. 3 Padding is applied on the boundary of the top face row composed of the Right, Front, and Left faces, and the bottom face row composed of the Top, Back, and Bottom faces.

Table I Scaling factor for each face

| Face | Axis | Scaling factor |
| --- | --- | --- |
| Front | u | 1 |
|  | v | face height-8face height |
| Back | u | face width-8face width |
|  | v | 1 |
| Right | u | face width-4face width |
|  | v | face height-8face height |
| Left | u | face width-4face width |
|  | v | face height-8face height |
| Top | u | face width-4face width |
|  | v | face height-8face height |
| Bottom | u | face width-4face width |
|  | v | face height-8face height |

Before converting back to ERP or generating viewports, blending is applied with a distance-based weighting function as shown below to update a target pixel, A, by blending with its corresponding pixel, A’, which is found by the mapping functions, i.e.,

|  | Aupdated=A∙d+A'∙(D-d)D | (3) |
| --- | --- | --- |

where D is the width of pixels to be blended on the boundary, and d is the distance from the target pixel A to the boundary.

However, pixels near the discontinuous edge may be dyed by another face, and blending with the dyed pixels is not desirable. For this reason, if the distance between the corresponding pixel and the discontinuous edge is less than 4 pixels, blending is not allowed.

# Experiment Results

The experiments are conducted on the 360Lib-6.0 [4] and the BMS-1.0 [5] followed by the JVET common test conditions (CTC) and evaluation procedures [6]. The configuration settings for the VTM-1.0 [7] and the BMS-1.0 are tested. Following the CTC, the frame size is set to 3552x2368 with the face size of 1184x1184. Compared to PERP, Table II demonstrates the average BD-rate reduction in terms of end-to-end WS-PSNR for MCP without padding under the VTM-1.0 and BMS-1.0 configuration settings. And Table III shows the average BD-rate reduction compared to CMP.

Table II BD-rate performance for MCP w/o padding compared to PERP

|  | VTM-1.0 |  |  | BMS-1.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -19.91% | -21.26% | -21.11% | -21.08% | -22.17% | -22.43% |
| ChairLift | -26.58% | -20.21% | -20.58% | -22.53% | -13.35% | -16.73% |
| KiteFlite | -10.57% | -2.49% | -4.51% | -11.28% | -3.48% | -7.14% |
| Harbor | -11.78% | -4.38% | -3.73% | -12.75% | -3.80% | -4.06% |
| Trolley | -9.12% | 1.21% | -3.96% | -10.37% | 3.13% | -3.29% |
| GasLamp | -6.37% | 2.53% | 8.98% | -7.58% | 1.71% | 7.92% |
| Balboa | -9.95% | -7.69% | -9.24% | -8.85% | -7.73% | -8.66% |
| Broadway | -7.22% | -6.94% | -6.13% | -7.14% | -7.12% | -5.44% |
| Landing2 | -3.86% | -4.60% | -6.89% | -3.71% | -3.73% | -7.00% |
| BranCastle2 | -11.98% | -10.35% | -9.98% | -12.71% | -11.13% | -9.93% |
| Class S1 | -14.06% | -7.43% | -7.49% | -14.26% | -6.33% | -7.62% |
| Class S2 | -8.25% | -7.40% | -8.06% | -8.10% | -7.43% | -7.76% |
| Overall | -11.73% | -7.42% | -7.72% | -11.80% | -6.77% | -7.68% |

Table III BD-rate performance for MCP w/o padding compared to CMP

|  | VTM-1.0 |  |  | BMS-1.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -11.86% | -10.31% | -10.69% | -12.26% | -11.16% | -11.79% |
| ChairLift | -9.56% | -6.63% | -6.44% | -9.13% | -7.32% | -5.52% |
| KiteFlite | -6.69% | -2.80% | -2.42% | -7.29% | -3.86% | -2.76% |
| Harbor | -12.82% | -2.74% | -2.93% | -13.50% | -2.89% | -2.72% |
| Trolley | -7.38% | -2.68% | -3.23% | -8.09% | -2.46% | -3.18% |
| GasLamp | -8.32% | -4.20% | -4.51% | -8.88% | -4.46% | -4.84% |
| Balboa | -5.16% | -7.34% | -7.79% | -5.89% | -9.36% | -8.91% |
| Broadway | -7.23% | -6.23% | -6.47% | -8.55% | -8.30% | -7.46% |
| Landing2 | -8.29% | -9.48% | -10.52% | -7.28% | -8.88% | -10.50% |
| BranCastle2 | -10.39% | -10.69% | -10.83% | -9.08% | -10.17% | -9.47% |
| Class S1 | -9.44% | -4.89% | -5.04% | -9.86% | -5.36% | -5.14% |
| Class S2 | -7.77% | -8.43% | -8.91% | -7.70% | -9.18% | -9.09% |
| Overall | -8.77% | -6.31% | -6.58% | -8.99% | -6.89% | -6.72% |

For MCP with padding, the average BD-rate reduction in terms of end-to-end WS-PSNR, compared to PERP and CMP, is reported in Table IV and Table V, respectively.

Table IV BD-rate performance for MCP w/ padding compared to PERP

|  | VTM-1.0 |  |  | BMS-1.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -19.92% | -22.01% | -21.86% | -21.15% | -22.96% | -23.37% |
| ChairLift | -26.69% | -19.96% | -20.59% | -22.61% | -13.55% | -16.98% |
| KiteFlite | -10.25% | -2.52% | -4.51% | -11.00% | -3.66% | -7.15% |
| Harbor | -11.61% | -4.75% | -4.91% | -12.58% | -5.73% | -5.99% |
| Trolley | -8.66% | 1.54% | -4.16% | -9.96% | 2.67% | -3.50% |
| GasLamp | -6.00% | 2.41% | 9.29% | -7.16% | 0.68% | 7.57% |
| Balboa | -9.98% | -7.61% | -9.17% | -8.89% | -8.04% | -8.35% |
| Broadway | -7.33% | -6.83% | -6.15% | -7.23% | -7.31% | -6.08% |
| Landing2 | -3.66% | -5.92% | -8.05% | -3.52% | -5.21% | -7.99% |
| BranCastle2 | -11.56% | -10.13% | -9.95% | -12.31% | -11.10% | -10.04% |
| Class S1 | -13.85% | -7.55% | -7.79% | -14.07% | -7.09% | -8.24% |
| Class S2 | -8.13% | -7.62% | -8.33% | -7.99% | -7.91% | -8.11% |
| Overall | -11.56% | -7.58% | -8.01% | -11.64% | -7.42% | -8.19% |

Table V BD-rate performance for MCP w/ padding compared to CMP

|  | VTM-1.0 |  |  | BMS-1.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -11.86% | -11.16% | -11.55% | -12.32% | -12.04% | -12.81% |
| ChairLift | -9.68% | -6.34% | -6.44% | -9.21% | -7.54% | -5.80% |
| KiteFlite | -6.33% | -2.82% | -2.41% | -6.97% | -4.05% | -2.78% |
| Harbor | -12.63% | -3.14% | -4.12% | -13.32% | -4.89% | -4.72% |
| Trolley | -6.88% | -2.37% | -3.39% | -7.64% | -2.91% | -3.41% |
| GasLamp | -7.95% | -4.33% | -4.24% | -8.45% | -5.43% | -5.16% |
| Balboa | -5.17% | -7.26% | -7.71% | -5.92% | -9.66% | -8.60% |
| Broadway | -7.32% | -6.13% | -6.50% | -8.62% | -8.50% | -8.08% |
| Landing2 | -8.06% | -10.66% | -11.58% | -7.07% | -10.19% | -11.42% |
| BranCastle2 | -9.94% | -10.47% | -10.80% | -8.62% | -10.13% | -9.57% |
| Class S1 | -9.22% | -5.03% | -5.36% | -9.65% | -6.14% | -5.78% |
| Class S2 | -7.62% | -8.63% | -9.15% | -7.56% | -9.62% | -9.42% |
| Overall | -8.58% | -6.47% | -6.87% | -8.81% | -7.53% | -7.23% |

# Conclusion

In this contribution, the performance of MCP with and without padding are investigated. The experimental data reports that MCP can reach higher BD-rate reduction than PERP and CMP. For subjective quality testing, the seam artifact can be effectively reduced with padding while still keeping high BD-rate performance.

# References

- A. Segall, V. Baroncini, J. Boyce, J. Chen, and T. Suzuki, “Joint Call for Proposals on Video Compression with Capability beyond HEVC,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-H1002, Oct. 2017, Macau, CN.

- P. Hanhart, and J.-L. Lin, “Description of Core Experiment 13: Projection formats,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1033, Apr. 2018, San Diego, USA.

- J.-L. Lin, Y.-H. Lee, C.-H. Shih, S.-Y. Lin, H.-C. Lin, S.-K. Chang, P. Wang, L. Liu, and C.-C. Ju, “Description of 360° video coding technology proposal by MediaTek,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J0019, Apr. 2018, San Diego, USA.

- 360Lib, https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/

- VTM Benchmark Set, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/

- P. Hanhart, J. Boyce, K. Choi, “JVET common test conditions and evaluation procedures for 360° video”, JVET-J1012, Apr. 2018, San Diego, USA.

- Versatile Video Coding Test Model, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_VTM/tags/

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).