| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0136-v1 |
| --- | --- |

| Title: | CE1: Redundant partition prevention with redundancy existence check (Test 3.0.6) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao, Haitao Yang, Jianle Chen | Email: | yin.zhao@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the results of adding a partition redundancy existence check for removing redundant ways to generate a quad-tree-like partitioning pattern, which fixes a bug that a node can never be split into a quad-tree-like pattern in some cases. Simulation results reportedly show on average 0.0% and 0.0% luma BD-rate gain for RA and LB configuration over VTM1.0, and 0.0% and 0.1% luma BD-rate gain for RA and LB configuration over BMS1.0.

Proposed method

There are three ways to split a node A into four child nodes Bi (i=0,..,3) in a quad-tree-like pattern, as shown in Figure 1.

(a) QT                  (b) VBT-HBT-HBT (VHH)    (c) HBT-VBT-VBT (HVV)

Figure 1 Three ways to generate a quad-tree-like pattern (node A may be a QT node or a MTT node).

In BMS1.0, for non-I slice, the following restriction rules apply:

Rule 1: HVV split combination in Figure 1(c) is not allowed,

Rule 2: VHH split combination in Figure 1(b) is not allowed only if QT split in Figure 1(a) is allowed.

Due to MTT split restriction (either the above rule or another rule that the center child node of a ternary tree split cannot use binary tree split in the same direction), node A may not be allowed to be split by either QT or VBT split in some cases, as shown in Figure 2, and thus node A cannot be split into a quad-tree-like pattern in these cases. This situation only occurs when MaxMTTDepth is greater than 2.

Figure 2 Two cases that a node A is not allowed to use VBT split as well as QT split. Left: node A is a center child node of a Vertical Ternary Tree (VTT) split. Right: node A is a bottom child node of a HBT split, while the top child node chooses a VBT split.

To fix this “bug”, a condition check is performed before disabling the HVV combination for node A, that is, the Rule 1 is modified from

HVV split combination in Figure 1(c) is not allowed

to

HVV split combination in Figure 1(c) is not allowed only if VBT split is allowed for node A.

Experimental results

The proposed method is tested under VVC CTC [1], and the results are listed in Table 1. Since the redundancy removal rule is applied on non-I slice, results under AI configuration are identical to the anchor. On average, there is 0.0% and 0.0% luma BD-rate gain for RA and LB configuration over VTM1.0, and 0.0% and 0.1% gain for RA and LB configuration over BMS1.0. It is also observed that 0.3% luma BD-rate gain is achieved on “Johnny” and “KristenAndSara” in LB configuration over BMS1.0.

Table 1 Coding performance of the proposed method

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.09% | 0.05% | 101% | 101% | 0.04% | 0.04% | -0.04% | 101% | 100% |
| Class A2 | -0.02% | -0.02% | 0.03% | 100% | 101% | 0.00% | -0.03% | 0.04% | 101% | 101% |
| Class B | -0.01% | -0.03% | 0.09% | 101% | 101% | -0.02% | -0.08% | -0.08% | 101% | 100% |
| Class C | -0.01% | -0.04% | 0.00% | 101% | 102% | 0.05% | -0.04% | -0.14% | 101% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.01% | -0.01% | 0.05% | 101% | 101% | 0.01% | -0.04% | -0.06% | 101% | 100% |
| Class D | 0.02% | 0.14% | -0.01% | 101% | 102% | 0.03% | -0.07% | -0.10% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.39% | 100% | 100% | -0.03% | -0.15% | 0.04% | 101% | 99% |
| Class C | 0.05% | -0.14% | 0.12% | 100% | 100% | -0.02% | 0.00% | -0.06% | 100% | 100% |
| Class E | 0.01% | 0.11% | 0.21% | 100% | 99% | -0.20% | 0.18% | -0.12% | 101% | 100% |
| Overall | 0.02% | -0.02% | 0.26% | 100% | 100% | -0.07% | -0.02% | -0.03% | 101% | 100% |
| Class D | -0.06% | -0.29% | -0.42% | 100% | 101% | -0.02% | 0.14% | 0.07% | 100% | 99% |

Conclusions

This contribution identifies a bug that a node is not allowed to be split into a quad-tree-like pattern in two cases due to partitioning redundancy prevention in BMS1.0. As a bugfix, a condition check is added before disabling the HVV split combination.

We propose to adopt this change into the test model.

References

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, April 2018.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).