| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0137-v1 |
| --- | --- |

| Title: | CE1: On configuration of the MTT (Test 3.0.8 and Test 3.0.9) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao, Haitao Yang, Jianle Chen | Email: | yin.zhao@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the results of configuring the MTT with different levels of encoder complexity by 1) limiting split direction for nodes of aspect ratio equal to or greater than 1:4, namely “aspect ratio limit”, or by 2) a mode that child nodes of a Ternary Tree (TT) split cannot further split, namely “TT on leaf” mode. This contribution also compares the trade-off of the proposed configuration with disabling TT and setting MaxMTTDepth as 2.

For “aspect ratio limit”, simulation results reportedly show on average 0.1%, 0.4% and 0.4% luma BD-rate loss for AI, RA and LB configuration over VTM1.0, with 6%, 16% and 14% EncT reduction, and 0.0%, 0.4%, 0.4% luma BD-rate loss for AI, RA and LB configuration over BMS1.0, with 6%, 12% and 11% EncT reduction.

For “TT on leaf”, simulation results reportedly show on average 0.6%, 1.2% and 1.2% luma BD-rate loss for AI, RA and LB configuration over VTM1.0, with 40%, 37% and 39% EncT reduction, and 0.5%, 1.2% and 1.1% luma BD-rate loss for RA and LB configuration over BMS1.0, with 38%, 39% and 37% EncT reduction. Compared with recursive TT under VTM configuration, “TT on leaf” mode achieves around 60% BD-rate gain with 1/3 encoding time increase.

Introduction

Finding the optimal split mode of a MTT node is a search problem. Compression efficiency may increase as the search space enlarges, while the encoding complexity and partition signalling cost may also become higher. The search space of the current MTT structure is considerably larger than H.265/HEVC, since a QT leaf node becomes a MTT tree. It may be desirable for low-complexity encoders that the partitioning search space can be narrowed down to different levels, so that there is no penalty from partition signalling when an encoder always does not try some split modes.

In BMS1.0, the encoder complexity of the MTT is affected mostly by two parameters: 1) the MaxMTTDepth and 2) whether TT is allowed in MTT. We tested the configuration of MaxMTTDepth = 2 (for both I and P/B) and that of disabling TT in MTT, as listed in Table 1.

Table 1 Coding performance of MaxMTTDepth =2 and MTT=0

| MaxMTTDepth = 2 (--MaxBTDepthISliceL=2 --MaxBTDepth=2) | Over VTM1.0 |  |  |  |  | Over BMS1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | 1.20% | 2.22% | 2.37% | 49% | 100% |  |  |  |  |  |
| RA | 1.95% | 3.25% | 3.27% | 53% | 100% |  |  |  |  |  |
| LB | 1.49% | 2.77% | 2.75% | 56% | 100% |  |  |  |  |  |
| Disabling TT (--MTT=0) | Over VTM1.0 |  |  |  |  | Over BMS1.0 |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | 1.51% | 1.75% | 1.86% | 42% | 100% |  |  |  |  |  |
| RA | 2.85% | 3.90% | 3.98% | 42% | 100% |  |  |  |  |  |
| LB | 3.02% | 4.39% | 4.76% | 41% | 100% |  |  |  |  |  |

The two settings reduce EncT by around 50% and 60%, respectively, but also compromise compression efficiency significantly. Therefore, this proposal studies two other ways to limit the partitioning search space of the MTT, and both of them achieve a better trade-off in terms of EncT and BD-rate than either configuring MaxMTTDepth = 2 or disabling TT. Together with the existing search parameters, encoder complexity can be configured in finer granularity, as illustrated in Figure 1 in Section 4.

Proposed configuration parameter 1: aspect ratio limit

It is observed at decoder that the probability of a node (e.g., aspect ratio 1:4) to be split into child nodes of higher aspect ratio (e.g., 1:8) is lower than that of the node to be split into child nodes of lower aspect ratio (e.g., 1:2). Therefore, limiting the search direction (horizontal or vertical) could be a straightforward way to shrink the search space with a good efficiency-complexity trade-off.

We tested aspect ratio limit as 1:2, 1:4 and 1:8, i.e., the nodes with aspect ratio lower than the threshold 1:2/1:4/1:8 can split in both direction; otherwise, the nodes can only split in one direction that reduces aspect ratio. The coding results are summarized in Table 2.

From Table 2, the coding performance of aspect ratio limit 1:8 is similar to the anchor. All nodes’ aspect ratio is no larger than 1:16 when aspect ratio limit 1:8 is used (because a 1:16 node can be generated from a 1:4 node with a TT split), and this experiment indicates that the node with aspect ratio larger than 1:16 is probably not helpful in coding efficiency. Therefore, VVC standard may limit aspect ratio of all nodes within 1:16, which can reduce the size of some aspect ratio related look-up tables and make implementation easier.

In addition, the aspect ratio limit 1:4 introduces 0.13%/0.42%/0.43% luma BD-rate loss with 6%/16%/14% EncT reduction, which seems a better trade-off than the current two parameters in BMS, as shown in Figure 1. The coding performance of aspect ratio limit 1:2 provides a trade-off slightly worse than disabling TT, as shown in Figure 1.

Therefore, this proposal recommend to add an aspect_ratio_limit_flag in SPS, with value 1 specifying that a node whose aspect ratio is smaller than 1:4 can split in both directions, otherwise the node can only split in one direction that reduces aspect ratio.

Table 2 Coding performance of aspect ratio (AR) limit 1:2, 1:4 and 1:8

| AR 1:8 | Over VTM1.0 |  |  |  |  | Over BMS1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | 0.00% | 0.00% | 0.00% | 100% | 102% | 0.00% | 0.00% | 0.00% | 100% | 102% |
| RA | 0.03% | 0.07% | 0.10% | 97% | 100% | 0.04% | 0.08% | 0.06% | 99% | 100% |
| LB | 0.03% | 0.23% | 0.17% | 97% | 100% | -0.01% | -0.01% | -0.06% | 99% | 100% |
| AR 1:4 | Over VTM1.0 |  |  |  |  | Over BMS1.0 |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | 0.13% | 0.68% | 0.67% | 94% | 102% | 0.11% | 0.63% | 0.58% | 94% | 102% |
| RA | 0.42% | 0.97% | 0.90% | 84% | 100% | 0.42% | 1.01% | 0.79% | 88% | 100% |
| LB | 0.43% | 1.38% | 1.55% | 86% | 100% | 0.37% | 0.99% | 1.01% | 89% | 100% |
| AR 1:2 | Over VTM1.0 |  |  |  |  | Over BMS1.0 |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | 0.92% | 2.63% | 2.66% | 66% | 100% |  |  |  |  |  |
| RA | 2.22% | 3.78% | 3.69% | 57% | 100% |  |  |  |  |  |
| LB | 2.42% | 4.43% | 4.65% | 58% | 100% |  |  |  |  |  |

Proposed configuration parameter 2: TT on leaf mode

The “TT on leaf” mode specifies that the child nodes from a TT split cannot further split, which configures the MTT as a binary tree cascaded with a ternary tree of maxTTDepth = 1.

Coding performance of “TT on leaf” is provided in Table 3. “TT on leaf” mode reduces around 39% EncT with 0.6%/1.2%/1.2% luma BD-rate loss in AI/RA/LB, while disabling TT reduces around 59% EncT with 1.5%/2.9%/3.0% luma BD-rate loss in AI/RA/LB.

The “TT on leaf” mode is signaled in SPS as two flags, one for I slice and one for P/B slice, namely tt_on_leaf_i_slice_flag and tt_on_leaf_non_i_slice_flag.

Table 3 Coding performance of TT disabled and “TT on leaf” mode

| “TT on Leaf” | Over VTM1.0 |  |  |  |  | Over BMS1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | 0.63% | 0.88% | 0.90% | 60% | 101% | 0.49% | 0.98% | 1.00% | 62% | 102% |
| RA | 1.18% | 1.75% | 1.80% | 63% | 100% | 1.17% | 1.95% | 1.86% | 61% | 101% |
| LB | 1.21% | 2.02% | 2.13% | 61% | 100% | 1.08% | 1.93% | 1.76% | 63% | 100% |
| TT disabled (--MTT=0) | Over VTM1.0 |  |  |  |  | Over BMS1.0 |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | 1.51% | 1.75% | 1.86% | 42% | 100% |  |  |  |  |  |
| RA | 2.85% | 3.90% | 3.98% | 42% | 100% |  |  |  |  |  |
| LB | 3.02% | 4.39% | 4.76% | 41% | 100% |  |  |  |  |  |

More discussion

The BD-rate loss and encoding time reduction of different MTT configurations in AI/RA/LB over VTM1.0 are plotted in Figure 1. It is indicated from the figure that the two new parameters create better trade-off points, located between full-complexity VTM1.0 and lower-complexity VTM1.0 with MaxMTTDepth2.

“1% BD-rate loss for 2x speed-up” may be considered a criterion for a good encoder fast algorithm. The trade-off points from the two proposed parameters are closer to this criterion (plotted as the dashed blue lines in Figure 1) than MaxMTTDepth2 or TT disabled.

In particular, the “TT on leaf” mode reduces 40% EncT with 0.6% BD-rate loss in AI configuration, which is below the “1% BD-rate loss for 2x speed up” bar. Since the intra frame encoding time of VTM is already around 10x of HM, we would suggest using this constraint for I slice in CTC.

Figure 1 BD-rate loss vs. EncT reduction of different encoder complexity configuration parameters

Conclusions

This contribution introduces two parameters for configuring encoder complexity, which seem to achieve better trade-off between coding efficiency and encoder complexity than the parameters in the test model.

For aspect ratio limit 1:4, simulation results reportedly show on average 0.1%, 0.4% and 0.4% luma BD-rate loss for AI, RA and LB configuration over VTM1.0, with 6%, 16% and 14% EncT reduction, and 0.0%, 0.4%, 0.4% luma BD-rate loss for AI, RA and LB configuration over BMS1.0, with 6%, 12% and 11% EncT reduction.

For “TT on leaf mode”, simulation results reportedly show on average 0.6%, 1.2% and 1.2% luma BD-rate loss for AI, RA and LB configuration over VTM1.0, with 40%, 37% and 39% EncT reduction, and 0.5%, 1.2% and 1.1% luma BD-rate loss for RA and LB configuration over BMS1.0, with 38%, 39% and 37% EncT reduction. Compared with recursive TT under VTM configuration, “TT on leaf” mode achieves around 60% BD-rate gain with 1/3 encoding time.

We propose to adopt the two parameters into the test model. To reduce the CTC simulation time, we also suggest turning on the “TT on leaf” mode in CTC, especially for I slices and BMS simulation.

References

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, April 2018.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).