| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0138-v1 |
| --- | --- |

| Title: | CE7.1.3 : Scan Region-based Coefficient Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yinji Piao Woongil Choi Chanyul Kim | Tel: Email: | [+82-10-6296-4261] [yj1003.piao@samsung.com] |
| Source: | Samsung Electronics Co.,Ltd |  |  |

_____________________________

# Abstract

This contribution presents coefficient coding method described in CE7[1] subtest 1.3. Scan region-based coefficient coding scheme was first introduced in CfP response JVET-J0024[2]. A rectangular region to be scanned is signaled by a coordinate in a TU. Only coefficients in the scan region are considered to be coded by significant flag, GT1flag, GT2flag and remaining level. Proposed coefficient coding scheme outperforms coefficient coding of VTM on average -0.92% BD-rate and -0.49% BD-rate in AI and RA configurations respectively. Compared to coefficient coding of BMS, only 0.1% BD-rate loss in AI and RA is observed with 27 less context models.

# Introduction

Coefficient coding in HEVC is processed in coefficient group (CG) basis. A TU to be coded is partitioned into CGs with size of 4x4. Coefficients in each CG are coded from the last non-zero coefficient of last CG in one of three scan orders, which are horizontal scan, vertical scan, and diagonal scan. Last position (lastX, lastY) is coded first to determine the starting position of inverse scan. Then significant group flag (SigCGflag) is signaled for each CG to indicate whether there are non-zero coefficients in that CG. If there is at least one non-zero coefficient, all coefficients in that CG are inverse scanned to signal non-zero coefficients’ positions with significant flag (referred as GT0flag), For those non-zero coefficients, greater than 1 flags (referred as GT1flag), and greater than 2 flags (referred as GT2flag) are further signaled to indicate whether absolute levels are greater than 1, or greater than 2. If absolute levels are greater than 1 or greater than 2, the remaining levels (AbsMinusX) are coded by Truncated Golomb-Rice and Exp-Gobomb codes. More specifically,

- LastX and lastY are coded by context modeling of prefix and bypass coding of suffix.

- Significant group flag is coded by context modeling based on the corresponding right and bottom significant group flags

- Significant flag is coded by context modeling based on the position in a CG, corresponding right and bottom significant group flags, color component, and transform size.

- GT1 is coded by context modeling based on the previously coded GT1 flag in corresponding CG and GT1 flag in previous CG, similar to GT2flag. Maximum 8 GT1 flag is considered to be signaled for each CG while only 1 GT2 flag is considered.

- Rice parameter for remaining level binarization is updated based on previously coded level. Both prefix and suffix of remaining level are bypass coded.

- Sign of each non-zero coefficient is bypass-coded except the hidden sign of last non-zero coefficient in each CG which can be implicitly derived by the parity of sum of previous decoded levels in that CG if the distance of first and last non-zero coefficients satisfy a given threshold.

Coefficient coding in JEM, follows basic coefficient coding scenario of HEVC except the context modeling of significant flag, GT1flag, GT2 flag and rice parameter derivation.

- Context index of a significant flag is derived by the number of significant flag (max.5) of previously coded five spatial neighboring coefficients, the position in a TU, color component and TU size.

- Context indices of GT1flag and GT2flag are derived by the number of corresponding flags (max.4) in neighborhood similar to significant flag. Depending on the located coefficient position in a TU and color component, different set of context models are used similar to significant flag.

- Rice parameter is determined by the sum of previously coded spatially neighboring five coefficients as significant flag.

# Proposed method

In this proposal, region to be scanned in a TU is determined by the scan region (SRx, SRy), where SRx is most right non-zero coefficients’ x–axis and SRy is most bottom non-zero coefficients’ y-axis as depicted in Figure 1 (a). Only coefficients in scan region are coded from the right-bottom corner of the scan region in inverse zigzag scan order as shown in Figure 2 (b). Similar to HEVC and JEM, a coefficient is presented by significant flag, GT1flag, GT2flag, remaining level and sign. Difference from JEM coefficient coding is the context derivation of significant flag, GT1flag, GT2flag as follows,

- SRx and SRy are coded by context modeling of prefix and bypass coding of suffix, which is exactly same with lastX and lastY coding.

- Context index of a significant flag is derived by the number of significant flag (max.4) of previously coded five spatial neighboring coefficients, the position in a TU, color component and TU size.

- Context indices of GT1flag and GT2flag are derived by the number of corresponding flags (max.3) in neighborhood similar to JEM. Depending on the position in scan region and color component, different set of context models are used similar to JEM. The numbers of GT1flag and GT2 flag to be coded are proportional to the size of scan region.

(a)                                        (b)

Figure 1 (a) Scan region, (b) Example of scan order in a scan region

Table I compares the difference between JEM coefficient coding and proposed SRCC. There are 27 less context models in the proposed method than JEM coefficient coding.

Table I. Comparison of JEM_CC & SR_CC

| JEM_CC |  | SR_CC |  |
| --- | --- | --- | --- |
| Scan | Diag | Scan | Diag |
| SigCGflag | 4 Ctx |  |  |
| Last_xy | 58(29x2) Ctx | SR_xy | 58(29x2) Ctx |
| Sigflag | 66 Ctx | Sigflag | 47 Ctx |
| GT1/GT2 | 22 Ctx | GT1/GT2 | 18 Ctx |
|  | 8GT1/16 1GT2/16 |  | max(scanR/2, 8)GT1 max(scanR/16,1)GT2 |
| RicePara | BMS | RicePara | VTM/BMS |
| total | 150 Ctx | total | 123 Ctx |

# Results

Proposed coefficient coding was implemented on top of BMS1.0 s/w. Tests were conducted with VTM configuration and BMS configuration using one s/w under common test condition [3]. For fair comparison, following changes are made to the s/w and original proposal,

- C1: Contexts of syntaxes used in coefficient coding are initialized with state corresponding to equal probability (154) both for anchor and proposed method.

- C2: Diagonal scan is used in proposed method instead of Zigzag scan.

- C3: Rice parameter derivation for remaining level coding is consistent with configuration, which means HEVC rice parameter derivation is used in VTM configuration and JEM rice parameter derivation is used in BMS configuration.

The summary results with three modifications are presented in following tables. More details can be found in accompanying excel sheets.

Table II. Test results under VTM and BMS common test condition

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0_ctxInitEP |  |  |  |  | Over BMS-1.0_ctxInitEP |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | 0.07% | 0.28% | 101% | 102% | 0.31% | 1.00% | 1.06% | 97% | 101% |
| Class A2 | -0.84% | 0.07% | -0.12% | 101% | 102% | 0.06% | 0.76% | 0.58% | 97% | 105% |
| Class B | -0.86% | 0.18% | 0.16% | 103% | 102% | 0.02% | 0.86% | 0.96% | 98% | 100% |
| Class C | -1.21% | -0.39% | -0.44% | 105% | 102% | 0.05% | 0.55% | 0.58% | 101% | 100% |
| Class E | -1.45% | -0.43% | -0.43% | 100% | 101% | 0.03% | 0.78% | 0.69% | 98% | 99% |
| Overall | -0.92% | -0.08% | -0.10% | 102% | 102% | 0.08% | 0.78% | 0.79% | 99% | 101% |
| Class D | -1.41% | -0.45% | -0.49% | 107% | 103% | -0.02% | 0.21% | 0.34% | 102% | 100% |
| Class F (optional) | -1.73% | -0.59% | -0.50% | 103% | 102% | 0.31% | 0.63% | 0.76% | 103% | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0_ctxInitEP |  |  |  |  | Over BMS-1.0_ctxInitEP |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -0.23% | 0.14% | 99% | 101% | 0.26% | 0.65% | 0.84% | 97% | 100% |
| Class A2 | -0.52% | 0.30% | -0.03% | 98% | 101% | -0.02% | 0.67% | 0.55% | 98% | 100% |
| Class B | -0.38% | 0.09% | 0.07% | 100% | 101% | 0.14% | 0.69% | 0.62% | 98% | 101% |
| Class C | -0.83% | -0.09% | -0.04% | 101% | 101% | 0.09% | 0.51% | 0.47% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.49% | 0.02% | 0.04% | 100% | 101% | 0.12% | 0.63% | 0.61% | 98% | 100% |
| Class D | -0.91% | -0.09% | -0.28% | 102% | 100% | 0.01% | -0.06% | 0.19% | 101% | 101% |
| Class F (optional) | -1.32% | -0.21% | -0.30% | 101% | 100% | 0.24% | 0.62% | 0.51% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0_ctxInitEP |  |  |  |  | Over BMS-1.0_ctxInitEP |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.21% | 0.33% | 0.27% | 99% | 99% | 0.50% | 0.66% | 0.55% | 96% | 99% |
| Class C | 0.09% | -0.03% | -0.11% | 100% | 100% | 0.48% | 0.16% | 0.07% | 97% | 101% |
| Class E | -0.33% | -1.42% | -0.66% | 97% | 100% | 0.32% | -0.16% | 0.84% | 96% | 101% |
| Overall | 0.03% | -0.23% | -0.09% | 99% | 100% | 0.45% | 0.29% | 0.46% | 96% | 100% |
| Class D | -0.29% | -0.21% | -0.26% | 101% | 100% | 0.24% | -0.41% | -0.25% | 98% | 99% |
| Class F (optional) | -0.55% | -0.53% | -0.33% | 99% | 100% | 0.39% | 0.14% | 0.46% | 98% | 99% |

With newly initialized context tables for SRCC, the test results compared to VTM and BMS anchor is as shown in Table III.

Table III. Test results under VTM and BMS common test condition with new context initialization

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | 0.08% | 0.30% | 179% | 100% | 0.31% | 1.00% | 0.97% | 107% | 99% |
| Class A2 | -0.82% | 0.07% | -0.11% | 167% | 102% |  |  |  |  |  |
| Class B | -0.84% | 0.21% | 0.24% | 234% | 100% | 0.05% | 1.02% | 1.00% | 117% | 98% |
| Class C | -1.10% | -0.26% | -0.30% | 142% | 97% | 0.16% | 0.67% | 0.72% | 170% | 98% |
| Class E | -1.34% | -0.36% | -0.22% | 376% | 94% | 0.16% | 1.04% | 0.75% | 140% | 97% |
| Overall | -0.87% | -0.03% | -0.01% | 205% | 99% |  |  |  |  |  |
| Class D | -1.11% | -0.12% | -0.11% | 107% | 100% | 0.28% | 0.90% | 1.01% | 227% | 99% |
| Class F (optional) | -1.66% | -0.52% | -0.52% | 189% | 101% | 0.41% | 0.87% | 0.57% | 162% | 97% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.22% | 0.32% | 98% | 104% | 0.33% | 0.84% | 0.88% | 100% | 98% |
| Class A2 | -0.49% | 0.28% | 0.00% | 97% | 100% | 0.04% | 0.72% | 0.65% | 97% | 99% |
| Class B | -0.28% | 0.31% | 0.14% | 96% | 96% | 0.28% | 0.78% | 0.71% | 88% | 100% |
| Class C | -0.63% | 0.09% | 0.00% | 97% | 94% | 0.35% | 0.62% | 0.42% | 86% | 98% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.38% | 0.14% | 0.11% | 97% | 98% | 0.26% | 0.74% | 0.66% | 92% | 99% |
| Class D | -0.59% | -0.05% | -0.28% | 96% | 94% | 0.45% | 0.43% | 0.32% | 84% | 95% |
| Class F (optional) | -1.15% | -0.26% | -0.26% | 89% | 93% | 0.47% | 0.65% | 0.55% | 78% | 94% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.27% | 0.76% | 1.19% | 98% | 98% |  |  |  |  |  |
| Class C | 0.27% | 0.33% | 0.67% | 99% | 93% | 0.75% | 1.26% | 0.97% | 96% | 95% |
| Class E | 0.36% | 0.14% | 0.44% | 96% | 98% | 0.40% | 0.35% | 1.13% | 94% | 95% |
| Overall | 0.29% | 0.46% | 0.83% | 98% | 96% |  |  |  |  |  |
| Class D | 0.23% | -0.10% | -0.35% | 100% | 94% | 0.81% | 0.72% | 0.34% | 97% | 95% |
| Class F (optional) | -0.42% | -0.51% | 0.33% | 99% | 98% | 0.83% | 0.90% | 0.85% | 97% | 95% |

With newly initialized context tables for SRCC and zigzag scan, the test results compared to VTM and BMS anchor is as shown in Table IV.

Table IV. Test results under VTM and BMS common test condition

# Conclusions

This document presents test results of coefficient coding scheme proposed in Cfp response [2] on top of VTM/BMS s/w. SRCC provides similar results to coefficient coding scheme in BMS with less context models. We recommend adoption of SRCC in test model for potential improvement utilizing scan region and its simplicity.

# Reference

- JVET, “Description of Core Experiment 7 (CE7): Quantization and coefficient coding”, JVET-J1027, 10th JVET Meeting, San Diego, US, 10-20 Apr., 2018

- JVET, “Description of SDR, HDR and 360° video coding technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario”, JVET-J0024, 10th JVET Meeting, San Diego, US, 10-20 Apr., 2018

- JVET, “JVET common test conditions and software reference configurations”, JVET-J1010, 10th JVET Meeting, San Diego, US, 10-20 Apr., 2018

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).