| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0139-v1 |
| --- | --- |

| Title: | CE6: Spatially Varying Transform (Test 6.1.12.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao, Haitao Yang, Jianle Chen | Email: | yin.zhao@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the results of integrating Spatially Varying Transform (SVT) proposed in JVET-J0024 to BMS 1.1. SVT is used for primary transform of inter-predicted blocks. Simulation results reportedly show on average –0.6%/–1.1% luma BD-rate in RA/LB configuration over VTM1.1, with 7%/15% EncT increase, and –0.3%/–0.6% luma BD-rate in RA/LB configuration over BMS1.1, with 3%/5% EncT increase.

Compared with InterEMT on top of VTM, SVT achieves around –0.1% luma BD-rate and –0.3% chroma BD rate with 25% EncT reduction in RA, and achieves –0.1% luma BD-rate and –0.9% chroma BD-rate with 31% EncT reduction in LB. Compared with InterEMT on top of BMS, SVT achieves 0.2% luma BD-rate and –0.3% chroma BD rate in RA, and –0.2% luma BD-rate and –0.9% chroma BD-rate in LB, while reduces EncT by 8% and 11% in RA and LB, respectively.

Proposed method

For an inter-predicted CU with root_cbf equal to 1, a svt_flag may be signaled to indicate whether the whole residual block or a sub-part of the residual block is decoded. The latter case is called a spatially varying transform (SVT), in which a part of the residual block is coded and the rest of the residual block is forced to be zero.

When SVT is used for a CU, SVT type and SVT position information are further decoded from the bitstream. There are two SVT types and three SVT positions, as indicated in Figure 1. For SVT-V (or SVT-H), the TU width (or height) equals to half of the CU width (or height), and three TU positions are equally spaced as (i/4) of the CU width (or height) to the top-left corner of the CU, where i=0, 1 and 2. The three positions are hence denoted as position 0, 1 and 2.

Both TU and CU boundaries are filtered in de-blocking filtering. SVT-V (or SVTH) is allowed when CU width (or height) is in the range of [8, 32]. To ensure the gap between two filtered boundaries are not smaller than 4 pixels, Position 1 of SVT-V (or SVT-H) is not allowed when CU width (or height) equals to 8.

Position-dependent core transform is applied in SVT. The three positions are associated with different core transforms. More specifically, the horizontal and vertical transforms for each SVT position is specified in Table 1.

To restrict the complexity of SVT, SVT is only applied to Merge Mode (for the first 2 Merge candidates) and AMVP mode, not to HEVC-beyond inter modes like Affine, FRUC and IMV in BMS1.0.

A fast algorithm is designed for SVT. For each SVT mode, a RD cost is estimated based on the SSD of the residual-skipped part. An SVT mode is skipped in RDO if the estimated RD cost of the SVT mode is larger than the actual RD cost of the best mode. In addition, only the best 3 SVT modes in terms of the estimated RD cost are tried in RDO.

Table 1 Horizontal and vertical transforms for different SVT types and positions

| SVT type, position | horizontal transform | vertical transform |
| --- | --- | --- |
| SVT-V, position 0 | DCT-8 | DST-7 |
| SVT-V, position 1 | DST-1 | DST-7 |
| SVT-V, position 2 | DST-7 | DST-7 |
| SVT-H, position 0 | DST-7 | DCT-8 |
| SVT-H, position 1 | DST-7 | DST-1 |
| SVT-H, position 2 | DST-7 | DST-7 |

Figure 1 Illustration of six SVT modes

Experimental results

The proposed method is tested under VVC CTC [1] with BMS1.1 used as anchor. Simulation results are summarized in Table 2 and Table 3. More detailed results are provided in Table 4 to Table 7.

Table 2 shows the coding performance of SVT on top of VTM and BMS. As listed in Test 1 in Table 2, SVT achieves on average 0.6% and 1.1% luma BD-rate gain in RA and LB configuration over VTM1.1, with 7% and 15% EncT increase, respectively, and achieves 0.3% and 0.6% luma BD-rate gain in RA and LB configuration over BMS1.1, with 3% and 5% EncT increase, respectively. In addition, NSST does not affect the performance of SVT (as Test 2), since NSST is applied to intra-coded block and SVT to inter-coded block.

SVT has some coding gain overlap with InterEMT, as listed in Table 3. Compared with InterEMT on top of VTM plus IntraEMT (as Test 3), SVT achieves around 0.1% luma BD-rate loss and 0.3% chroma BD rate gain in RA, and achieves around 0.1% luma BD-rate gain and 0.9% chroma BD-rate gain in LB. The encoding time increase by SVT is much lower than that by InterEMT, i.e., 6% vs. 31% in RA and 14% vs. 45% in LB.

By replacing InterEMT with SVT on top of BMS (as Test 4 in Table 3), there are 0.2% luma BD-rate loss and 0.3% chroma BD rate gain in RA, and 0.2% luma BD-rate gain and 0.9% chroma BD-rate gain in LB, while the total encoding time is reduced by 8% and 11% in RA and LB, respectively.

Table 2 Summary of overall performance of SVT over VTM1.1 and BMS1.1

| Test 1 | “VTM + SVT” vs. VTM |  |  |  |  | “BMS + SVT” vs. BMS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -0.62% | 0.19% | 0.16% | 107% | 102% | -0.27% | 0.07% | 0.05% | 103% | 101% |
| LB | -1.09% | 0.45% | 0.54% | 115% | 104% | -0.56% | -0.22% | -0.08% | 105% | 102% |
| Test 2 | “VTM + NSST + SVT” vs. “VTM1.1 + NSST” |  |  |  |  | “BMS + NSST + SVT” vs. “BMS + NSST” |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -0.59% | 0.19% | 0.18% | 106% | 102% | -0.28% | 0.07% | 0.00% | 103% | 100% |
| LB | -1.07% | 0.29% | 0.26% | 114% | 103% | -0.50% | -0.09% | -0.15% | 106% | 100% |

Table 3 Summary of overall performance of SVT and InterEMT over VTM and BMS

| Test 3 | “VTM + IntraEMT + SVT” vs. “VTM + IntraEMT” |  |  |  |  | “VTM + IntraEMT + InterEMT” vs. “VTM + IntraEMT” |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -0.59% | 0.12% | 0.10% | 106% | 101% | -0.65% | 0.47% | 0.51% | 131% | 104% |
| LB | -1.08% | 0.41% | 0.61% | 114% | 102% | -0.97% | 1.46% | 1.41% | 145% | 107% |
| Test 4 | “BMS - InterEMT + SVT” vs. “BMS” |  |  |  |  |  |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | 0.17% | -0.34% | -0.37% | 92% | 100% |  |  |  |  |  |
| LB | -0.16% | -1.03% | -0.89% | 89% | 98% |  |  |  |  |  |

Conclusions

This contribution describes Spatially Varying Transform (SVT) implemented on BMS1.1.

- On top of VTM1.1, SVT achieves 0.6%/1.1% luma BD-rate gain in RA/LB with 7%/15% EncT increase.

- On top of BMS1.1, SVT achieves 0.3%/0.6% luma BD-rate gain in RA/LB with 3%/5% EncT increase.

- Compared with InterEMT in VTM, SVT achieves similar coding efficiency with much less encoding time, i.e., 6%/14% additional EncT by SVT vs. 31%/45% by InterEMT in RA/LB.

- If replacing InterEMT by SVT in BMS setting, coding efficiency on average across RA and LB configurations is close to that of BMS1.1, while the EncT is reduced by around 10%.

We propose to adopt SVT into the test model.

Table 4 Coding performance of SVT on top of VTM and BMS

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.40% | 0.21% | 0.13% | 104% | 101% | -0.15% | 0.05% | 0.01% | 102% | 101% |
| Class A2 | -0.80% | -0.01% | -0.04% | 107% | 102% | -0.33% | 0.07% | -0.09% | 103% | 102% |
| Class B | -0.66% | 0.23% | 0.29% | 107% | 102% | -0.30% | 0.05% | 0.18% | 103% | 101% |
| Class C | -0.61% | 0.27% | 0.17% | 110% | 101% | -0.28% | 0.10% | 0.01% | 102% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.62% | 0.19% | 0.16% | 107% | 102% | -0.27% | 0.07% | 0.05% | 103% | 101% |
| Class D | -0.64% | 0.42% | 0.29% | 112% | 100% | -0.29% | 0.10% | 0.01% | 103% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.20% | 0.41% | 0.59% | 115% | 104% | -0.54% | -0.03% | -0.12% | 106% | 102% |
| Class C | -1.15% | 0.16% | 0.28% | 116% | 104% | -0.62% | -0.15% | -0.11% | 105% | 102% |
| Class E | -0.83% | 0.89% | 0.80% | 113% | 103% | -0.53% | -0.64% | 0.02% | 104% | 100% |
| Overall | -1.09% | 0.45% | 0.54% | 115% | 104% | -0.56% | -0.22% | -0.08% | 105% | 102% |
| Class D | -1.14% | 0.88% | 0.45% | 118% | 102% | -0.52% | -0.19% | 0.23% | 106% | 101% |

Table 5 Coding performance of SVT on top of “VTM + NSST” and “BMS − NSST”

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 + NSST |  |  |  |  | Over BMS-1.1 - NSST |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.36% | 0.18% | 0.08% | 103% | 102% | -0.18% | 0.22% | -0.06% | 102% | 100% |
| Class A2 | -0.78% | 0.10% | 0.00% | 106% | 102% | -0.33% | 0.06% | -0.18% | 103% | 101% |
| Class B | -0.64% | 0.19% | 0.35% | 107% | 102% | -0.30% | 0.01% | 0.15% | 104% | 101% |
| Class C | -0.57% | 0.26% | 0.16% | 108% | 102% | -0.27% | 0.03% | -0.03% | 104% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.59% | 0.19% | 0.18% | 106% | 102% | -0.28% | 0.07% | 0.00% | 103% | 100% |
| Class D | -0.62% | 0.18% | 0.28% | 110% | 100% | -0.34% | 0.07% | 0.19% | 104% | 101% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.1 + NSST |  |  |  |  | Over BMS-1.1 - NSST |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.16% | 0.32% | 0.46% | 114% | 104% | -0.54% | -0.05% | 0.32% | 106% | 100% |
| Class C | -1.08% | 0.22% | 0.23% | 114% | 103% | -0.55% | -0.27% | -0.32% | 106% | 100% |
| Class E | -0.90% | 0.34% | -0.02% | 113% | 102% | -0.36% | 0.07% | -0.70% | 105% | 101% |
| Overall | -1.07% | 0.29% | 0.26% | 114% | 103% | -0.50% | -0.09% | -0.15% | 106% | 100% |
| Class D | -1.12% | 0.96% | 0.33% | 116% | 102% | -0.62% | -0.05% | -0.57% | 107% | 100% |

Table 6 Coding performance of SVT and InterEMT on top of “VTM1.1 + IntraEMT”

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | “VTM-1.1 + intraEMT + SVT ” (--EMT=1 --EMTFast=3) vs. VTM-1.1 + intraEMT (--EMT=1 --EMTFast=3) |  |  |  |  | “VTM-1.1 + intraEMT + InterEMT ” (--EMT=3 --EMTFast=3) vs. VTM-1.1 + intraEMT (--EMT=1 --EMTFast=3) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.38% | 0.21% | 0.10% | 103% | 101% | -0.50% | 0.57% | 0.70% | 131% | 105% |
| Class A2 | -0.76% | 0.01% | -0.07% | 106% | 102% | -0.93% | 0.42% | 0.50% | 129% | 103% |
| Class B | -0.62% | 0.18% | 0.22% | 106% | 101% | -0.71% | 0.39% | 0.40% | 130% | 103% |
| Class C | -0.60% | 0.06% | 0.08% | 108% | 101% | -0.47% | 0.52% | 0.51% | 135% | 103% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.59% | 0.12% | 0.10% | 106% | 101% | -0.65% | 0.47% | 0.51% | 131% | 104% |
| Class D | -0.61% | 0.18% | 0.16% | 110% | 100% | -0.54% | 0.46% | 0.68% | 133% | 101% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | “VTM-1.1 + intraEMT + SVT ” (--EMT=1 --EMTFast=3) vs. VTM-1.1 + intraEMT (--EMT=1 --EMTFast=3) |  |  |  |  | “VTM-1.1 + intraEMT + InterEMT ” (--EMT=3 --EMTFast=3) vs. VTM-1.1 + intraEMT (--EMT=1 --EMTFast=3) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.20% | 0.49% | 0.71% | 114% | 103% | -1.40% | 1.21% | 1.02% | 145% | 109% |
| Class C | -1.14% | 0.13% | 0.04% | 114% | 102% | -0.93% | 1.21% | 1.14% | 145% | 106% |
| Class E | -0.81% | 0.67% | 1.19% | 113% | 100% | -0.32% | 2.21% | 2.40% | 146% | 106% |
| Overall | -1.08% | 0.41% | 0.61% | 114% | 102% | -0.97% | 1.46% | 1.41% | 145% | 107% |
| Class D | -1.05% | 0.41% | 0.86% | 116% | 100% | -0.90% | 0.98% | 1.69% | 139% | 102% |

Table 7 Coding performance of SVT replacing InterEMT on BMS

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.1 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |  |  |  |  |
| Class A1 | 0.22% | -0.25% | -0.50% | 92% | 100% |  |  |  |  |  |
| Class A2 | 0.34% | -0.35% | -0.50% | 92% | 100% |  |  |  |  |  |
| Class B | 0.21% | -0.27% | -0.26% | 92% | 100% |  |  |  |  |  |
| Class C | -0.02% | -0.47% | -0.33% | 91% | 100% |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.17% | -0.34% | -0.37% | 92% | 100% |  |  |  |  |  |
| Class D | 0.02% | -0.41% | -0.34% | 91% | 100% |  |  |  |  |  |
|  | Over BMS-1.1 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |  |  |  |  |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.20% | -0.78% | -0.83% | 90% | 98% |  |  |  |  |  |
| Class C | -0.24% | -1.08% | -0.98% | 88% | 99% |  |  |  |  |  |
| Class E | -0.64% | -1.40% | -0.88% | 89% | 97% |  |  |  |  |  |
| Overall | -0.16% | -1.03% | -0.89% | 89% | 98% |  |  |  |  |  |
| Class D | -0.10% | -0.77% | -0.67% | 89% | 99% |  |  |  |  |  |

References

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, April 2018.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).