| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0140-v1 |
| --- | --- |

| Title: | CE7: Adaptive quantization step size scaling (Test 7.3.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao, Haitao Yang, Jianle Chen | Email: | yin.zhao@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the results of integrating Adaptive Quantization Step scaling (AQS) proposed in JVET-J0025 to BMS 1.0. AQS only applies on luma component. Simulation results reportedly show on average 4.7%/3.8%/3.4% luma MsSSIM-BD-rate gain and 2.5%/2.4%/3.0% luma PSNR-BD-rate loss in AI/RA/LB configuration over VTM1.0, and 3.5%/3.1%/2.7% luma MsSSIM-BD-rate gain and 1.5%/1.8%/2.2% luma PSNR-BD-rate loss in AI/RA/LB configuration over BMS1.0.

Proposed method

Visual distortion of a block is influenced by the background of the block. Luminance masking and contrast masking effect of the background are considered in the quantizer design, in order to make a more perceptually uniform quantizer. The supporting background are two lines around the current CU.

A luminance factor fl(Ba) is derived based on the average luminance of the background Ba (normalized to 8-bit),

flBa=1+60-Ba150,       Ba∈0,60   1,                      Ba∈60,170       1+Ba-170425,    Ba∈170,255.

A contrast masking factor fcBc is derived based on the Mean Absolute Difference (MAD) between the background luminance and the average luminance (i.e., a way to measure the contrast Bc).

fcBc=1,            Bc≤5Bc+510,     Bc>5.

The scaler is based on the product of the two factors and is constrained in a range of [2/2,2]. The scaler is then applied to scale the quantization step size. For the flexibility of the quantizer, a slice-header parameter s is signaled to further adjust the scaler. At encoder, the parameter s is derived from the geometric mean of masking factors calculated from original luma samples of 16x16 blocks inside the slice, and is written into the slice header.

qs'= qsQP×scaler=qsQP×clipflBa×fcBc×s2, 0.5, 2.0

To avoid floating-point calculation, the factors and scaler are all integerized into a 9-bit integers, and the squared root operation is realized by a look-up table with 49 entries.

The quantization step size scaler is derived based on the reconstructed luma samples around the current CU, and thus is the same in encoder and decoder. The scaler is then used in the quantization process to adjust the quantization step size.

A general decoder pipeline structure was introduced in JVET-J0081 [1]. Compared with this pipeline structure, the AQS scheme in JVET-J0025 in fact introduced a loop from “intra prediction + reconstruction” stage back to “inverse quantization and inverse transform” stage, which introduces a significant burden on the hardware decoder pipeline design. To solve this problem, the scaling of quantization step size is changed to scaling of residual at decoder. Accordingly, the “inverse quantization + inverse transform” stage is not changed, and the residual scaling can be performed in parallel with “intra prediction” without increasing the intra prediction critical loop, as shown in Figure 1. The forward transform and quantization at encoder can still be implemented with the quantization step size scaling technique.

Figure 1 Illustration of change to hardware decoder from AQS (using residual scaling).

Experimental results

The proposed method is tested under VVC CTC [2]. By incorporating masking effects, AQS achieves subjective quality improvement, but introduces PSNR BD-rate loss. Since MS-SSIM has better correlation with subjective quality than PSNR, we use MS-SSIM as an additional quality metric. The MS-SSIM calculation code was borrowed from JEM7.0, which was implemented by Ericsson [3] (encapsulated by the JVET_F0064_MSSSIM macro). Simulation results of AQS using residual scaling and quantization step size scaling are summarized in Table 1 and Table 2, respectively. In general, the residual scaling achieves very close results as the quantization step size scaling. More detailed results on AQS with residual scaling can be found in Table 3 and Table 4.

As listed in Table 1, AQS achieves on average 4.7%/3.8%/3.4% luma MsSSIM-BD-rate gain and 2.5%/2.4%/3.0% luma PSNR-BD-rate loss in AI/RA/LB configuration over VTM1.0, and 3.5%/3.1%/2.7% luma MsSSIM-BD-rate gain and 1.5%/1.8%/2.2% luma PSNR-BD-rate loss in AI/RA/LB configuration over BMS1.0. The coding performance of AQS over BMS is smaller than that over VTM, since BMS tools provide better prediction and less residual, lowering the effect from the adaptive residual scaling.

Table 1 Summary of overall performance of AQS with residual scaling

|  | Over VTM, PSNR BD-rate |  |  |  |  | Over VTM, MS-SSIM BD-rate |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | 2.53% | 0.85% | 0.89% | 103% | 102% | -4.74% | 0.79% | 0.84% | 103% | 102% |
| RA | 2.39% | 0.52% | 0.51% | 101% | 100% | -3.81% | 0.41% | 0.53% | 101% | 100% |
| LB | 2.95% | -1.06% | -1.07% | 96% | 99% | -3.43% | -1.01% | -0.99% | 96% | 99% |
|  | Over BMS, PSNR BD-rate |  |  |  |  | Over VTM, MsSSIM BD-rate |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | 1.45% | 0.83% | 0.81% | 103% | 104% | -3.53% | 0.71% | 0.75% | 103% | 104% |
| RA | 1.80% | 0.35% | 0.21% | 99% | 101% | -3.07% | 0.37% | 0.00% | 99% | 101% |
| LB | 2.18% | -1.68% | -1.58% | 94% | 100% | -2.68% | -1.81% | -1.45% | 94% | 100% |

Table 2 Summary of overall performance of AQS with quantization step size scaling

|  | Over VTM, PSNR BD-rate |  |  |  |  | Over VTM, MS-SSIM BD-rate |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | 2.51% | 0.80% | 0.89% | 103% | 102% | -4.78% | 0.74% | 0.87% | 103% | 102% |
| RA | 2.34% | 0.53% | 0.43% | 101% | 100% | -3.84% | 0.35% | 0.47% | 101% | 100% |
| LB | 2.96% | -1.37% | -1.31% | 96% | 98% | -3.42% | -1.49% | -1.64% | 96% | 98% |
|  | Over BMS, PSNR BD-rate |  |  |  |  | Over VTM, MsSSIM BD-rate |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | 1.45% | 0.81% | 0.81% | 103% | 103% | -3.56% | 0.68% | 0.81% | 103% | 103% |
| RA | 1.78% | 0.41% | 0.23% | 100% | 101% | -3.08% | 0.29% | 0.20% | 100% | 101% |
| LB | 2.13% | -1.69% | -1.71% | 95% | 99% | -2.73% | -1.76% | -1.31% | 95% | 99% |

Conclusions

This contribution describes the algorithm and results of adaptive quantization step size scaling. To alleviate the cost for decoder implementation, AQS is modified to adaptive residual scaling (ARS). In terms of MS-SSIM, ARS brings 4.7%/3.8%/3.4% luma BD-rate gain in AI/RA/LB configuration over VTM1.0 and 3.5%/3.1%/2.7% luma BD-rate gain in AI/RA/LB configuration over BMS1.0.

We propose to adopt MS-SSIM into the test model as an additional quality metric. In addition, we propose to adopt ARS into the test model and establish an AHG that covers the scope of perceptual coding with normative processing for further study.

References

- M. Zhou, W. Wan, T. Hellman, P. Chen, O. Hugosson, D. Symes, A. Duenas, and E. Chai, "Comments on Test Model Development", JVET-J0081, April 2018.

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, April 2018.

- M. Pettersson, R. Sjöberg, P. Wennersten, K. Andersson, J. Ström, and J. Enhorn, “MS-SSIM as an additional mandatory metric to PSNR for future video coding”, JVET-F0064, March 2017.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 3 Coding performance of AQS with residual scaling on top of VTM

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 (metric: PSNR) |  |  |  |  | Over VTM-1.0 (metric: MS-SSIM) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 1.02% | 2.06% | 2.02% | 103% | 103% | -2.57% | 1.52% | 1.54% | 103% | 103% |
| Class A2 | 2.59% | 0.56% | 0.35% | 103% | 104% | -4.75% | 0.42% | 0.13% | 103% | 104% |
| Class B | 2.83% | 1.30% | 1.51% | 104% | 101% | -5.41% | 1.23% | 1.48% | 104% | 101% |
| Class C | 2.88% | 0.67% | 0.75% | 103% | 101% | -5.59% | 0.86% | 0.95% | 103% | 101% |
| Class E | 3.02% | -0.60% | -0.51% | 104% | 100% | -4.69% | -0.38% | -0.39% | 104% | 100% |
| Overall | 2.53% | 0.85% | 0.89% | 103% | 102% | -4.74% | 0.79% | 0.84% | 103% | 102% |
| Class D | 3.00% | 1.97% | 2.28% | 103% | 102% | -7.13% | 2.00% | 2.69% | 103% | 102% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 (metric: PSNR) |  |  |  |  | Over VTM-1.0 (metric: MS-SSIM) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 1.64% | 1.40% | 1.41% | 102% | 100% | -2.18% | 0.24% | 0.84% | 102% | 100% |
| Class A2 | 3.14% | 1.43% | 1.27% | 101% | 101% | -2.82% | 1.19% | 1.57% | 101% | 101% |
| Class B | 2.70% | 0.10% | 0.19% | 100% | 99% | -4.89% | 0.42% | 0.10% | 100% | 99% |
| Class C | 2.00% | -0.30% | -0.33% | 100% | 101% | -4.44% | -0.04% | 0.06% | 100% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 2.39% | 0.52% | 0.51% | 101% | 100% | -3.81% | 0.41% | 0.53% | 101% | 100% |
| Class D | 2.04% | -0.15% | -0.62% | 100% | 102% | -5.25% | 0.24% | -0.42% | 100% | 102% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 (metric: PSNR) |  |  |  |  | Over VTM-1.0 (metric: MS-SSIM) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 2.19% | -1.69% | -1.20% | 96% | 100% | -4.12% | -1.80% | -0.76% | 96% | 100% |
| Class C | 2.02% | -1.16% | -1.14% | 97% | 98% | -3.84% | -1.32% | -0.83% | 97% | 98% |
| Class E | 5.44% | 0.10% | -0.79% | 94% | 97% | -1.72% | 0.71% | -1.60% | 94% | 97% |
| Overall | 2.95% | -1.06% | -1.07% | 96% | 99% | -3.43% | -1.01% | -0.99% | 96% | 99% |
| Class D | 2.16% | -0.91% | -1.28% | 97% | 101% | -3.34% | -0.50% | -1.63% | 97% | 101% |

Table 4 Coding performance of AQS with residual scaling on top of BMS

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 (metric: PSNR) |  |  |  |  | Over BMS-1.0 (metric: MS-SSIM) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.64% | 1.77% | 1.60% | 104% | 105% | -2.02% | 0.75% | 0.99% | 104% | 105% |
| Class A2 | 1.38% | 0.54% | 0.43% | 104% | 105% | -3.61% | 0.45% | 0.28% | 104% | 105% |
| Class B | 1.71% | 1.34% | 1.46% | 103% | 105% | -4.02% | 1.31% | 1.49% | 103% | 105% |
| Class C | 1.69% | 0.60% | 0.66% | 102% | 103% | -4.03% | 0.78% | 0.85% | 102% | 103% |
| Class E | 1.59% | -0.35% | -0.48% | 101% | 103% | -3.50% | -0.17% | -0.37% | 101% | 103% |
| Overall | 1.45% | 0.83% | 0.81% | 103% | 104% | -3.53% | 0.71% | 0.75% | 103% | 104% |
| Class D | 1.79% | 1.77% | 1.79% | 102% | 103% | -5.36% | 1.92% | 2.09% | 102% | 103% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 (metric: PSNR) |  |  |  |  | Over BMS-1.0 (metric: MS-SSIM) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 1.26% | 1.05% | 0.91% | 101% | 102% | -1.81% | 0.32% | 0.06% | 101% | 102% |
| Class A2 | 2.43% | 1.06% | 0.93% | 100% | 102% | -2.24% | 1.07% | 0.63% | 100% | 102% |
| Class B | 2.06% | 0.13% | 0.17% | 98% | 100% | -4.07% | 0.41% | 0.16% | 98% | 100% |
| Class C | 1.42% | -0.42% | -0.80% | 98% | 101% | -3.39% | -0.18% | -0.72% | 98% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 1.80% | 0.35% | 0.21% | 99% | 101% | -3.07% | 0.37% | 0.00% | 99% | 101% |
| Class D | 1.50% | -0.46% | -0.45% | 98% | 99% | -4.20% | 0.35% | -0.16% | 98% | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 (metric: PSNR) |  |  |  |  | Over BMS-1.0 (metric: MS-SSIM) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 1.81% | -2.06% | -1.84% | 95% | 101% | -3.38% | -2.01% | -2.12% | 95% | 101% |
| Class C | 1.57% | -1.25% | -1.50% | 95% | 101% | -2.80% | -1.19% | -1.15% | 95% | 101% |
| Class E | 3.60% | -1.62% | -1.25% | 93% | 98% | -1.34% | -2.30% | -0.75% | 93% | 98% |
| Overall | 2.18% | -1.68% | -1.58% | 94% | 100% | -2.68% | -1.81% | -1.45% | 94% | 100% |
| Class D | 1.66% | -1.23% | -1.71% | 94% | 100% | -2.36% | 0.76% | -0.54% | 94% | 100% |