| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0141-v2 |
| --- | --- |

| Title: | AHG8: 360°-based inter/intra prediction for cubemap projection |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheng-Hsuan Shih, Jian-Liang Lin, Hung-Chih Lin, Shen-Kai Chang, Chi-Cheng Ju No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Jian-Liang Lin +886-3-5670766 ext. 25250 {ch.shih, jl.lin, hungchih.lin, shenkai.chang, cc.ju}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a 360°-based inter/intra prediction for cubemap projection. This proposed method modifies the referenced pixels in inter and intra prediction by using the geometric neighboring pixels when those referenced pixels are outside the frame boundary (layout boundary) or across the discontinuous edge(s) in the projection layout. As compared to the conventional inter and intra prediction applied to the cubemap layout, the experimental results reportedly demonstrate that the proposed method, in terms of end-to-end WS-PSNR-Y/U/V, achieves -0.97%/-1.26%/-1.28% and -1.40%/-0.60/-0.79% BD-rate reduction, under VTM-1.0 and BMS-1.0, respectively, while the seam artifacts caused by the frame boundary is also reduced.

# Introduction

In conventional video coding systems, the replication padding for reference pixel data is used in inter prediction when the reference region is outside the frame boundary. Fig. 1 shows a typical implementation for replication padding which extends the boundary pixels in the border region. In intra prediction, the referenced neighboring pixel data used by predictor construction would be unavailable if those referenced data are outside frame boundary or have not been coded yet.

Fig. 1 Replication padding in conventional video system

In this contribution, the 360°-based inter/intra prediction for cubemap projection is proposed to use the geometric neighboring pixel data for efficient prediction based on the geometric continuity property in 360° video. In the proposed method, the inter prediction replaces the replication padding with the geometry padding when the reference pixels are outside the frame boundary (layout boundary) or across the discontinuous edge(s). Similarly, the referenced neighboring pixels in the intra prediction are changed to use the geometric neighboring reconstructed pixels when those pixels are outside the frame boundary (layout boundary) or across the discontinuous edge(s).

# 360°-based inter and intra prediction

The geometry padding in Fig. 2 demonstrates a pixel position qb qboutside the Bottom face is represented by its geometric pixel position qa, which is located at the Front face. This geometric pixel position is obtained by the projection mapping functions. When the derived coordinate qa is fractional, the 6-tap and 4-tap Lanczos interpolation filters [1] are applied for the luminance and chrominance components, respectively.

Fig. 2 Geometry padding process

To achieve the geometry padding [2][3] in inter prediction, the proposed implementation maximally utilizes face continuity by alternatively constructing the top sub-frame and the bottom sub-frame for each reference frame data, as illustrated in Fig. 3. Each sub-frame buffer has one 3x1 face region with an extended padding region. The pixel data in the padding region are obtained by the geometry padding process as described above. Note that the padding size is the LCU size with additional 16 pixels, which yields the width of the padding area is 144 pixels.

Fig. 3 Top and bottom sub-frame buffers: 3x1 face region with geometry padding area

Moreover, the intra prediction constructs efficient predictors by using the neighboring reconstructed pixels of the current CU. Similar to the geometry padding in inter prediction, the reference pixels of intra prediction can be derived from geometric neighboring pixels. In the proposed method, when a CU is located at the frame boundary (layout boundary) or the discontinuous edge(s), its neighboring pixels are replaced by the geometric neighboring reconstructed pixels, which are located at other faces. As the example shown in Fig. 4, the neighboring pixels of the current CU are replaced by the top and left geometric neighboring pixels which are located at the Left face and Front face, respectively. Note that the geometric neighboring reference pixels are unavailable when these pixels have not been coded yet.

Fig. 4 Geometric neighboring pixels

# Experimental Results

The simulations on the proposed method are conducted on the Modified Cubemap Projection (MCP) layout proposed in JVET-J0019 [4]. The software is implemented on the 360Lib-6.0 [5] and the BMS-1.0 [6] followed by the JVET common test conditions [7] and evaluation procedures for 360° video [8] with  two encoder configurations, VTM-1.0 [9] and BMS-1.0 [6]. Moreover, in order to align with the CTU size, the face resolution is changed to 1280x1280. That is, the frame resolution of the MCP is 3840x2560. As compared to the conventional inter and intra prediction applied to the MCP, the BD-rate performance, in terms of end-to-end WS-PSNR and S-PSNR-NN, provided by the proposed method under VTM-1.0 and BMS-1.0 is listed in Table I and Table II, respectively.

Table I. BD-rate performance in VTM test condition for MCP projection

| Sequences | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -1.56% | -2.02% | -2.03% | -1.59% | -2.05% | -1.99% |
| ChairLift | -2.15% | -2.06% | -1.83% | -2.14% | -2.04% | -1.84% |
| KiteFlite | -0.05% | 0.09% | 0.06% | -0.03% | 0.08% | 0.07% |
| Harbor | -0.14% | -0.25% | -0.42% | -0.13% | -0.28% | -0.38% |
| Trolley | -0.16% | -0.17% | -0.11% | -0.18% | -0.17% | -0.10% |
| GasLamp | -0.24% | -0.27% | -0.21% | -0.24% | -0.24% | -0.20% |
| Balboa | -1.69% | -2.33% | -2.43% | -1.68% | -2.31% | -2.44% |
| Broadway | -1.32% | -1.82% | -2.07% | -1.32% | -1.80% | -2.06% |
| Landing2 | -0.99% | -1.67% | -1.51% | -0.96% | -1.66% | -1.49% |
| BranCastle2 | -1.39% | -2.09% | -2.25% | -1.41% | -2.08% | -2.24% |
| Average | -0.97% | -1.26% | -1.28% | -0.97% | -1.26% | -1.27% |

Table II. BD-rate performance in BMS test condition for MCP projection

| Sequences | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -1.92% | -1.00% | -0.25% | -1.99% | -1.03% | -0.27% |
| ChairLift | -2.55% | -1.63% | -1.81% | -2.56% | -1.64% | -1.79% |
| KiteFlite | -0.48% | 0.23% | 0.06% | -0.41% | 0.19% | 0.02% |
| Harbor | -0.07% | 1.15% | 2.26% | 0.04% | 1.33% | 2.46% |
| Trolley | -0.80% | 0.11% | -0.70% | -0.85% | 0.18% | -0.69% |
| GasLamp | -0.72% | 0.85% | 0.25% | -0.80% | 0.94% | 0.34% |
| Balboa | -2.02% | -1.55% | -2.23% | -2.02% | -1.57% | -2.20% |
| Broadway | -1.52% | -1.61% | -2.29% | -1.52% | -1.61% | -2.26% |
| Landing2 | -1.50% | -0.19% | -0.69% | -1.51% | -0.13% | -0.66% |
| BranCastle2 | -2.38% | -2.36% | -2.55% | -2.38% | -2.32% | -2.54% |
| Average | -1.40% | -0.60% | -0.79% | -1.40% | -0.57% | -0.76% |

# Subjective Quality Testing

In Fig. 5 to Fig. 10, the resulted viewport images with the conventional inter/intra prediction and the proposed 360°-based inter/intra prediction are demonstrated (under VTM-1.0 encoder configuration with QP = 37) to evaluate the seam artifacts appeared at frame boundary.

Fig. 5 The cropped image of Landing2 (frame 30) using conventional inter/intra prediction

Fig. 6 The cropped image of Landing2 (frame 30) using 360°-based inter/intra prediction

Fig. 7 The cropped image of Broadway (frame 299) using conventional inter/intra prediction

Fig. 8 The cropped image of Broadway (frame 299) using 360°-based inter/intra prediction

Fig. 9 The cropped image of SkateboardInLot (frame 194) using conventional inter/intra prediction

Fig. 10 The cropped image of SkateboardInLot (frame 194) using 360°-based inter/intra prediction

# Conclusion

The contribution proposes a 360°-based inter/intra prediction for cubemap projection. The referenced pixels used in inter and intra prediction fetch the pixel data at the geometric neighboring positions for superior compression efficiency when those pixels are outside the frame boundary (layout boundary) or across the discontinuous edge(s) in the projection layout. As compared to the conventional inter and intra prediction applied to the cubemap layout, the proposed method, in terms of end-to-end WS-PSNR-Y/U/V, achieves -0.97%/-1.26%/-1.28% and -1.40%/-0.60/-0.79% BD-rate reduction, under VTM-1.0 and BMS-1.0, respectively, while also noticeably reduces the visual seam artifacts.

# References

- Lanczos resampling, https://en.wikipedia.org/wiki/Lanczos_resampling

- J. Sauer and M.Wien, “Geometry correction for motion compensation of planar-projected 360VR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-D0067, Oct. 2016, Chengdu, China

- Y. He, Y. Ye, P. Hanhart, and X. Xiu , “AHG8: Geometry padding for 360 video coding,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-D0075, Oct. 2016, Chengdu, China

- J.-L. Lin, Y.-H. Lee, C.-H. Shih, S.-Y. Lin, H.-C. Lin, S.-K. Chang, P. Wang, L. Liu, and C.-C. Ju, “Description of 360° video coding technology proposal by MediaTek,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-J0019, Apr. 2018, San Diego, USA

- 360Lib software, https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/

- VTM Benchmark Set, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-J1010, Apr. 2018, San Diego, USA

- P. Hanhart, J. Boyce, and K. Choi, “JVET common test conditions and evaluation procedures for 360° video,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-J1012, Apr. 2018, San Diego, USA

- Versatile Video Coding Test Model, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_VTM/tags/

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).