| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0142-v2 |
| --- | --- |

| Title: | AHG8: 360°-based in-loop filters for cubemap projection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sheng-Yen Lin, Lin Liu, Cheng-Hsuan Shih, Jian-Liang Lin, Hung-Chih Lin, Shen-Kai Chang, Chi-Cheng Ju No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Jian-Liang Lin +886-3-5670766 ext. 25250 {Sheng-Yen.Lin, Lin.Liu, ch.shih, jl.lin, hungchih.lin, shenkai.chang, cc.ju}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes two 360°-based in-loop filtering methods that modify the conventional in-loop filters for appropriately filtering on the cubemap layout. One of them is to disable in-loop filtering processes at the discontinuous edge, while the other one uses the property of geometric continuity in 360° video to properly filter the pixel data located at the geometric positions. As compared to the conventional in-loop filters applied to the cubemap layout, the experimental results demonstrate that the two proposed methods, in terms of end-to-end WS-PSNR-Y, achieve 0.19% and 0.26% BD-rate reduction under VTM-1.0 and achieve 0.16% and 0.23% BD-rate reduction under BMS-1.0, respectively. In addition, the proposed methods noticeably enhance subjective quality by alleviating the seam artifacts.

# Introduction

The in-loop filters in a conventional video coding system, such as de-blocking filter and the sample adaptive offset filter, are used to remove undesired artifacts and enhance encoding quality. For 360° video, the layout for a specific projection format usually has one or multiple discontinuous edges. Applying in-loop filters on these discontinuous edges directly may result in poor visual quality and coding efficiency because the accessed pixels (to be referenced and/or filtered) cross discontinuous edges are jointly processed (filtered). To address this problem, we propose two 360°-based in-loop filtering methods such that the discontinuous edge in the cubemap layout can be properly processed.

# Proposed Methods

In this contribution, the in-loop filters, including the de-blocking (DB) filter and the sample adaptive offset (SAO) filter, are performed necessarily changes to avoid inappropriately filtering at the layout boundary and/or the discontinuous edge in the cubemap layout.

## Method 1: Disabled Filtering Process cross the Discontinuous Edge

### The first proposed method is a straightforward approach that disables the in-loop filtering process when the accessed pixels cross the discontinuous edge. The seam artifacts are thus significantly reduced without any additional memory buffer.

### De-blocking Filter

The de-blocking filter is used to efficiently reduce visible blocky artifacts appeared at prediction block and transform block edges. The boundary strength of each block edge determines whether the edge filtering is performed. The edge filtering is disabled (the boundary strength is set to 0) when the filtered pixels are cross the discontinuous edge in the cubemap layout.

### Sample Adaptive Offset Filter

In the SAO filtering process, each pixel is adjusted by referencing its neighboring pixels. When the referenced pixels cross the discontinuous edge, the proposed modification disables the SAO filtering process of the current pixel to avoid inadequate filtering process.

## Method 2: Filtering Process Based on Geometric Continuity

### For further improvement, the second method is implemented based on the geometric continuity. The in-loop filtering process makes use of the pixel data located at the geometric positions for filtering so that the discontinuous edge, as well as the frame boundary (layout boundary), can be appropriately processed. Because the accessed pixels outside the layout boundary and discontinuous edge are obtained from other faces, extra memory buffers are needed.

### De-blocking Filter

Based on the concept of geometric continuity, the de-blocking filter fetches the geometric pixel positions located at another face for edge filtering when the filtered pixels cross the frame boundary (layout boundary) or the discontinuous edge as illustrated in Fig. 1. The pixel data in the geometric positions are copied directly from the corresponding face. After the pixel data are collected for edge filtering, the motion vector located at the geometric pixel positions is also adjusted by the motion projection, which is proposed in JVET-0019 [1] for appropriate boundary strength derivation.

Fig. 1 Geometric pixel positions from another face for 360°-based DB filter based on geometric continuity

### Sample Adaptive Offset filter

As illustrated in Fig. 2, when the referenced neighboring pixels cross the discontinuous edge, the proposed implementation uses the geometric pixel positions located at another face for SAO filtering. Moreover, the pixel data in the geometric positions are copied directly from the corresponding face.

Fig. 2 Geometric pixel positions from another face for 360°-based SAO filter based on geometric continuity

# Experimental Results

The simulations on the proposed 360°-based in-loop filtering methods are conducted on the Modified Cubemap Projection (MCP) layout proposed in JVET-J0019 [1]. The software is implemented on the 360Lib-6.0 [2] and the BMS-1.0 [3] followed by the JVET common test conditions [5] and evaluation procedures for 360° video [6] with two encoder configurations, VTM-1.0 [4] and BMS-1.0 [3].  The resolution of each face of the MCP is set to1184x1184, which yields the frame resolution is 3552x2368. As compared to the conventional in-loop filter applied to the MCP, the BD-rate performance, in terms of end-to-end WS-PSNR, provided by the two proposed 360°-based in-loop filtering methods under VTM-1.0 and BMS-1.0 is listed in Table I and Table II, respectively.

Table I. BD-rate performance for the two proposed 360°-based in-loop filtering methods compared to the conventional in-loop filter under VTM-1.0

|  | Method 1 |  |  | Method 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -0.23% | -0.91% | -1.40% | -0.41% | -1.18% | -1.81% |
| ChairLift | -0.32% | -0.11% | -0.25% | -0.46% | -0.38% | -0.52% |
| KiteFlite | -0.07% | -0.18% | -0.23% | -0.08% | -0.28% | -0.27% |
| Harbor | -0.25% | -0.97% | -1.36% | -0.27% | -1.13% | -1.62% |
| Trolley | -0.06% | -0.42% | -0.30% | -0.07% | -0.55% | -0.43% |
| GasLamp | -0.09% | -0.58% | -0.47% | -0.08% | -0.66% | -0.54% |
| Balboa | -0.30% | -0.48% | -0.44% | -0.40% | -0.57% | -0.63% |
| Broadway | -0.30% | -0.43% | -0.37% | -0.40% | -0.37% | -0.45% |
| Landing2 | -0.20% | -1.53% | -1.27% | -0.28% | -1.75% | -1.48% |
| BranCastle2 | -0.05% | -0.09% | -0.18% | -0.12% | -0.29% | -0.42% |
| Class S1 | -0.17% | -0.53% | -0.67% | -0.23% | -0.70% | -0.87% |
| Class S2 | -0.21% | -0.63% | -0.56% | -0.30% | -0.74% | -0.75% |
| Overall | -0.19% | -0.57% | -0.63% | -0.26% | -0.72% | -0.82% |

Table II. BD-rate performance for the two proposed 360°-based in-loop filtering methods compared to the conventional in-loop filter under BMS-1.0

|  | Method 1 |  |  | Method 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -0.17% | -0.73% | -0.90% | -0.24% | -0.71% | -1.32% |
| ChairLift | -0.26% | -0.03% | -0.10% | -0.39% | -0.18% | -0.37% |
| KiteFlite | -0.09% | -0.01% | -0.06% | -0.11% | -0.15% | -0.16% |
| Harbor | -0.24% | -0.68% | -0.87% | -0.25% | -0.94% | -1.10% |
| Trolley | -0.08% | -0.24% | -0.18% | -0.08% | -0.39% | -0.34% |
| GasLamp | -0.11% | -0.40% | -0.29% | -0.12% | -0.57% | -0.40% |
| Balboa | -0.21% | -0.28% | -0.08% | -0.34% | -0.20% | -0.15% |
| Broadway | -0.20% | 0.12% | -0.19% | -0.33% | -0.16% | -0.55% |
| Landing2 | -0.20% | -1.30% | -1.01% | -0.29% | -1.52% | -1.30% |
| BranCastle2 | -0.06% | -0.10% | -0.06% | -0.11% | -0.28% | -0.34% |
| Class S1 | -0.16% | -0.35% | -0.40% | -0.20% | -0.49% | -0.62% |
| Class S2 | -0.17% | -0.39% | -0.33% | -0.27% | -0.54% | -0.58% |
| Overall | -0.16% | -0.36% | -0.37% | -0.23% | -0.51% | -0.60% |

# Subjective Quality Testing

In Fig. 3 to Fig. 8, the cropped viewport images of the conventional in-loop filters and the two proposed 360°-based in-loop filtering methods are demonstrated (under VTM-1.0 with QP = 37) in order to evaluate the seam artifacts appeared at the discontinuous edge.

Fig. 3 The subjective quality of Balboa (frame 3) using conventional in-loop filtering

Fig. 4 The subjective quality of Balboa (frame 3) using proposed 360°-based in-loop filtering method 1

Fig. 5 The subjective quality of Balboa (frame 3) using proposed 360°-based in-loop filtering method 2

Fig. 6 The subjective quality of BranCastle2 (frame 65) using conventional in-loop filtering

Fig. 7 The subjective quality of BranCastle2 (frame 65) using proposed 360°-based in-loop filtering method 1

Fig. 8 The subjective quality of BranCastle2 (frame 65) using proposed 360°-based in-loop filtering method 2

# Conclusion

This contribution proposes two 360°-based in-loop filtering methods for cubemap projection. The concept of these two methods aims to avoid improper edge filtering at the layout boundary and discontinuous edge. As compared to the conventional in-loop filters applied to the cubemap layout, the proposed methods can remarkably reduce the visible seam artifacts with slightly improved compression efficiency.

# References

- J.-L. Lin, Y.-H. Lee, C.-H. Shih, S.-Y. Lin, H.-C. Lin, S.-K. Chang, P. Wang, L. Liu, and C.-C. Ju,” Description of 360° video coding technology proposal by MediaTek,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J0019, Apr. 2018, San Diego, USA.

- 360Lib, https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/

- VTM Benchmark Set, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/

- Versatile Video Coding Test Model, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_VTM/tags/

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1010, Apr. 2018, San Diego, USA.

- P. Hanhart, J. Boyce, and K. Choi, “JVET common test conditions and evaluation procedures for 360° video,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1012, Apr. 2018, San Diego, USA.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).