| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 Jul. 2018 | Document: JVET-K0144-v1 |
| --- | --- |

| Title: | CE10: Triangular prediction unit mode (CE10.3.1 and CE10.3.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao Chong Soon Lim | Tel: Email: | +65-6550-5346 ruling.liao@sg.panasonic.com chongsoon.lim@sg.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

This contribution provides test results for the triangular prediction unit mode which is described in CE10. In the triangular prediction unit mode, a CU can be split using two triangular prediction units, either in diagonal or inverse diagonal direction. Each triangular prediction unit in the CU has its own motion vectors and reference frame index which are derived from a merge candidate list. Triangular partitioning is only applied to motion compensated prediction, which means that the transform and quantization process is applied to the whole CU formed by combining together the two triangles. Two different settings for the triangular prediction unit mode are tested and their coding results are reported as follows:

- (CE10.3.1) applying the triangular prediction unit mode to all CU size:

  - (VTM configuration) (RA) -1.00% BD-rate with 119% encoding time and 103% decoding time

(LB) -1.70% BD-rate with 125% encoding time and 103% decoding time

  - (BMS configuration) (RA) -0.96% BD-rate with 109% encoding time and   99% decoding time

(LB) -1.44% BD-rate with 113% encoding time and 100% decoding time

- (CE10.3.2) applying the triangular prediction unit mode to CU size larger than or equal to 8×8:

  - (VTM configuration) (RA) -0.95% BD-rate with 118% encoding time and 103% decoding time

(LB) -1.63% BD-rate with 122% encoding time and 102% decoding time

  - (BMS configuration) (RA) -0.91% BD-rate with 108% encoding time and 100% decoding time

(LB) -1.41% BD-rate with 112% encoding time and 101% decoding time

# Introduction

The concept of the triangular prediction unit mode is to introduce a new triangular partition for motion compensated prediction. As shown in Figure 1, it splits a CU into two triangular prediction units, either in diagonal or inverse diagonal direction. Each triangular prediction unit in the CU is inter-predicted using its own motion vectors and reference frame index which are derived from a merge candidate list.

Figure 1: An illustration of splitting a CU into two triangular prediction units

An adaptive weighting process is applied to the diagonal or inverse diagonal edge between the two triangular prediction units to derive the final prediction for the whole CU using the prediction pixels of the two triangular prediction units. Two weighting factors group are listed as follows:

- 1st weighting factors group: {7/8, 6/8, 4/8, 2/8, 1/8} and {7/8, 4/8, 1/8} are used for the luminance and the chrominance samples, respectively;

- 2nd weighting factors group: {7/8, 6/8, 5/8, 4/8, 3/8, 2/8, 1/8} and {6/8, 4/8, 2/8} are used for the luminance and the chrominance samples, respectively.

One out of two weighting factors group is selected based on the comparison of the motion vectors of two triangular prediction units. An example of the weighting process when the 1st weighting factors group is selected is shown in Figure 2. After the adaptive weighting process is performed, the transform and quantization process is applied to the whole CU.

Figure 2: An example of a CU applying the 1st weighting factors group

The triangular prediction unit mode is only applied to CUs in skip or merge mode. For a CU coded in a skip or a merge mode, a CU level flag is signalled to indicate whether the triangular prediction unit mode is applied or not for the current CU. When the triangular prediction unit mode is applied to the CU, an index indicating the direction of splitting the CU into two triangular prediction units and the motion vectors of the two triangular prediction units are signalled. The index ranges from 0 to 39 and a look-up table is used for deriving splitting direction and motion vectors from the index.

# Experimental Results

The following experiments were conducted using the BMS-1.0 software with the VTM and BMS configurations and the JVET common test conditions [1] to collect the BD-rate, encoding and decoding time of the proposed techniques in the Random Access and Low Delay B configurations. Two different settings described in CE10 [2] are tested:

- (CE10.3.1) applying the triangular prediction unit mode to all CU size;

- (CE10.3.2) applying the triangular prediction unit mode to CU size larger than or equal to 8×8.

Results of CE10.3.1 and CE10.3.2 are summarized in Table 1 and Table 2, respectively. As compared to VTM-1.0, roughly 1% BD-rate reduction with 20% encoding time increase can be obtained in the RA configuration. Higher BD-rate reduction which is up to 1.7% can be observed in the LB configuration. Besides, it is reported that similar coding gain can be obtained using the BMS configuration, which means that the triangular prediction unit mode is independent of other BMS tools.

Table 1: Experimental results of CE10.3.1 using the VTM and BMS configurations

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 (CE10.3.1) |  |  |  |  | Over BMS-1.0 (CE10.3.1) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.69% | -1.53% | -1.65% | 114% | 102% | -0.63% | -1.04% | -1.08% | 107% | 99% |
| Class A2 | -1.02% | -1.58% | -1.51% | 118% | 103% | -0.97% | -1.48% | -1.77% | 108% | 99% |
| Class B | -0.97% | -1.25% | -1.15% | 120% | 103% | -0.92% | -1.24% | -1.19% | 109% | 99% |
| Class C | -1.24% | -1.82% | -1.91% | 123% | 103% | -1.23% | -1.59% | -1.76% | 110% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.00% | -1.52% | -1.53% | 119% | 103% | -0.96% | -1.34% | -1.44% | 109% | 99% |
| Class D | -0.76% | -1.14% | -1.15% | 127% | 103% | -0.92% | -1.32% | -1.56% | 111% | 99% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 (CE10.3.1) |  |  |  |  | Over BMS-1.0 (CE10.3.1) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.37% | -1.77% | -1.88% | 125% | 103% | -1.14% | -1.93% | -1.55% | 113% | 101% |
| Class C | -1.54% | -2.31% | -2.51% | 126% | 103% | -1.48% | -2.40% | -2.61% | 114% | 100% |
| Class E | -2.45% | -2.09% | -2.38% | 123% | 102% | -1.90% | -2.06% | -1.29% | 112% | 97% |
| Overall | -1.70% | -2.03% | -2.21% | 125% | 103% | -1.44% | -2.12% | -1.84% | 113% | 100% |
| Class D | -1.35% | -1.79% | -1.62% | 126% | 103% | -1.31% | -1.67% | -1.90% | 115% | 100% |

Table 2: Experimental results of CE10.3.2 using the VTM and BMS configurations

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 (CE10.3.2) |  |  |  |  | Over BMS-1.0 (CE10.3.2) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.70% | -1.65% | -1.56% | 114% | 102% | -0.63% | -1.12% | -1.14% | 106% | 100% |
| Class A2 | -0.99% | -1.53% | -1.39% | 117% | 103% | -0.94% | -1.49% | -1.64% | 107% | 100% |
| Class B | -0.95% | -1.20% | -1.17% | 119% | 103% | -0.91% | -1.16% | -1.11% | 108% | 100% |
| Class C | -1.12% | -1.51% | -1.68% | 121% | 103% | -1.11% | -1.30% | -1.51% | 109% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.95% | -1.44% | -1.43% | 118% | 103% | -0.91% | -1.25% | -1.33% | 108% | 100% |
| Class D | -0.64% | -0.90% | -0.91% | 123% | 103% | -0.76% | -0.95% | -1.05% | 110% | 99% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 (CE10.3.2) |  |  |  |  | Over BMS-1.0 (CE10.3.2) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.35% | -1.77% | -1.54% | 122% | 103% | -1.10% | -1.76% | -1.09% | 112% | 102% |
| Class C | -1.44% | -2.03% | -2.35% | 123% | 102% | -1.38% | -1.99% | -2.31% | 114% | 101% |
| Class E | -2.36% | -2.11% | -1.83% | 122% | 102% | -1.96% | -1.39% | -1.61% | 111% | 99% |
| Overall | -1.63% | -1.94% | -1.89% | 122% | 102% | -1.41% | -1.74% | -1.63% | 112% | 101% |
| Class D | -1.14% | -1.57% | -1.77% | 121% | 102% | -1.04% | -0.90% | -1.55% | 113% | 100% |

# Encoder complexity optimization

To reduce the encoding time of the triangular prediction unit mode, the encoder fast algorithm which is aiming to reduce the number of indices being tested for this mode is introduced. Firstly, the number of indices being tested is determined for a CU based on its size as follows:

If CU width / CU height ≥ 16 or CU height / CU width ≥ 16,

the number of indices being tested is set to 2;

Otherwise, if CU width / CU height ≥ 8 or CU height / CU width ≥ 8,

the number of indices being tested is set to 14;

Otherwise, if CU width / CU height ≥ 4 or CU height / CU width ≥ 4,

the number of indices being tested is set to 30;

Otherwise,

the number of indices being tested is set to 40.

Secondly, only the combinations of the first four merge candidates with lower cost are tested. Therefore, the maximum number of indices being tested is 24.

Results of CE10.3.1 and CE10.3.2 with encoder fast algorithm are summarized in Table 3 and Table 4, respectively. In summary, roughly 10% encoding time reduction can be achieved with 0.05% BD-rate dropped in the RA configuration. On the other hands, 10% encoding time reduction can be obtained with 0.2% BD-rate dropped in the LB configuration.

Table 3: Experimental results of CE10.3.1 with encoder fast algorithm using the VTM configuration

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 (CE10.3.1 w/o encoder fast algorithm) |  |  |  |  | Over VTM-1.0 (CE10.3.1 w/ encoder fast algorithm) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.69% | -1.53% | -1.65% | 114% | 102% | -0.67% | -1.49% | -1.45% | 109% | 103% |
| Class A2 | -1.02% | -1.58% | -1.51% | 118% | 103% | -0.95% | -1.35% | -1.29% | 113% | 103% |
| Class B | -0.97% | -1.25% | -1.15% | 120% | 103% | -0.91% | -1.20% | -1.12% | 112% | 103% |
| Class C | -1.24% | -1.82% | -1.91% | 123% | 103% | -1.15% | -1.65% | -1.81% | 113% | 102% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.00% | -1.52% | -1.53% | 119% | 103% | -0.93% | -1.41% | -1.41% | 112% | 103% |
| Class D | -0.76% | -1.14% | -1.15% | 127% | 103% | -0.71% | -1.05% | -1.08% | 116% | 102% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 (CE10.3.1 w/o encoder fast algorithm) |  |  |  |  | Over VTM-1.0 (CE10.3.1 w/ encoder fast algorithm) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.37% | -1.77% | -1.88% | 125% | 103% | -1.23% | -1.58% | -1.51% | 117% | 102% |
| Class C | -1.54% | -2.31% | -2.51% | 126% | 103% | -1.40% | -1.83% | -2.05% | 115% | 102% |
| Class E | -2.45% | -2.09% | -2.38% | 123% | 102% | -2.12% | -1.68% | -1.60% | 111% | 102% |
| Overall | -1.70% | -2.03% | -2.21% | 125% | 103% | -1.51% | -1.69% | -1.71% | 115% | 102% |
| Class D | -1.35% | -1.79% | -1.62% | 126% | 103% | -1.17% | -1.56% | -1.27% | 115% | 103% |

Table 4: Experimental results of CE10.3.2 with encoder fast algorithm using the VTM configuration

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 (CE10.3.2 w/o encoder fast algorithm) |  |  |  |  | Over VTM-1.0 (CE10.3.2 w/ encoder fast algorithm) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.70% | -1.65% | -1.56% | 114% | 102% | -0.65% | -1.12% | -1.14% | 106% | 100% |
| Class A2 | -0.99% | -1.53% | -1.39% | 117% | 103% | -0.93% | -1.49% | -1.64% | 107% | 100% |
| Class B | -0.95% | -1.20% | -1.17% | 119% | 103% | -0.87% | -1.16% | -1.11% | 108% | 100% |
| Class C | -1.12% | -1.51% | -1.68% | 121% | 103% | -1.06% | -1.30% | -1.51% | 109% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.95% | -1.44% | -1.43% | 118% | 103% | -0.89% | -1.25% | -1.33% | 108% | 100% |
| Class D | -0.64% | -0.90% | -0.91% | 123% | 103% | -0.60% | -0.95% | -1.05% | 110% | 99% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 (CE10.3.2 w/o encoder fast algorithm) |  |  |  |  | Over VTM-1.0 (CE10.3.2 w/ encoder fast algorithm) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.35% | -1.77% | -1.54% | 122% | 103% | -1.24% | -1.47% | -1.33% | 111% | 102% |
| Class C | -1.44% | -2.03% | -2.35% | 123% | 102% | -1.32% | -1.75% | -1.84% | 113% | 102% |
| Class E | -2.36% | -2.11% | -1.83% | 122% | 102% | -2.17% | -1.89% | -1.83% | 110% | 101% |
| Overall | -1.63% | -1.94% | -1.89% | 122% | 102% | -1.50% | -1.67% | -1.63% | 111% | 102% |
| Class D | -1.14% | -1.57% | -1.77% | 121% | 102% | -1.01% | -1.86% | -1.29% | 112% | 102% |

# Memory bandwidth

Table 5 shows the number of reference samples required for a 128×128 CTU in the worst case scenario.

Table 5: Number of reference samples required for a 128×128 CTU in the worst case scenario

|  | worst case scenario | number of reference samples required |
| --- | --- | --- |
| VTM-1.0 | 4×4 bi-prediction | 299008 |
| BMS-1.0 | 4×4 bi-prediction with DMVR | 446464 |
| CE10.3.1 | 4×4 triangular prediction unit mode | 598016 |
| CE10.3.2 | 8×8 triangular prediction unit mode | 280576 |

# Conclusion

In this contribution, the triangular prediction unit mode is proposed and tested. Two tests of the triangular prediction unit mode achieve similar coding gains which are up to 1.7% in the LB configuration. According to the interesting results, we recommend to adopt the triangular prediction unit mode into the next version of VTM.

# References

- J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations”, JVET-J1010, San Diego, April 2018.

- C.-W. Hsu, M. Winken and X. Xiu, “Description of Core Experiment 10: Combined and multi-hypothesis prediction”, JVET-J1030, San Diego, April 2018.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).