| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0145-v2 |
| --- | --- |

| Title: | Non-CE1:On Transform Unit Partition-Uniform Transform Unit Structure |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianqing Zhu, Jie Yao, Wenting Cai, Kimihiko Kazui 601,Yongda International Tower, 2277 Longyang Road Shanghai, 201204, China | Tel: Email: | +86-21-63350606 ext. 5809 zhujianqing@cn.fujitsu.com yaojie@cn.fujitsu.com caiwenting@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |
| Source: | Fujitsu |  |  |

_____________________________

- Abstract

This contribution proposes uniform TU (UTU) structure. A CU can be partitioned into TUs uniformly, i.e. each TU in a CU has same size. A syntax element, utu_mode, is signalled in CU syntax. When utu_mode is zero, it means no partitioning. Otherwise, the value of utu_mode indicates partition structure.UTU structure is only applied on intra CU. Test results show gain of 0.65 % (Y), 1.26% (Cb) and 1.5% (Cr) in the case of UTU only performed on I-slice and Luma component.

Introduction

In the first version of VVC test model, VTM1.0[1], multi-type of trees are introduced in CU partitioning. And many kinds of partition schemes were studied in CE1. This contribution is also related to partitioning.

In HEVC, CU is further split into TU in Quad-tree structure. In VTM1.0, CU will not be split into TU except for implicit splits when the CU size is too large for the maximum transform size. However, it does not mean further TU partitioning is meaningless since HEVC has proved that TU partitioning has gain.

This contribution proposes uniform TU (UTU) structure, which is a simpler and more efficient (less flag bits for same partition) TU partitioning scheme than that in HEVC.

Proposed method

In UTU structure, a CU can partitioned into TUs uniformly, i.e. each TU in a CU has same size. Each TU is in square shape if split occurs.

For a square CU with size 2Nx2N, its partition can be as shown in Fig 1. The TU size can be 2Nx2N (no partition), NxN (partitioned into 4 TUs), N/2xN/2 (partitioned into 16 TUs), N/4xN/4  (partitioned into 64 TUs) and so on, till to allowed smallest TU size (4x4 in VTM1.0) .

Figure 1 UTU structure for squre CU

For a non-square CU with size 2mx2n (m>n) as shown in Figure2 (left), TU size can be no_split, 2nx2n (call it NxN. 2m-n TUs), 2n-1x2n-1 ( call it N/2xN/2. 4x2m-n TUs), 2n-2x2n-2 ( call it N/4xN/4. 16x2m-n TUs) and so on, till to allowed smallest TU size (4x4 in VTM1.0).

For a non-square CU with size 2nx2m (m>n) as shown in Figure2 (right), TU size can be no_split, 2nx2n (call it NxN. 2m-n TUs), 2n-1x2n-1 (call it N/2xN/2. 4x2m-n TUs), 2n-2x2n-2 ( call it N/4xN/4. 16x2m-n TUs) and so on, till to allowed smallest TU size (4x4 in HEVC).

Figure 2 UTU structure for non-squre CU

Use a syntax element, utu_mode, to indicate which structure is used in CU level syntax.

- utu_mode=0 means no partition; utu_mode=1 means NxN; utu_mode=2 means N/2xN/2; utu_mode=3 means N/4xN/4;and so on.

In this proposal, maximum value of utu_mode is  tested in several values.

Luma and chroma components can share a UTU structure or use independent UTU structure.

UTU structure can be applied on intra CU or inter CU. In this proposal, UTU structure is only applied on intra CU.

Binarization of utu_mode

A unary binarization process is applied for the utu_mode binarization (Table 1) and at most 3 bins are required since the largest possible value of utu_mode is set to 3. While the largest possible value of utu_mode may varies according to CU size. For example when CU size is 8x8, the maximum value of utu_mode changes to 1 since minimum TU size is 4x4. Table 2 lists such special cases when the utu_mode binarization process is CU size dependent.

| utu_mode | Bin string |  |  |
| --- | --- | --- | --- |
| 0 | 0 |  |  |
| 1 | 1 | 0 |  |
| 2 | 1 | 1 | 0 |
| 3 | 1 | 1 | 1 |
| binIdx | 0 | 1 | 2 |

Table 1. bin string of utu_mode binarization

| CU size | utu_mode |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 64x8 8x64 32x8 8x32 16x16 16x8 8x16 | 0 | 10 | 11 | X |
| 64x4 4x64 32x4 4x32 16x4 4x16 8x8 8x4 4x8 | 0 | 1 | X | X |
| 4x4 | X | X | X | X |

Table 2. CU size dependent cases of utu_mode binarization

The syntax of utu_mode is illustrated as following part in red.

| coding_unit( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if( slice_type != I ) { |  |
| pred_mode_flag | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| utu_mode[ x0 ][ y0 ] | ae(v) |
| [Ed. (BB): Intra prediction yet to be added, pending further specification development.] |  |
| } else { |  |
| [Ed. (BB): Inter prediction yet to be added, pending further specification development.] |  |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ) |  |
| cu_cbf | ae(v) |
| if( cu_cbf ) { |  |
| transform_tree( x0, y0, cbWidth, cbHeight ) |  |
| } |  |

| transform_tree( x0, y0, tbWidth, tbHeight ) { | Descriptor |
| --- | --- |
| if( tbWidth > MaxTbSizeY ¦ ¦ tbHeight > MaxTbSizeY ) { |  |
| trafoWidth = ( tbWidth > MaxTbSizeY ) ? (tbWidth / 2) : tbWidth |  |
| trafoHeight = ( tbHeight > MaxTbSizeY ) ? (tbHeight / 2) : tbHeight |  |
| transform_tree( x0, y0, trafoWidth, trafoHeight ) |  |
| if( tbWidth > MaxTbSizeY ) |  |
| transform_tree( x0 + trafoWidth, y0, trafoWidth, trafoHeight ) |  |
| if( tbHeight > MaxTbSizeY ) |  |
| transform_tree( x0, y0 + trafoHeight, trafoWidth, trafoHeight ) |  |
| if( tbWidth > MaxTbSizeY && tbHeight > MaxTbSizeY ) |  |
| transform_tree( x0 + trafoWidth, y0 + trafoHeight, trafoWidth, trafoHeight ) |  |
| } else if( utu_mode[ x0 ][ y0 ]) { |  |
| utuDepth = utu_mode[ x0 ][ y0 ] |  |
| utuSize = ( tbWidth == tbHeight ) ? tbWidth >> utuDepth : min( tbWidth, tbHeight ) >> ( utuDepth – 1 ) |  |
| utuNumWidth = tbWidth / utuSize |  |
| utuNumHeight = tbHeight / utuSize |  |
| for(j = 0; j < utuNumHeight; j++ ) |  |
| for(i = 0; i < utuNumWidth; i++ ) |  |
| transform_unit( x0 + i * utuSize, y0 + j * utuSize, utuSize, utuSize ) |  |
| } else { |  |
| transform_unit( x0, y0, tbWidth, tbHeight ) |  |
| } |  |
| } |  |

Speed-up of UTUS in RDO

Speed-up Method1:Early termination of utu_mode loop

The utu_mode loop will be terminated in advance once Rd-cost of current utu_mode becomes worse than that of previous one.

CheckUTUSBestMode() {

for (utu_mode =0; utu_mode<3; utu_mode++)

Rd_cost[utu_mode] = CheckUTUSCost(utu_mode);

if Rd_cost[utu_mode] > Rd_cost[utu_mode - 1]

break;

}

Speed-up Method2: Fast decision of pred_mode candidates

The best pred_mode of in cycles of utu_mode=0 and 1 will be saved. In cycles of utu_mode=2 and3, only check this pred_mode.

CheckUTUSBestMode() {

for (utu_mode =0; utu_mode<3; utu_mode++)

if (utu_mode ≤1)

while (pred_mode in full_mode_list)

CheckRdCost(pred_mode);

if (utu_mode == 1)

pred_mode_candidate = best_pred_mode;

else

CheckRdCost(pred_mode_candidate);

}

Experimental results

Test1 (UTU only on I-slice, Luma and chroma share a UTU structure, without Speed-up)

The anchor is VTM1.0. Test condition is JVET CTC on SDR[3].

Maximum value of utu_mode is 3.

Table3 Result of Test1 - all Intra

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.35% | 0.84% | 0.75% | 222% | 104% |
| Class A2 | -0.40% | 0.55% | 0.27% | 223% | 104% |
| Class B | -0.44% | -0.19% | -0.11% | 228% | 104% |
| Class C | -0.73% | -0.55% | -0.44% | 222% | 106% |
| Class E | -0.61% | -0.58% | -0.56% | 207% | 102% |
| Overall | -0.51% | -0.04% | -0.05% | 221% | 104% |
| Class D | -0.54% | -0.41% | -0.49% | 184% | 107% |

Table4 Result of Test2 - random access

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.10% | 0.19% | 107% | 102% |
| Class A2 | -0.01% | 0.39% | 0.25% | 107% | 102% |
| Class B | -0.09% | -0.24% | -0.31% | 108% | 103% |
| Class C | -0.25% | -0.27% | -0.31% | 116% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.06% | -0.10% | 110% | 102% |
| Class D | -0.15% | -0.38% | -0.44% | 128% | 108% |

Test2 (UTU only on I-slice and only on Luma, without Speed-up)

The anchor is VTM1.0. Test condition is JVET CTC on SDR[3].

Test2.1

Maximum value of utu_mode is 3.

Table5 Result of Test2.1 - all Intra

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.50% | -0.14% | -0.12% | 212% | 112% |
| Class A2 | -0.53% | -0.93% | -0.60% | 212% | 113% |
| Class B | -0.60% | -1.22% | -1.97% | 217% | 112% |
| Class C | -0.91% | -1.68% | -2.02% | 201% | 115% |
| Class E | -0.67% | -2.21% | -2.33% | 210% | 111% |
| Overall | -0.65% | -1.26% | -1.50% | 210% | 113% |
| Class D | -0.69% | -1.58% | -1.75% | 194% | 114% |

Table6 Result of Test2.1 - random access

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.13% | -0.11% | 105% | 102% |
| Class A2 | -0.07% | -0.45% | -0.23% | 105% | 104% |
| Class B | -0.19% | -1.06% | -2.07% | 106% | 103% |
| Class C | -0.33% | -0.73% | -0.98% | 110% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.17% | -0.66% | -1.02% | 107% | 102% |
| Class D | -0.21% | -1.28% | -1.58% | 114% | 105% |

Test2.2

Maximum value of utu_mode is 2.

Table7 Result of Test2.2 - all Intra

|  | Over VTM-1.0 |  |  |  |  | Over utu_mode=0,1,2,3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |  | Y | U | V | EncT | DecT |  |
| Class A1 | -0.48% | -0.18% | -0.08% | 199% | 110% |  | 0.01% | -0.03% | 0.04% | 94% | 99% |  |
| Class A2 | -0.52% | -0.95% | -0.58% | 199% | 111% |  | 0.00% | -0.02% | 0.02% | 94% | 98% |  |
| Class B | -0.60% | -1.28% | -1.97% | 200% | 111% |  | 0.01% | -0.06% | 0.00% | 92% | 99% |  |
| Class C | -0.91% | -1.58% | -1.98% | 174% | 114% |  | 0.00% | 0.10% | 0.04% | 87% | 99% |  |
| Class E | -0.69% | -2.19% | -2.33% | 176% | 108% |  | -0.02% | 0.02% | 0.00% | 84% | 97% |  |
| Overall | -0.65% | -1.26% | -1.49% | 190% | 111% |  | 0.00% | 0.00% | 0.02% | 90% | 98% |  |
| Class D | -0.69% | -1.59% | -1.75% | 145% | 113% |  | 0.00% | -0.01% | 0.00% | 75% | 99% |  |

Test2.3

Maximum value of utu_mode is 1.

Table8 Result of Test2.3 - all Intra

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.41% | -0.13% | -0.14% | 161% | 109% |
| Class A2 | -0.45% | -0.77% | -0.49% | 162% | 110% |
| Class B | -0.52% | -1.10% | -1.59% | 165% | 110% |
| Class C | -0.82% | -1.36% | -1.76% | 145% | 113% |
| Class E | -0.63% | -1.83% | -1.99% | 146% | 108% |
| Overall | -0.58% | -1.06% | -1.27% | 156% | 110% |
| Class D | -0.68% | -1.51% | -1.71% | 132% | 113% |

Test3 (UTU on all slices and only on Luma, without Speed-up)

For B/P slice, only intra CU uses UTU structure.

The anchor is VTM1.0. Test condition is JVET CTC on SDR[3].

Maximum value of utu_mode is 3.

Table9 Result of Test3 - all Intra

|  | Over VTM-1.0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.50% | -0.14% | -0.12% | 207% | 110% |
| Class A2 | -0.53% | -0.93% | -0.60% | 206% | 111% |
| Class B | -0.60% | -1.22% | -1.97% | 208% | 112% |
| Class C | -0.91% | -1.68% | -2.02% | 179% | 114% |
| Class E | -0.67% | -2.21% | -2.33% | 183% | 109% |
| Overall | -0.65% | -1.26% | -1.50% | 196% | 111% |
| Class D | -0.69% | -1.58% | -1.75% | 150% | 113% |

Table10 Result of Test3 - random access

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | 0.56% | 0.68% | 124% | 105% |
| Class A2 | -0.11% | -0.11% | 0.12% | 116% | 104% |
| Class B | -0.29% | -0.93% | -1.90% | 118% | 104% |
| Class C | -0.60% | -0.85% | -1.22% | 121% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.30% | -0.45% | -0.80% | 120% | 104% |
| Class D | -0.36% | -1.10% | -1.55% | 116% | 107% |

Test4 (UTU only on I-slice and only on Luma, with Speed-up method1)

The anchor is VTM1.0. Test condition is JVET CTC on SDR[3] but encoded frame number is 40.

Maximum value of utu_mode is 3.

Table11 Result of Test4 - all Intra

|  | Over VTM-1.0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.48% | -0.13% | -0.15% | 172% | 110% |
| Class A2 | -0.50% | -0.85% | -0.53% | 175% | 114% |
| Class B | -0.51% | -0.92% | -1.30% | 179% | 112% |
| Class C | -0.85% | -1.54% | -1.85% | 190% | 119% |
| Class E | -0.70% | -1.79% | -1.49% | 177% | 138% |
| Overall | -0.61% | -1.06% | -1.13% | 179% | 118% |
| Class D | -0.66% | -1.77% | -2.28% | 193% | 114% |

Test5 (UTU only on I-slice and only on Luma, with Speed-up method2)

The anchor is VTM1.0. Test condition is JVET CTC on SDR[3] but encoded frame number is 40.

Maximum value of utu_mode is 3.

Table12 Result of Test5 - all Intra

|  | Over VTM-1.0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.53% | -0.17% | 0.07% | 181% | 111% |
| Class A2 | -0.54% | -0.95% | -0.59% | 179% | 114% |
| Class B | -0.51% | -0.73% | -1.50% | 183% | 112% |
| Class C | -0.94% | -1.58% | -1.92% | 185% | 117% |
| Class E | -0.70% | -2.01% | -1.87% | 182% | 138% |
| Overall | -0.65% | -1.08% | -1.24% | 182% | 117% |
| Class D | -0.71% | -1.28% | -2.06% | 184% | 116% |

Test6 (UTU only on I-slice and only on Luma, with Speed-up method1+method2)

The anchor is VTM1.0. Test condition is JVET CTC on SDR[3] but encoded frame number is 40.

Maximum value of utu_mode is 3.

Table13 Result of Test6 - all Intra

|  |  | Over VTM-1.0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U | V | EncT | DecT |
| Class A1 | -0.48% |  | -0.21% | -0.09% | 170% | 110% |
| Class A2 | -0.48% |  | -0.88% | -0.55% | 172% | 112% |
| Class B | -0.48% |  | -1.07% | -1.41% | 175% | 111% |
| Class C | -0.86% |  | -1.39% | -1.89% | 180% | 116% |
| Class E | -0.66% |  | -1.93% | -1.68% | 175% | 139% |
| Overall | -0.59% |  | -1.11% | -1.20% | 175% | 116% |
| Class D | -0.70% |  | -1.83% | -2.04% | 181% | 114% |

References

- J. Chen, and E. Alshina, “Algorithm description for Versatile Video Coding and Test Model 1 (VTM 1),” JVET-J1002, San Diego, US, April. 2018.

- W. Pu, X. Guo, P. Onno, P. Lai, and J. Xu, “Description of Core Experiment: Partitioning,” J JVET-J1021, San Diego, US, April. 2018.

- J. Boyce, K. Suehring, X. Li, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, US, April. 2018.

Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).