| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–20 Jul. 2018 | Document: JVET-K0147 |
| --- | --- |

| Title: | CE10.1.10: Dual Merge Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang Yongbing Lin Quanhe Yu Jianhua Zheng | Tel: Email: | +86 18601967458 zhangna25@hisilicon.com zhengjianhua@hisilicon.com |
| Source: | HiSilicon Technologies Co., Ltd |  |  |

_____________________________

# Abstract

This contribution reports the results of CE10.1.10 for the dual merge mode. Simulation results show that compared to VTM-1.0, the proposed method can provide 3.41% BD bitrate savings in LP condition with encoding time increase of 10%, decoding time decrease of 1%, and compared to BMS-1.0, the proposed method can provide 1.67% BD bitrate savings in LP condition with the same encoding time, decoding time decrease of 5%.

# Introduction

In current VTM-1.0 and BMS-1.0, the set of possible candidates for the merge mode consists of spatial neighbour candidates, ATMVP candidates, a temporal candidate, and generated candidates.

It may not be effective that each reference list has only one set of motion information.

# Proposed Method

The dual merge mode allows each reference list to use two sets of motion information. It is determined by the SATD criterion whether each reference list uses two sets of motion information or one set of motion information and indicated with a flag. The new set of motion information for each reference list is obtained as follows:

RefX’= (RefX == 0) ? 1 : 0

MvX’ = dX’ * MVX /dX

As shown in Figure 1, the X represents the reference list 0 or the reference list 1. RefX represents the reference index of the original motion information in the reference list X. RefX' represents the reference index of the new set of motion information in the reference list X. Similarly, MVX represents the motion vector of the original motion information in the reference list X. MVX' represents the motion vector of the new set of motion information in the reference list X. dX represents the temporal distance between the current picture and the reference picture specified by the RefX. dX’ represents the temporal distance between the current picture and the reference picture specified by the RefX’.

Figure 1. The sketch map of dual merge mode

Original prediction value is

PredUni = PredX

PredBi=(Pred0+Pred1) >>1

Modified prediction value is

PredUni = (PredX+ PredX’) >>1

PredBi=( (Pred0+Pred1) >>1+ (Pred0’+Pred1’) >>1 ) >>1

where PredUni represents the prediction value of uni-prediction, PredBi represents the prediction value of bi-prediction, PredX represents the prediction value compensated by the original motion information of the reference list X, and PredX’ represents the prediction value compensated by the new set of motion information in the reference list X.

For the merge mode, the original motion information can be the motion information of each candidate.

A flag under merge mode is signalled to indicate whether the dual merge mode is used.

This scheme is used only in low delay P configuration, as shown in Figure 2.

Figure 2. The sketch map of dual merge mode in LDP

# Experimental Results

The proposed method has been implemented on BMS-1.0[1], and all the tests were conducted under the common test conditions [2]. The experimental results of the dual merge mode compared to VTM-1.0 and BMS-1.0 are shown in Table1.

Table 1. Experimental results of the dual merge mode

# Conclusion

This contribution proposes a dual merge mode. Simulation results show that compared to VTM-1.0, the proposed method can provide 3.41% BD bitrate savings in LP condition with encoding time increase of 10%, decoding time decrease of 1%, and compared to BMS-1.0, the proposed method can provide 1.67% BD bitrate savings in LP condition with the same encoding time, decoding time decrease of 5%.

# Reference

- BMS-1.0, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0

- J.Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, US, 10–20 Apr. 2018.

# Patent rights declaration(s)

HiSilicon Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).