| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 Jul. 2018 | Document: JVET-K0148-v2 |
| --- | --- |

| Title: | CE10 related: Combining multi-hypothesis prediction with triangular prediction unit mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao Chong Soon Lim | Tel: Email: | +65-6550-5346 ruling.liao@sg.panasonic.com chongsoon.lim@sg.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

This contribution provides test results for combining the multi-hypothesis prediction with the triangular prediction unit mode. Two tests of multi-hypothesis prediction, the CE10.1.4 proposed by MediaTek and the CE10.1.8 proposed by Fraunhofer HHI, are used to combine with the CE10.3.2 triangular prediction unit mode. Two different combination are tested and their coding results are reported as follows:

- CE10.1.4 plus CE10.3.2:

  - (VTM configuration) (RA) -1.88% BD-rate with 139% encoding time and 107% decoding time

(LB) -2.04% BD-rate with 149% encoding time and 106% decoding time

- CE10.1.8 plus CE10.3.2:

  - (VTM configuration) (RA) -2.55% BD-rate with 150% encoding time and 106% decoding time

(LB) -3.75% BD-rate with 169% encoding time and 106% decoding time

# Introduction

In this contribution, the combination of the multi-hypothesis prediction and the triangular prediction unit mode is tested. The CE10.1.4, CE10.1.8 and CE10.3.2 software [2] are used and directly integrated together. A CU can be coded in either the multi-hypothesis prediction mode or the triangular prediction unit mode. Besides, the maximum hypothesis for a CU is set to 4. Note that in case of the triangular prediction unit mode, the additional prediction parameters (i.e. multi-hypothesis prediction parameter) of the selected merging candidates cannot be used for the current CU.

# Experimental Results

The following experiments were conducted using the BMS-1.0 software with the VTM configuration and the JVET common test conditions [1] to collect the BD-rate, encoding and decoding time of the proposed techniques in the Random Access and Low Delay B configurations. Two different combination are tested:

- CE10.1.4 plus CE10.3.2;

- CE10.1.8 plus CE10.3.2.

Results are summarized in Table 1 and Table 2, respectively.

Table 1: Experimental results of combining CE10.1.4 and CE10.3.2 using the VTM configuration

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 (CE10.1.4) |  |  |  |  | Over VTM-1.0 (CE10.3.2) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.19% | -2.16% | -1.70% | 115% | 107% | -0.70% | -1.65% | -1.56% | 114% | 102% |
| Class A2 | -0.94% | -1.17% | -0.92% | 118% | 106% | -0.99% | -1.53% | -1.39% | 117% | 103% |
| Class B | -1.90% | -1.74% | -1.54% | 119% | 107% | -0.95% | -1.20% | -1.17% | 119% | 103% |
| Class C | -0.86% | -0.76% | -0.95% | 123% | 104% | -1.12% | -1.51% | -1.68% | 121% | 103% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.29% | -1.45% | -1.29% | 119% | 106% | -0.95% | -1.44% | -1.43% | 118% | 103% |
| Class D | -0.32% | -0.20% | -0.28% | 127% | 105% | -0.64% | -0.90% | -0.91% | 123% | 103% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 (CE10.1.4 + CE10.3.2) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.71% | -3.16% | -2.86% | 130% | 108% |
| Class A2 | -1.63% | -2.21% | -1.99% | 137% | 107% |
| Class B | -2.39% | -2.43% | -2.33% | 140% | 108% |
| Class C | -1.58% | -2.11% | -2.33% | 145% | 106% |
| Class E |  |  |  |  |  |
| Overall | -1.88% | -2.45% | -2.37% | 139% | 107% |
| Class D | -0.71% | -0.91% | -0.89% | 151% | 105% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 (CE10.1.4) |  |  |  |  | Over VTM-1.0 (CE10.3.2) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.18% | -1.76% | -1.76% | 125% | 106% | -1.35% | -1.77% | -1.54% | 122% | 103% |
| Class C | -0.68% | -1.12% | -1.30% | 126% | 103% | -1.44% | -2.03% | -2.35% | 123% | 102% |
| Class E | -0.17% | -0.87% | -1.54% | 128% | 105% | -2.36% | -2.11% | -1.83% | 122% | 102% |
| Overall | -0.76% | -1.32% | -1.55% | 126% | 105% | -1.63% | -1.94% | -1.89% | 122% | 102% |
| Class D | -0.55% | -1.31% | -1.46% | 127% | 105% | -1.14% | -1.57% | -1.77% | 121% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 (CE10.1.4 + CE10.3.2) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.08% | -3.15% | -3.26% | 148% | 106% |
| Class C | -1.77% | -2.64% | -3.23% | 149% | 106% |
| Class E | -2.31% | -2.60% | -3.00% | 150% | 106% |
| Overall | -2.04% | -2.84% | -3.18% | 149% | 106% |
| Class D | -1.36% | -2.63% | -2.68% | 149% | 105% |

Table 2: Experimental results of combining CE10.1.8 and CE10.3.2 using the VTM configuration

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 (CE10.1.8) |  |  |  |  | Over VTM-1.0 (CE10.3.2) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.96% | -1.15% | -1.25% | 119% | 101% | -0.70% | -1.65% | -1.56% | 114% | 102% |
| Class A2 | -1.21% | -0.87% | -0.72% | 124% | 103% | -0.99% | -1.53% | -1.39% | 117% | 103% |
| Class B | -2.77% | -1.86% | -1.63% | 126% | 110% | -0.95% | -1.20% | -1.17% | 119% | 103% |
| Class C | -1.80% | -0.98% | -1.02% | 129% | 102% | -1.12% | -1.51% | -1.68% | 121% | 103% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.84% | -1.29% | -1.21% | 125% | 105% | -0.95% | -1.44% | -1.43% | 118% | 103% |
| Class D | -2.09% | -0.52% | -0.69% | 131% | 101% | -0.64% | -0.90% | -0.91% | 123% | 103% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 (CE10.1.8 + CE10.3.2) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.53% | -2.41% | -2.60% | 137% | 104% |
| Class A2 | -1.99% | -2.25% | -2.04% | 146% | 105% |
| Class B | -3.34% | -2.78% | -2.67% | 153% | 107% |
| Class C | -2.74% | -2.44% | -2.63% | 160% | 106% |
| Class E |  |  |  |  |  |
| Overall | -2.55% | -2.51% | -2.52% | 150% | 106% |
| Class D | -2.68% | -1.48% | -1.63% | 165% | 105% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 (CE10.1.8) |  |  |  |  | Over VTM-1.0 (CE10.3.2) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.47% | -1.14% | -1.00% | 135% | 106% | -1.35% | -1.77% | -1.54% | 122% | 103% |
| Class C | -2.05% | -0.75% | -1.04% | 135% | 110% | -1.44% | -2.03% | -2.35% | 123% | 102% |
| Class E | -2.22% | 1.60% | 1.12% | 148% | 109% | -2.36% | -2.11% | -1.83% | 122% | 102% |
| Overall | -2.27% | -0.32% | -0.48% | 138% | 108% | -1.63% | -1.94% | -1.89% | 122% | 102% |
| Class D | -1.39% | -0.63% | -0.97% | 135% | 105% | -1.14% | -1.57% | -1.77% | 121% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 (CE10.1.8 + CE10.3.2) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -3.54% | -2.76% | -2.68% | 163% | 106% |
| Class C | -3.43% | -2.96% | -3.44% | 168% | 106% |
| Class E | -4.54% | -0.63% | -0.92% | 182% | 106% |
| Overall | -3.75% | -2.29% | -2.49% | 169% | 106% |
| Class D | -2.45% | -2.09% | -2.05% | 166% | 104% |

# Conclusion

In this contribution, the combination of the multi-hypothesis prediction and the triangular prediction unit mode is tested. According to the coding results, it is observed that the multi-hypothesis prediction is independent of the triangular prediction unit mode.

# References

- J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations”, JVET-J1010, San Diego, April 2018.

- C.-W. Hsu, M. Winken and X. Xiu, “Description of Core Experiment 10: Combined and multi-hypothesis prediction”, JVET-J1030, San Diego, April 2018.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).