| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0150-v1 |
| --- | --- |

| Title: | CE1: Split restriction for narrow TT block (Test 3.0.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Boon Teo Chong Soon Lim | Tel: Email: | +65 6550 5417 hanboon.teo@sg.panasonic.com chongsoon.lim@sg.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

This contribution presents the results of CE1 test 3.0.3. The proposed technique restricts the TT block to reach very narrow size (block width > twice block height or block height > twice block width). It is reported that the proposed method provides 6% encoding complexity reduction with 0.07% luma coding loss for RA and 5% encoding complexity reduction with 0.08% luma coding loss for LD using VTM configurations and 4% encoding complexity reduction with 0.08% luma coding loss for RA and 4% encoding complexity reduction with 0.01% luma coding gain for LD using BMS configurations. It is also reported that the proposed technique can help to remove the subjective quality artifacts caused by the narrow TT block.

# Introduction

Panasonic’s call for proposal (JVET-J0020, [1]) includes a feature to restrict TT partitioning. TT vertical partitioning is invalid when the block width is smaller than the larger value between MinBTTTSize and 16, or when the block height is greater than twice the block width. When TT vertical partitioning is invalid, TT_HOR (TT horizontal partitioning) is automatically chosen. TT horizontal partitioning is invalid when the block height is smaller than the larger value between MinBTTTSize and 16 or when the block width is greater than twice the block height. When TT horizontal partitioning is invalid, TT_VER (TT vertical partitioning) is automatically chosen.

Figure 1: An example of narrow horizontal TT block

# Simulation results

The simulation was conducted using the BMS-1.0 software with VTM and BMS configurations and the JVET common test conditions [2]. For these simulations, the configuration files were additionally set with SEIDecodedPictureHash=1 and SIMD=SSE42.

It is also noted that All Intra setting was not simulated because narrow TT conditions are not satisfied in Intra slices.

Table 1: Random Access Main10 results of CE1 test 3.0.3 using VTM configurations

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.06% | 0.07% | 91% | 100% |
| Class A2 | 0.14% | 0.22% | 0.19% | 93% | 100% |
| Class B | 0.07% | 0.17% | 0.19% | 95% | 100% |
| Class C | 0.02% | 0.02% | 0.02% | 96% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.12% | 0.12% | 94% | 100% |
| Class D | 0.01% | 0.01% | 0.04% | 97% | 100% |

Table 2: Low delay B Main10 results of CE1 test 3.0.3 using VTM configurations

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | 0.35% | 0.54% | 94% | 100% |
| Class C | 0.07% | 0.04% | 0.34% | 96% | 100% |
| Class E | 0.15% | 0.40% | 0.23% | 95% | 100% |
| Overall | 0.08% | 0.26% | 0.40% | 95% | 100% |
| Class D | 0.01% | -0.19% | -0.09% | 97% | 100% |

Table 3: Random Access Main10 results of CE1 test 3.0.3 using BMS configurations

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.05% | 0.12% | 95% | 100% |
| Class A2 | 0.18% | 0.20% | 0.22% | 95% | 100% |
| Class B | 0.04% | 0.15% | 0.07% | 97% | 100% |
| Class C | 0.08% | 0.07% | -0.02% | 98% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | 0.12% | 0.09% | 96% | 100% |
| Class D | 0.06% | -0.07% | 0.06% | 99% | 100% |

Table 4: Low delay B Main10 results of CE1 test 3.0.3 using BMS configurations

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | 0.06% | 0.46% | 95% | 101% |
| Class C | 0.00% | -0.01% | -0.21% | 97% | 100% |
| Class E | -0.14% | 0.12% | -0.25% | 97% | 99% |
| Overall | -0.01% | 0.05% | 0.06% | 96% | 100% |
| Class D | 0.04% | 0.18% | -0.51% | 98% | 99% |

# Effect on subjective quality

When a TT block is partitioned into narrow blocks, it is possible that some subjective artifacts will appear. An example of the artifact is shown in Figure 2. Due to the partition cost of a smaller partition of the TT block makeup of only ¼ of the total TT block costs, there is a small chance that the TT block with an artifact in one of the smaller partition is selected as the best partitioning block. When this happens, obvious subjective artifact appears.

Figure 2: Subjective artifact caused by narrow TT block partitioning

# Conclusions

In this contribution, the split restriction of narrow TT block is proposed and tested. On some occasions, narrow TT block causes subjective quality artifacts and by restricting narrow TT blocks, we can get 4% to 6% of encoding time savings with negligible effect on coding performance. We recommend adopting the split restriction of narrow TT blocks into the next version of VTM.

# References

- T. Toma, T. Nishi, K. Abe, R. Kanoh, C. Lim, J. Li, R. Liao, S. Pavan, H. Sun, H. Teo, V. Drugeon (Panasonic), “Description of SDR video coding technology proposal by Panasonic”, JVET-J0020.

- J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations”, JVET-J1010, San Diego, April 2018.

- J. Ma, F. Le Léannec, M. W. Park, “Description of Core Experiment: Partitioning”, JVET-J1021.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).