| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0151-v1 |
| --- | --- |

| Title: | CE2: Modified ALF classification (CE2-4.2.4) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaru Ikeda Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81 50 3750 2782 masaru.ikeda@sony.com teruhiko.s@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

Results are reported for JVET-J1022 [1] High-Efficiency ALF CE2-4.2.4. The experiments show that modified ALF classification saves the BD-Rate for Y, U and V by 3.10% (x%), 3.37% (x%) and 3.89% (x%), respectively, over VTM (BMS-ALF) with 249% (x%) decoding time at AI.

At RA, it saves the BD-Rate for Y, U and V by 5.33% (5.06%), 2.18% (2.21%) and 1.79% (1.50%) over VTM (BMS) with 343% (x%) decoding time.

At LDB, it saves the BD-Rate for Y, U and V by 4.58% (x%), 1.66% (x%) and 2.01% (x%) over VTM (BMS) with 259% (x%) decoding time.

Introduction

This technology, which was modified classification on top of GALF, was originally proposed as a part of JVET-J0028 [2] in the CfP response in San Diego meeting. There are two differences for the derivation of the directionality from GALF. One is derivation of direction and the other one is that similarity of the gradient is used for classification instead of the maximum and minimum values of the gradients. The detail is described in section 2.

Proposed Technology

Each 2x2 block is categorized into one out of 25 classes. The classification index C is derived based on its directionality D and a quantized value of activity , as follows:

|  | . | (46) |
| --- | --- | --- |

In the proposed classification, the derivation of the directionality is modified. The horizontal and vertical gradients are calculated using Sobel filter as follows:

Then the representative vector is calculated so that a given condition below is satisfied for gradient vectors within a certain window size (e.g. 6x6). According to , the direction and transformation are identified.

Then the similarity between the representative vector and each gradient vector within the window are calculated using the following inner product.

To derive the value of the directionality D, the value S is compared to two thresholds th1 and th2:

Step 1.	If S > th1 is true, D is set to 0.

Step 2.	If , D is set to 0; otherwise D is set to 1.

Step 3.	If , D is set to 4; otherwise D is set to 3.

The activity value  is calculated in the same way as GALF.

For both chroma components in a picture, no classification method is applied, i.e. a single set of ALF coefficients is applied for each chroma component.

Results

BD-Rate and run times for AI, RA and LDB over VTM-1.0 and BMS-1.0 - ALF are shown in 3.1 and 3.2, respectively. Parallel simulation was used for RA. The encoding time is not reliable since the simulation was conducted on different machines.

Over VTM-1.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.35% | -3.28% | -3.46% | 103% | 354% |
| Class A2 | -3.02% | -3.78% | -3.61% | 106% | 279% |
| Class B | -2.79% | -2.26% | -2.93% | 114% | 269% |
| Class C | -3.33% | -2.69% | -3.57% | 103% | 200% |
| Class D | -2.40% | -2.78% | -3.31% | 110% | 180% |
| Class E | -5.04% | -6.63% | -7.39% | 117% | 284% |
| Overall | -3.10% | -3.37% | -3.89% | 109% | 249% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Figure 1  Coding results of AI

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -4.34% | -2.61% | -1.51% | 100% | 466% |
| Class A2 | -6.44% | -1.38% | -0.31% | 103% | 449% |
| Class B | -5.91% | -3.38% | -2.25% | 118% | 369% |
| Class C | -4.57% | -1.06% | -1.95% | 114% | 270% |
| Class D | -5.29% | -2.07% | -2.38% | 146% | 257% |
| Class E |  |  |  |  |  |
| Overall (Ref) | -5.33% | -2.18% | -1.79% | 117% | 343% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Figure 2  Coding results of RA

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -4.67% | -1.55% | -1.48% | 130% | 329% |
| Class C | -4.37% | -1.73% | -1.84% | 114% | 237% |
| Class D | -4.30% | -1.55% | -1.91% | 133% | 217% |
| Class E | -5.10% | -1.91% | -3.25% | 129% | 249% |
| Overall (Ref) | -4.58% | -1.66% | -2.01% | 126% | 259% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Figure 3  Coding results of LDB

Over BMS-1.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0-ALF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.11% | -3.87% | -2.75% | 101% | #DIV/0! |
| Class A2 | -2.80% | -3.25% | -2.94% | 101% | #DIV/0! |
| Class B | -2.28% | -2.31% | -2.67% | 101% | #DIV/0! |
| Class C | -2.59% | -2.46% | -3.09% | 101% | #DIV/0! |
| Class D | -1.74% | -2.55% | -2.90% | 101% | #DIV/0! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class F (optional) | -1.61% | -1.32% | -1.21% | 101% | #DIV/0! |

Figure 4  Coding results of AI

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0-ALF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -4.03% | -2.82% | -0.37% | 101% | #DIV/0! |
| Class A2 | -6.35% | -0.81% | -0.38% | 103% | #DIV/0! |
| Class B | -5.50% | -3.53% | -2.19% | 103% | #DIV/0! |
| Class C | -4.24% | -1.16% | -1.60% | 105% | #DIV/0! |
| Class D | -5.14% | -2.19% | -2.22% | 107% | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall (Ref) | -5.06% | -2.21% | -1.50% | 104% | #DIV/0! |
| Class F (optional) | -1.81% | -1.17% | -1.13% | 110% | #DIV/0! |

Figure 5  Coding results of RA

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0-ALF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -4.47% | -1.55% | -1.26% | 101% | #DIV/0! |
| Class C | -4.33% | -2.19% | -2.29% | 101% | #DIV/0! |
| Class D | -4.64% | -1.99% | -1.96% | 102% | #DIV/0! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Overall (Ref) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class F (optional) | -3.66% | -1.60% | -1.62% | 105% | #DIV/0! |

Figure 6  Coding results of LDB

Conclusion

It is suggested that the modification is further studied in CE since the coding performance is not so changed and the decoder complexity is higher than for GALF.

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

References

- “Description of Core Experiment 2 (CE2): In-Loop Filters”, JVET-J1022.

- “Description of SDR and HDR video coding technology proposal by Sony”, JVET-J1028.