| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0154-v1 |
| --- | --- |

| Title: | On encoding distortion evaluation of VTM/BMS software |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomohiro Ikai | Tel: Email: | +81-43-299-8526 chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

The bit depth of previous test sequences is 8-bit, and it is effective to evaluate the 8-bit distortion when 10-bit coding using IBDI. However, the bit depth of most of test sequences is 10-bit, and it is considered that 10-bit distortion evaluation should be appropriate for those sequences. In this contribution, it is proposed to fix BMS software to use 10-bit distortion. All changes are encoder only where the major points in the software are lambda setting and distortion accumulator. The experimental results reportedly show there are some gains on average for most of sequences and there are large chroma gains for HDR PQ sequences. It is recommended to use this fix for the next VVC software and CTC.

Introduction

The bitdepth of previous test sequences was 8-bit, and it is effective to evaluate the 8-bit distortion when 10-bit coding using IBDI [1]. The current VTM and BTM software architecture is based on HM software. The method of distortion evaluation is that was proposed by the software of JCTVC-A 127 [2] originally. Currently, the bitdepth of many test sequences is 10-bit and the bitdepth may be increased in the future. Although VTM software has been already cleaned up and FULL_NBIT macro is enable to set on, BMS software has several problems. Since in most of CEs, BMS software is used now, we would like to propose to be able to change BMS software to an arbitrary bitdepth evaluation. This is an encoding technology. In addition, we would like to recommend 10-bit distortion evaluation on CTC.

In the following sections, a summary of changes of the program and results of experiments are described.

Changes

In this contribution, we would like to propose several changes of BMS software. There are two main problems; lambda setting and distortion accumulation.

Lambda setting

In order to adjust the distortion evaluation, DISTORTION_PRECISION_ADJUSTMENT(x) macro is defined on current BMS software. We propose to change the argument of this define statement to the internal bitdepth, not the difference between the internal bitdepth and 8-bit and to define DISTORTION_ESTIMATION_BITS macro independently.

Therefore, we would like to change the following define statements in Typedef.h.

#define DISTORTION_LAMBDA_BUGFIX				1

#define DISTORTION_TYPE_BUGFIX				1

#define DISTORTION_ESTIMATION_BITS				10

#if DISTORTION_LAMBDA_BUGFIX

#define NON_PRECISION_ADJUSTMENT				0

#if NON_PRECISION_ADJUSTMENT

#define DISTORTION_PRECISION_ADJUSTMENT(x)		0

#else

#define DISTORTION_PRECISION_ADJUSTMENT(x)

((x>DISTORTION_ESTIMATION_BITS)? ((x)-DISTORTION_ESTIMATION_BITS) : 0)

#endif

#else

#if FULL_NBIT

#define DISTORTION_PRECISION_ADJUSTMENT(x)	0

#else

#define DISTORTION_PRECISION_ADJUSTMENT(x)	(x)

#endif

#endif

Since the argument of DISTORTION_PRECISION_ADJUSTMENT(x) is changed to the bitdepth and DISTORTION_ESTIMATION_BITS macro is defined, we can set arbitrary value of distortion evaluation.

NON_PRECISION_ADJUSTMENT macro is defined additionally. NON_PRECISION_ADJUSTMENT macro is the same as the meaning of FULL_NBIT macro. When NON_PRECISION_ADJUSTMENT is equal to 0 and both DISTORTION_ESTIMATION_BITS and InternalBitDepth is equal to 10, DISTORTION_PRECISION_ADJUSTMENT (10) is set to 0. Therefore, this case is the same as the meaning of FULL_NBIT macro, too.

In addition, since this does not affect the results on CTC because of range extension problems, it is not necessary immediately, but I would like to point out the following changes in RdCost.cpp.

#if DISTORTION_LAMBDA_BUGFIX

dLambda = 0.57 * pow(2.0, ((LOSSLESS_AND_MIXED_LOSSLESS_RD_COST_TEST_QP_PRIME – 12

+ 6 * ((bitDepths.recon[CHANNEL_TYPE_LUMA] - 8)

– DISTORTION_PRECISION_ADJUSTMENT(bitDepths.recon[CHANNEL_TYPE_LUMA]))) / 3.0));

#else

#if FULL_NBIT

dLambda = 0.57 * pow(2.0, ((LOSSLESS_AND_MIXED_LOSSLESS_RD_COST_TEST_QP_PRIME – 12) / 3.0));

#else

dLambda = 0.57 * pow(2.0, ((LOSSLESS_AND_MIXED_LOSSLESS_RD_COST_TEST_QP_PRIME – 12

- 6 * (bitDepths.recon[CHANNEL_TYPE_LUMA] - 8)) / 3.0));

#endif

#endif

Distortion accumulation

Second problem is the accumulator of distortion. In order to avoid the overflow of accumulator, it is necessary to change the variable from UInt to UInt64. Therefore, the following define statements are defined in Typedef.h.

#if DISTORTION_TYPE_BUGFIX

Typedef	UInt64		Distortion;

#else

#if FULL_NBIT

Typedef	UInt64		Distortion;

#else

Typedef	UInt		Distortion;

#endif

Since several BMS tools use UInt instead of Distortion, those parts should be changed. In addition, max and min values should be unified max and min of Distortion.

Other problems

Another problem is that some tools contain bugs. On the current BMS configuration, ALF is set to 2. There seem to be some bugs of filter coefficients related to bitdepth. In current ALF, both m_nInternalBitDepth and m_nInputBitDepth is set to 10, when InternalBitDepth is set on 10. Historically, m_nInputBitDepth is set on 8 for 8-bit sequence and 10 for 10-bit sequence respectively, when InputBitDepth was described on SPS. As a result, the relationship between m_nInputBitDepth and m_nBitIncrement is incorrect on the current BMS software.

For example, there are the following codes in AdaptiveLoopFilter.cpp.

#if DISTORTION_LAMBDA_BUGFIX

Int shift = (11 + m_nInternalBitDepth - 8);

#else

#if FULL_NBIT

Int shift = (11 + m_nBitIncrement + m_nInputBitDepth - 8);

#else

Int shift = (11 + m_nBitIncrement);

#endif

#endif

Since m_nBitIncrement has been already set to nInternalBitDepth-8, it is incorrect to be added to m_nInputBitDepth. Therefore, the precision of the filter coefficients should be consistent with the internal bitdepth.

There seem to be several bugs of tools with non-BMS configuration; however we will not discuss them at this time.

Experimental results

The simulation based on the both VTM and BMS configurations, JVET-J1010 [3] for SDR sequences and JVET-J1011 [4] for HDR sequences with only fixed Qp have been conducted.

SDR

VTM configuration

Table 1 Summary of experimental results of SDR sequences on VTM confiiguration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BTM-1.1(VTM) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.19% | -0.23% | 101% | 103% |
| Class A2 | -0.02% | -0.03% | 0.03% | 101% | 102% |
| Class B | -0.03% | 0.01% | -0.02% | 100% | 101% |
| Class C | -0.01% | 0.00% | -0.09% | 100% | 101% |
| Class E | -0.10% | -0.17% | -0.18% | 101% | 104% |
| Overall | -0.04% | -0.06% | -0.09% | 100% | 102% |
| Class D | -0.01% | 0.06% | 0.03% | 101% | 101% |
| Class F (optional) | -0.11% | -0.20% | -0.23% | 95% | 96% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over BTM-1.1(VTM) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.26% | -0.27% | 99% | 101% |
| Class A2 | -0.04% | -0.02% | -0.08% | 99% | 101% |
| Class B | 0.00% | 0.01% | -0.27% | 100% | 101% |
| Class C | -0.01% | 0.06% | 0.03% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.04% | -0.15% | 100% | 101% |
| Class D | 0.01% | 0.12% | -0.02% | 99% | 100% |
| Class F (optional) | -0.12% | -0.29% | -0.28% | 91% | 94% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over BTM-1.1(VTM) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.03% | 0.01% | 99% | 101% |
| Class C | 0.06% | 0.15% | 0.17% | 99% | 103% |
| Class E | -0.09% | -1.24% | 0.76% | 98% | 100% |
| Overall | -0.01% | -0.28% | 0.25% | 99% | 102% |
| Class D | 0.01% | -0.20% | -0.20% | 100% | 101% |
| Class F (optional) | -0.14% | -0.35% | -0.59% | 99% | 99% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over BTM-1.1(VTM) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.28% | -0.04% | 99% | 101% |
| Class C | -0.04% | 0.04% | -0.16% | 99% | 100% |
| Class E | -0.06% | -0.98% | 0.86% | 98% | 99% |
| Overall | -0.03% | -0.11% | 0.14% | 99% | 100% |
| Class D | -0.04% | -0.60% | 0.32% | 99% | 97% |
| Class F (optional) | -0.22% | -0.55% | -0.34% | 99% | 101% |

BMS configuration

Table 2 Summary of experimental results of SDR sequences on BMS confiiguration

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.1 (BTM) |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |
| Class A1 | -0.07% | -0.38% | -0.48% | 101% | 102% |  |
| Class A2 | -0.03% | 0.06% | 0.02% | 100% | 101% |  |
| Class B | -0.05% | 0.02% | -0.09% | 100% | 102% |  |
| Class C | -0.04% | 0.09% | -0.01% | 99% | 99% |  |
| Class E | -0.12% | -0.12% | -0.36% | 100% | 102% |  |
| Overall | -0.06% | -0.05% | -0.16% | 100% | 101% |  |
| Class D | -0.03% | 0.15% | 0.14% | 100% | 102% |  |
| Class F (optional) | -0.25% | -0.30% | -0.74% | 100% | 100% |  |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over BMS-1.1 (BTM) |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |
| Class A1 | -0.01% | -0.26% | -0.54% | 100% | 101% |  |
| Class A2 | -0.01% | 0.00% | 0.00% | 100% | 100% |  |
| Class B | -0.02% | -0.07% | -0.14% | 99% | 100% |  |
| Class C | 0.01% | 0.17% | -0.19% | 100% | 101% |  |
| Class E |  |  |  |  |  |  |
| Overall | -0.01% | -0.03% | -0.20% | 100% | 101% |  |
| Class D | 0.02% | 0.09% | 0.10% | 100% | 102% |  |
| Class F (optional) | -0.27% | -0.34% | -0.87% | 100% | 100% |  |
|  | Low delay B Main10 |  |  |  |  |  |
|  | Over BMS-1.1 (BTM) |  |  |  |  |  |
|  | Y | U | V | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.01% | 0.10% | -0.08% | 100% |  | 94% |
| Class C | -0.05% | -0.08% | -0.32% | 100% |  | 95% |
| Class E | -0.33% | -0.94% | -1.15% | 100% |  | 82% |
| Overall | -0.10% | -0.22% | -0.43% | 100% |  | 91% |
| Class D | 0.02% | 0.18% | 0.09% | 100% |  | 90% |
| Class F (optional) | -0.26% | -0.61% | -0.45% | 113% |  | 91% |
|  | Low delay P Main10 |  |  |  |  |  |
|  | Over BMS-1.1 (BTM) |  |  |  |  |  |
|  | Y | U | V | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.05% | -0.27% | -0.04% | 100% |  | 94% |
| Class C | 0.06% | 0.22% | 0.18% | 99% |  | 92% |
| Class E | 0.03% | -1.27% | -0.81% | 101% |  | 91% |
| Overall | 0.01% | -0.36% | -0.16% | 100% |  | 93% |
| Class D | 0.01% | 0.41% | 0.00% | 100% |  | 93% |
| Class F (optional) | -0.27% | -0.81% | -0.20% | 96% |  | 83% |

HDR

VTM configuration

Table 3 Summary of experimental results of HDR sequences on VTM confiiguration (Fixed Qp)

|  |  | All Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over BMS-1.1 WCG_EXT=1 (VTM) |  |  |  |  |  |
|  |  | BD-rate |  |  |  |  |  |
|  |  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| HDR-A | DayStreet |  |  |  | 0.0% | 0.0% | -0.3% |
|  | FlyingBirds |  |  |  | 0.0% | 0.0% | -0.4% |
|  | PeopleInShoppingCenter |  |  |  | 0.0% | 0.0% | -0.1% |
|  | SunsetBeach |  |  |  | 0.0% | 0.0% | -0.2% |
| HDR-B | Market | 0.0% | -3.3% | -8.6% | 0.0% | -3.0% | -6.9% |
|  | SunRise | 0.2% | -8.0% | -19.7% | 0.3% | -7.4% | -18.8% |
|  | ShowGirl2 | 0.0% | -4.3% | -9.9% | -0.1% | -4.4% | -8.8% |
|  | BalloonFestival | -0.1% | -1.5% | -4.1% | 0.0% | -1.2% | -3.4% |
|  | Hurdles | 0.0% | -2.6% | -3.9% | 0.1% | -2.4% | -3.8% |
|  | Starting | 0.0% | -2.3% | -3.9% | 0.1% | -1.6% | -3.1% |
|  | Cosmos1 | 0.0% | -0.8% | -6.6% | 0.0% | -0.7% | -6.4% |
|  | Average HDR-A |  |  |  | 0.0% | 0.0% | -0.2% |
|  | Average HDR-B | 0.0% | -3.3% | -8.1% | 0.1% | -2.9% | -7.3% |
|  | Average all |  |  |  | 0.0% | -1.9% | -4.7% |
|  | Enc Time[%] | 100% |  |  |  |  |  |
|  | Dec Time[%] | 102% |  |  |  |  |  |
|  |  | Random Access |  |  |  |  |  |
|  |  | Over BMS-1.1 WCG_EXT=1 (VTM) |  |  |  |  |  |
|  |  | BD-rate |  |  |  |  |  |
|  |  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| HDR-A | DayStreet |  |  |  | 0.0% | -0.3% | -0.8% |
|  | FlyingBirds |  |  |  | 0.0% | 0.1% | -0.1% |
|  | PeopleInShoppingCenter |  |  |  | 0.0% | 0.0% | -0.2% |
|  | SunsetBeach |  |  |  | 0.0% | 0.0% | -0.3% |
| HDR-B | Market | 0.1% | -5.0% | -11.0% | 0.0% | -4.4% | -8.9% |
|  | SunRise | 0.3% | -11.8% | -26.5% | 0.3% | -10.6% | -23.3% |
|  | ShowGirl2 | -0.1% | -11.4% | -16.8% | -0.2% | -12.0% | -18.1% |
|  | BalloonFestival | -0.1% | -2.9% | -5.7% | 0.0% | -2.3% | -4.8% |
|  | Hurdles | 0.0% | -3.2% | -4.4% | 0.1% | -2.7% | -4.3% |
|  | Starting | 0.0% | -3.7% | -5.7% | 0.1% | -2.4% | -4.6% |
|  | Cosmos1 | -0.1% | -1.7% | -9.9% | -0.1% | -1.3% | -9.0% |
|  | Average HDR-A |  |  |  | 0.0% | -0.1% | -0.3% |
|  | Average HDR-B | 0.0% | -5.7% | -11.4% | 0.0% | -5.1% | -10.4% |
|  | Average all |  |  |  | 0.0% | -3.3% | -6.8% |
|  | Enc Time[%] | 98% |  |  |  |  |  |
|  | Dec Time[%] | 101% |  |  |  |  |  |

BMS configuration

Table 4 Summary of experimental results of HDR sequences on BMS confiiguration (Fixed Qp)

|  |  | All Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over BMS-1.1 WCG_EXT=1 (BMS) |  |  |  |  |  |
|  |  | BD-rate |  |  |  |  |  |
|  |  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| HDR-A | DayStreet |  |  |  | 0.0% | -0.1% | -0.3% |
|  | FlyingBirds |  |  |  | 0.0% | -0.3% | -0.6% |
|  | PeopleInShoppingCenter |  |  |  | 0.0% | 0.0% | -0.1% |
|  | SunsetBeach |  |  |  | 0.0% | 0.0% | -0.5% |
| HDR-B | Market | 0.0% | -4.6% | -10.8% | 0.0% | -4.1% | -9.1% |
|  | SunRise | 0.3% | -14.9% | -29.4% | 0.4% | -12.1% | -27.9% |
|  | ShowGirl2 | 0.0% | -8.0% | -18.7% | -0.1% | -8.5% | -19.6% |
|  | BalloonFestival | -0.1% | -2.6% | -7.0% | 0.0% | -1.9% | -5.8% |
|  | Hurdles | 0.1% | -5.1% | -8.9% | 0.1% | -4.5% | -8.3% |
|  | Starting | 0.0% | -4.0% | -7.4% | 0.1% | -2.6% | -5.5% |
|  | Cosmos1 | 0.0% | -2.1% | -13.0% | 0.0% | -1.9% | -11.6% |
|  | Average HDR-A |  |  |  | 0.0% | -0.1% | -0.4% |
|  | Average HDR-B | 0.0% | -5.9% | -13.6% | 0.1% | -5.1% | -12.5% |
|  | Average all |  |  |  | 0.0% | -3.3% | -8.1% |
|  | Enc Time[%] | 100% |  |  |  |  |  |
|  | Dec Time[%] | 102% |  |  |  |  |  |
|  |  | Random Access |  |  |  |  |  |
|  |  | Over BMS-1.1 WCG_EXT=1 (BMS) |  |  |  |  |  |
|  |  | BD-rate |  |  |  |  |  |
|  |  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| HDR-A | DayStreet |  |  |  | 0.0% | -0.3% | -0.8% |
|  | FlyingBirds |  |  |  | 0.0% | 0.3% | -1.1% |
|  | PeopleInShoppingCenter |  |  |  | 0.0% | 0.1% | -0.2% |
|  | SunsetBeach |  |  |  | 0.0% | 0.1% | -0.6% |
| HDR-B | Market | 0.0% | -7.1% | -13.4% | 0.1% | -6.1% | -11.4% |
|  | SunRise | 0.2% | -19.3% | -36.8% | 0.4% | -15.8% | -32.5% |
|  | ShowGirl2 | -0.1% | -16.4% | -28.1% | -0.1% | -17.8% | -30.1% |
|  | BalloonFestival | -0.2% | -4.4% | -9.5% | -0.1% | -3.3% | -7.7% |
|  | Hurdles | 0.0% | -5.8% | -8.9% | 0.0% | -4.8% | -8.0% |
|  | Starting | -0.1% | -5.6% | -8.8% | 0.0% | -3.6% | -6.8% |
|  | Cosmos1 | -0.1% | -3.5% | -17.3% | -0.1% | -2.9% | -15.6% |
|  | Average HDR-A |  |  |  | 0.0% | 0.1% | -0.7% |
|  | Average HDR-B | 0.0% | -8.9% | -17.5% | 0.0% | -7.7% | -16.0% |
|  | Average all |  |  |  | 0.0% | -4.9% | -10.4% |
|  | Enc Time[%] | 100% |  |  |  |  |  |
|  | Dec Time[%] | 97% |  |  |  |  |  |

Conclusion

In this contribution, an improvement of encoding technology for BMS software has been proposed. As experimental results, there are small gains on average for SDR and HLG sequences and there are large chroma gains for PQ sequences. The encoding and decoding times has not been changed.

We would like to recommend that BMS software should be changed to an arbitrary bitdepth evaluation and 10-bit distortion evaluation should be introduced to CTC.

Reference

[1]	T. Chujoh and R. Noda, Internal bit depth increase for coding efficiency, ITU-T SG16 Q6 Document, VCEG-AE13, Marrakech, Jan. 2007.

[2]	K. McCann, W.-J. Han and I.-K. Kim, Samsung’s Response to the Call for Proposals on Video Compression Technology, JCTVC-A124.

[3]	J. Boyce, K. Sühring, X. Li and V. Seregin, JVET common test conditions and software reference configurations, Joint Video Experts Team Document, JVET-J1010, San Diego, Apr. 2018.

[4]	A. Segall, E. François and D. Rusanovskyy, JVET common test conditions and evaluation procedures for HDR/WCG video, Joint Video Experts Team Document, JVET-J1011, San Diego, Apr. 2018.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).