| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0155-v1 |
| --- | --- |

| Title: | AHG12: Flexible Tile Partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yukinobu Yasugi Tomohiro Ikai 1-9-2, Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | +81-43-299-8526 yasugi.yukinobu@sharp.co.jp ikai.tomohiro@sharp.cojp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution proposes a tile functionality that allows to split pictures into flexible partitioning tile, where the width or height of the unit of tile can be multiplies of 4 (the minimum CU size), 8, 16, 32 and 64, i.e. smaller than CTU. In the proposed tile, pictures are split into constant size CTUs as the conventional tile while the size of the right most and bottom most CTUs in tile boundary can be different from the constant CTU size. This flexible feature provides better load balancing since all tile can be almost the same size in uniform spacing mode. Furthermore, it is also asserted that this feature is useful for 360 video sequences or frame packing sequences since the corresponding tile can fit the arbitrary face size.

The experimental results with this feature (unit=32) reportedly show that luma BD-rate coding losses on average are 0.66 % for All Intra and 0.99 % for Random Access under the common test condition (CTC) for SDR sequences.

The experimental results without this feature (i.e. HEVC like tile) reportedly show that luma BD-rate coding losses on average are 0.64 % for All Intra and 0.97 % for Random Access under the common test condition (CTC) for SDR sequences.

Introduction

The size of CTU has become larger, such as 64x64, 128x128 and even 256x256, as the resolution of image has increased. At the same time, the granularity of tile segmentation has also become coarser, i.e. larger. However, pictures that are made of compositing smaller pictures, for example, frame packing images and 360 contents, are not always layout along to CTU boundaries. With those type of contents, the conventional tile segmentation by CTU is insufficient for tile to fit frame packing boundaries. In addition, it is sometimes difficult to split a picture into the equal size of CTUs for load balance by the conventional tile.

In this contribution, we introduced a novel tile splitting method with fine granularity unit.

Technical description

Figure 1 shows an example of conventional 2x2 tile provided that the syntax element  uniform_spacing_flag is equal to 1. In this case, the picture is split into four tiles along the boundaries of the maximum CTB size, and the shape and number of CTUs for the resulting tiles are different each other. The odd CTUs appear only in the right and bottom edge of the picture. Figure 2 shows an example of the proposed tile segmentation. The picture is split into the same size tiles using boundaries of smaller blocks than CTU. There are CTUs of odd size in the right and bottom edge of each tile. The odd CTUs are encoded and decoded as same as in picture boundary. The conversion between the location of the top-left luma sample of each CTU and the raster scan address of CTU CtuRsAddr is done by using pre-calculated look-up tables.

Figure 2: Example of 2x2 segmentation by using boundaries of smaller units
 (uniform_spacing_flag = 1)

Figure 1: Example of segmentation by conventional 2x2 tiles
 (uniform_spacing_flag=1)

Text

Coding tree block raster and tile scanning conversion process

The list colWidth[ i ] for i ranging from 0 to num_tile_columns_minus1, inclusive, specifying the width of the i-th tile column in units of tile unit blocks, is derived as follows:

if ( tile_unit_size_idc > 0 )

TileUnitSizeY = MinCbSizeY << (tile_unit_size_idc  1)

else

TileUnitSizeY = CtbSizeY

PicWidthInTileUnitsY = Ceil( pic_width_in_luma_samples ÷ TileUnitSizeY

if ( uniform_spacing_flag )

for ( i = 0; i <= num_tile_columns_minus1; i++ ) {

posR = ( ( i + 1 ) * PicWidthInTileUnitsY ) / ( num_tile_columns_minus1 + 1 )

posL =  ( i     * PicWidthInTileUnitsY ) / ( num_tile_columns_minus1 + 1 )

colWidth[ i ] = min(PicWidthInTileUnitsY – posL, posR – posL)

}

else {

colWidth[ num_tile_columns_minus1 ] = PicWidhInTileUnitsY

for ( i = 0; i <= num_tile_columns_minus1; i++ ) {

colWidth[ i ] = column_width_minus1[ i ] + 1

colWidth[ num_tile_columns_minus1 ] = colWidth[ i ]

}

}

The list rowHeight[ j ] for j ranging from 0 to num_tile_rows_minus1, inclusive, specifying the height of the j-th tile row in units of tile unit blocks, is derived as follows:

if ( uniform_spacing_flag )

for ( j = 0; j <= num_tile_rows_minus1; j++ ) {

posB = ( ( j + 1 ) * PicHeightInTileUnitsY ) / ( num_tile_rows_minus1 + 1 )

posT =  ( j     * PicHeightInTileUnitsY ) / ( num_tile_rows_minus1 + 1 )

rowHeight [ j ] = min(PicHeightInTileUnitsY – posT, posB – posT )

}

else {

rowHeight [ num_tile_rows_minus1 ] = PicHeightInTileUnitsY

for ( j = 0; j <= num_tile_rows_minus1; j++ ) {

rowHeight [ j ] = row_height_minus1[ j ] + 1

rowHeight[ num_tile_rows_minus1 ] = rowHeight [ j ]

}

}

The list colX[ i ] for i ranging from 0 to num_tile_columns_minus1 + 1, inclusive, specifying the X location of the top-left luma sample of the i-th tile column, is derived as follows:

for ( colX[ 0 ] = 0, i = 0; i <= num_tile_columns_minus1; i++ )

colX[ i + 1 ] = colX[ i ] + colWidth[ i ] * tileUnitSizeY

The list rowY[ j ] for j ranging from 0 to num_tile_rows_minus1 + 1, inclusive, specifying the Y location of the top-left luma sample of the j-th tile row, is derived as follows:

for ( rowY[ 0 ] = 0, j = 0; j <= num_tile_rows_minus1; j++ )

rowY[ j + 1 ] = rowY[ j ] + rowHeight [ j ] * tileUnitSizeY

The list colBd[ i ] for i ranging from 0 to num_tile_columns_minus1 + 1, inclulsive, specifying the location of the i-th tile column boundary in coding tree blocks, is derived as follows:

for ( colBd[ 0 ] = 0, i = 0; i <= num_tile_columns_minus1; i++ )

colBd[ i + 1 ] = colBd[ i ] + Ceil( colWidth[ i ] * tileUnitSizeY ÷ CtbSizeY )

The list rowBd[ j ] for j ranging from 0 to num_tile_rows_minus1 + 1, inclulsive, specifying the location of the j-th tile row boundary in units of coding tree blocks, is derived as follows:

for ( rowBd[ 0 ] = 0, j = 0; j <= num_tile_rows_minus1; j++ )

rowBd[ j + 1 ] = rowBd[ j ] + Ceil( rowHeight[ j ] * tileUnitSizeY ÷ CtbSizeY )

The variables PicWidthInCtbsY and PicHeightInCtbsY are derived as follows:

for(PicWidthInCtbsY = 0, i = 0; i <= num_tile_columns_minus1; i++ )

PicWidthInCtbsY += colBd[ i ]

for(PicHeightInCtbsY = 0, j = 0; j <= num_tile_rows_minus1; j++ )

PicHeightInCtbsY += rowBd[ j ]

Note that the summation of colBd[ i ] and rowBd[ j ] in this tile system are not always equal to Ceil( pic_width_in_luma_samples ÷ CtbSizeY ) and Ceil( pic_height_in_luma_samples ÷ CtbSizeY ), respectively.

Syntax and semantics

Coding tree unit syntax

| xCtbAddr = CtbAddrInRs % PicWidthInCtbsY |  |
| --- | --- |
| yCtbAddr = CtbAddrInRs / PicWidthInCtbsY |  |
| tileId = TileId[ CtbAddrRsToTs[ CtbAddrInRs ] ] |  |
| col = tileId % ( num_tile_columns_minus1 + 1 ) |  |
| row = tileId / ( num_tile_columns_minus1 + 1 ) |  |
| xCtb = colX[ col ] + (xCtbAddr – colBd[col]) << CtbLog2SizeY |  |
| yCtb = rowY[ row ] + (yCtbAddr – rowBd[row]) << CtbLog2SizeY |  |
| … |  |

General Picture Parameter set RBSP syntax

| … |  |
| --- | --- |
| if ( tile_enabled_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| tile_size_unit_idc | ue(v) |
| if ( !uniform_spacing_flag ) { |  |
| for ( i=0; i < num_tile_columns_minus1: i++ ) |  |
| column_width_minus1[i] | ue(v) |
| for ( i=0; i < num_tile_rows_minus1: i++ ) |  |
| row_height_minus1[i] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| … |  |

tile_size_unit_idc specifies that the unit size of tiles. The variable TileUnitSizeY is derived as follows:

if ( tile_unit_size_idc > 0 )

TileUnitSizeY = MinCbSizeY << (tile_unit_size_idc  1)

else

TileUnitSizeY = CtbSizeY

Note: In this contribution, the unified syntax element tile_unit_size_idc was implemented for simplicity instead of separate syntax elements, i.e. tile_unit_width_idc and tile_unit_height_idc. However, it is considered a little more effective to use them in some cases.

Experimental results

The proposed software was implemented on BMS software, with the following macro definitions.

HEVC_TOOLS = 1

HEVC_TILES_WPP = 1

(Other definitions for HEVC tools were set to zero.)

SHARP_TILE_EXTENSION = 1

CTC experiments

The authors implemented the proposed method on BMS-1.0 and tested the performance of the tile system for five test cases with 2x2 splitting and the uniform spacng mode enabled  (--NumTileColumnsMinus1=1 --NumTileRowsMinus1=1 --UniformSpacingFlag=1). The other conditions are in accordance with the JVET CTC for SDR. The experiments were carried out using BMS software with VTM configuration.

The tested five settings are as follows:

Test0: 128x128 block based tile (conventional)

Test1: 4x4 block based tile

Test2: 8x8 block based tile

Test3: 16x16 block based tile

Test4: 32x32 block based tile

The resulting size of 2x2 tiles for classA-C with each setting are described in Table 1.

Table 1: Resulting tile sizes in each test

|  | classA (3840x2160) | classB (1920x1080) | classC (832x480) |
| --- | --- | --- | --- |
| Test0 | (1920+1920) x (1024+1136) | (896+1024) x (512+568) | (384+448) x (256+224) |
| Test1 | (1920+1920) x (1080+1080) | (960+960) x (540+540) | (416+416) x (240+240) |
| Test2 | (1920+1920) x (1080+1080) | (960+960) x (536+544) | (416+416) x (240+240) |
| Test3 | (1920+1920) x (1072+1088) | (960+960) x (544+536) | (416+416) x (240+240) |
| Test4 | (1920+1920) x (1088+1072) | (960+960) x (544+536) | (416+416) x (224+256) |

Table 2 and Table 3	 show a summary of experimental results of the tests for IntraOnly and RandomAccess condition, respectively. We observed that the larger tile units (Test3 or Test4) have about the same performance of the conventional 128x128 CTU based tile (Test0).

Table 2: Summary of experimental results of IntraOnly condition

| All Intra Main 10 / Over VTM-1.0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Y BD-rate | Test0 (unit=128) | Test1 (unit=4) | Test2 (unit=8) | Test3 (unit=16) | Test4 (unit=32) |
| Class A1 | 0.38% | 0.73% | 0.73% | 0.41% | 0.40% |
| Class A2 | 0.24% | 0.43% | 0.43% | 0.21% | 0.25% |
| Class B | 0.43% | 1.24% | 0.48% | 0.46% | 0.46% |
| Class C | 0.86% | 0.93% | 0.93% | 0.93% | 0.84% |
| Class E | 1.34% | 1.86% | 1.86% | 1.41% | 1.41% |
| Overall | 0.64% | 1.06% | 0.84% | 0.68% | 0.66% |
| Class D | 1.46% | 1.96% | 1.96% | 1.54% | 1.46% |
| Class F (optional) | 0.87% | 1.03% | 1.03% | 0.94% | 0.96% |

Table 3: Summary of experimental results of RandomAccess condition

| Random Access Main 10 / Over VTM-1.0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Y BD-rate | Test0 (unit=128) | Test1 (unit=4) | Test2 (unit=8) | Test3 (unit=16) | Test4 (unit=32) |
| Class A1 | 0.70% | 2.11% | 2.11% | 0.65% | 0.61% |
| Class A2 | 0.46% | 1.32% | 1.32% | 0.72% | 0.50% |
| Class B | 0.99% | 2.78% | 1.10% | 1.02% | 1.02% |
| Class C | 1.55% | 2.32% | 2.32% | 2.32% | 1.63% |
| Class E |  |  |  |  |  |
| Overall | 0.97% | 2.23% | 1.67% | 1.23% | 0.99% |
| Class D | 2.92% | 4.91% | 4.91% | 3.21% | 2.95% |
| Class F (optional) | 1.62% | 2.03% | 2.03% | 1.79% | 1.73% |

Basically, the proposed method increases odd size CTUs like picture boundary and this could causes loss, especially when the smaller tile unit is used in continuous regions. If any boundary partitioning tool is implemented in VTM/BMS software, it would work well together with the proposed tile system and the loss would be reduced.

Conclusion

This contribution describes an extension of the conventional tile that provides more flexible fitting to frame-packed videos or better load balancing. The proposed tile method uses odd-size CTUs in order to fit to boundaries of packed regions or to split frame as equally as possible. We propose studying them in Ad-Hoc Group 12.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).