| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0156-v2 |
| --- | --- |

| Title: | CE11: Results on composite reference picture (test 11.1.1, 11.1.2, 11.1.3, 11.1.4, 11.2.1 and 11.2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weiran Li, Xiaozhen Zheng DJI | Tel: Email: | xiaozhen.zheng@dji.com |
| Source: | SZ DJI Technology Co., Ltd., |  |  |

_____________________________

- Abstract

This contribution proposes to add composite reference as additional reference picture in VTM and BMS platform. The technique is studied in CE11. This contribution reports the DJI’s test results on test 1 and test 2 follow common test condition. In test 1, simulations show that the proposed technique can achieve up to 1.05% to 2.15% BD rate reduction on average at various refresh rates for low delay B (LB) Main10 configurations at VTM, and up to 1.27% to 2.68% BD rate reduction on average at various refresh rates for low delay B (LB) Main10 configurations at BMS. In test 2, the test results for random access (RA) can get 0.34% BD rate increase and 0.13% BD rate reduction when intra period is increased to two second and five second respectively.

Introduction

Composite reference picture was proposed in JVET Call for Proposal [1] and it was planned to study in CE11 [2]. In the CE11, test 1 and test 2 concentrate on the use of the proposed composite reference technique, and test 3 targets at the use of HEVC-like encoder-only mechanism.

The fundamental concept of composite reference picture is to partial update a long-term reference at CTU level. A refresh rate is used to control the total amount of CTUs which are used to update the composed reference. The refresh rate per frame is set to unlimited, 1/2, 1/8 and 1/10 per frame in test 1. Test 2 targets at the test of larger random access GOP whose IDR period is set to two seconds and five seconds. The best block update refresh rate conducted at test 1 is used as the default refresh rate at test 2.

Proposed method

General idea

Composite reference generates and updates a long-term reference by using information of static areas, which are always regarded as background, among a video scene.

Usually, background areas have few motion in a long temporal window. Therefore the blocks with minor difference between the background and current frame are picked up, and are used to replace the co-located blocks in the long-term reference that targets at the renewal of the background information.

When composite reference picture is enable, the reference lists of current coding frame will be modified. The composed long-term reference with POC equal to -1 will be added to the end of both reference list A and reference list B of the current frame.

If current frame is an IDR picture, all the CTUs in the slice will be marked to update the long-term reference, else if current slice is a P or a B frame, a control flag will be encoded at CTU level. If the value of the flag is equal to 1, the CTU will be marked to update the composed long-term reference.

After a frame is decoded and reconstructed completely, the process of updating the composed long-term reference will be performed. For every CTU marked to update the long-term reference, its Luma and Chroma reconstructed pixels will be used to replace the co-located pixels in the long-term reference, as illustrated in fig. 1.

Fig 1. Illustration of block-level long-term reference update process

In the decoding process, if composed long-term reference is used as reference, motion vector scaling and decoder motion refine operation that use motion trajectory are invalid because the distance between the composed long-term reference and the current frame is not available and motion trajectory model doesn’t work for this composed reference. Meanwhile, since this reference picture is composed from various coded frames, it’s unreasonable to perform motion vector refinement at such picture. Therefore, the tools like BIO、DMVR、FRUC are set to disable if any of the motion vector is referred to the composed long-term reference.

CTU selection strategy for composite reference update at encoder side

According to the principle of composite reference picture, IDR frame is used to update the composite reference. Regarding P and B frame, CTU-level similarity metric between current original frame and two previous original frames is calculated before the coding of current frame. Based on CTU-level similarity metric, a certain number of CTUs of current frame are used and marked to updated composed reference. A control parameter, which is called as refresh rate and denoted as R, is used at encoder side which imposes the limitation on the total amount of CTUs in current frame that can be used to update composed reference. The CTU selection strategy is described below.

Denoted current frame’s coding order as N. The original frames with coding order N-1 and N-2 are denoted as img_ref and img_ref2 respectively. For each CTU, the following parameters are calculated:

DistY: the sum up of absolute difference between CTU in current frame and co-located CTU in img_ref in Y component;

DistU: the sum up of absolute difference between CTU in current frame and co-located CTU in img_ref in U component;

DistV: the sum up of absolute difference between CTU in current frame and co-located CTU in img_ref in V component;

DistYRef2: the sum up of absolute difference between CTU in current frame and co-located CTU in img_ref2 in Y component;

DistURef2: the sum up of absolute difference between CTU in current frame and co-located CTU in img_ref2 in U component;

DistVRef2: the sum up of absolute difference between CTU in current frame and co-located CTU in img_ref2 in V component;

LargeDist: the number of Luma pixel whose absolute difference value between pixel in current CTU and co-located CTU located at img_ref or img_ref2 is larger than 200<<(bitdepth - 8)

PixCount: Number of Luma pixels in current CTU

If either following conditionA or conditionB is true, the current CTU is marked as a candidate to update composed long-term reference:

- conditionA: all of the following conditions are true

- LargeDist / PixCount < 0.01

- DistY / PixCount < (3.5 << (bitdepth - 8))

- DistU / PixCount < (0.5 << (bitdepth - 8))

- DistV / PixCount < (0.5 << (bitdepth - 8))

- conditionB: all of the following conditions are true

- LargeDist / PixCount < 0.02

- DistY / PixCount < (6 << (bitdepth - 8))

- DistU / PixCount < (0.5 << (bitdepth - 8))

- DistV / PixCount < (0.5 << (bitdepth - 8))

- DistYRef2 / PixCount < (6 << (bitdepth - 8))

- DistURef2 / PixCount < (0.5 << (bitdepth - 8))

- DistVRef2 / PixCount < (0.5 << (bitdepth - 8))

After pick-up all candidate CTU, order all the candidates by using followed cost function:

cost = DistY / PixCount + DistU / PixCount + DistV / PixCount

Choose up to R CTUs with smallest cost as updated CTUs, and use their reconstruction pixels to replace the co-located blocks in composed long-term reference.

Experimental results

The software is implemented based on BMS-1.0. And it is committed in SVN server [3]. The test on refresh rate (test 1) and test on different random access GOP (test 2) are evaluated. Meanwhile, several subtests with different configurations are also evaluation. The tests and subtests are summarized as below:

Test 1– Block update refresh rate per frame

| CE11.1.1 | No limitation on block update refresh |
| --- | --- |
| CE11.1.2 | 1/2 block update refresh rate |
| CE11.1.3 | 1/8 block update refresh rate |
| CE11.1.4 | 1/10 block update refresh rate |

Test 2– IDR GOP setting for random access

| CE11.2.1 | Intra period as two seconds based on best refresh rate at test 1 |
| --- | --- |
| CE11.2.2 | Intra period as five seconds based on best refresh rate at test 1 |

Each testing item is evaluated using BD rates, encoding time, decoding time according to Common Test Conditions (CTC) in [4].

Simulation environment

For the simulations, same platform machines as below in PC cluster were used.

- Encode

  - OS: Windows Sever 2008 R2 Enterprise

  - Compiler: Visual Studio 2017

  - CPU: Intel Xeon E5-2687W v4 3.0GHz

    - SIMD: AVX2 instruction is enabled.

  - Memory: 256 GB

- Decode

  - OS: Windows Sever 2008 R2 Enterprise

  - Compiler: Visual Studio 2017

  - CPU: Intel Xeon E5-2687W v4 3.0GHz

    - SIMD: AVX2 instruction is enabled.

  - Memory: 256 GB

Test 1

The results of test 1 with different refresh rate are summarized in the Table 1 to Table 16.

Table 1 : Over VTM-1.0 (refresh rate: unlimitation, LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.76% | -0.86% | -1.14% | 106% | 95% |
| Class C | -2.53% | -4.18% | -3.50% | 106% | 90% |
| Class E | 0.46% | 0.26% | 0.00% | 106% | 100% |
| Overall | -1.05% | -1.69% | -1.64% | 106% | 95% |
| Class D | 0.08% | -0.11% | 0.28% | 108% | 96% |

Table 2 : Over VTM-1.0 (refresh rate: unlimitation, RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.83% | 0.63% | 0.72% | 118% | 92% |
| Class A2 | 0.71% | 0.58% | 0.65% | 128% | 91% |
| Class B | 0.38% | -0.03% | 0.14% | 124% | 92% |
| Class C | 0.58% | 0.47% | 0.38% | 115% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.59% | 0.36% | 0.42% | 121% | 93% |
| Class D | 1.24% | 0.92% | 0.91% | 111% | 98% |

Table 3 : Over VTM-1.0 (refresh rate: 50%, LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.75% | -0.76% | -1.06% | 105% | 91% |
| Class C | -2.50% | -4.23% | -3.31% | 106% | 90% |
| Class E | -0.77% | -1.59% | -1.92% | 108% | 94% |
| Overall | -1.34% | -2.12% | -2.02% | 106% | 91% |
| Class D | 0.07% | -0.44% | 0.13% | 109% | 97% |

Table 4 : Over VTM-1.0 (refresh rate: 50%, RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.82% | 0.58% | 0.70% | 125% | 98% |
| Class A2 | 0.66% | 0.50% | 0.54% | 137% | 102% |
| Class B | 0.42% | -0.05% | 0.13% | 133% | 104% |
| Class C | 0.60% | 0.46% | 0.31% | 123% | 107% |
| Class E |  |  |  |  |  |
| Overall | 0.60% | 0.32% | 0.37% | 130% | 103% |
| Class D | 1.24% | 0.88% | 0.86% | 118% | 109% |

Table 5 : Over VTM-1.0 (refresh rate: 12.5%, LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.70% | -0.84% | -0.83% | 106% | 94% |
| Class C | -3.72% | -6.71% | -5.64% | 106% | 90% |
| Class E | -2.10% | -2.58% | -2.54% | 108% | 87% |
| Overall | -2.06% | -3.23% | -2.86% | 107% | 91% |
| Class D | 0.06% | -0.28% | 0.38% | 109% | 97% |

Table 6 : Over VTM-1.0 (refresh rate: 12.5%, RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.82% | 0.58% | 0.70% | 125% | 98% |
| Class A2 | 0.66% | 0.50% | 0.54% | 137% | 102% |
| Class B | 0.42% | -0.05% | 0.13% | 133% | 104% |
| Class C | 0.60% | 0.46% | 0.31% | 123% | 107% |
| Class E |  |  |  |  |  |
| Overall | 0.60% | 0.32% | 0.37% | 130% | 103% |
| Class D | 1.24% | 0.88% | 0.86% | 118% | 109% |

Table 7 : Over VTM-1.0 (refresh rate: 10%, LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.72% | -0.92% | -1.16% | 107% | 94% |
| Class C | -3.90% | -7.00% | -5.71% | 106% | 91% |
| Class E | -2.19% | -2.49% | -2.60% | 110% | 90% |
| Overall | -2.15% | -3.34% | -3.04% | 107% | 92% |
| Class D | 0.06% | -0.28% | 0.38% | 109% | 96% |

Table 8 : Over VTM-1.0 (refresh rate: 10%, RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.81% | 0.56% | 0.67% | 122% | 93% |
| Class A2 | 0.72% | 0.65% | 0.62% | 129% | 90% |
| Class B | 0.36% | -0.14% | 0.12% | 126% | 92% |
| Class C | 0.56% | 0.45% | 0.38% | 115% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.58% | 0.32% | 0.40% | 123% | 93% |
| Class D | 1.25% | 0.95% | 0.92% | 112% | 101% |

Table 9 : Over BMS-1.0 (refresh rate: unlimitation, LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.99% | -1.75% | -1.63% | 109% | 99% |
| Class C | -2.72% | -5.14% | -5.02% | 107% | 100% |
| Class E | 0.18% | -0.14% | -0.37% | 110% | 102% |
| Overall | -1.27% | -2.48% | -2.44% | 109% | 100% |
| Class D | -0.04% | 0.38% | -0.32% | 109% | 103% |

Table 10 : Over BMS-1.0 (refresh rate: unlimitation, RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.70% | 0.56% | 0.66% | 108% | 96% |
| Class A2 | 0.41% | 0.21% | 0.06% | 109% | 94% |
| Class B | 0.15% | -0.07% | 0.04% | 107% | 94% |
| Class C | 0.41% | 0.21% | 0.15% | 104% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.38% | 0.19% | 0.20% | 107% | 95% |
| Class D | 0.67% | 0.28% | 0.42% | 103% | 98% |

Table 11 : Over BMS-1.0 (refresh rate: 50%, LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.97% | -1.77% | -1.53% | 109% | 99% |
| Class C | -2.68% | -4.72% | -4.72% | 106% | 100% |
| Class E | -1.27% | -2.31% | -2.55% | 112% | 98% |
| Overall | -1.61% | -2.88% | -2.85% | 109% | 99% |
| Class D | 0.02% | -0.05% | -0.38% | 109% | 101% |

Table 12 : Over BMS-1.0 (refresh rate: 50%, RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.70% | 0.56% | 0.66% | 108% | 97% |
| Class A2 | 0.41% | 0.21% | 0.06% | 109% | 94% |
| Class B | 0.15% | -0.07% | 0.04% | 107% | 95% |
| Class C | 0.41% | 0.21% | 0.15% | 103% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.38% | 0.19% | 0.20% | 107% | 95% |
| Class D | 0.67% | 0.28% | 0.42% | 102% | 99% |

Table 13 : Over BMS-1.0 (refresh rate: 12.5%, LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.93% | -1.89% | -1.65% | 109% | 99% |
| Class C | -3.94% | -6.75% | -6.61% | 105% | 97% |
| Class E | -3.45% | -5.89% | -6.11% | 112% | 93% |
| Overall | -2.56% | -4.51% | -4.42% | 109% | 97% |
| Class D | -0.12% | -0.15% | -0.77% | 108% | 99% |

Table 14 : Over BMS-1.0 (refresh rate: 12.5%, RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.68% | 0.53% | 0.61% | 108% | 96% |
| Class A2 | 0.43% | 0.16% | 0.15% | 109% | 94% |
| Class B | 0.14% | -0.07% | 0.04% | 107% | 95% |
| Class C | 0.40% | 0.16% | 0.08% | 104% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.37% | 0.16% | 0.19% | 107% | 95% |
| Class D | 0.67% | 0.29% | 0.44% | 102% | 98% |

Table 15 : Over BMS-1.0 (refresh rate: 10%, LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.96% | -2.08% | -1.47% | 109% | 99% |
| Class C | -4.15% | -7.07% | -6.60% | 105% | 97% |
| Class E | -3.56% | -6.11% | -5.72% | 111% | 93% |
| Overall | -2.68% | -4.75% | -4.24% | 108% | 97% |
| Class D | -0.12% | -0.15% | -0.77% | 108% | 99% |

Table 16 : Over BMS-1.0 (refresh rate: 10%, RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.67% | 0.48% | 0.60% | 108% | 96% |
| Class A2 | 0.42% | 0.23% | 0.15% | 110% | 95% |
| Class B | 0.13% | -0.11% | 0.04% | 108% | 95% |
| Class C | 0.38% | 0.21% | 0.09% | 104% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.36% | 0.16% | 0.19% | 107% | 96% |
| Class D | 0.68% | 0.24% | 0.42% | 102% | 98% |

The average percentage of CTUs per frame that are used to update a composite reference is summarized at the below table:

|  |  | unlimit |  | 50% |  | 25% |  | 10% |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | update_rate |  | update_rate |  | update_rate |  | update_rate |  |
| MarketPlace_1920x1080_60fps_10bit_420 |  | 23.14% |  | 20.28% |  | 11.17% |  | 9.36% |  |
| RitualDance_1920x1080_60fps_10bit_420 |  | 20.69% |  | 20.68% |  | 11.29% |  | 9.41% |  |
| Cactus_1920x1080_50 |  | 18.37% |  | 18.37% |  | 11.85% |  | 9.63% |  |
| BasketballDrive_1920x1080_50 |  | 15.60% |  | 15.60% |  | 11.17% |  | 9.38% |  |
| BQTerrace_1920x1080_60 |  | 13.44% |  | 13.40% |  | 7.34% |  | 6.74% |  |
| BasketballDrill_832x480_50 |  | 34.90% |  | 34.76% |  | 10.70% |  | 7.14% |  |
| BQMall_832x480_60 |  | 7.68% |  | 7.56% |  | 4.59% |  | 3.67% |  |
| PartyScene_832x480_50 |  | 2.40% |  | 2.20% |  | 2.00% |  | 1.94% |  |
| RaceHorses_832x480_30 |  | 1.33% |  | 1.33% |  | 1.33% |  | 1.21% |  |
| BasketballPass_416x240_50 |  | 11.50% |  | 11.50% |  | 5.36% |  | 5.36% |  |
| BQSquare_416x240_60 |  | 1.29% |  | 1.27% |  | 0.86% |  | 0.86% |  |
| BlowingBubbles_416x240_50 |  | 7.72% |  | 7.72% |  | 6.59% |  | 6.59% |  |
| RaceHorses_416x240_30 |  | 0.08% |  | 0.08% |  | 0.08% |  | 0.08% |  |
| FourPeople_1280x720_60 |  | 84.94% |  | 50.00% |  | 11.67% |  | 10.00% |  |
| Johnny_1280x720_60 |  | 90.56% |  | 50.00% |  | 11.67% |  | 10.00% |  |
| KristenAndSara_1280x720_60 |  | 81.62% |  | 50.00% |  | 11.67% |  | 10.00% |  |

From the test results, it can be observed that 1/10 refresh rate can get highest coding performance at lowdelay B configuration. All of subtests with different refresh rate can not get coding gain at random access configuration. The reason for that is related to the principle of composite reference which performs partially update frame by frame. It means that a composite reference is even not completely constructed at the end of a random access GOP. Therefore. it’s desirable to test the performance behavior in the case the random access GOP is extended from one second to two seconds and five seconds, whose results are summarised at section 3.3.

Test 2

The results of random access GOP with two seconds and five seconds are summarized in the table 9 and table 10.

Table 9 : Over VTM-1.0 (two seconds)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.85% | 0.64% | 0.70% | 123% | 91% |
| Class A2 | 0.60% | 0.46% | 0.39% | 133% | 91% |
| Class B | 0.21% | -0.51% | -0.21% | 129% | 91% |
| Class C | -0.07% | -0.61% | -0.44% | 122% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.34% | -0.11% | 0.03% | 127% | 93% |
| Class D | 1.24% | 0.88% | 1.07% | 117% | 107% |

Table 10 : Over VTM-1.0 (five seconds)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.91% | 0.45% | 0.48% | 117% | 89% |
| Class A2 | 0.26% | -0.09% | -0.05% | 127% | 87% |
| Class B | 0.03% | -0.67% | -0.52% | 129% | 89% |
| Class C | -1.40% | -2.82% | -2.45% | 121% | 95% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -0.91% | -0.74% | 124% | 90% |
| Class D | 0.96% | -0.01% | 0.76% | 118% | 99% |

Conclusion

This contribution reports simulation result of CE test 11.1.1, 11.1.2, 11.1.3, 11.1.4, 11.2.1 and 11.2.2. According to test 1’s results, 1/10 refresh rate can get best coding performance at lowdelay B configuration, which means only 1/10 of CTUs in a frame can be used to updated composite reference. The average coding gain for 1/10 refresh rate is -2.15/-3.34/-3.04 for Y/U/V components with about 7% encoding time increased, and -2.68%/-4.75%/-4.24% for Y/U/V components with about 8% encoding time increased at BMS.

Regarding random access configuration, almost all of subtests get minor coding lost, and subtest 11.2.2 can get minor coding gain. It’s should be noted that composited reference need to be refreshed with IDR frame and be reconstructed at both encoder and decoder side at each random access GOP. The construction process might not finish at the end of each GOP. Therefore, the capability of composite reference can not be exploited as much as possible at random access configuration.

References

[1] Z. Wang, X. Meng, C. Jia, J. Cui, S. H. Wang, S. S. Wang, S. Ma, W. Li, Z. Miao and X. Zheng, “Description of SDR and HDR video coding technology proposal by DJI and Peking University” JVET-J0011, JVET 10th Meeting: San Diego, US, April 2018.

[2] X. Zheng, G. Li and Y. Li, “Description of CE11: Composite reference picture” JVET-J1031, JVET 10th Meeting: San Diego, US, April 2018.

[3] https://jvet.hhi.fraunhofer.de/svn/svn_VCC_Software_BMS/branches/CE/J_SanDiego/CE11/

[4] Jill Boyce, K. Suehring, X. Li, and Vadim Seregin, “JVET common test conditions and software reference configurations” JVET-J1010, April 2018.

Patent rights declaration(s)

SZ DJI Technology Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).