| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0157-v1 |
| --- | --- |

| Title: | CE11: HEVC-like encoder only solution for composite reference picture |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weiran Li, Xiaozhen Zheng DJI | Tel: Email: | xiaozhen.zheng@dji.com |
| Source: | SZ DJI Technology Co., Ltd., |  |  |

_____________________________

- Abstract

This contribution provide a HEVC-like encoder only solution for composite reference that is evaluated in CE11. Implementation details and test results followed by common test condition are provided in the document. Simulations show that the proposed technique can achieve -2.46% and -1.46% coding gain over VTM1.0 and BMS1.0 at Lowdelay B Main10 configuration (LDB) with around 20% encoding time increase.

Introduction

Composite reference picture was proposed in JVET Call for Proposal [1] and it is studied in CE11 [2]. As an alternative approach, HEVC-like encoder only implementation is also evaluated at CE11 test 3. This document provides an encoder only composite reference implementation and reports the test results.

The basic generation process of composite reference picture in test 3 is same as the process described in [3]. This document concentrates on the description of encoding workflow and the use of RPS syntax by exploiting HEVC mechanism.

Proposed method

General idea

Composite reference generates and updates a long-term reference by using information of static areas, which are always regarded as background, among a video scene. Usually, background areas have few motion in a long temporal window. Therefore the blocks with minor difference between the background and current frame are picked up, and are used to replace the co-located blocks in the long-term reference that targets at the renewal of the background information.

Different to CE11 test 1 and test 2 that introduces a new composed virtual reference frame, CE11 test 3 targets at evaluating the use of the HEVC long-term reference mechanism, potentially in combination with signaling of no output coded pictures (pic_output_flag = 0). This combination could theoretically achieve similar functionality as that provided by composite reference pictures, e.g. by synthesizing and signaling a no-output reference picture that only contains background information which is illustrated at Fig. 1.

Fig. 1: Example use of no output reference pictures

As illustrated in Fig. 1, the frames I0, B1, B2, B3 and B4 are coded as short-term reference. During the encoding of those short-term frames, an alternative frame is composed by exploiting previous coded frames’ content. When an encoder determines the alternative frame has been completely constructed, such frame will be coded as long-term reference with pic_output_flag=0.

To harmonize current coding tools in VTM and BMS, the tools with motion vector scaling and motion vector refinement at decoder side are modified when they use reference data from such long-term reference. In details, if the long-term reference is used as reference, motion vector scaling and decoder motion refine operation that use motion trajectory are invalid because the distance between the composed long-term reference and the current frame is not available and motion trajectory model doesn’t work for this composed reference. Meanwhile, since this reference picture is composed from various coded frames, it’s unreasonable to perform motion vector refinement at such picture. Therefore, the tools like BIO、DMVR、FRUC are set to disable if any of the motion vector is referred to the long-term reference.

Implementation details

Modification of RPS setting in cfg file

In order to keep dpb process and the motion vector scaling process same as before when a long-term reference is encoded, all of short-term frames’ POC is doubled from “x” to “2x”. The RPS setting for short-term frames without long-term reference and short-term frames with long-term reference are showed at Fig. 2 and Fig. 3.

Fig2. RPS setting for short-term frames without long-term reference

Fig3. RPS setting for short-term frames with long-term reference

RPS syntax for long-term reference

A composed long-term reference is always coded after the first short-term frame and coded before the first frame of a particular GOP. Therefore, the parameter at SPS long_term_ref_pics_present_flag is set to 1 and num_long_term_ref_pics_sps is set 0.

The parameter at the composed long-term frame’s PPS Output_flag_present_flag is set to 1, deblocking_filter_control_present_flag and pps_deblocking_filter_disabled_flag is set to 1.

Since RPS for the composed long-term frame is different to RPS at SPS, short_term_ref_pic_set_sps_flag is set to 0 at the composed long-term’s slice header.

Meanwhile, the reference pictures for the composed long-term frame are same as the reference pictures for the first frame in a particular coding GOP. Thus, delta POC for composed reference frame is set to -1, -9, -17, -25, deltaRPS is set to 1, ref_idcs is set to 5 and reference idcs is set to 1, 1, 1, 1, 0. Fig.4 show an example for POC and deltaPOC setting when short-term frames are coded without composed long-term frame and with composed long-term frame.

Fig. 4: Illustration of short-term and long-term reference’s references

When a composed long-term reference is coded, a new short-term RPS need to be encoded with inter_ref_pic_set_prediction_flag is set to 1, delta_rps, used_by_curr_pic_flag[j] and use_delta_flag[j] are set to 1.

After the coded of the composed long-term frame, num_long_term_pics, used_by_curr_pic_lt_flag and num_ref_idx_active_override_flag in the following short-term frames’ RPS is set to 1. num_ref_idx_l0_active_minus1 is set to 4.

CTU selection strategy for composite reference update at encoder side

According to the principle of composite reference picture, CTU-level similarity metric between current original frame and two previous original frames is calculated before the coding of current frame. Based on CTU-level similarity metric, a certain number of CTUs of current frame are used and marked to updated composed reference. A control parameter, which is called as refresh rate and denoted as R, is used at encoder side which imposes the limitation on the total amount of CTUs in current frame that can be used to update composed reference. The value of R is set to 25% CTU number in a coding frame.

The details of CTU selection strategy can be found at [3].

Experimental results

The software is implemented based on BMS-1.0. And it is committed in SVN server [4]. The test is evaluated using BD rates, encoding time, decoding time according to Common Test Conditions (CTC) in [5].

Simulation environment

For the simulations, same platform machines as below in PC cluster were used.

- Encode

  - OS: Windows Sever 2008 R2 Enterprise

  - Compiler: Visual Studio 2017

  - CPU: Intel Xeon E5-2687W v4 3.0GHz

    - SIMD: AVX2 instruction is enabled.

  - Memory: 256 GB

- Decode

  - OS: Windows Sever 2008 R2 Enterprise

  - Compiler: Visual Studio 2017

  - CPU: Intel Xeon E5-2687W v4 3.0GHz

    - SIMD: AVX2 instruction is enabled.

  - Memory: 256 GB

Test results

The results of test 1 with different refresh rate are summarized in the Table 1 and Table 2.

Table 1 : Over VTM-1.0 (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.76% | -0.69% | -0.56% | 120% | 90% |
| Class C | -4.31% | -7.03% | -5.70% | 116% | 87% |
| Class E | -2.84% | -2.68% | -2.40% | 133% | 82% |
| Overall | -2.46% | -3.30% | -2.73% | 122% | 87% |
| Class D | 0.16% | 0.70% | 0.10% | 122% | 91% |

Table 2 : Over BMS-1.0 (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.16% | -0.42% | 0.03% | 118% | 94% |
| Class C | -3.52% | -6.11% | -5.59% | 112% | 90% |
| Class E | -1.41% | -3.91% | -3.75% | 134% | 82% |
| Overall | -1.46% | -3.19% | -2.79% | 119% | 90% |
| Class D | 1.63% | 1.38% | 1.00% | 118% | 90% |

From the test results, it can be observed that the proposed encoder-only composite reference solution can still get obvious coding performance improvement over VTM1.0 and BMS1.0, in particular in the scenario with large static area.

Meanwhile, according to the principle of composite reference, a composite long-term reference need to perform partially update frame by frame. It means that a composite long-term reference is even not completely constructed at the end of a random access GOP. Therefore it’s reasonable that the proposed composite long-term reference can’t get coding performance improvement. Thus, the coding results for composite long-term reference at random access configuration are not provided.

Conclusion

This contribution reports simulation result of CE test 3 that implements HEVC-like encoder solution for composite reference. The average coding gain for the test is -2.46%/-3.30%/-2.73% for Y/U/V components over VTM1.0 and -1.46%/-3.19%/-2.79% for Y/U/V components over BMS1.0 with about 20% encoding time increased.

Besides mandatory and minor harmonization with motion vector refinement tools in VTM / BMS to support long-term reference coding, the proposed method is a completely encoder-only solution that can get stratified coding performance improvement. Therefore, it’s suggested to adopt the proposed HEVC-like encoder solution for composite reference to the next version of VTM / BMS software.

References

[1], “Description of SDR and HDR video coding technology proposal by DJI and Peking University” JVET-J0011, JVET 10th Meeting: San Diego, US, April 2018.

[2] X. Zheng, G. Li and Y. Li, “Description of CE11: Composite reference picture” JVET-J1031, JVET 10th Meeting: San Diego, US, April 2018.

[3] W. Li and X. Zheng, “CE11: Results on composite reference picture (test 11.1.1, 11.1.2, 11.1.3, 11.1.4, 11.2.1 and 11.2.2)” JVET-K0156, JVET 11th Meeting: Ljubljana, SI, 10–18 July 2018

[4] https://jvet.hhi.fraunhofer.de/svn/svn_VCC_Software_BMS/branches/CE/J_SanDiego/CE11/

[5] Jill Boyce, K. Suehring, X. Li, and Vadim Seregin, “JVET common test conditions and software reference configurations” JVET-J1010, April 2018.

Patent rights declaration(s)

SZ DJI Technology Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).