| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0159-v1 |
| --- | --- |

| Title: | CE11-related: Hardware friendly composite reference picture solution |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weiran Li, Xiaozhen Zheng DJI | Tel: Email: | xiaozhen.zheng@dji.com |
| Source: | SZ DJI Technology Co., Ltd., |  |  |

_____________________________

- Abstract

This contribution proposes a hardware friendly composite reference design. It can allow composite reference update at block level when a CTU’s coding is finished, which shows benefits on hardware design. The proposed method can achieve almost same coding performance of composite reference and get lower encoding runtime increase.

Introduction

Composite reference picture was proposed in JVET Call for Proposal [1] and it was planned to study in CE11 [2]. The fundamental concept of composite reference picture is to partial update a long-term reference at CTU level. If current frame is an IDR frame, all the CTUs in the frame will be marked to update the long-term reference, else if current frame is a P or a B frame, a control flag will be encoded at CTU level. If the value of the flag is equal to 1, the CTU will be marked to update the composed long-term reference. The partial update of the composite reference is illustrated at Fig. 1.

Fig 1. Illustration of block-level long-term reference update process

Base on CE11’s composite reference design, the process of updating the composed long-term reference will be performed after a frame is decoded and reconstructed completely. For every CTU marked to update the long-term reference, its Luma and Chroma reconstructed pixels will be used to replace the co-located pixels in the composite reference. Thus, current composite reference design can’t allow block-level update of a composite reference, which might have negative impact on hardware design.

This contribution proposes a new hardware composite reference design that allow block-level update of a composite reference and keep almost original composite reference’s coding performance.

Proposed method

Since the content of composite reference is partially updated from a coded frame’s selected CTUs, update the reference with a selected CTU when it’s finished the reconstruction immediately might cause later coded CTUs located in the same frame use the data from previous selected reconstructed CTU. It might complicate hardware design. Meanwhile, update the reference immediately when a selected CTU is finished coding or update the reference when a whole frame is reconstructed will cause different results.

To achieve the target of block level update for composite reference as well as avoid complicating hardware design, this contribution proposes a motion vector restriction method that restricts motion vector search on the area of co-located CTU located at composite reference. Fig. 1 illustrates the restriction process. When a CTU need to use reference data from a composite reference, it can only get reference data from its co-located CTU area in composite reference. It means that current CTU’s reference data will not be impacted by previous CTUs even if they are selected to update composite reference.

With the help of motion vector restriction at encoder side, a selected CTU can update a composite reference immediately when its reconstruction process is finished and its update process will not impact later coded CTUs because they can’t use reference data from previous selected CTU. Meanwhile, this restriction can also achieve identical results no matter update the reference immediately when a selected CTU is finished coding or update the reference when a whole frame is reconstructed will cause different results. It will bring benefits to both hardware and software implementation.

Fig. 1: Illustration of motion vector restriction when a CTU use reference data from composite reference

Experimental results

The software is implemented based on BMS-1.0. The test is evaluated using BD rates, encoding time, decoding time according to Common Test Conditions (CTC) in [2].

Simulation environment

For the simulations, same platform machines as below in PC cluster were used.

- Encode

  - OS: Windows Sever 2008 R2 Enterprise

  - Compiler: Visual Studio 2017

  - CPU: Intel Xeon E5-2687W v4 3.0GHz

    - SIMD: AVX2 instruction is enabled.

  - Memory: 256 GB

- Decode

  - OS: Windows Sever 2008 R2 Enterprise

  - Compiler: Visual Studio 2017

  - CPU: Intel Xeon E5-2687W v4 3.0GHz

    - SIMD: AVX2 instruction is enabled.

  - Memory: 256 GB

Test results

The results of test 1 with different refresh rate are summarized in the Table 1 to Table 2.

Table 1 : Over VTM-1.0 (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -3.71% | -6.73% | -5.64% | 100% |  |
| Class E | -1.75% | -1.94% | -2.29% | 98% |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.08% | -0.02% | -0.52% | 103% | 2% |

Table 2 : Over VTM-1.0 (refresh rate: unlimitation, RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |

Conclusion

This contribution proposes a hardware-friendly composite reference design that allows to update a composite reference immediately when a selected CTU’s construction process is finished. It can also get identical results with the update process when a whole frame’s construction process is finished. Thus the proposed method can get benefits for both hardware and software design.

The proposed method can get almost same coding performance as original composite reference picture. It’s suggested to add the proposed method to CE for further study.

References

[1], “Description of SDR and HDR video coding technology proposal by DJI and Peking University” JVET-J0011, JVET 10th Meeting: San Diego, US, April 2018.

[2] Jill Boyce, K. Suehring, X. Li, and Vadim Seregin, “JVET common test conditions and software reference configurations” JVET-J1010, April 2018.

Patent rights declaration(s)

SZ DJI Technology Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).