| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0160-v1 |
| --- | --- |

| Title: | CE2: Non-local Structure-based Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Xuewei Meng, Chuanmin Jia, Shanshe Wang, Siwei Ma Peking University Xiaozhen Zheng DJI | Tel: Email: Tel: Email: | swma@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | SZ DJI Technology Co., Ltd., Peking University |  |  |

_____________________________

- Abstract

This contribution proposes NLSF (Non-local Structure-based Filter) technique that was described in JVET-J0011 and evaluated in CE2 test 5. The simulate results are reported followed by common test condition.

Introduction

NLSF (Non-Local Structure-based Filter) is a new in-loop filter that targets at the reduction of noise by using non-local reconstructed pixels proposed in JVET-J0011. It is employed between Deblocking Filter (DF) and Sample Adaptive Offset (SAO) in BMS-1.0 and VTM-1.0, which includes group construction by block matching and SVD-based filtering processes.

Group construction by block matching

As illustrated in Fig 1, the input reconstructed frame is firstly divided into K overlapped blocks, and each block serves as the basic loop unit. Let Bs × Bs denote the size of loop unit, which is set to 6 by default. For the current loop unit, no more than c (defaulted to 20) similar blocks are collected within the corresponding search window to construct the block group. As the example of Fig 1, the red square represents the current loop unit, meanwhile the blue dotted square denotes the search window. The size of search window is set to 32x32 by default, and SSD (Sum of Square Differences) is adopted as the matching criterion. Details of block matching process are given bellow.

Fig 1 Illustrations for Non-Local Structure-based Filter

Step one: As shown in Fig 2, an eight-points diamond template is applied to search the most similar blocks in the search window. The center point of eight-points diamond template is the current loop unit labelled by “0” in Fig 2. The search process starts from the center point and successively search the points labelled by “1”, “2”, “4” and “8”. During the searching process, at most T (defaulted to 5) most similar blocks (golden points labelled by “8” in Fig 3) with SSD smaller than threshold ε are selected and their coordinates are recorded. Estimation of threshold ε is described in section 2.3. The number of final selected blocks in step one is denoted as n (n≤T).

Fig 2 Block matching template

Step two: The n similar blocks selected in step one serve as the centers of the second round search, as shown in Fig 3. The similar blocks selected in step two are denoted by red points in Fig 3. After removing the reduplicative similar blocks, the number of selected blocks in the end is no more than c. A parameter m is used to represent the number of similar blocks selected in block matching process.

Fig 3 Block matching process

SVD-based filtering

For each block, it is viewed as a vector xk∈RBs × Bs, k=1,2,…,K. Then, the current loop unit and the selected similar blocks generates a matrix XGk=[xGk⊗1, xGk⊗2, …, xGk⊗m]. Then apply SVD to the matrix.

Encoder and decoder description of NLSF

Switch on/off control, parameters estimation in NLSF, line buffer usage and bitstream structured are described as follows:

Frame level on/off control

For one frame, there are two flags signaled in slice header, slice_nlsf_luma_flag and slice_nlsf_chroma_flag, determined according to the rate-distortion costs of luma and Cb component respectively w/o NLSF. If the luma component of current frame performs better with NLSF than without, switch on the slice_nlsf_luma_flag and then calculate the rate-distortion costs of Cb component. If Cb component peform better, switch on the slice_nlsf_luma_flag. Otherwise, switch off it. If luma component of current frame performs worse, switch off the slice_nlsf_luma_flag and don’t need to calculate the rate-distortion costs of Cb component. And the slice_nlsf_chroma_flag will not be signaled.

CTU level on/off control

NLSF is CTU level loop filter tool. The on/off control of each CTU is determined according to the SSE (Sum of Square Error) of luma component between CTU before/after NLSF and its corresponding CTU in original video sequence, i.e. dfSSE and nlsfSSE. If dfSSE > nlsfSSE, switch on the CTU level on/off control. Otherwise, it is switched off.

Parameter ε estimation

As the similarity criterion of similarity evaluation is SSD, the parameter ε is related to the size of loop unit and the bit depth of video. In this proposal, parameter ε is determined as

ε=Bs2×(1≪(2×bitDepth))×λ,

where Bs denotes the size of loop unit, bitDepth represents the bit depth of input sequence, and λ is the parameter obtained according to the statistics of SSD between loop unit and the selected similar blocks.

Parameter τ estimation

Gaussian model is adopted to characterize the quantization noise between the original frame and the reconstructed frame. Here, the estimation of τ is converted to estimate the variance σ2 of the quantization noise. There exists a linear relationship between σ and the quantization step size Qstep, as

σ=a×Qstep+b,

where a and b are parameters trained offline. Qstep can be computed according to the QP as

Qstep= 2QP-46

Moreover, the relation between σ and τ can be expressed as

τ=σ×(Bs2+ c)

The parameters a and b we set in BMS-1.0 and VTM-1.0 are shown below.

|  | I frame |  | B frame |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Luma | Chroma | Luma |  | Chroma |
|  |  |  | Forward | Bidirectional |  |
| a | 0.2759 | 0.07467 | 0.1490 | 0.18979 | 0.05102 |
| b | 3.292 | 4.586 | 6.269 | 5.118 | 5.612 |

Line buffer usage

As shown in Fig 4, the red square block is a CTU and the grey region represents the use of the neighbouring pixels. 8 lines reconstructed pixels and 16 colmun line reconstructed pixels are used at NLSF processing.

Fig 4 Line buffer usage for NLSF

Bitstream structure

There are three frame level on/off controls of NLSF, slice_nlsf_luma_flag, slice_nlsf_cb_flag and slice_nlsf_cr_flag, which are signalled in the slice header to indicate whether NLSF are enabled for luma, cb and cr components, respectively.

| slice_segment_header( ){ | Descriptor |
| --- | --- |
| …… |  |
| if (nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP){ |  |
| …… |  |
| if(sps_temporal_mvp_enable_flag) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| } |  |
| if(nonlocal_structure_filter_enabled_flag){ |  |
| slice_nlsf_luma_flag | u(1) |
| if(slice_nlsf_luma_flag){ |  |
| slice_nlsf_cb_flag | u(1) |
| slice_nlsf_cr_flag | u(1) |
| } |  |
| } |  |
| if(sample_adaptive_offset_enabled_flag){ |  |
| slice_sao_luma_flag | u(1) |
| if(slice_sao_luma_flag) |  |
| slice_sao_chroma_flag | u(1) |
| } |  |
| …… |  |
| } |  |

Experimental results

The software is implemented based on BMS-1.0. And it is committed in SVN server [3]. The test are evaluated follow by common test condition [4], and its results are summarized in Table 1 to Table 6.

Table 1 Comparison of Proposed method with BMS 1.0 (AI)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.74% | -1.27% | -2.17% | 101% | #DIV/0! |
| Class C | -0.98% | -2.82% | -3.61% | 99% | #DIV/0! |
| Class E | -1.77% | -3.11% | -2.48% | 99% | #DIV/0! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.62% | -1.82% | -2.03% | 100% | #DIV/0! |

Table 2 Comparison of Proposed method with BMS 1.0 (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -1.08% | -0.56% | -1.02% | 102% | #DIV/0! |
| Class E | -2.05% | -2.07% | 0.04% | 105% | #DIV/0! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.32% | 0.18% | -0.17% | 104% | #DIV/0! |

Table 3 Comparison of Proposed method with BMS 1.0 (RA)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -0.96% | -3.12% | -3.59% | 101% | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.40% | -1.67% | -1.79% | 103% | #DIV/0! |

Table 4 Comparison of Proposed method with VTM 1.0 (AI)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.04% | -4.01% | -4.70% | 106% |  |
| Class A2 | -2.19% | -3.28% | -2.91% | 107% |  |
| Class B | -2.17% | -3.42% | -4.64% | 108% |  |
| Class C | -2.24% | -4.20% | -5.39% | 108% |  |
| Class E | -4.45% | -6.99% | -7.65% | 108% |  |
| Overall | -2.55% | -4.26% | -5.03% | 108% |  |
| Class D | -1.86% | -3.75% | -4.43% | 111% |  |

Table 5 Comparison of Proposed method with VTM 1.0 (LB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.55% | 0.20% | -1.04% | 109% | #DIV/0! |
| Class C | -1.30% | 0.05% | -0.14% | 110% | #DIV/0! |
| Class E | -2.84% | -3.76% | -3.17% | 122% | #DIV/0! |
| Overall | -2.20% | -0.84% | -1.27% | 112% | #DIV/0! |
| Class D | -0.74% | 0.19% | 0.15% | 112% | #DIV/0! |

Table 6 Comparison of Proposed method with VTM 1.0 (RA)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -2.26% | -2.23% | -3.15% | 111% | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -1.38% | -0.71% | -1.42% | 115% | #DIV/0! |

Conclusion

This contribution reports simulation results of CE2 test 2-5.1. The results show that the proposed NLSF technique can up to -2.55%/-4.26%/-5.03% coding gain for Y/U/V component on average at VTM platform.

References

[1] Z. Wang, X. Meng, C. Jia, et al. “Description of SDR video coding technology proposal by DJI and Peking University,” JVET-J0011, April., 2018.

[2], “Description of CE2: In-Loop Filters” JVET-J1022, JVET 10th Meeting: San Diego, US, April 2018.

[3] https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE2/CE2-5.1/

[4] K. Suehring, X. Li, “JVET common test conditions and software reference configurations,” JVET-C1010, May, 2016.

Patent rights declaration(s)

SZ DJI Technology Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).