| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0162 |
| --- | --- |

| Title: | CE3.5: Multiple Reference Intra Prediction (tests 5.2.1 and 5.2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gagan Rath Fabrice Urban Fabien. Racapé | Email: | gagan.rath@technicolor.com fabrice.urban@technicolor.com fabien.racape@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports results of CE3.5.2, related to Multiple Reference sample-based Intra Prediction (MRIP) and Boundary Filtering (BF), described in JVET-J0022, implemented within the JVET BMS-1.0. For MRIP alone (CE3 test 5.2.1), simulation results reportedly show an average luma BD-rate variation of -0.31%, resp. -0.18%, in All Intra (AI), resp. Random Access (RA) configuration, compared with VTM, and -0.21%, resp. -0.10%, in AI, resp. RA, configuration, compared to BMS. For MRIP+BF (CE3 test 5.2.2) simulation results show an average luma BD-rate variation of -0.91%, resp. -0.44%, in All Intra (AI), resp. Random Access (RA) configuration, compared with VTM, and -0.97%, resp. -0.49%, in AI, resp. RA, configuration, compared to BMS.

# Introduction

Intra prediction using multiple reference lines or samples, also called Multiple Reference Intra Prediction (MRIP), was proposed in JVET-J0022, along with a Boundary Filtering (BF) that also involves multiple lines of neighbor samples. This contribution reports the results of MRIP only (CE3 test 5.2.1), and MRIP + BF (CE3 test 5.2.2), when implemented in the BMS software with the Common Test Conditions.

# Description and encoder options

Two reference layers are used for intra prediction, where each reference layer consists of a left reference array and a top reference array (Figure 1). Each reference layer is constructed by reference sample substitution and then pre-filtered. In PLANAR and DC prediction modes, and in the LMChroma mode for Chroma blocks, only one reference layer is used.

For angular prediction modes, except the vertical and the horizontal modes, a prediction for a block is constructed using each reference layer. The final prediction is formed as a weighted average of the predictions made from the two reference layers. The prediction from the closer reference layer is given higher weight than the prediction from the farther layer. The used weights are 3 and 1.

At the decoder, if the decoded prediction mode is PLANAR or DC, or LMChroma for a Chroma block, only one reference layer is constructed for the prediction process.

Figure 1. Intra prediction with 2 reference layers. The layer #1 consists of the usual reference array, the layer #2 consists of the next reference array on top and on left of layer #1.

To avoid sharp changes in intensity at the block boundaries, multiple reference layers are used to smoothen the boundary samples for certain prediction modes. The process is mode dependent. For example,

- For DC prediction mode (mode 2), the boundary filtering remains the same as in VTM/BMS.

- For the two modes closest to the vertical/horizontal modes (e.g. modes 16, 17, 19, 20 for the horizontal modes in BMS), the predicted pixel values on the first column/line of a block are filtered as:

Vertical modes – for y = 0 to H – 1

P(0, y) = Clip(P(0,y) + (3*((P(-1, y)-P(-1, -1)) >> 2) + ((P(-2,y)-P(-2,-2)) >> 2)) >> 2)

Horizontal modes – for x = 0 to W – 1

P(x, 0) = Clip(P(x,0) + (3*((P(x, -1)-P(-1, -1)) >> 2) + ((P(x,-2)-P(-2,-2)) >> 2)) >> 2)

This filtering is applied to Luma blocks only.

- For angular modes having positive directions (mode indices from 2 to HOR_IDX-2 and from VER_IDX+2 to VDIA_IDX), a gradient based filtering is applied to the predicted samples on the left/top boundary as follows:

Down-left modes (mode # VER_IDX +2 to VDIA_IDX):

– for x = 0 to W – 1, y = 0 to H – 1

P(x,y) = Clip(P(x,y) + (D (y)* L(x) + 32) >> 6))

where D(y) = P(-1,y)-((3*P(s1,-1)+P(s2,-2)+2) >> 2)

and   L(x) = (32 >> ((x << 1) >> scale)),

with scale = (log2(W)+log2(H)-2)>>2.

Up-right modes (mode # 2 to HOR_IDX-2)

– for x = 0 to W – 1, y = 0 to H – 1

P(x,y) = Clip(P(x,y) + (D (x)* L(y) + 32) >> 6))

where D(x) = P(x,-1)-((3*P(-1,t1)+P(-2,t2)+2) >> 2)

and   L(y) = (32 >> ((y << 1) >> scale)),

with scale = (log2(W)+log2(H)-2)>>2.

where D(y) / D(x) corresponds to the gradient at the first left/top reference array at distance y / x from the top/left of the block (Figure 2). s1 and s2 are the reference samples in the first and second reference layers, respectively, of the sample P(-1,y), for the considered directional mode in the range [VER_IDX+2, VDIA_IDX]. Similarly, t1 and t2 are the reference samples in the first and second reference layers, respectively, of the sample P(x,-1), for the considered directional mode in the range [2, HOR_IDX-2]. This filtering is applied to Luma blocks only.

Figure 2. Gradient based boundary filtering for a vertical mode. The gradient is computed at the left reference array by taking the difference of the actual sample value and its prediction using the reference arrays on the top.

- For the vertical/horizontal modes, the filtering is done using the gradients at the top and left reference arrays. For the vertical mode, the gradients are calculated as:

Dx= Px,-1- Px,-2;

D(y) = P(-1,y) – P(-1,-1)

Dy=D(x)+W2W

If  Dy is less than a preset threshold, the filtering is performed as:

– for x = 0 to W – 1, y = 0 to H – 1

Px,y= ClipPx,y+ Dx* Ly+ Dy* Lx+ 32>> 6.

Else, the filtering is performed as

– for x = 0 to W – 1, y = 0 to H – 1

P(x,y) = Clip(P(x,y) + (D(y) * L(x) + 32) >> 6),

where :

L(y) = (32 >> ((y << 1) >> scale)),

L(x) = (32 >> ((x << 1) >> scale))

scale = (log2(W)+log2(H)-2)>>2,

The filtering for the horizontal mode is performed in an analogous manner.

# Results

The tools described above are implemented within the BMS-1.0 software. The following tables report simulation results using VTM and BMS configuration files, as defined in the Common Test Conditions. The same binaries are used (JEM_TOOLS macros activated), and VTM is activated using configuration parameters.

## Multiple reference lines, test 5.2.1

### VTM

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.39% | -0.47% | -0.61% | 126% | 103% |
| Class A2 | -0.25% | -0.27% | -0.26% | 124% | 107% |
| Class B | -0.35% | -0.48% | -0.50% | 123% | 104% |
| Class C | -0.23% | -0.52% | -0.66% | 121% | 106% |
| Class E | -0.32% | -0.81% | -0.69% | 123% | 107% |
| Overall | -0.31% | -0.51% | -0.54% | 123% | 105% |
| Class D | -0.04% | -0.24% | -0.34% | 119% | 109% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | -0.36% | -0.46% | 107% | 105% |
| Class A2 | -0.14% | -0.19% | -0.10% | 105% | 105% |
| Class B | -0.20% | -0.26% | -0.46% | 105% | 101% |
| Class C | -0.14% | -0.33% | -0.39% | 105% | 104% |
| Overall | -0.18% | -0.29% | -0.37% | 106% | 103% |
| Class D | -0.01% | -0.21% | -0.40% | 105% | 106% |

### BMS

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | -0.06% | -0.21% | 105% | 100% |
| Class A2 | -0.23% | -0.12% | -0.18% | 105% | 101% |
| Class B | -0.26% | -0.24% | -0.21% | 106% | 102% |
| Class C | -0.21% | -0.28% | -0.34% | 104% | 102% |
| Class E | -0.14% | -0.31% | -0.26% | 105% | 98% |
| Overall | -0.21% | -0.21% | -0.24% | 105% | 101% |
| Class D | 0.01% | -0.09% | -0.20% | 105% | 106% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.11% | -0.20% | 102% | 101% |
| Class A2 | -0.09% | -0.16% | -0.17% | 102% | 101% |
| Class B | -0.15% | -0.23% | -0.21% | 102% | 99% |
| Class C | -0.05% | -0.19% | -0.66% | 101% | 103% |
| Overall | -0.10% | -0.18% | -0.32% | 102% | 101% |
| Class D | -0.03% | -0.20% | -0.14% | 101% | 97% |

## Multiple reference lines + boundary filtering, test 5.2.2

### VTM

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.85% | -1.35% | -1.26% | 138% | 106% |
| Class A2 | -0.90% | -1.60% | -1.42% | 135% | 111% |
| Class B | -0.91% | -1.25% | -1.34% | 134% | 108% |
| Class C | -0.62% | -1.12% | -1.41% | 131% | 111% |
| Class E | -1.35% | -1.89% | -2.11% | 132% | 107% |
| Overall | -0.91% | -1.40% | -1.49% | 134% | 109% |
| Class D | -0.38% | -0.82% | -0.79% | 129% | 113% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.45% | -1.22% | -1.02% | 109% | 107% |
| Class A2 | -0.41% | -1.10% | -0.66% | 107% | 107% |
| Class B | -0.49% | -0.98% | -1.13% | 107% | 104% |
| Class C | -0.39% | -0.68% | -1.14% | 107% | 106% |
| Overall | -0.44% | -0.97% | -1.02% | 107% | 106% |
| Class D | -0.17% | -0.34% | -0.61% | 106% | 105% |

### BMS

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.89% | -1.12% | -1.16% | 109% | 102% |
| Class A2 | -1.06% | -1.50% | -1.48% | 110% | 105% |
| Class B | -0.98% | -1.29% | -1.41% | 109% | 102% |
| Class C | -0.68% | -1.02% | -1.03% | 107% | 106% |
| Class E | -1.33% | -1.60% | -2.20% | 108% | 107% |
| Overall | -0.97% | -1.29% | -1.43% | 108% | 104% |
| Class D | -0.36% | -0.70% | -0.80% | 107% | 106% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.47% | -0.93% | -0.96% | 104% | 101% |
| Class A2 | -0.51% | -1.18% | -0.83% | 104% | 101% |
| Class B | -0.56% | -1.04% | -1.42% | 105% | 100% |
| Class C | -0.39% | -0.98% | -1.36% | 104% | 101% |
| Overall | -0.49% | -1.03% | -1.19% | 104% | 101% |
| Class D | -0.14% | -0.77% | -0.85% | 104% | 99% |

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).