| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0163 |
| --- | --- |

| Title: | CE3.7: Bi-Directional Intra Prediction (test 7.1.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gagan Rath Fabrice Urban Fabien Racapé | Email: | gagan.rath@technicolor.com fabrice.urban@technicolor.com fabien.racape@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports the results of CE3.7, related to Bi-Directional Intra Prediction, described in JVET-J0022, implemented within the JVET BMS-1.0. Simulation results reportedly show an average luma BD-rate variation of -0.26%, resp. -0.20%, in All Intra (AI), resp. Random Access (RA) configuration, compared with TM, and an average luma BD-rate variation of -0.38%, resp. -0.27%, in AI, resp. RA configuration, compared to BMS.

# Introduction

Bi-Directional Intra Prediction (BDIP) was proposed in JVET-J0022. The underlying idea is that a target sample value is predicted using two reference samples on both ends of a positive prediction direction, as shown in the Figure 1. A CU-level flag (BDIPflag) indicates the application of bi-directional intra prediction. The flag is signaled only for luma blocks with prediction modes which cross both left and above reference sample lines. For the remaining modes, the traditional uni-directional prediction is performed.

Figure 1. In bi-directional prediction, two reference samples are used for computing the prediction for a target pixel. The second reference sample is located at the intersection of the prediction direction with the second reference array.

## Bit stream organization

The flag for CU-level activation, i.e., BDIPflag,  is coded with a CABAC context. The flag is encoded only for the eligible prediction modes, which are modes 2 to 9 and modes 59 to 66 in BMS. For the remaining prediction modes, BDIPflag is not encoded.

# Description and encoder options

For a positive vertical prediction direction, the first reference sample is at displacement ∆x from x-coordinate of the target pixel at (x,y). As specified in VTM/BMS,

∆x =1+y*A

where A denotes the angle associated with the prediction mode. The second reference sample is at displacement ∆y from the y-coordinate of the target pixel at (x,y).  ∆y can be computed as:

∆y= 1+x≪10A

The values of  ∆y are pre-computed for all positive A and x = 0, and stored in a table. For any value of x, the value of  ∆y is computed by incrementing this stored value by its magnitude x times. Denoting Ay≡ 1≪10A, the tabulated values for different positive A values are given in Table 1 in case of using 65 modes as in BMS.

Table 1. Parameter Ay for different positive values of A in BMS.

| A | 1 | 2 | 3 | 5 | 7 | 9 | 11 | 13 | 15 | 17 | 19 | 21 | 23 | 26 | 29 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ay | 1024 | 512 | 341 | 205 | 146 | 114 | 93 | 79 | 68 | 60 | 54 | 49 | 45 | 39 | 35 | 32 |

If the coordinate of the second reference sample lies beyond the length of the left reference array, the last reference sample on the left reference array is taken to be the second reference sample value.

Using the first and the second reference samples, an update term is computed and added to the initial prediction (which is the first reference sample value):

ΔP = P2-P1*∆x1+x≪5+ ∆x

P=P1+ ∆P

Here P1 and P2 are the first and the second reference sample values for a target pixel, ∆P is the update, and P is the final prediction value. For positive horizontal prediction directions, the computations are similar.

To keep the additional complexity low, the proposed prediction method is applied only to a subset of modes associated with positive directions (modes 2 to 9 and modes 59 to 66).  For an eligible prediction mode, the encoder applies both the uni-directional and bidirectional predictions, and selects the one having better RD performance. It encodes a flag (termed here as BDIPflag) to indicate the used prediction. A value of 1 indicates bi-directional prediction whereas a value of 0 indicates the existing uni-directional prediction. The decoder decodes the BDIPflag wherever the decoded prediction mode is eligible for bi-directional prediction.

# Results

The tool described above is implemented within the BMS-1.0 software. The following tables report simulation results using VTM and BMS configuration files, as defined in the Common Test Conditions. The same binaries are used (JEM_TOOLS macros activated).

## VTM

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.28% | -0.35% | -0.28% | 113% | 97% |
| Class A2 | -0.20% | -0.25% | -0.22% | 112% | 101% |
| Class B | -0.29% | -0.29% | -0.29% | 112% | 96% |
| Class C | -0.25% | -0.30% | -0.44% | 110% | 95% |
| Class E | -0.29% | -0.23% | -0.20% | 111% | 97% |
| Overall | -0.26% | -0.29% | -0.30% | 111% | 97% |
| Class D | -0.28% | -0.34% | -0.27% | 110% | 98% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | -0.35% | -0.17% | 103% | 102% |
| Class A2 | -0.12% | -0.14% | -0.19% | 102% | 101% |
| Class B | -0.23% | -0.34% | -0.50% | 102% | 101% |
| Class C | -0.25% | -0.34% | -0.33% | 102% | 103% |
| Overall | -0.20% | -0.30% | -0.33% | 102% | 102% |
| Class D | -0.13% | -0.33% | -0.48% | 102% | 105% |

## BMS

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.61% | -0.99% | -0.95% | 103% | 99% |
| Class A2 | -0.31% | -0.68% | -0.56% | 104% | 98% |
| Class B | -0.38% | -0.61% | -0.69% | 102% | 96% |
| Class C | -0.31% | -0.39% | -0.41% | 102% | 93% |
| Class E | -0.31% | -0.49% | -0.68% | 100% | 95% |
| Overall | -0.38% | -0.62% | -0.65% | 102% | 96% |
| Class D | -0.23% | -0.36% | -0.53% | 102% | 97% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.45% | -0.96% | -1.07% | 103% | 100% |
| Class A2 | -0.19% | -0.56% | -0.28% | 102% | 101% |
| Class B | -0.26% | -0.54% | -0.84% | 103% | 100% |
| Class C | -0.22% | -0.30% | -0.49% | 102% | 100% |
| Overall | -0.27% | -0.56% | -0.68% | 103% | 100% |
| Class D | -0.15% | -0.16% | -0.47% | 103% | 100% |

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).