| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0164_r1 |
| --- | --- |

| Title: | CE2: Subsampled sum-modified-Laplacian (Test 4.1.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sung-Chang Lim Jungwon Kang Hahyun Lee Jinho Lee Seunghyun Cho Hui Yong Kim 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-5772 sclim@etri.re.kr jungwon@etri.re.kr hanilee@etri.re.kr jinosoul@etri.re.kr shcho@etri.re.kr hykim5@etri.re.kr |
| Source: | Electronics and Telecommunications Research Institute (ETRI) |  |  |

_____________________________

- Abstract

This contribution reports the experimental results of CE2-4.1.1 test proposed in call for proposal (CfP) response document of JVET-J0013. In CE2-4.1.1 test, 4x4 block classification based on subsampled sum-modified-Laplacian (SSML) was tested in order to reduce the computational complexity of the block classification of adaptive loop filter (ALF) in benchmark set (BMS), especially for worst case operation count. And, 2x2 block classification based on SSML is also tested. The CE2-4.1.1 method for 4x4 block classification based on SSML reportedly reduces approximately 78% of worst case operation count compared to block classification compared to ALF in BMS. The CE2-4.1.1 method for 2x2 block classification based on SSML reportedly reduces approximately 50% of worst case operation count compared to block classification compared to ALF in BMS. The experimental results for 4x4 block classification based on SSML reportedly show average BD-rates of 0.19%, 0.18%, and 0.15% with decoding times of 96%, 96%, and 96% for AI, RA, and LDB cases, respectively, compared to VVC test model (VTM) 1.0 anchor with ALF. And, the experimental results for 4x4 block classification based on SSML reportedly show average BD-rates of 0.15%, 0.17%, and 0.10% with decoding times of 97%, 97%, and 97% for AI, RA, and LDB cases, respectively, compared to BMS 1.0 anchor. And, the experimental results for 2x2 block classification based on SSML reportedly show average BD-rates of 0.12%, 0.11%, and 0.10% with decoding times of 98%, 97%, and 98% for AI, RA, and LDB cases, respectively, compared to VTM 1.0 anchor with ALF. And, the experimental results for 2x2 block classification based on SSML reportedly show average BD-rates of 0.10%, 0.11%, and 0.00% with decoding times of 98%, 98%, and 98% for AI, RA, and LDB cases, respectively, compared to BMS 1.0 anchor.

Test descriptions

To reduce complexity of adaptive loop filter (ALF) in benchmark set (BMS) [1], four tests were suggested to be evaluated in the core experiment 2 (CE2) on in-loop filters [2] including CE2-4.1.1 test. In CE2-4.1.1 test, 4x4 block classification is performed instead of 2x2 block classification of ALF, thus, ALF coefficients may switch for each 4x4 luma block. To classify each 4x4 block where filtering operation is performed, 4x4 block classification using subsampled sum-modified-Laplacian (SSML) is used as proposed in [3]. Each 4x4 block is categorized into one out of 25 classes. The classification index C is derived based on its directionality, D, and a quantized value of activity, A, as follows:

| C=5D+A. |  |
| --- | --- |

To calculate D and A, gradients of the horizontal, vertical and two diagonal directions are calculated using 1-D SSML:

gv=k=-12l=-25Vi+2k,j+l,  Vp,q=2Rp,q-Rp,q-1-Rp,q+1,

gh=k=-25l=-12Hi+k,j+2l,  Hp,q=2Rp,q-Rp-1,q-Rp+1,q,

gd1=k=-12l=-12(D1i+2k,j+2l+D1i+2k+1,j+2l+1), D1p,q=2Rp,q-Rp-1,q-1-Rp+1,q+1,

gd2=k=-12j=-12(D2i+2k+1,j+2l+D2i+2k,j+2l+1),D2p,q=2Rp,q-Rp-1,q+1-Rp+1,q-1,

where indices i and j refer to the coordinates of the upper left sample in the 4x4 block and R(i,j) indicates a reconstructed sample at coordinate (i,j).

Fig. 1 (a), (b), (c), and (d) show positions of calculating 1-D Laplacian, Vp,q, Hp,q, D1p,q and D2p,q for gradients, gv, gh, gd1 and gd2, respectively, for a 4x4 block. Since 1-D Laplacian values, Vp,q, Hp,q, D1p,q and D2p,q are calculated at the subsampled positions within 8x8 window, it can be claimed that the computational complexity in calculating gradients is reduced. The rest of derivation process for D and A with the calculated gradients, gv, gh, gd1 and gd2, is the same as ALF in BMS.

(a) Calculation of vertical gradient, gv                 (b) Calculation of horizontal gradient, gh

(c) Calculation of diagonal gradients, gd1              (d) Calculation of diagonal gradients, gd2

Fig. 1	Calculation of gradients (gv, gh, gd1, and gd2)

In order to reduce a coding loss of 4x4 block classification based on SSML, 2x2 block classification based on SSML is also tested. For 2x2 block classification based on SSML, since 1-D Laplacian values, Vp,q, Hp,q, D1p,q and D2p,q are calculated at the subsampled positions within 6x6 window and gradients, gv, gh, gd1 and gd2 are assigned to each 2x2 block, it can be claimed that the computational complexity in calculating gradients is also reduced.

Experiment results

The CE2-4.1.1 experiments were performed using CE2-4.1.1 software [4], based on BMS 1.0 software, with BMS configuration file and VVC test model (VTM) configuration file under JVET common test condition [5]. To check mismatch between encoder and decoder, both BMS configuration file and VTM configuration file are modified to enable MD5 decoded picture hash SEI message. The experiments are conducted on the platform of Intel Xeon E5-1620 v2 (3.7GHz) and 32GB RAM with GCC 4.7.2 compiler under CentOS Linux.

The following macro definitions in the CE2-4.1.1 software were used to run the experiments for 4x4 block classification based on SSML.

#define CE2411 1 ///< CE2.4.1.1 Reduced Complexity ALF (JVET-J0013) #if CE2411 #define CE2411_4x4BC 1 ///< 4x4 block classification #define CE2411_SSML 1 ///< Subsampled sum-modified-Laplacian (SSML) #endif

And, the following macro definitions in the CE2-4.1.1 software were used to run the experiments for 2x2 block classification based on SSML.

#define CE2411 1 ///< CE2.4.1.1 Reduced Complexity ALF (JVET-J0013) #if CE2411 #define CE2411_4x4BC 0 ///< 4x4 block classification #define CE2411_SSML 1 ///< Subsampled sum-modified-Laplacian (SSML) #endif

The experimental results of CE2-4.1.1 test for 4x4 block classification based on SSML compared to VTM 1.0 anchor and VTM 1.0 anchor with ALF are summarized in Table I. The average BD-rates of -3.11%, -5.18%, and -4.41% are observed for AI, RA, and LDB cases, respectively, compared to VTM 1.0 anchor. And, the average BD-rates of 0.19%, 0.18%, and 0.15% are observed for AI, RA, and LDB cases, respectively, compared to VTM 1.0 anchor with ALF. Compared to VTM 1.0 anchor with ALF, decoding time of 96% for AI, RA, and LDB cases is observed.

Table I. Experimental results of CE2-4.1.1 for 4x4 block classification based on SSML over VTM 1.0 anchor and VTM 1.0 anchor with ALF

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over VTM-1.0 + ALF |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.48% | -3.30% | -3.48% | 111% | 240% | -0.02% | 0.00% | 0.00% | 101% | 95% |
| Class A2 | -2.95% | -3.80% | -3.62% | 110% | 210% | 0.11% | 0.00% | 0.00% | 101% | 96% |
| Class B | -2.67% | -2.30% | -2.97% | 110% | 203% | 0.13% | 0.00% | 0.00% | 101% | 96% |
| Class C | -2.97% | -2.73% | -3.61% | 110% | 171% | 0.42% | 0.02% | 0.02% | 101% | 97% |
| Class E | -4.84% | -6.71% | -7.48% | 111% | 218% | 0.30% | 0.00% | 0.00% | 101% | 96% |
| Overall | -3.11% | -3.55% | -4.06% | 110% | 205% | 0.19% | 0.00% | 0.00% | 101% | 96% |
| Class D | -2.29% | -2.82% | -3.36% | 110% | 165% | 0.43% | 0.03% | 0.03% | 100% | 97% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over VTM-1.0 + ALF |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -4.66% | -2.71% | -1.50% | 111% | 310% | -0.12% | -0.04% | 0.05% | 101% | 95% |
| Class A2 | -6.34% | -1.37% | -0.28% | 113% | 311% | 0.13% | 0.02% | 0.00% | 100% | 95% |
| Class B | -5.56% | -3.46% | -2.28% | 115% | 260% | 0.26% | -0.06% | 0.02% | 100% | 96% |
| Class C | -4.23% | -1.08% | -1.99% | 114% | 224% | 0.34% | -0.09% | 0.00% | 100% | 96% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -5.18% | -2.26% | -1.65% | 113% | 268% | 0.18% | -0.05% | 0.02% | 100% | 96% |
| Class D | -5.15% | -2.06% | -2.68% | 120% | 236% | 0.32% | 0.08% | -0.11% | 100% | 97% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over VTM-1.0 + ALF |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -4.41% | -1.67% | -1.35% | 116% | 234% | 0.12% | -0.07% | -0.04% | 100% | 96% |
| Class C | -4.00% | -1.56% | -1.47% | 115% | 203% | 0.17% | 0.05% | 0.30% | 100% | 97% |
| Class E | -4.95% | -2.34% | -3.19% | 132% | 204% | 0.19% | 0.06% | 0.08% | 100% | 97% |
| Overall | -4.41% | -1.80% | -1.85% | 119% | 216% | 0.15% | 0.00% | 0.10% | 100% | 96% |
| Class D | -4.14% | -1.82% | -1.96% | 118% | 206% | 0.25% | 0.39% | 0.41% | 100% | 96% |

The experimental results of CE2-4.1.1 test for 4x4 block classification based on SSML compared to BMS 1.0 anchor without ALF and BMS 1.0 anchor are summarized in Table II. The average BD-rates of -2.59%, -4.91%, and -4.14% are observed for AI, RA, and LDB cases, respectively, compared to BMS 1.0 anchor without ALF. And, the average BD-rates of 0.15%, 0.17%, and 0.10% are observed for AI, RA, and LDB cases, respectively, compared to BMS 1.0 anchor. Compared to BMS 1.0 anchor, decoding time of 97% for AI, RA, and LDB cases is observed.

Table II. Experimental results of CE2-4.1.1 for 4x4 block classification based on SSML over BMS 1.0 anchor without ALF and BMS 1.0 anchor

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 - ALF |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.23% | -3.89% | -2.76% | 101% | 188% | -0.03% | 0.00% | 0.00% | 101% | 96% |
| Class A2 | -2.76% | -3.26% | -2.96% | 101% | 176% | 0.09% | -0.01% | -0.01% | 100% | 96% |
| Class B | -2.19% | -2.35% | -2.71% | 101% | 169% | 0.11% | 0.00% | 0.00% | 100% | 97% |
| Class C | -2.32% | -2.50% | -3.13% | 101% | 150% | 0.32% | 0.02% | 0.02% | 101% | 97% |
| Class E | -3.83% | -6.16% | -6.98% | 101% | 184% | 0.27% | 0.01% | 0.01% | 101% | 96% |
| Overall | -2.59% | -3.43% | -3.57% | 101% | 171% | 0.15% | 0.01% | 0.01% | 101% | 97% |
| Class D | -1.66% | -2.57% | -2.92% | 100% | 142% | 0.34% | 0.04% | 0.04% | 100% | 98% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 - ALF |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -4.37% | -2.88% | -0.43% | 101% | 209% | -0.11% | -0.10% | -0.02% | 100% | 96% |
| Class A2 | -6.27% | -0.75% | -0.37% | 102% | 202% | 0.14% | -0.02% | 0.00% | 100% | 97% |
| Class B | -5.20% | -3.59% | -2.22% | 101% | 181% | 0.22% | -0.03% | -0.02% | 100% | 97% |
| Class C | -3.92% | -1.15% | -1.77% | 101% | 157% | 0.33% | 0.02% | -0.12% | 100% | 97% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -4.91% | -2.23% | -1.37% | 101% | 183% | 0.17% | -0.03% | -0.04% | 100% | 97% |
| Class D | -4.99% | -2.12% | -2.32% | 102% | 153% | 0.34% | -0.01% | -0.14% | 100% | 98% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 - ALF |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -4.24% | -1.35% | -1.15% | 101% | 186% | 0.09% | -0.07% | 0.35% | 100% | 97% |
| Class C | -4.03% | -2.21% | -1.98% | 101% | 162% | 0.11% | 0.01% | -0.09% | 100% | 96% |
| Class E | -4.13% | -1.39% | -1.73% | 106% | 158% | 0.09% | -0.01% | -0.14% | 100% | 96% |
| Overall | -4.14% | -1.65% | -1.57% | 102% | 171% | 0.10% | -0.03% | 0.08% | 100% | 97% |
| Class D | -4.49% | -2.36% | -1.95% | 102% | 156% | 0.18% | -0.08% | -0.04% | 100% | 96% |

In order to reduce a coding loss of 4x4 block classification based on SSML, the following experiments are additionally conducted. The experimental results of CE2-4.1.1 test for 2x2 block classification based on SSML compared to VTM 1.0 anchor and VTM 1.0 anchor with ALF are summarized in Table III. The average BD-rates of -3.19%, -5.25%, and -4.45% are observed for AI, RA, and LDB cases, respectively, compared to VTM 1.0 anchor. And, the average BD-rates of 0.12%, 0.11%, and 0.10% are observed for AI, RA, and LDB cases, respectively, compared to VTM 1.0 anchor with ALF. Compared to VTM 1.0 anchor with ALF, decoding times of 98%, 97%, and 98% for AI, RA, and LDB cases is observed, respectively.

Table III. Experimental results of CE2-4.1.1 for 2x2 block classification based on SSML over VTM 1.0 anchor and VTM 1.0 anchor with ALF

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over VTM-1.0 + ALF |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.41% | -3.30% | -3.48% | 111% | 246% | 0.05% | 0.00% | 0.00% | 101% | 98% |
| Class A2 | -3.01% | -3.79% | -3.62% | 110% | 215% | 0.04% | 0.00% | 0.00% | 101% | 98% |
| Class B | -2.73% | -2.29% | -2.97% | 110% | 207% | 0.07% | 0.00% | 0.00% | 101% | 98% |
| Class C | -3.11% | -2.74% | -3.62% | 110% | 174% | 0.28% | 0.01% | 0.01% | 101% | 99% |
| Class E | -5.02% | -6.72% | -7.49% | 111% | 223% | 0.11% | 0.00% | 0.00% | 101% | 98% |
| Overall | -3.19% | -3.55% | -4.06% | 111% | 209% | 0.12% | 0.00% | 0.00% | 101% | 98% |
| Class D | -2.48% | -2.84% | -3.38% | 111% | 168% | 0.23% | 0.01% | 0.01% | 101% | 99% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over VTM-1.0 + ALF |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -4.49% | -2.62% | -1.46% | 111% | 317% | 0.06% | 0.04% | 0.08% | 101% | 97% |
| Class A2 | -6.40% | -1.32% | -0.30% | 113% | 317% | 0.05% | 0.08% | -0.01% | 101% | 97% |
| Class B | -5.69% | -3.41% | -2.28% | 115% | 265% | 0.12% | -0.01% | 0.03% | 101% | 98% |
| Class C | -4.39% | -1.01% | -1.97% | 115% | 228% | 0.17% | -0.02% | 0.02% | 100% | 98% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -5.25% | -2.19% | -1.64% | 114% | 273% | 0.11% | 0.02% | 0.03% | 101% | 97% |
| Class D | -5.35% | -2.18% | -2.58% | 121% | 242% | 0.11% | -0.06% | -0.02% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over VTM-1.0 + ALF |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -4.47% | -1.63% | -1.43% | 116% | 239% | 0.05% | -0.02% | -0.12% | 100% | 98% |
| Class C | -4.04% | -1.46% | -1.54% | 115% | 206% | 0.12% | 0.14% | 0.22% | 101% | 98% |
| Class E | -4.96% | -2.57% | -3.31% | 132% | 206% | 0.17% | -0.18% | -0.05% | 100% | 97% |
| Overall | -4.45% | -1.81% | -1.94% | 120% | 219% | 0.10% | -0.01% | 0.01% | 100% | 98% |
| Class D | -4.30% | -1.65% | -2.11% | 118% | 209% | 0.09% | 0.57% | 0.26% | 100% | 97% |

The experimental results of CE2-4.1.1 test for 2x2 block classification based on SSML compared to BMS 1.0 anchor without ALF and BMS 1.0 anchor are summarized in Table IV. The average BD-rates of -2.65%, -4.96%, and -4.24% are observed for AI, RA, and LDB cases, respectively, compared to BMS 1.0 anchor without ALF. And, the average BD-rates of 0.10%, 0.11%, and 0.00% are observed for AI, RA, and LDB cases, respectively, compared to BMS 1.0 anchor. Compared to BMS 1.0 anchor, decoding times of 98%, 98%, and 98% for AI, RA, and LDB cases is observed, respectively. And, the detailed results of Table I, Table II, Table III, and Table IV can be found in the accompanying Excel spreadsheet.

Table IV. Experimental results of CE2-4.1.1 for 2x2 block classification based on SSML over BMS 1.0 anchor without ALF and BMS 1.0 anchor

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 - ALF |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.15% | -3.89% | -2.76% | 101% | 192% | 0.05% | 0.00% | 0.00% | 101% | 98% |
| Class A2 | -2.80% | -3.26% | -2.96% | 104% | 179% | 0.04% | 0.00% | 0.00% | 104% | 98% |
| Class B | -2.23% | -2.35% | -2.71% | 100% | 172% | 0.07% | 0.00% | 0.00% | 100% | 98% |
| Class C | -2.42% | -2.51% | -3.14% | 100% | 152% | 0.21% | 0.01% | 0.01% | 100% | 99% |
| Class E | -3.99% | -6.17% | -7.00% | 100% | 187% | 0.11% | 0.00% | 0.00% | 100% | 98% |
| Overall | -2.65% | -3.43% | -3.57% | 101% | 174% | 0.10% | 0.00% | 0.00% | 101% | 98% |
| Class D | -1.81% | -2.59% | -2.95% | 100% | 144% | 0.18% | 0.01% | 0.01% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 - ALF |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -4.20% | -2.80% | -0.47% | 101% | 214% | 0.07% | -0.02% | -0.06% | 100% | 97% |
| Class A2 | -6.36% | -0.80% | -0.36% | 102% | 205% | 0.04% | -0.06% | 0.00% | 100% | 98% |
| Class B | -5.31% | -3.57% | -2.24% | 101% | 183% | 0.10% | -0.01% | -0.03% | 100% | 98% |
| Class C | -4.05% | -1.28% | -1.67% | 101% | 159% | 0.19% | -0.11% | -0.02% | 100% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -4.96% | -2.25% | -1.36% | 101% | 186% | 0.11% | -0.05% | -0.03% | 100% | 98% |
| Class D | -5.20% | -2.02% | -2.28% | 102% | 155% | 0.12% | 0.08% | -0.10% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 - ALF |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -4.27% | -1.45% | -1.29% | 106% | 188% | 0.06% | -0.18% | 0.20% | 105% | 98% |
| Class C | -4.05% | -2.20% | -1.89% | 101% | 165% | 0.09% | 0.04% | -0.01% | 100% | 98% |
| Class E | -4.43% | -1.53% | -1.80% | 107% | 159% | -0.22% | -0.14% | -0.20% | 100% | 97% |
| Overall | -4.24% | -1.72% | -1.62% | 105% | 173% | 0.00% | -0.10% | 0.03% | 102% | 98% |
| Class D | -4.62% | -2.10% | -2.27% | 102% | 159% | 0.05% | 0.19% | -0.37% | 100% | 98% |

Computational complexity analysis

It is claimed that the CE2-4.1.1 method can reduce the computational complexity compared to ALF in BMS as described in the following: In the 4x4 block classification based on SSML, 1-D Laplacian values, Vp,q, Hp,q, D1p,q and D2p,q calculated at the subsampled positions within 8x8 window are summed up for each 4x4 block to derive gradients, gv, gh, gd1 and gd2, respectively. And, in the 2x2 block classification based on SSML, 1-D Laplacian values, Vp,q, Hp,q, D1p,q and D2p,q calculated at the subsampled positions within 6x6 window are summed up for each 2x2 block to derive gradients, gv, gh, gd1 and gd2, respectively. On the other hand, in the 2x2 block classification based on sum-modified-Laplacian (SML) of ALF in BMS, 1-D Laplacian values calculated at every positions within 6x6 window are summed up for each 2x2 block to derive gradients. Therefore, 32 * (2 additions + 1 shift) + 31 additions are required to derive each gradient for 4x4 block by applying CE2-4.1.1 method for 4x4 block classification based on SSML and 18 * (2 additions + 1 shift) + 17 additions are required to derive each gradient for 2x2 block by applying CE2-4.1.1 method for 2x2 block classification based on SSML and while 36 * (2 additions + 1 shift) + 35 additions are required to derive each gradient for 2x2 block by applying 2x2 block classification of ALF in BMS 1.0 as tabulated in Table V. If the computational complexity for a given block size (e.g., 4x4 block) are compared, the CE2-4.1.1 method for 4x4 block classification based on SSML reduces approximately 78% of worst case operation count compared to block classification of ALF. And, the CE2-4.1.1 method for 2x2 block classification based on SSML reduces approximately 50% of worst case operation count compared to block classification of ALF.

Table V. Computational complexity comparison between ALF in BMS 1.0 and CE2-4.1.1

|  | ALF in BMS 1.0 | CE2-4.1.1 for 4x4 block classification based on SSML | CE2-4.1.1 for 2x2 block classification based on SSML |
| --- | --- | --- | --- |
| Block classification granularity | 2x2 (Luma) | 4x4 (Luma) | 2x2 (Luma) |
| Operation count for gradients calculation (gv, gh, gd1, and gd2) | For each of gradients of 2x2 block: 36 * (2 additions + 1 shift) + 35 additions = 107 additions + 36 shifts For each of gradients of 4x4 block: 4 * (107 additions + 36 shifts) = 428 additions + 144 shifts | For each of gradients of 4x4 block: 32 * (2 additions + 1 shift) + 31 additions = 95 additions + 32 shifts | For each of gradients of 2x2 block: 18 * (2 additions + 1 shift) + 17 additions = 53 additions + 18 shifts For each of gradients of 4x4 block: 4 * (53 additions + 18 shifts) = 212 additions + 72 shifts |

Conclusions

This contribution reports the experimental results of CE2-4.1.1 test. In CE2-4.1.1 test, the 4x4 block classification based on SSML was tested in order to reduce the computational complexity of the block classification of ALF in BMS. And, 2x2 block classification based on SSML is also tested. The CE2-4.1.1 method for 4x4 block classification based on SSML reportedly reduces approximately 78% of worst case operation count compared to block classification method of ALF in BMS. The CE2-4.1.1 method for 2x2 block classification based on SSML reportedly reduces approximately 50% of worst case operation count compared to block classification compared to ALF in BMS. The experimental results for 4x4 block classification based on SSML reportedly show average BD-rates of 0.19%, 0.18%, and 0.15% with decoding times of 96%, 96%, and 96% for AI, RA, and LDB cases, respectively, compared to VTM 1.0 anchor with ALF. And, the experimental results for 4x4 block classification based on SSML reportedly show average BD-rates of 0.15%, 0.17%, and 0.10% with decoding times of 97%, 97%, and 97% for AI, RA, and LDB cases, respectively, compared to BMS 1.0 anchor. And, the experimental results for 2x2 block classification based on SSML reportedly show average BD-rates of 0.12%, 0.11%, and 0.10% with decoding times of 98%, 97%, and 98% for AI, RA, and LDB cases, respectively, compared to VTM 1.0 anchor with ALF. And, the experimental results for 2x2 block classification based on SSML reportedly show average BD-rates of 0.10%, 0.11%, and 0.00% with decoding times of 98%, 98%, and 98% for AI, RA, and LDB cases, respectively, compared to BMS 1.0 anchor. Therefore, it is recommended to adopt this test to reduce the computational complexity of ALF in BMS.

References

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0

- L. Zhang, K. Andersson, and C.-Y. Chen, “Description of Core Experiment 2 (CE2) on In-Loop Filters,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-J1022, Apr. 2018

- J. Kang, H. Lee, S.-C. Lim, J. Lee, H. Y. Kim, N.-U. Kim, and Y.-L. Lee, “Description of SDR video coding technology proposal by ETRI and Sejong University,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-J0013, Apr. 2018

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE2/CE2-4.1.1

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-J1010, Apr. 2018

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).