| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0165-v1 |
| --- | --- |

| Title: | CE3: Combined filter (Test 2.6.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jinho Lee Hahyun Lee Sung-Chang Lim Jungwon Kang Hui Yong Kim 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-5656 jinosoul@etri.re.kr hanilee@etri.re.kr sclim@etri.re.kr jungwon@etri.re.kr hykim5@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

- Abstract

This contribution presents test results of CE3.2.6.1 described in JVET-J1023. CE3.2.6.1 presents 6-tap combined filter which is combining 3-tap reference sample smoothing filter and 4-tap interpolation filter defined in JEM7.0 for intra prediction. When the combined filter is applied, reference sample smoothing process is removed. For luma angular prediction, the combined filter is applied, and the bilinear filter in HEVC is used for chroma angular prediction. It is reported that the average luma BD-rate gain for AI and RA cases of the combined filter are 0.55% and 0.29% compared to VTM1.0, and 0.39% and 0.14% compared to BMS1.0, respectively.

- Algorithm description

In JVET-J0013[1], combined filter (CF) for intra prediction is proposed and included in CE3[2]. The CF combines two filters which are 3-tap reference sample smoothing filter and 4-tap interpolation filter defined in JEM7.0. Therefore, 6-tap interpolation filter is used for intra angular prediction, and the reference sample smoothing process which is applying low-pass filter according to intra prediction mode and block size is removed. The CF is applied for luma angular prediction, on the other hand, bilinear filter in HEVC is used for chroma angular prediction. The CF has two filters and it is selectively used based on whether a width or height of a block is larger than 8 as described in Table 1 and 2. For some angular modes having 45, 135, or 225 degrees, a 3-tap [1, 2, 1] filter is used for prediction when a block’s width or height is larger than 8.

Table 1. Filter types according to color component and block size

| Intra Prediction Mode | Luma |  | Chroma |
| --- | --- | --- | --- |
|  | Width <= 8 | Width > 8 |  |
| Angular modes (except V/H, 45, 135, and 225 degrees modes) | CF A | CF B | Bilinear |
| 45, 135, and 225 degrees modes | Copy | 3-tap filter [1, 2, 1] | Copy |

Table 2. Combined filters

| CF A (width <= 8) | CF B (width > 8) |
| --- | --- |
| { 0, 0, 256, 0, 0, 0 }, { 0, -4, 253, 9, -2, 0 }, { 1, -7, 249, 17, -4, 0 }, { 1, -10, 245, 25, -6, 1 }, { 1, -13, 241, 34, -8, 1 }, { 2, -16, 235, 44, -10, 1 }, { 2, -18, 229, 53, -12, 2 }, { 2, -20, 223, 63, -14, 2 }, { 2, -22, 217, 72, -15, 2 }, { 3, -23, 209, 82, -17, 2 }, { 3, -24, 202, 92, -19, 2 }, { 3, -25, 194, 101, -20, 3 }, { 3, -25, 185, 111, -21, 3 }, { 3, -26, 178, 121, -23, 3 }, { 3, -25, 168, 131, -24, 3 }, { 3, -25, 159, 141, -25, 3 }, { 3, -25, 150, 150, -25, 3 } | { 47, 255, 416, 256, 49, 1 } { 43, 247, 416, 264, 53, 1 } { 40, 240, 414, 270, 58, 2 } { 37, 233, 413, 277, 62, 2 } { 34, 226, 412, 284, 66, 2 } { 31, 218, 410, 292, 71, 2 } { 28, 210, 407, 299, 77, 3 } { 26, 203, 404, 306, 82, 3 } { 23, 195, 401, 313, 88, 4 } { 21, 188, 398, 320, 93, 4 } { 19, 180, 393, 327, 100, 5 } { 17, 173, 389, 333, 106, 6 } { 16, 167, 385, 339, 111, 6 } { 14, 159, 380, 346, 118, 7 } { 13, 153, 375, 351, 124, 8 } { 11, 145, 370, 358, 131, 9 } { 10, 138, 364, 364, 138, 10 } |

Experimental results

The experiments were performed based on BMS 1.0 software [3] with BMS and VTM configuration files under JVET common test condition [4]. To check mismatch between encoder and decoder, MD5 decoded picture hash SEI message is enabled.

Table 3 shows experimental results of the CE3.2.6.1 test compared to VTM1.0 and BMS1.0, respectively. Compared to VTM1.0, the luma BD-rate gain of 0.55% for AI case and 0.29% for RA case are observed. And, the luma BD-rate gain of 0.39% for AI case and 0.14% for RA case are observed compared to BMS1.0.

Table 3. Results of CE3.2.6.1 over VTM1.0 and BMS1.0

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.25% | 0.02% | 0.02% | 109% | 101% | -0.09% | -0.20% | -0.22% | 101% | 100% |
| Class A2 | -0.03% | 0.34% | 0.32% | 108% | 101% | 0.00% | 0.28% | 0.22% | 102% | 100% |
| Class B | -0.45% | 0.04% | 0.19% | 108% | 102% | -0.35% | -0.09% | -0.10% | 101% | 101% |
| Class C | -1.16% | -0.38% | -0.39% | 107% | 103% | -0.93% | -0.50% | -0.52% | 101% | 101% |
| Class E | -0.70% | 0.02% | 0.00% | 108% | 103% | -0.45% | 0.04% | -0.11% | 101% | 101% |
| Overall | -0.55% | -0.01% | 0.02% | 108% | 102% | -0.39% | -0.11% | -0.16% | 101% | 101% |
| Class D | -0.70% | -0.15% | -0.25% | 107% | 103% | -0.50% | -0.14% | -0.12% | 101% | 101% |
| Class F (optional) | -0.51% | -0.28% | -0.28% | 108% | 102% | -0.42% | -0.38% | -0.47% | 101% | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.18% | 0.04% | 102% | 100% | -0.05% | -0.18% | -0.14% | 100% | 100% |
| Class A2 | -0.08% | 0.28% | 0.18% | 101% | 100% | -0.01% | 0.20% | 0.14% | 100% | 100% |
| Class B | -0.29% | 0.21% | 0.11% | 102% | 100% | -0.18% | 0.22% | 0.07% | 100% | 99% |
| Class C | -0.56% | 0.16% | 0.17% | 102% | 100% | -0.28% | 0.15% | -0.33% | 100% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.29% | 0.13% | 0.13% | 102% | 100% | -0.14% | 0.12% | -0.07% | 100% | 100% |
| Class D | -0.34% | 0.25% | -0.34% | 102% | 100% | -0.23% | 0.15% | 0.17% | 100% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Additionally, Table 4 shows test results of CE3.2.6.1 when the CF is applied to both luma and chroma angular prediction. It is reported that the luma BD-rate is similar to CE3.2.6.1 with luma only, but chroma U and V BD-rate gains for AI case are 0.55% and 0.62% compared to VTM1.0, and 0.62% and 0.59% compared to BMS1.0, respectively

Table 4. Results of CE3.2.6.1 (luma & chroma) over VTM1.0 and BMS1.0

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -0.53% | -0.69% | 110% | 102% | -0.12% | -0.44% | -0.26% | 102% | 100% |
| Class A2 | 0.00% | 0.10% | -0.14% | 109% | 101% | 0.00% | -0.06% | -0.13% | 102% | 101% |
| Class B | -0.45% | -0.64% | -0.50% | 109% | 103% | -0.38% | -0.71% | -0.66% | 102% | 101% |
| Class C | -1.29% | -1.59% | -1.89% | 108% | 104% | -1.06% | -1.21% | -1.13% | 102% | 101% |
| Class E | -0.59% | 0.34% | 0.49% | 109% | 103% | -0.37% | -0.43% | -0.54% | 102% | 101% |
| Overall | -0.55% | -0.55% | -0.62% | 109% | 103% | -0.42% | -0.62% | -0.59% | 102% | 101% |
| Class D | -0.75% | -0.71% | -0.89% | 109% | 104% | -0.53% | -0.60% | -0.79% | 102% | 101% |
| Class F (optional) | -0.68% | -1.03% | -1.27% | 109% | 103% | -0.55% | -0.83% | -0.92% | 102% | 100% |

Conclusions

This contribution reports the experimental result of CE3.2.6.1 test. In the test, a combined filter which is combining 3-tap reference sample smoothing filter and 4-tap interpolation filter defined in JEM7.0 is proposed. In addition, the reference sample construction is simplified by removing the reference sample smoothing process. The experimental results reportedly show that the average luma BD-rates gain of 0.55% and 0.29% for AI and RA cases compared to VTM1.0, and 0.39% and 0.14% for AI and RA cases compared to BMS1.0, respectively.

Reference

- J. Kang, H. Lee, S.-C. Lim, J. Lee, H. Y. Kim, N.-U. Kim, and Y.-L. Lee, “Description of SDR video coding technology proposal by ETRI and Sejong University,” JVET-J0013, April. 2018

- G. Van der Auwera, J. Heo, and A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” JVET-J1023, April 2018.

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, April 2018.

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).