| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0166-v1 |
| --- | --- |

| Title: | CE3: Multi-line based intra prediction (Test 5.3.1, 5.3.2, 5.3.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jinho Lee Hahyun Lee Sung-Chang Lim Jungwon Kang Hui Yong Kim 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-5656 jinosoul@etri.re.kr hanilee@etri.re.kr sclim@etri.re.kr jungwon@etri.re.kr hykim5@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

- Abstract

This contribution presents test results of CE3.5.3.1, CE3.5.3.2, and CE3.5.3.3 described in JVET-J1023. CE3.5.3 presents multi-line based intra prediction that performs weighted average of two prediction values generated from first reference line and second reference line. Interpolation filters applying to the first reference line are set to 6-tap combined filter, 4-tap filter, and bilinear filter in test CE3.5.3.1, CE3.5.3.2, and CE3.5.3.3, respectively. For the second reference line, bilinear filter is used for all the sub tests. It is reported that the average luma BD-rate gain for AI case of CE3.5.3.1, CE3.5.3.2, and CE3.5.3.3 are 0.75%, 0.76%, and 0.29% compared to VTM1.0, and 0.57%, 0.56%, and 0.19% compared to BMS1.0, respectively.

- Algorithm description

In JVET-J0013[1], multi-line based intra prediction (MIP) is proposed and included in CE3[2]. The MIP performs intra prediction by weighted average of two prediction values generated from first reference line and second reference line. Figure 1 depicts an example of CE3.5.3.1 which is applying 6-tap combined filter (CF) to the first reference line (1st Ref. line) and bilinear filter to the second reference line (2nd Ref. line). The 6-tap CF is same as the one in CE3.2.6.1[2] and reference sample smoothing process is removed when the CF is applied. Likewise, 4-tap filter defined in JEM7.0, and bilinear filter in HEVC is used for the first reference line in CE3.5.3.2 and CE3.5.3.3, respectively. Table 1 shows detail filter types for angular prediction according to color component and reference line.

Fig. 1. An example of MIP in CE3.5.3.1

Table 1. Filter types according to color component and reference line

| Test | Luma |  | Chroma |
| --- | --- | --- | --- |
|  | 1st Ref. line | 2nd Ref. line |  |
| CE3.5.3.1 | 6-tap CF | Bilinear | 6-tap CF |
| CE3.5.3.2 | 4-tap | Bilinear | 4-tap |
| CE3.5.3.3 | Bilinear | Bilinear | Bilinear |

The MIP is applied to luma samples when an intra prediction mode is one of vertical, horizontal, and angular modes. For the weighted average, weighting factor 3/4 is used for the first reference line and weighting factor 1/4 is used for the second reference line. When the MIP is applied, reference sample smoothing process for the second line is not performed to reduce complexity. In addition, the MIP is restricted when a prediction sample is located at the upper boundary in a coding tree block(CTB).

Experimental results

The experiments were performed based on BMS 1.0 software [3] with BMS and VTM configuration files under JVET common test condition [4]. To check mismatch between encoder and decoder, MD5 decoded picture hash SEI message is enabled.

Table 2, 3, and 4 show experimental results of the CE3.5.3.1, CE3.5.3.2, and CE3.5.3.3 compared to VTM1.0 and BMS1.0, respectively.

In CE3.5.3.1, the average luma BD-rate gain of 0.75% and 0.38% for AI and RA cases are observed compared to VTM1.0, and 0.57% and 0.26% for AI and RA cases are observed compared to BMS1.0. Similar results are observed in CE3.5.3.2 with the average luma BD-rate gain of 0.76% and 0.40% for AI and RA cases compared to VTM1.0, and 0.56% and 0.27% for AI and RA cases compared to BMS1.0 as described in Table 3. Lastly, applying bilinear filter both the first and second reference line as represented in CE3.5.3.3 gives average luma BD-rate gain of 0.29% and 0.16% for AI and RA cases compared to VTM1.0, and 0.19% and 0.09% for AI and RA cases compared to BMS1.0, respectively.

Table 2. Results of CE3.5.3.1 over VTM1.0 and BMS1.0

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.43% | -0.43% | -0.58% | 120% | 106% | -0.17% | -0.17% | -0.18% | 106% | 101% |
| Class A2 | -0.29% | 0.00% | -0.25% | 119% | 106% | -0.21% | -0.05% | -0.15% | 105% | 102% |
| Class B | -0.67% | -0.70% | -0.63% | 119% | 106% | -0.58% | -0.77% | -0.69% | 105% | 102% |
| Class C | -1.44% | -1.78% | -2.19% | 117% | 107% | -1.21% | -1.25% | -1.26% | 104% | 103% |
| Class E | -0.73% | 0.21% | 0.51% | 119% | 107% | -0.47% | -0.41% | -0.43% | 105% | 103% |
| Overall | -0.75% | -0.63% | -0.72% | 119% | 107% | -0.57% | -0.60% | -0.60% | 105% | 102% |
| Class D | -0.86% | -0.92% | -0.99% | 117% | 106% | -0.60% | -0.50% | -0.57% | 104% | 103% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -0.50% | -0.71% | 105% | 101% | -0.13% | -0.12% | -0.15% | 102% | 100% |
| Class A2 | -0.15% | -0.54% | -0.74% | 104% | 101% | -0.10% | -0.28% | -0.33% | 102% | 100% |
| Class B | -0.37% | -1.23% | -1.11% | 104% | 101% | -0.29% | -0.85% | -0.62% | 102% | 100% |
| Class C | -0.64% | -1.47% | -1.57% | 104% | 101% | -0.45% | -0.63% | -0.81% | 102% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.38% | -1.01% | -1.08% | 104% | 101% | -0.26% | -0.53% | -0.52% | 102% | 101% |
| Class D | -0.41% | -0.92% | -1.40% | 104% | 101% | -0.24% | -0.64% | -0.48% | 102% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 3. Results of CE3.5.3.2 over VTM1.0 and BMS1.0

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.47% | -0.29% | -0.51% | 117% | 105% | -0.25% | -0.29% | -0.32% | 105% | 101% |
| Class A2 | -0.45% | -0.31% | -0.44% | 116% | 105% | -0.32% | -0.21% | -0.27% | 104% | 102% |
| Class B | -0.75% | -0.68% | -0.62% | 116% | 106% | -0.63% | -0.62% | -0.60% | 104% | 102% |
| Class C | -1.17% | -1.47% | -1.72% | 114% | 107% | -0.91% | -0.78% | -0.86% | 103% | 103% |
| Class E | -0.82% | -0.15% | 0.19% | 116% | 107% | -0.54% | -0.48% | -0.50% | 104% | 103% |
| Overall | -0.76% | -0.64% | -0.68% | 116% | 106% | -0.56% | -0.51% | -0.54% | 104% | 102% |
| Class D | -0.75% | -0.78% | -0.76% | 114% | 106% | -0.48% | -0.52% | -0.47% | 104% | 103% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.31% | -0.44% | -0.65% | 104% | 101% | -0.15% | -0.29% | -0.11% | 102% | 100% |
| Class A2 | -0.23% | -0.56% | -0.73% | 103% | 101% | -0.15% | -0.43% | -0.31% | 102% | 101% |
| Class B | -0.43% | -0.87% | -1.12% | 103% | 101% | -0.34% | -0.58% | -0.53% | 102% | 100% |
| Class C | -0.55% | -1.15% | -1.30% | 103% | 101% | -0.35% | -0.50% | -0.61% | 102% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.40% | -0.80% | -1.00% | 103% | 101% | -0.27% | -0.47% | -0.42% | 102% | 100% |
| Class D | -0.35% | -0.66% | -1.24% | 103% | 101% | -0.20% | -0.29% | -0.74% | 102% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 4. Results of CE3.5.3.3 over VTM1.0 and BMS1.0

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.25% | -0.06% | -0.11% | 111% | 103% | -0.10% | 0.09% | 0.01% | 103% | 101% |
| Class A2 | -0.28% | -0.08% | -0.10% | 110% | 103% | -0.17% | 0.03% | -0.02% | 103% | 101% |
| Class B | -0.30% | -0.12% | -0.13% | 110% | 103% | -0.23% | -0.03% | -0.04% | 103% | 101% |
| Class C | -0.34% | -0.28% | -0.30% | 109% | 104% | -0.29% | -0.14% | -0.12% | 102% | 102% |
| Class E | -0.25% | -0.17% | 0.01% | 111% | 103% | -0.12% | 0.13% | 0.02% | 103% | 101% |
| Overall | -0.29% | -0.15% | -0.13% | 110% | 104% | -0.19% | 0.00% | -0.03% | 103% | 101% |
| Class D | -0.15% | -0.09% | -0.26% | 110% | 104% | -0.06% | 0.19% | 0.11% | 103% | 102% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.13% | -0.08% | 102% | 101% | -0.05% | 0.05% | 0.07% | 101% | 100% |
| Class A2 | -0.12% | -0.09% | -0.04% | 102% | 100% | -0.07% | 0.02% | 0.02% | 101% | 101% |
| Class B | -0.16% | -0.01% | -0.30% | 102% | 100% | -0.12% | 0.12% | -0.15% | 101% | 100% |
| Class C | -0.20% | -0.14% | -0.13% | 102% | 100% | -0.09% | 0.02% | -0.33% | 101% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.16% | -0.08% | -0.16% | 102% | 100% | -0.09% | 0.06% | -0.12% | 101% | 100% |
| Class D | -0.09% | 0.13% | -0.23% | 102% | 100% | 0.02% | -0.18% | 0.25% | 101% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Conclusions

This contribution reports the experimental results of CE3.5.3.1, CE3.5.3.2, and CE3.5.3.3 tests. In the tests, multi-line based intra prediction which is averaging two prediction values generated from first reference line and second reference line is proposed. For the first reference line, 6-tap, 4-tap, or bilinear filter is used. On the other hand, for the second reference line, reference sample smoothing process is not performed and bilinear filter is used for prediction to reduce complexity. The experimental results of CE3.5.3.1 reportedly show that the average luma BD-rates gain of 0.75% and 0.38% for AI and RA cases compared to VTM1.0, and 0.57% and 0.26% for AI and RA cases compared to BMS1.0, respectively.

Reference

- J. Kang, H. Lee, S.-C. Lim, J. Lee, H. Y. Kim, N.-U. Kim, and Y.-L. Lee, “Description of SDR video coding technology proposal by ETRI and Sejong University,” JVET-J0013, April. 2018

- G. Van der Auwera, J. Heo, and A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” JVET-J1023, April 2018.

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, April 2018.

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).