| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0167_r2 |
| --- | --- |

| Title: | CE6: DST-VII with residual flipping (Test 1.4) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sung-Chang Lim Jungwon Kang Hahyun Lee Jinho Lee Seunghyun Cho Hui Yong Kim ETRI, 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA Nam-Uk Kim Yung-Lyul Lee Sejong University, 209 Neungdong-ro, Gwangjin-gu, Seoul, 05006, KOREA Dae-Yeon Kim Wook Je Jeong Chips&Media, 11/12/13F, V&S Tower, 26 Samseong-ro 85 gil, Gangnam-Gu, Seoul, 06194, KOREA | Tel: Email: | +82-42-860-5772 sclim@etri.re.kr jungwon@etri.re.kr hanilee@etri.re.kr jinosoul@etri.re.kr shcho@etri.re.kr hykim5@etri.re.kr nukim@sju.ac.kr yllee@sejong.ac.kr brad.kim@chipsnmedia.com nick.jeong@chipsnmedia.com |
| Source: | Electronics and Telecommunications Research Institute (ETRI), Sejong University, and Chips&Media |  |  |

_____________________________

- Abstract

This contribution reports the experimental results of CE6-1.4 test proposed in call for proposal (CfP) response document of JVET-J0013. In CE6-1.4 test, DST-VII with residual flipping for up to 64x64 transform is tested in order to reduce the required memory sizes for storing transform matrices compared to the adaptive multiple transform (AMT) in benchmark set (BMS). And, DST-VII with residual flipping for up to 32x32 transform is also tested. The CE6-1.4 method with up to 64x64 transform reportedly saves about 60% of memory for transform matrices compared to AMT and 75% of memory for additional transform matrices in addition to DCT-II compared to AMT. The CE6-1.4 method with up to 32x32 transform reportedly saves about 75% of memory for transform matrices compared to AMT and about 94% of memory for additional transform matrices in addition to DCT-II compared to AMT. The experimental results with up to 64x64 transform reportedly show average BD-rates of -3.24%, -2.06%, and -1.49% with encoding/decoding times of 198%/120%, 163%/109%, and 163%/111% for AI, RA, and LDB cases, respectively, compared to VTM 1.1 anchor. And, the experimental results with up to 64x64 transform reportedly show average BD-rates of 0.25%, 0.13%, and 0.01% with encoding/decoding times of 94%/99%, 96%/100%, and 98%/100% for AI, RA, and LDB cases, respectively, compared to BMS 1.1 anchor. And, the experimental results with up to 32x32 transform reportedly show average BD-rates of -3.23%, -1.94%, and -1.37% for AI, RA, and LDB cases, respectively, compared to VTM 1.1 anchor. And, the experimental results with up to 32x32 transform reportedly show average BD-rates of 0.27%, 0.26%, and 0.15% for AI, RA, and LDB cases, respectively, compared to BMS 1.1 anchor.

Test descriptions

To investigate the coding tools related to transforms and transform signalling on top of the VVC test model (VTM), the tradeoff between compression efficiency and complexity of different transform tools was suggested to be investigated in the core experiment 6 (CE6) on transforms and transform signalling [1] including CE6-1.4 test. In CE6-1.4 test, as a primary transform, the adaptive multiple transform (AMT) in BMS [2] is replaced with DST-VII with residual flipping as proposed in [3] in order to reduce the required memory sizes for storing transform matrices compared to AMT. The DST-VII transform matrices used in this test are the same as defined in AMT. In addition, DST-VII with residual flipping applies to the CUs with both width and height smaller than or equal to 64 same as AMT. And, DST-VII with residual flipping applies to the CUs with both width and height smaller than or equal to 32 is also tested.

When intra prediction is performed for a CU, the prediction errors usually increase as the distance from the reference sample increases. Also, when inter prediction is performed for a CU and the motion vector of the CU is different from the neighboring CU, the prediction errors are usually higher near the CU boundary than in the middle of the CU. Therefore, the N-point DST-VII is efficient for the residual distribution having the shape of the v0 basis vector (low frequency component), as shown in Fig. 1.

Fig. 1	Basis vectors of DST-VII

Thus, in CE6-1.4 method, it is asserted that, for the CU having a large prediction error in the CU boundary, coding efficiency can be improved if DST-VII is applied after residual is flipped to be suitable for DST-VII at the encoder, as shown in Fig. 2.

Fig. 2	Primary transform using DST-VII with residual flipping

And, for the decoder, the inverse residual flipping is applied after DST-VII is performed as shown in Fig 3. Since the inverse residual flipping process is exactly the same as the forward residual flipping process, the residual flipping method performed in the decoder is performed in the same way as in the encoder.

Fig. 3	Inverse primary transform using DST-VII with residual flipping

A CU-level flag is signalled whether DST-VII with residual flipping is used or not. When the CU-level flag is false, DCT-II is used. When the CU-level flag is true, DST-VII is used and an index is signalled to indicate which type of residual flipping is to be used with DST-VII. Four types of residual flipping are shown in Fig. 4.

Fig. 4	Four types of residual flipping

And, each residual flipping type can be implemented as follows:

No-Flip (no flipping): 𝑝′(𝑥, 𝑦) = 𝑝(𝑥, 𝑦)

H-Flip (horizontal flipping): 𝑝′(𝑥, 𝑦) = 𝑝(𝑊−1−𝑥, 𝑦)

V-Flip (vertical flipping): 𝑝′(𝑥, 𝑦) = 𝑝(𝑥, 𝐻−1−𝑦)

HV-Flip (horizontal & vertical flipping): 𝑝′(𝑥, 𝑦) = 𝑝(𝑊−1−𝑥, 𝐻−1−𝑦)

where 𝑝(𝑥, 𝑦) is residual, 𝑝′(𝑥, 𝑦) is flipped residual, and 𝑊 and 𝐻 are the width and height of the CU, respectively.

In this test, the method of selecting the optimal primary transform among DCT-II and DST-VII with four types of residual flipping is performed in a manner similar to AMT in BMS. For intra residual, due to the different residual statistics of different intra prediction modes, a mode-dependent residual flipping candidate selection process is used in a similar way to AMT. The four residual flipping candidate sets are pre-defined for intra prediction mode as tabulated in Table I. One of the residual flipping candidates in the pre-defined residual flipping candidate set for the given intra prediction mode is selected based on the signalled index. For inter residual, one of four residual flipping methods is selected based on the signalled index.

Table I: Four pre-defined residual flipping candidate sets

| Intra Prediction Mode | Residual Flipping Candidates |
| --- | --- |
| Diagonal modes (34 ± 4) | {No-Flip} |
| Horizontal direction modes (18 ± 4) | {No-Flip and V-Flip} |
| Vertical direction mode (50 ± 4) | {No-Flip and H-Flip} |
| Others | {No-Flip, H-Flip, V-Flip, and HV-Flip} |

Experiment result

The CE6-1.4 experiments were performed using CE6-1.4 software [4], based on BMS 1.1 software [2], with BMS configuration file and versatile test model (VTM) configuration file under the JVET common test condition [5]. To check mismatch between encoder and decoder, both BMS configuration file and VTM configuration file are modified to enable MD5 decoded picture hash SEI message. The experiments are conducted on the platform of Intel Xeon E5-1620 v2 (3.7GHz) and 32GB RAM with GCC 4.7.2 compiler under CentOS Linux.

The following macro definitions in the CE6-1.4 software were used to run the experiments with up to 64x64 transform.

#define CE614 1 ///< CE6.1.4 DST7 with Residual Flipping (JVET-J0013) #if CE614 #define CE614_MAX_CU_SIZE 64 #define CE614_INTRA_MODE_TH 4 #define CE614_SIG_NUM_TH 2 #endif

And, the following macro definitions in the CE6-1.4 software were used to run the experiments with up to 32x32 transform.

#define CE614 1 ///< CE6.1.4 DST7 with Residual Flipping (JVET-J0013) #if CE614 #define CE614_MAX_CU_SIZE 32 #define CE614_INTRA_MODE_TH 4 #define CE614_SIG_NUM_TH 2 #endif

The experimental results of CE6-1.4 test with up to 64x64 transform compared to VTM 1.1 anchor are summarized in the left side of Table II. The average BD-rates of -3.24%, -2.06%, and -1.49% are observed for AI, RA, and LDB cases, respectively, compared to VTM 1.1 anchor. For the runtime comparison, encoding/decoding times of 198%/120%, 163%/109%, and 163%/111% for AI, RA, and LDB cases are observed, respectively. And, the experimental results of CE6-1.4 test with non-separable secondary transform (NSST) and up to 64x64 transform compared to VTM 1.1 anchor with NSST are summarized in the right side of Table II. The average BD-rates of -2.17%, -1.62%, and -1.32% are observed for AI, RA, and LDB cases, respectively, compared to VTM 1.1 anchor with NSST. For the runtime comparison, encoding/decoding times of 217%/115%, 190%/108%, and 175%/110% for AI, RA, and LDB cases are observed, respectively.

Table II. Experimental results of CE6-1.4 with up to 64x64 transform over VTM 1.1 anchor (left side) and CE6-1.4 with NSST and up to 64x64 transform over VTM 1.1 anchor with NSST (right side)

| All Intra Main10. Over VTM-1.1. Y; U; V; EncT; DecT. Class A1; -3.09%; -1.35%; -1.23%; 195%; 131%. Class A2; -2.98%; -1.70%; -1.57%; 199%; 123%. Class B; -3.23%; -1.58%; -1.57%; 203%; 123%. Class C; -2.89%; -1.18%; -1.35%; 204%; 110%. Class E; -4.16%; -3.00%; -2.87%; 187%; 120%. Overall; -3.24%; -1.71%; -1.68%; 198%; 120%. Class D; -2.82%; -0.97%; -0.82%; 214%; 106%. Random Access Main 10. Over VTM-1.1. Y; U; V; EncT; DecT. Class A1; -1.82%; -0.04%; 0.02%; 170%; 115%. Class A2; -1.97%; 0.03%; 0.34%; 155%; 109%. Class B; -2.40%; -0.33%; -0.53%; 161%; 109%. Class C; -1.90%; 0.07%; -0.11%; 165%; 106%. Class E. Overall; -2.06%; -0.09%; -0.13%; 163%; 109%. Class D; -1.76%; 0.32%; -0.36%; 162%; 102%. Low delay B Main10. Over VTM-1.1. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; -1.90%; 0.72%; 0.86%; 168%; 113%. Class C; -1.26%; 0.75%; 1.04%; 163%; 109%. Class E; -1.11%; 1.60%; 1.20%; 154%; 108%. Overall; -1.49%; 0.95%; 1.01%; 163%; 111%. Class D; -1.17%; 0.92%; 0.40%; 151%; 103%. | All Intra Main10. Over VTM-1.1 + NSST. Y; U; V; EncT; DecT. Class A1; -1.99%; -1.23%; -1.02%; 210%; 123%. Class A2; -2.14%; -1.47%; -1.09%; 216%; 118%. Class B; -2.26%; -1.31%; -1.50%; 221%; 117%. Class C; -1.79%; -0.75%; -1.03%; 225%; 106%. Class E; -2.75%; -2.75%; -2.73%; 206%; 114%. Overall; -2.17%; -1.44%; -1.45%; 217%; 115%. Class D; -1.74%; -0.71%; -0.77%; 235%; 104%. Random Access Main 10. Over VTM-1.1 + NSST. Y; U; V; EncT; DecT. Class A1; -1.32%; 0.00%; 0.03%; 198%; 113%. Class A2; -1.67%; 0.01%; 0.38%; 180%; 108%. Class B; -1.95%; -0.48%; -0.68%; 190%; 108%. Class C; -1.39%; 0.04%; -0.23%; 192%; 105%. Class E. Overall; -1.62%; -0.14%; -0.20%; 190%; 108%. Class D; -1.33%; 0.16%; 0.11%; 190%; 102%. Low delay B Main10. Over VTM-1.1 + NSST. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; -1.74%; 0.76%; 0.57%; 184%; 113%. Class C; -1.15%; 1.00%; 0.61%; 177%; 109%. Class E; -0.86%; 0.78%; 0.34%; 157%; 109%. Overall; -1.32%; 0.84%; 0.53%; 175%; 110%. Class D; -1.02%; 0.98%; 0.82%; 161%; 103%. |
| --- | --- |

The experimental results of CE6-1.4 test with up to 64x64 transform compared to BMS 1.1 anchor are summarized in the left side of Table III. The average BD-rates of 0.25%, 0.13%, and 0.01% are observed for AI, RA, and LDB cases, respectively, compared to BMS 1.1 anchor. For the runtime comparison, encoding/decoding times of 94%/99%, 96%/100%, and 98%/100% for AI, RA, and LDB cases are observed, respectively. And, the experimental results of CE6-1.4 test without NSST and with up to 64x64 transform compared to BMS 1.1 anchor without NSST are summarized in the right side of Table III. The average BD-rates of 0.29%, 0.13%, and 0.09% are observed for AI, RA, and LDB cases, respectively, compared to BMS 1.1 anchor without NSST. For the runtime comparison, encoding/decoding times of 96%/99%, 98%/100%, and 99%/100% for AI, RA, and LDB cases are observed, respectively. And, the detailed results of Table II and Table III can be found in the accompanying Excel spreadsheet.

Table III. Experimental results of CE6-1.4 with up to 64x64 transform over BMS 1.1 anchor (left side) and CE6-1.4 without NSST and with up to 64x64 transform over BMS 1.1 anchor without NSST (right side)

| All Intra Main10. Over BMS-1.1. Y; U; V; EncT; DecT. Class A1; 0.33%; 0.33%; 0.20%; 96%; 98%. Class A2; 0.29%; 0.21%; -0.03%; 94%; 99%. Class B; 0.27%; 0.15%; 0.10%; 94%; 99%. Class C; 0.16%; 0.02%; 0.08%; 92%; 99%. Class E; 0.20%; -0.02%; -0.04%; 94%; 99%. Overall; 0.25%; 0.13%; 0.06%; 94%; 99%. Class D; 0.15%; 0.07%; 0.16%; 92%; 99%. Random Access Main 10. Over BMS-1.1. Y; U; V; EncT; DecT. Class A1; 0.16%; 0.26%; 0.17%; 95%; 100%. Class A2; 0.13%; 0.30%; -0.07%; 96%; 100%. Class B; 0.11%; 0.30%; 0.06%; 96%; 100%. Class C; 0.13%; 0.29%; -0.22%; 95%; 100%. Class E. Overall; 0.13%; 0.29%; -0.02%; 96%; 100%. Class D; 0.08%; -0.02%; 0.30%; 96%; 100%. Low delay B Main10. Over BMS-1.1. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; 0.01%; 0.15%; 0.10%; 97%; 100%. Class C; 0.05%; 0.11%; 0.20%; 98%; 100%. Class E; -0.05%; -0.20%; 0.36%; 99%; 99%. Overall; 0.01%; 0.05%; 0.20%; 98%; 100%. Class D; 0.02%; -0.52%; -0.31%; 98%; 100%. | All Intra Main10. Over BMS-1.1 - NSST. Y; U; V; EncT; DecT. Class A1; 0.32%; 0.14%; 0.12%; 97%; 98%. Class A2; 0.27%; 0.02%; -0.11%; 96%; 99%. Class B; 0.29%; -0.08%; -0.04%; 96%; 99%. Class C; 0.26%; -0.09%; -0.05%; 94%; 99%. Class E; 0.28%; -0.02%; -0.18%; 96%; 99%. Overall; 0.29%; -0.02%; -0.05%; 96%; 99%. Class D; 0.21%; -0.11%; -0.11%; 94%; 99%. Random Access Main 10. Over BMS-1.1 - NSST. Y; U; V; EncT; DecT. Class A1; 0.13%; 0.28%; 0.23%; 98%; 99%. Class A2; 0.14%; 0.02%; -0.08%; 98%; 100%. Class B; 0.13%; 0.00%; 0.28%; 98%; 100%. Class C; 0.13%; 0.01%; 0.04%; 98%; 99%. Class E. Overall; 0.13%; 0.06%; 0.13%; 98%; 100%. Class D; 0.07%; -0.24%; -0.02%; 98%; 100%. Low delay B Main10. Over BMS-1.1 - NSST. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; 0.02%; -0.11%; 0.51%; 99%; 101%. Class C; 0.04%; -0.05%; -0.37%; 99%; 100%. Class E; 0.27%; -0.47%; -0.67%; 99%; 100%. Overall; 0.09%; -0.18%; -0.08%; 99%; 100%. Class D; 0.01%; -0.50%; -0.52%; 99%; 100%. |
| --- | --- |

In order to further reduce the required memory sizes for storing transform matrices, the following experiments are additionally conducted. The experimental results of CE6-1.4 test with up to 32x32 transform compared to VTM 1.1 anchor are summarized in the left side of Table IV. The average BD-rates of -3.23%, -1.94%, and -1.37% are observed for AI, RA, and LDB cases, respectively, compared to VTM 1.1 anchor. And, the experimental results of CE6-1.4 test with up to 32x32 transform compared to BMS 1.1 anchor are summarized in the right side of Table IV. The average BD-rates of 0.27%, 0.26%, and 0.15% are observed for AI, RA, and LDB cases, respectively, compared to BMS 1.1 anchor.

Table IV. Experimental results of CE6-1.4 with up to 32x32 transform over VTM 1.1 anchor (left side) and CE6-1.4 with up to 32x32 transform over BMS 1.1 anchor (right side)

| All Intra Main10. Over VTM-1.1. Y; U; V; EncT; DecT. Class A1; -3.04%; -1.09%; -1.04%; 285%; 126%. Class A2; -2.94%; -1.61%; -1.56%; 271%; 120%. Class B; -3.23%; -1.48%; -1.47%; 298%; 121%. Class C; -2.88%; -1.12%; -1.41%; 283%; 110%. Class E; -4.15%; -2.89%; -2.76%; 274%; 116%. Overall; -3.23%; -1.59%; -1.62%; 284%; 118%. Class D; -2.81%; -0.96%; -0.85%; 309%; 106%. Random Access Main 10. Over VTM-1.1. Y; U; V; EncT; DecT. Class A1; -1.51%; 0.04%; 0.15%; 199%; 108%. Class A2; -1.85%; 0.08%; 0.30%; 192%; 106%. Class B; -2.30%; -0.19%; -0.46%; 212%; 107%. Class C; -1.88%; -0.06%; -0.21%; 216%; 105%. Class E. Overall; -1.94%; -0.05%; -0.12%; 206%; 106%. Class D; -1.72%; 0.29%; -0.32%; 233%; 103%. Low delay B Main10. Over VTM-1.1. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; -1.68%; 0.65%; 0.86%; 218%; 109%. Class C; -1.23%; 0.76%; 1.04%; 201%; 107%. Class E; -1.02%; 1.18%; 1.81%; 196%; 105%. Overall; -1.37%; 0.82%; 1.16%; 207%; 107%. Class D; -1.20%; 1.06%; 0.87%; 202%; 103%. | All Intra Main10. Over BMS-1.1. Y; U; V; EncT; DecT. Class A1; 0.38%; 0.64%; 0.49%; 109%; 96%. Class A2; 0.33%; 0.23%; 0.03%; 110%; 98%. Class B; 0.28%; 0.18%; 0.29%; 100%; 98%. Class C; 0.17%; 0.04%; 0.05%; 92%; 99%. Class E; 0.23%; 0.07%; -0.06%; 93%; 97%. Overall; 0.27%; 0.22%; 0.17%; 100%; 98%. Class D; 0.16%; 0.02%; 0.07%; 92%; 99%. Random Access Main 10. Over BMS-1.1. Y; U; V; EncT; DecT. Class A1; 0.46%; 0.58%; 0.53%; 85%; 98%. Class A2; 0.24%; 0.36%; 0.12%; 93%; 100%. Class B; 0.22%; 0.51%; 0.30%; 99%; 99%. Class C; 0.17%; 0.31%; -0.09%; 88%; 100%. Class E. Overall; 0.26%; 0.44%; 0.21%; 92%; 99%. Class D; 0.08%; 0.01%; 0.34%; 91%; 100%. Low delay B Main10. Over BMS-1.1. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; 0.23%; 0.24%; 0.04%; 101%; 98%. Class C; 0.03%; 0.06%; -0.03%; 91%; 98%. Class E; 0.17%; -0.11%; 0.35%; 90%; 98%. Overall; 0.15%; 0.09%; 0.09%; 95%; 98%. Class D; 0.07%; 0.04%; -0.51%; 94%; 100%. |
| --- | --- |

Complexity analysis

Metrics regarding encoding complexity, memory requirements, and other operations

For intra prediction, there are four types of combinations for each intra prediction mode to calculate RD cost in AMT, but in CE6-1.4 method, there are up to four types of flipping candidates in the pre-defined residual flipping candidate set for each intra prediction mode, as tabulated in Table I. Therefore, it is claimed that the average encoding time could be reduced by using the CE6-1.4 method compared to AMT, as tabulated in Table III and Table IV.

While four additional transforms are needed for AMT as primary transforms, one additional transform and residual flipping are needed in CE6-1.4 method. The required memory sizes for storing transform matrices of primary transforms in HEVC, BMS 1.1, CE6-1.4 method with up to 64x64 transform, and CE6-1.4 method with up to 32x32 transform are tabulated in Table V and the required memory sizes for storing transform matrices for the additional core transforms, in addition to DCT-II, in BMS 1.1, CE6-1.4 method with up to 64x64 transform, CE6-1.4 method with up to 32x32 transform are tabulated in Table VI. Therefore, it is claimed that CE6-1.4 method with up to 64x64 transform saves about 60% of memory for transform matrices compared to AMT and 75% of memory for additional transform matrices in addition to DCT-II compared to AMT. And, it is claimed that CE6-1.4 method with up to 32x32 transform saves about 75% of memory for transform matrices compared to AMT and about 94% of memory for additional transform matrices in addition to DCT-II compared to AMT.

Table V. Required memory sizes for transform matrices in HEVC, AMT in BMS 1.1, CE6-1.4 with up to 64x64 transform, and CE6-1.4 with up to 32x32 transform

| Transform size | Matrix size | HEVC (DCT-II and 4x4 DST-VII) |  |  | AMT in BMS 1.1 (DCT-II, DCT-V, DCT-VIII, DST-I, and DST-VII) |  |  | CE6-1.4 with up to 64x64 transform (DCT-II and DST-VII) |  |  | CE6-1.4 with up to 32x32 transform (DCT-II and DST-VII) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Num. of kernels | Bits per elements | Memory size | Num. of kernels | Bits per elements | Memory size | Num. of kernels | Bits per elements | Memory size | Num. of kernels | Bits per elements | Memory size |
| 2 | 4 | 0 | 7 | 0 | 1 | 10 | 5 | 1 | 10 | 5 | 1 | 10 | 5 |
| 4 | 16 | 2 | 7 | 28 | 5 | 10 | 100 | 2 | 10 | 40 | 2 | 10 | 40 |
| 8 | 64 | 1 | 7 | 56 | 5 | 10 | 400 | 2 | 10 | 160 | 2 | 10 | 160 |
| 16 | 256 | 1 | 7 | 224 | 5 | 10 | 1,600 | 2 | 10 | 640 | 2 | 10 | 640 |
| 32 | 1024 | 1 | 7 | 896 | 5 | 10 | 6,400 | 2 | 10 | 2,560 | 2 | 10 | 2,560 |
| 64 | 4096 | 0 | 7 | 0 | 5 | 10 | 25,600 | 2 | 10 | 10,240 | 1 | 10 | 5,120 |
| Total (bytes) |  |  |  | 1204 |  |  | 34,105 |  |  | 13,645 |  |  | 8,525 |

Table VI. Additional memory sizes for transform matrices in AMT in BMS 1.1, CE6-1.4 with up to 64x64 transform, and CE6-1.4 with up to 32x32 transform

| Transform size | Matrix size | AMT in BMS 1.1 (DCT-V, DCT-VIII, DST-I, and DST-VII) |  |  | CE6-1.4 with up to 64x64 transform (DST-VII) |  |  | CE6-1.4 with up to 32x32 transform (DST-VII) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Num. of kernels | Bits per elements | Memory size | Num. of kernels | Bits per elements | Memory size | Num. of kernels | Bits per elements | Memory size |
| 2 | 4 | 0 | 10 | 0 | 0 | 10 | 0 | 0 | 10 | 0 |
| 4 | 16 | 4 | 10 | 80 | 1 | 10 | 20 | 1 | 10 | 20 |
| 8 | 64 | 4 | 10 | 320 | 1 | 10 | 80 | 1 | 10 | 80 |
| 16 | 256 | 4 | 10 | 1,280 | 1 | 10 | 320 | 1 | 10 | 320 |
| 32 | 1024 | 4 | 10 | 5,120 | 1 | 10 | 1,280 | 1 | 10 | 1,280 |
| 64 | 4096 | 4 | 10 | 20,480 | 1 | 10 | 5,120 | 0 | 10 | 0 |
| Total (bytes) |  |  |  | 27,280 |  |  | 6,820 |  |  | 1,700 |

In addition, it is claimed that the residual flipping and its inverse only require reordering the input and output of DST-VII, respectively, without any additional process. And, the reordering process nearly does not require computational complexity in both software and hardware codec implementations.

Complexity metrics

Regarding CE6-1.4 method, complexity metrics needed to be reported in the CE6 summary report are summarized as tabulated in Table VII.

Table VII. Complexity metrics for CE6-1.4

| Complexity metrics | CE6-1.4 |
| --- | --- |
| Operation counts (number of multiplications, additions, and bit-shift operations) for individual transforms | Same as DST-VII in BMS |
| Memory requirements for storing transform cores | Refer to Table V and Table VI in section 3.1 |
| Specify the minimum bit-precision required by the transforms | Same as BMS |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output | Same as BMS |
| Other operations and memory requirements relevant to the proposed tool | Refer to residual flipping in section 3.1 |
| Encoder and decoder times obtained on the same computing platform for both reference and proposed tools | Refer to Table II, Table III, and Table IV in section 2 |
| List of all combinations of transform types and transforms block sizes used for primary transformation. | Refer to Table V in section 3.1 |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms | Same as DST-VII in BMS |
| Additional computations at the encoder or decoder | No |

Conclusions

This contribution reports the experimental results of CE6-1.4 test. In CE6-1.4 test, DST-VII with residual flipping for up to 64x64 transform was tested in order to reduce the required memory sizes for storing transform matrices compared to AMT in BMS. And, DST-VII with residual flipping for up to 32x32 transform was also tested. The CE6-1.4 method with up to 64x64 transform reportedly saves about 60% of memory for transform matrices compared to AMT and 75% of memory for additional transform matrices in addition to DCT-II compared to AMT. The CE6-1.4 method with up to 32x32 transform reportedly saves about 75% of memory for transform matrices compared to AMT and about 94% of memory for additional transform matrices in addition to DCT-II compared to AMT. The experimental results with up to 64x64 transform reportedly show average BD-rates of -3.24%, -2.06%, and -1.49% with encoding/decoding times of 198%/120%, 163%/109%, and 163%/111% for AI, RA, and LDB cases, respectively, compared to VTM 1.1 anchor. And, the experimental results with up to 64x64 transform reportedly show average BD-rates of 0.25%, 0.13%, and 0.01% with encoding/decoding times of 94%/99%, 96%/100%, and 98%/100% for AI, RA, and LDB cases, respectively, compared to BMS 1.1 anchor. And, the experimental results with up to 32x32 transform reportedly show average BD-rates of -3.23%, -1.94%, and -1.37% for AI, RA, and LDB cases, respectively, compared to VTM 1.1 anchor. And, the experimental results with up to 32x32 transform reportedly show average BD-rates of 0.27%, 0.26%, and 0.15% for AI, RA, and LDB cases, respectively, compared to BMS 1.1 anchor. Therefore, it is recommended to adopt this test to reduce the required memory size for transform matrices of AMT in BMS.

References

- A. Said and X. Zhao, “Description of Core Experiment 6: Transforms and transform signalling,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-J1026, Apr. 2018

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.1

- J. Kang, H. Lee, S.-C. Lim, J. Lee, H. Y. Kim, N.-U. Kim, and Y.-L. Lee, “Description of SDR video coding technology proposal by ETRI and Sejong University,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-J0013, Apr. 2018

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE6/CE6-1.4

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-J1010, Apr. 2018

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sejong University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Chips&Media does not have any current or pending patent rights relating to the technology described in this contribution.