| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0168-v1 |
| --- | --- |

| Title: | CE9: Template Matched Merge (Test 9.3.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hahyun Lee Jungwon Kang Sung-Chang Lim Jinho Lee Hui Yong Kim 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-6138 hanilee@etri.re.kr jungwon@etri.re.kr sclim@etri.re.kr jinosoul@etri.re.kr hykim5@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

- Abstract

This contribution reports the experimental results of CE9.3.1 Template Matched Merge. This performs CU-level motion vector refinement using template matching around an initial motion vector derived from the signalled index. The experimental results show an average BD-rate gains are 4.25% and 1.23% while the average encoding times are increased by 157% and 175% and the average decoding times are increased by 72% and 41% for RA and LDB configuration, respectively, compared to VTM-1.0. And the experimental results show an average BD-rate gains are 2.22% and 1.10% while the average encoding times are increased by 48% and 63% and the average decoding times are increased by 13% and 17% for RA and LDB configuration, respectively, compared to BMS-1.0.

Algorithm description

At the previous JVET Meeting, Template Matched Merge (TMM), the decoder side motion vector derivation tool, was proposed in JVET-J0013 [1] and included in CE9.3 sub-test [2]. It is aimed to reduce the decoding complexity while minimizing coding efficiency degradation compared to template matching mode of pattern matched motion vector derivation (PMMVD) in JEM7 [3].

TMM is a special merge mode with a CU-level motion vector refinement based on template matching at the decoder side. In this mode, initial motion vector is derived from a signalled index, then CU-level motion vector refinement around the initial motion vector is performed using template matching. When a block is coded as SKIP/MERGE, a flag (i.e., TMM flag) is signalled to indicate whether TMM mode is or not. When a flag is false, the general SKIP/MERGE process is used. When the flag is true, a candidate index (i.e., TMMIndex) is signalled to indicate which candidate is to be used to derive initial motion information for the CU.

Motion derivation process in TMM has two steps: 1) initial motion vector derivation process based on a signalled index and 2) CU-level motion vector refinement process based on the template matching.

Initial motion vector derivation process

When a block is coded as TMM mode, TMM candidate list is first constructed by inserting the motion vectors and the reference index of the spatial neighboring and temporal neighboring blocks into the candidate. A maximum number of TMM candidates (i.e., MaxNumTMMMergeCand) can be up to seven. In the derivation of TMM spatial merge candidates, a maximum of seven candidates can be selected among TMM spatial merge candidates located in positions as shown in Figure 1 (a). The order of derivation is A  B  C  D  E  F  G. The remaining derivation process for TMM spatial merge candidates is same as in HEVC. If the number of derived TMM spatial merge candidates is less than MaxNumTMMMergeCand, at most one TMM temporal merge candidate is selected from two candidates (Figure 1 (b)) that are derived as same as in HEVC. If the number of derived TMM spatial and temporal merge candidates does not reach to MaxNumTMMMergeCand, the combined bi-predictive merge candidates and zero motion vector candidates are added at the end of the list as in HEVC.

At the encoder side, the CU-level motion refinement process with the template matching is performed around MV derived from each TMM candidate in the TMM candidate list. TMM candidates are checked for a CU by using RD cost selection. The one leading to minimum RD cost is selected as the best TMM candidate and signals the index indicating the best TMM candidate. At the decoder side, the motion vector from the TMM candidates indicated by the signalled index and is set to an initial motion vector

(a) Positions of spatial merge candidate    (b) Positions of temporal merge candidate

Figure 1: Positions of TMM candidates

CU-level motion vector refinement process

In the CU-level motion vector refinement process, a local search based on the template matching around the initial motion vector is performed and the motion vector with the minimum matching cost (SAD) is taken as the final motion vector for the whole CU. The template matching method applied in TMM is the same template matching scheme of PMMVD described in JEM7 [3].

- Maximum iteration of SAD calculation = Not defined (an unrestricted center-biased diamond  search and adaptive cross search )

- Maximum Search Range = 8 luma samples

- Bi-linear interpolation for template matching

- No sub-CU level refinement

Experimental Results

The CE9.3.1 [4] is implemented on BMS-1.0 software and tested under JVET common test condition for both VTM and BMS tests [5]. To check mismatch between encoder and decoder, MD5 decoded picture hash SEI message is enabled. The encoding is executed on a homogeneous computer clusters with Intel Xeon E5-1620 v2 (3.7GHz, AVX) and gcc4.7.2. The decoding is executed on a single machine with Intel i7 X990 (3.47GHz, SSE42) and gcc4.7.2.

Table 1 shows the experimental results of CE9.3.1 compared to VTM1.0 and BTM1.0. The average luma BD-rate gain of 4.25% and 1.23% for RA and LDB cases are observed compared to VTM1.0 and 2.22% and 1.10% for RA and LDB cases are observed compared to BMS1.0.

Table 1: Experimental results of the CE9.3.1 over VTM-1.0 and BMS-1.0

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -5.09% | -4.60% | -5.40% | 232% | 168% | -2.03% | -1.92% | -2.36% | 139% | 111% |
| Class A2 | -6.26% | -6.21% | -6.43% | 250% | 188% | -2.72% | -2.45% | -2.64% | 150% | 112% |
| Class B | -3.88% | -3.73% | -3.65% | 258% | 165% | -2.20% | -1.82% | -1.88% | 148% | 111% |
| Class C | -2.58% | -2.98% | -3.22% | 281% | 173% | -2.01% | -1.85% | -2.16% | 153% | 120% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -4.25% | -4.20% | -4.44% | 257% | 172% | -2.22% | -1.97% | -2.20% | 148% | 113% |
| Class D | -2.33% | -2.44% | -2.96% | 301% | 173% | -1.83% | -1.43% | -1.81% | 157% | 120% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.47% | -1.40% | -1.32% | 278% | 137% | -1.18% | -1.03% | -0.53% | 161% | 113% |
| Class C | -1.40% | -1.52% | -1.87% | 285% | 154% | -1.35% | -1.49% | -1.76% | 168% | 123% |
| Class E | -0.61% | -0.90% | -0.27% | 258% | 132% | -0.63% | -0.29% | -0.44% | 161% | 114% |
| Overall | -1.23% | -1.31% | -1.24% | 275% | 141% | -1.10% | -1.00% | -0.92% | 163% | 117% |
| Class D | -1.23% | -1.28% | -1.75% | 282% | 152% | -1.02% | -0.60% | -0.71% | 173% | 124% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Conclusions

This contribution reports the experimental results of CE9.3.1 Template Matched Merge. This performs CU-level motion vector refinement using template matching around an initial motion vector derived from the signalled index. The experimental results show an average BD-rate gains are 4.25% and 1.23% while the average encoding times are increased by 157% and 175% and the average decoding times are increased by 72% and 41% for RA and LDB configuration, respectively, compared to VTM-1.0. And the experimental results show an average BD-rate gains are 2.22% and 1.10% while the average encoding times are increased by 48% and 63% and the average decoding times are increased by 13% and 17% for RA and LDB configuration, respectively, compared to BMS-1.0.

References

- J. Kang, H. Lee, S.-C. Lim, J. Lee, H. Y. Kim, N.-U. Kim, and Y.-L. Lee, “Description of SDR video coding technology proposal by ETRI and Sejong University,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-J0013, Apr. 2018

- S. Esenlik, Y. W. Chen, “Description of Core Experiment 9 (CE9): Decoder Side Motion Vector Derivation”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-J1029, Apr. 2018.

- J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce, “Algorithm Description of Joint Exploration Test Model 7 (JEM7)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-G1001, Jul. 2017.

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE9/CE9-3.1

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-J1010, Apr. 2018

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).