| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0169 |
| --- | --- |

| Title: | CE3-related: Block Shape Adaptive Intra Prediction Directions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gagan Rath Fabrice Urban Fabien Racapé | Email: | gagan.rath@technicolor.com fabrice.urban@technicolor.com fabien.racape@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution presents a block shape-based adaptation of the set of intra prediction directions for the QTBT structure. If the shape of the block is square, then the already defined directions in the JVET VTM/BMS remain unchanged. Otherwise, the searched directions are modified according to the shape of the block. The total number of angular directions for any block remains the same as in VTM/BMS. Simulation results reportedly show an average luma BD-rate variation of -0.24%, in All Intra (AI) configuration, compared with VTM.

# Introduction

VTM/BMS specifies 65 angular intra prediction modes in addition to the planar and DC modes. These prediction directions are defined from 45 degrees to -135 degrees in clockwise direction. For a target block of size W×H pixels, the top reference samples array and the left reference samples array are each of size (W+H+1), which is required to cover the afore-mentioned angle range for all target pixels. This definition of the set of angles in VTM/BMS was done for compatibility with HEVC specified directions, without considering the QTBT structure.

In the case of a square block, there is a structural symmetry. A square block is not only symmetric along both height and width but also is symmetrically placed with respect to the two reference arrays, as seen in Figure 1.

Figure 1: A square target block with its top and left reference arrays. The target block is symmetrically placed w.r.t the two reference arrays. The prediction directions are defined from 45 deg to -135 deg in clockwise direction where mode 2 corresponds to 45 deg angle and mode 66 corresponds to 135 deg angle.

In the case of a rectangular block, there is no structural symmetry with the current defined directions. When the block is flat (i.e, W > H), it is placed closer to the top reference array, but asymmetrically. Similarly, when the block is tall (i.e., W < H), it is placed closer to the left reference array, but asymmetrically. This asymmetry is shown in Figure 2.

Figure 2: Rectangular target blocks (flat block on the left, tall block on the right) with their top and left reference arrays. The target blocks are asymmetrically placed w.r.t. the two reference arrays.

The result of this asymmetry is that for some prediction modes, the target pixel will be predicted from a farther reference array, whereas the nearer reference array will be excluded because of the defined angles. This is shown in Figure 3.

Figure 3: Rectangular target blocks (flat block on the left, tall block on the right) with their top and left reference arrays.  On the left, the target pixel A has the predictor sample L on the left reference array with a horizontal prediction direction; Though the sample T on the top reference array is nearer, the vertical prediction direction, so that T could be the predictor sample for A, is not allowed in JEM. The right figure shows the analogous case for a target pixel in a tall block.

The flatter or taller a block is (that is, the higher the ratio of the longer side to the shorter side), this asymmetry is more pronounced. Because of the inaccuracy in prediction, such prediction modes become less likely, as seen in actual test results.

Another argument for inefficiency in current defined directions is the equality in number of prediction directions, horizontally and vertically. When a block is rectangular with one side longer than the other, the number of prediction directions in the longer side should be higher simply because of the potential possibility of higher number of structural directionalities in that side. A related point to consider is the length of the reference arrays.  For rectangular blocks, if the width is greater than height, the top reference array should have more samples than the left one. Similarly, for blocks with height greater than width, the left reference array should have more samples than the top one.

# Description and encoder options

For any target block of width W and height H, we will associate mode 2 to the direction from vertex D towards vertex B, mode 66 to the direction from vertex B towards vertex D, along the secondary diagonal, as shown in Figure 4. The diagonal mode is associated with the direction from vertex A towards vertex C along the principal diagonal. All modes from mode 2 up to the diagonal mode will be termed Left modes. Similarly, all modes from the diagonal mode to mode 66, including the diagonal mode, will be termed Top modes. Left modes are associated with horizontal directions. Similarly, Top modes are associated with vertical directions.

Figure 4: Definition of prediction directions for a rectangular block. Mode 2 is along the secondary diagonal from vertex D towards vertex B. Mode 66 is along the opposite direction to mode 2. Diagonal mode is along the principal diagonal from vertex A towards vertex C.

To support these prediction directions, it is proposed to define the top reference with length 2W+1, and the left reference with length 2H+1, as shown in Figure 5.

Figure 5: Top reference array has length 2W+1, left reference array has length 2H+1.In the case of a square block, the directions and the lengths of the reference arrays are identical to those in JEM.

Here, the modes 2 and 66 are not defined to correspond to directions along 45 degrees angle and -135 degrees angle, rather the directions along the secondary diagonal. In the case of a flat block, some directions near, and including, 45 degrees are removed, but an equal number of directions are added exactly in the opposite directions beyond -135-degree angle. Similarly, in the case of a tall block, some directions near, and including, -135-degree are removed, but an equal number of directions are added exactly in the opposite directions beyond 45-degree angle.

Since the newly introduced directions are exactly opposite to the removed directions, their angle parameters are obtained from those of the removed directions using the relationship:

An= round(1≪10A)

The number of directions removed/added for different block shapes is specified by a parameter called modeShift. Indexing the original directions from 0 to 64, the new directions can be indexed beyond 64 or 0. The following table shows the directions (dirIndex) and the modeShift values for different block shapes specified as the ratio of width to height (W/H). Note that, in the current version of JEM with QTBT structure, the maximum value of W/H is 8.

| W/H | dirIndex | modeShift |
| --- | --- | --- |
| 1 | 0 - 64 | 0 |
| 2 | 6 - 70 | 6 |
| 4 | 10 - 74 | 10 |
| 8 | 12 - 76 | 12 |
| 1/2 | -6 : 58 | -6 |
| 1/4 | -10: 54 | -10 |
| 1/8 | -12:52 | -12 |

This new definition of modes requires to adapt the MPM-based prediction mode coding. Since the neighbor blocks of a target block can have different shapes from the target block, their prediction modes need to be mapped to the prediction modes of the target block. A simple solution is to use the original mode indexing with respective directions. In this case, the mode indices corresponding to the removed directions for a target block are mapped to the closest opposite direction which are newly included. For flat blocks, the mode m is mapped to the direction (m + (VDIA_IDX -1) - 2) whereas for tall blocks the mode m is mapped to the direction (m – (VDIA_IDX - 1) - 2). If the direction of prediction associated with a mode is still valid for a target block, there is no mapping required. The above mapping is done only for generating the prediction for a target block.

For example, for a flat block, mode 2 performs prediction along direction indexed as 65. Similarly, for a tall block, mode 66 performs prediction along direction indexed as -1. In this way, an MPM mode will correspond to the original direction, or to a direction close to the opposite direction.

This contribution has some conceptual similarity with the wide angle intra prediction method presented in JVET-J0012. The latter method preserves the original directions while also considering the opposite directions for 10 modes closest to the diagonal modes depending on the shape of the rectangular block. For those modes, to distinguish between an original prediction direction and its opposite direction, a 1-bit flag is encoded. Clearly, this method requires more complexity at the encoder side to check the RD performance with the new directions with wide angles for the rectangular blocks.  Furthermore, parsing of the bits at the decoder becomes mode-dependent for rectangular blocks. The reported method in this contribution does not have such problems since there are no wide-angle directions as the prediction directions always span 180 degrees, depending on the block shape.  The complexity for encoder intra mode search remains the same as that of VTM/BMS and no extra signaling flag is encoded.

# Results

The adaptation of intra directions is implemented within the BMS-1.0 software (JEM_TOOLS macros deactivated to emulate VTM). Table 1 reports the BD-rate performance compared to VTM in AI configuration, as defined in the Common Test Conditions.

Table 1: BD-rate gains over TM in AI configuration

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.12% | -0.21% | 100% | 100% |
| Class A2 | -0.17% | -0.09% | -0.16% | 100% | 100% |
| Class B | -0.19% | -0.27% | -0.20% | 100% | 101% |
| Class C | -0.40% | -0.59% | -0.72% | 101% | 101% |
| Class E | -0.25% | -0.17% | -0.18% | 100% | 100% |
| Overall | -0.24% | -0.27% | -0.31% | 100% | 100% |
| Class D | -0.30% | -0.39% | -0.41% | 101% | 101% |

# Conclusion

The proposed method reportedly shows an average luma BD-rate gain of -0.24% over TM in AI configuration, with no added complexity.  It is then recommended to study this tool in CE3.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).