| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET -K0170-v1 |
| --- | --- |

| Title: | CE5: Counter-based probability estimation (Test 2.4) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiho Choi, Yinji Piao, Chanyul Kim Seongchon-gil 34, Seocho-gu, Seoul, 06765 Korea | Tel: Email: | kiho14.choi@samsung.com yj1003.piao@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

_____________________________

- Abstract

This contribution presents a technology described in CE 5.2.4 that is a counter-based probability estimation scheme using two-probability update model. Testing results on the proposed method show 0.8%, 0.6%, 0.4%, and 0.3% BD rate savings on average for All Intra (AI), Random Access (RA), Low Delayed B (LDB) and Low Delayed P (LDP) configurations, respectively, compared to VTM1.0, and . 0.9%, 0.6%, 0.5%, and 0.5% BD rate savings on average for AI, RA, LDB and LDP configurations, respectively, compared to BMS1.0.

Proposed method

Similar to JEM, two-probability update model is employed in J0024 [1]. The probability is updated with linear combination for probability estimation that consists of only two summands corresponding N0=16 (short window) and N1=128 (long window):

P0 = (Y >> 4) + P0 – (P0 >> 4)

P1 = (Y >> 7) + P1 – ( P1>> 7)

P = (P0 + P1 + 1) >> 1

where Y = 2^15.

To start probability estimation it is necessary to initialize probability. Different from JEM, only short window size model is updated until the counter reaches 31. When the counter reaches 31, long window model starts updating with initial value borrowed from short window model as shown in Figure 1. This approach significantly reduces negative effect from wrong initialization and also reduces the complexity due to less update of long window size model.

Figure 1 Example of probability using different initialized value

The probability estimation process is as follows,

- bFastProbUpdate = counter < 31

- counter++

- MPS pass:

  - P0mps = P0 – (P0 >> 4)

  - P1mps = bFastProbUpdate ? P0mps: P1 – (P1 >> 7)

- LPS pass

  - P0lps = P0 + ((2^15 – P0) >> 4)

  - P1lps = bFastProbUpdate ? P0lps : P1 + ((2^15 – P1) >> 7)

  - If(P0 + P1 > 2^15)

    - mps = 1 – mps

    - P1lps = 2^15 – P1lps

    - P0lps = 2^15 – P0lps

- P= (P0 + P1) >> 1

Test result

Test conditions follows the description in CE5 document [2]. BMS SW was used for these experiments, as specified in JVET-J1010 [3]. For testing, mCABAC = 1 is set to VTM and BMS with a counter-based probability estimation scheme using two-probability update model.

VTM test result

For testing proposed method on VTM, VTM configuration files were used with BMS 1.0 software, and results are shown in Table 1. The average BD-rate gains for AI, RA, LDB, and LDP, are 0.8%, 0.6%, 0.4%, and 0.3%, respectively.

BMS test result

For testing proposed method on BMS, BMS configuration files were used with BMS 1.0 software, and results are shown in Table 1. The average BD-rate gains for AI, RA, LDB, and LDP, are 0.9%, 0.6%, 0.5%, and 0.5%, respectively.

Table 1: Test results on CE 5.2.4

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.88% | -0.88% | -1.22% | 104% | 101% | -0.92% | -1.67% | -1.81% | 105% | 106% |
| Class A2 | -1.00% | -0.79% | -0.89% | 104% | 102% | -1.05% | -1.01% | -1.24% | 104% | 102% |
| Class B | -0.79% | -0.88% | -0.75% | 105% | 102% | -0.89% | -0.75% | -0.95% | 104% | 101% |
| Class C | -0.73% | -0.88% | -0.74% | 104% | 101% | -0.82% | -0.76% | -0.84% | 104% | 103% |
| Class E | -0.67% | -1.00% | -0.71% | 104% | 101% | -0.79% | -0.95% | -0.76% | 104% | 103% |
| Overall | -0.81% | -0.89% | -0.84% | 104% | 101% | -0.89% | -0.98% | -1.08% | 104% | 103% |
| Class D | -0.71% | -0.73% | -0.60% | 105% | 104% | -0.80% | -0.29% | -0.53% | 105% | 103% |
| Class F (optional) | -0.52% | -0.37% | -0.23% | 105% | 105% | -0.50% | -0.31% | -0.72% | 104% | 102% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.66% | -0.98% | -1.33% | 102% | 103% | -0.70% | -1.57% | -1.80% | 101% | 101% |
| Class A2 | -0.55% | -0.61% | -0.67% | 102% | 101% | -0.68% | -0.62% | -0.89% | 101% | 101% |
| Class B | -0.55% | -0.67% | -0.57% | 102% | 102% | -0.63% | -0.60% | -0.81% | 101% | 101% |
| Class C | -0.51% | -0.76% | -0.60% | 102% | 103% | -0.58% | -0.89% | -1.11% | 101% | 101% |
| Overall | -0.56% | -0.74% | -0.75% | 102% | 102% | -0.64% | -0.88% | -1.10% | 101% | 101% |
| Class D | -0.48% | -0.40% | -0.22% | 104% | 104% | -0.56% | -0.75% | -0.67% | 102% | 102% |
| Class F (optional) | -0.35% | -0.36% | -0.05% | 103% | 104% | -0.38% | -0.15% | -0.41% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class B | -0.51% | -0.31% | 0.05% | 103% | 102% | -0.54% | -0.51% | 0.34% | 100% | 101% |
| Class C | -0.47% | -0.89% | -0.30% | 103% | 101% | -0.56% | -0.82% | -0.93% | 101% | 101% |
| Class E | -0.09% | -0.15% | 0.91% | 101% | 102% | -0.52% | -0.59% | -1.16% | 99% | 100% |
| Overall | -0.39% | -0.46% | 0.15% | 102% | 102% | -0.54% | -0.64% | -0.46% | 100% | 101% |
| Class D | -0.55% | 0.73% | 0.31% | 103% | 101% | -0.50% | 0.93% | -0.25% | 102% | 103% |
| Class F (optional) | -0.57% | -1.03% | -0.26% | 103% | 103% | -0.39% | -0.11% | 0.10% | 100% | 103% |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class B | -0.46% | 0.19% | 0.71% | 114% | 114% | -0.53% | 0.07% | -0.05% | 101% | 101% |
| Class C | -0.43% | -0.84% | -0.81% | 114% | 120% | -0.52% | -0.85% | -0.84% | 102% | 105% |
| Class E | -0.02% | 0.08% | 0.78% | 111% | 119% | -0.39% | 0.40% | -1.07% | 94% | 102% |
| Overall | -0.34% | -0.18% | 0.22% | 113% | 117% | -0.49% | -0.15% | -0.57% | 100% | 103% |
| Class D | -0.49% | 0.99% | 0.76% | 114% | 129% | -0.53% | 0.76% | 0.46% | 102% | 105% |
| Class F (optional) | -0.51% | -0.59% | -0.18% | 112% | 120% | -0.40% | -0.19% | -0.02% | 102% | 105% |

Conclusion

In this contribution, a technology described in CE 5.2.4 that is a counter-based probability estimation scheme using two-probability update model is proposed. Testing results show good coding performance in all scenarios. It is recommend to adopt the proposed method into the next version of test model of VVC.

Reference

[1] A. Alshin et al, “Description of SDR, HDR and 360° video coding technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario”, JVET-J0024, San Diego, US, 10–20 Apr. 2018

[2] T. Nguyen, A. Said, “Description of Core Experiment 5 (CE5): Arithmetic Coding Engine”, JVET-1025, San Diego, US, 10–20 Apr. 2018

[3] J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, JVET-1010, San Diego, US, 10–20 Apr. 2018

Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard.