| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0172 |
| --- | --- |

| Title: | CE3-related: Gradient-Based Boundary Filtering in Intra Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gagan Rath Fabrice Urban Fabien Racapé | Email: | gagan.rath@technicolor.com fabrice.urban@technicolor.com fabien.racape@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution presents new boundary filtering methods for some intra prediction modes that include the positive angular directions, the vertical and the horizontal modes, and the DC and the planar modes.  For these modes, the gradient at the reference samples on the immediate left or top of the target block is computed. A fractional part of this gradient is added to the prediction values near the boundary of the target block in order to produce a gradual change in intensity values. Simulation results reportedly show an average luma BD-rate variations of -0.65%, in All Intra (AI) configuration, compared with TM. The method can be combined with Multiple Reference Intra Prediction (MRIP), which was presented in JVET-J0022 and studied in CE3.5. Coupled with MRIP, BD-rate variations reportedly reach -0.87% on luma in AI configuration.

# Introduction

In Intra prediction, some prediction modes may include a post-filtering step which attempts to create smooth intensity changes at the block boundaries between the current block and causal neighbors. In this context, HEVC specifies post-processing for the DC, vertical and horizontal modes. In JEM and BMS, this post-processing has been extended further to some other angular prediction modes. In this contribution, we aim to improve this post-processing step using a gradient based approach for directions that have the possibility of creating discontinuities at the target block boundary.

# Gradient-based Boundary Filtering

In the following, 67 prediction modes are considered, as in JEM/BMS. Also, for generality, a rectangular target block having width W and height H is considered.

## Boundary Filtering with Angular Modes

The prediction modes in the range 50 – 66 correspond to directions from -90 degrees to -135 degrees in clockwise direction. For these modes, the reference samples on the left of the target block, i.e., R(-1,y), 0 <=y < H, can be predicted just like any target block pixel.

The gradient at -1,y, 0 <=y<H,  can be computed as

∆(y) = R(-1,y) – Ry.

where Ry denotes the predictor for the reference sample R(-1,y) (illustrated in Figure 1 for mode 66). The predictor sample value is computed in the same manner as for target pixels in JEM/BMS.

Figure 1: Predictor for a reference pixel on left reference in mode 66.

The predicted sample values on row y are modified as

P’(x,y) = Clip(P(x,y) + (∆(y)* L(x) + 32) >> 6), 0<= x < W)

where L(x) = (32 >> ((x << 1) >> s)), and  s = (log2(W) -1 + log2(H) -1) >> 2.

For mode 50, which corresponds to the vertical direction, the predictor for all reference samples R(-1,y), 0<=y < H, is the top-left reference sample, which is R(-1,-1). In this special case, the gradient is calculated as:

∆(y) = R(-1,y) – R(-1,-1), 0 <= y < H.

For modes 51 and 49, which are nearly vertical, the predictors for all reference samples R(-1,y), 0<=y < H, are close to the top-left reference sample R(-1,-1), if the height of the target block is small. In this case, the gradient can be calculated in the same manner.

In an analogous manner, for modes in the range [2; -18], the predicted samples on column x are modified as

P’(x,y) = Clip(P(x,y) + (∆(x)* L(y) + 32) >> 6), 0<= y < H

where L(y) = (32 >> ((y << 1) >> s)), and s = (log2(W) -1 + log2(H) -1) >> 2. The gradient at the reference sample R(x,-1) is computed as ∆(x) = R(x,-1) – Rx , which is illustrated in Figure 2 for mode 2.

Figure 2: Predictor for a reference pixel on top reference in mode 2.

For mode 18, which is the horizontal direction, the predictor for all reference samples R(x,-1), 0 <=x < W, is the top-left reference sample R(-1,-1). In this special case, the gradient is computed as:

∆(x) = R(x,-1) – R(-1,-1), 0 <= x < W.

For modes 19 and 17, which are nearly horizontal, the predictors for all reference samples R(x,-1), 0<=x < W, are close to the top-left reference sample R(-1,-1), if the width of the target block is small. In these cases, the gradient can be computed in the same manner.

Since Chroma predictions are rather smooth, the above post-filtering is applied to only luma blocks.

## Boundary filtering with PLANAR mode

In the PLANAR mode, the prediction at a target pixel is obtained from two interpolations as:

Px,y=W*Pvx,y+ H*Phx,y+H*W2*H*W, 0≤x<W, 0≤y<H,

where Pvx,y and Phx,y are the vertical and horizontal interpolations obtained as

Pvx,y=H-1-y*Rx,-1+1+y*R-1,H, 0≤x<W,0≤y<H;

Phx,y=W-1-x*R-1,y+1+x*RW,-1, 0≤x<W,0≤y<H.

The vertical interpolation can cause discontinuity at the left edge of the block. Similarly, the horizontal interpolation can cause discontinuity at the top edge of the block. The gradient at the top reference can be estimated as

∆vx=Rx,-1- (Phx,-1≫log2(W)),  0≤x<W

where

Phx,-1=W-1-x*R-1,-1+1+x*RW,-1, 0≤x<W

The horizontal interpolation can be filtered as

P'hx,y= Phx,y + (W*∆vx*Ly+32)≫6,  0≤x<W; 0≤y<H

where Ly=(32≫y≪1≫s), where s=log2W-1+log2H-1≫2. Similarly, the vertical interpolation can be filtered as

P'vx,y= Pvx,y + (H*∆hy*Lx+32)≫6,  0≤x<W; 0≤y<H

where, Lx=(32≫x≪1≫s), and s =log2W-1+log2H-1≫2. The gradient at the left reference is estimated as

∆hy=R-1,y-(Pv-1,y≫log2(H)), 0≤y<H

Equivalently, the final prediction is filtered by taking the weighted average of (2) and (3):

P'x,y=Clip(Px,y + ((∆vx*Ly+ ∆hy*Lx+64)≫7).

## Boundary filtering in DC mode

The computation of the DC value is split in two steps. A horizontal DC value is computed taking only the reference samples on the immediate left. Similarly, a vertical DC value is computed taking only the reference samples on the immediate top. The final DC value is computed taking the average of the horizontal and vertical DC values.

DCh= y=0H-1R-1,yH ,

DCv= x=0W-1Rx,-1W .

Px,y=dcVal=DCh+ DCv2 for 0≤x<W;0≤y<H.

The vertical gradient, horizontal gradient, and the gradient at the top left sample are computed as:

∆vx=Rx,-1- dcVal,  0≤x<W

∆hy=R-1,y- dcVal,  0≤y<H

∆d=(R-1,0+R0,-1≫1)-dcVal.

The predicted pixel values are then filtered as:

P'x,y=Clip( dcVal + ((∆vx*Ly+ ∆hy*Lx- ∆d*L + 32)≫6))

where Lx and Ly are as given in the previous section and  L=min⁡(Lx, Ly).  As in JEM, the DC filtering is applied only to luma blocks having size smaller than or equal to 16x16.

# Gradient-based boundary filtering with Multiple Reference Intra Prediction (MRIP)

In this section, an approach coupling Multiple reference intra prediction (MRIP) and the Boundary Filtering approach described above is described.

## Multiple reference intra prediction

Multiple reference intra prediction (MRIP) is investigated in CE3.5. It aims at constructing a better prediction based on two reference layers than the usual prediction based on a single reference layer, where a reference layer consists of a left reference array and a top reference array. In JVET-J0022, two reference layers are used for intra prediction. For angular prediction modes, except the vertical and the horizontal modes, a prediction for a target block is constructed using each reference layer. The final prediction is formed as a weighted average of the predictions made from the two reference layers.  For PLANAR, DC, or LMChroma for a Chroma target block, the usual prediction based on only one reference layer is used.

## Boundary Filtering with Angular Modes

The prediction modes in the range 50 – 66 correspond to directions from -90 degrees to -135 degrees in clockwise direction. For these modes, the reference samples on the left of the target block, i.e., R(-1,y), 0≤y<H, can be predicted just like any target block pixel. The gradient at (-1,y), 0≤y<H,  can be computed as

∆y=R-1,y-Ry,

where Ry denotes the predictor for the reference sample R-1,y. Compared to uni-layer case, the predictor is computed as:

Ry=3*Rs1,-1+Rs2,-2+2≫2,

where Rs1,-1 and Rs2,-2 are the predictors for R-1,y using the 1st and the 2nd reference layers respectively.  (illustrated in Figure 3 for mode 66). The predictors Rs1,-1 and Rs2,-2 are computed in the same manner as for target pixels in BMS 1.0.

Figure 3: Predictor for a reference pixel on left reference in mode 66.

The predicted sample values on row y are modified as

P'x,y=ClipPx,y+∆ y*Lx+32≫6,  0<= x < W

where Lx=(32≫x≪1≫s), and  s=log2W-1+log2H-1≫2.

The predicted samples Px,y are the weighted average of the predictions made using reference layer 1 and reference layer 2. The weights used are 3 and 1.

For modes 50, 49, and 51, which correspond to the vertical direction and the two adjacent directions, the gradient is simply calculated as:

∆ y=R-1,y-R-1,-1, 0≤y<H.

In an analogous manner, for modes in the range 2 -18, the predicted samples on column x are modified as

P'x,y=ClipPx,y+∆ x*Ly+32≫6,  0<= y < H

where  Ly=(32≫y≪1≫s), and  s=log2W-1+log2H-1≫2.. The gradient at the reference sample R(x,-1) is computed as

∆x=Rx,-1-Rx,

where the predictor Rx is computed as:

Rx=3*R-1,t1+R-2,t2+2≫2.

R-1,t1 and R-2,t2 are the predictors for Rx,-1 using the 1st and the 2nd reference layers respectively.  (illustrated in Figure 4 for mode 2). The predictors R-1,t1 and R-2,t2 are computed in the same manner as for target pixels in BMS 1.0.

Figure 4: Predictor for a reference pixel on top reference in mode 2.

For modes 17,18, and 19, which correspond to the horizontal direction and the two adjacent directions, the gradient is simply computed as:

∆ x=Rx,-1-R-1,-1, 0≤x<W.

Since Chroma predictions are rather smooth, the above post-filtering is applied to only Luma blocks.

# Results

The proposed boundary filtering tool is implemented within the BMS-1.0 software (JEM_TOOLS macros deactivated to emulate VTM). The following tables report BD-rate performance compared to VTM in AI configuration, as defined in the Common Test Conditions.

## Boundary filtering for Angular Modes only

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.45% | -0.85% | -0.69% | 102% | 99% |
| Class A2 | -0.66% | -1.26% | -1.14% | 104% | 101% |
| Class B | -0.52% | -0.65% | -0.69% | 104% | 103% |
| Class C | -0.31% | -0.54% | -0.69% | 107% | 105% |
| Class E | -0.94% | -1.21% | -1.34% | 106% | 101% |
| Overall | -0.55% | -0.86% | -0.87% | 105% | 102% |
| Class D | -0.20% | -0.30% | -0.50% | 105% | 100% |

## Boundary filtering for Planar and DC modes only

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.05% | 0.10% | 101% | 102% |
| Class A2 | -0.26% | -0.49% | -0.43% | 98% | 100% |
| Class B | -0.37% | -0.33% | -0.12% | 98% | 100% |
| Class C | -0.25% | -0.20% | -0.27% | 97% | 99% |
| Class E | -0.61% | -0.41% | -0.31% | 97% | 101% |
| Overall | -0.32% | -0.30% | -0.20% | 98% | 100% |
| Class D | -0.22% | -0.05% | -0.12% | 99% | 99% |

## Boundary filtering for Planar, DC, and angular modes

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.45% | -0.79% | -0.59% | 107% | 105% |
| Class A2 | -0.75% | -1.50% | -1.29% | 106% | 101% |
| Class B | -0.68% | -0.76% | -0.74% | 104% | 102% |
| Class C | -0.31% | -0.38% | -0.48% | 102% | 100% |
| Class E | -1.19% | -1.24% | -1.23% | 101% | 101% |
| Overall | -0.65% | -0.88% | -0.83% | 104% | 102% |
| Class D | -0.16% | -0.25% | -0.26% | 102% | 99% |

## MRIP with Boundary filtering for Angular Modes only

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.84% | -1.36% | -1.34% | 136% | 111% |
| Class A2 | -0.90% | -1.59% | -1.36% | 134% | 111% |
| Class B | -0.87% | -1.16% | -1.27% | 134% | 114% |
| Class C | -0.57% | -1.07% | -1.26% | 129% | 115% |
| Class E | -1.26% | -1.86% | -1.90% | 135% | 114% |
| Overall | -0.87% | -1.36% | -1.40% | 133% | 113% |
| Class D | -0.25% | -0.68% | -0.76% | 129% | 115% |

# Conclusion

A new way of performing boundary filtering between the intra prediction of a current block and its causal neighborhood is presented in this document. With only marginal complexity increase the method brings around 0.65% BD-rate gain over VTM for the luma component in All Intra configuration, when the method is activated for all intra modes in luma. Combined with MRIP, the gains reportedly reach 0.87% on the luma component in AI configuration. It is then recommended to consider such a tool in CE3.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).