| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0175-v1 |
| --- | --- |

| Title: | CE3-related: Advanced MPM based on intra reference line selection scheme |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hui-Yu Jiang Yao-Jen Chang | Email: | teresa.hy.jiang@foxconn.com morriss.yj.chang@foxconn.com |
| Source: | Foxconn Technology Group |  |  |

_____________________________

- Abstract

An advanced MPM (AMPM) is proposed to signal the selected reference line with the best reference samples to the decoder for each CU coded by angular MPM modes. On top of VTM AI configuration, the proposed AMPM provides 0.5% luma coding gain with 103%/99% coding runtime as compared to VTM anchor. On top of VTM AI configuration with ‘Intra65Ang = 1’ and 6MPM disabled, i.e., original 3 MPMs in VTM enabled, the proposed AMPM provides 1.x% luma coding gain with 10x%/10x% coding runtime as compared to VTM anchor.

Fig. 1. Intra reference samples around the CU

Introduction

In VTM [1], the candidate list for 3 MPMs is composed of the intra modes from left and above neighboring blocks, the derived angular modes and the default modes (Planar and DC). An MPM flag is signaled to indicate whether to enable the MPM for each CU. If the MPM is enabled for this CU, an MPM index flag is further signaled to indicate which MPM mode is selected. Conventional MPMs always use the reference samples out of the nearest reference line (RL) for prediction, i.e., RL0 in Fig. 1. This contribution proposes an advanced MPM (AMPM) based on intra reference line selection scheme to signal the best reference line with the best intra reference samples among RL0, RL1, RL2 and RL3 for the CU coded by angular MPM. Note that RL0, RL1, RL2 and RL3 are as depicted in Fig. 1.

Method Description

The proposed AMPM signals a RL index flag after the MPM flag and the MPM index flag to select the RL with the best reference samples for each angular MPM. The RL index flag signals 0, 10, 110, and 111 for RL0, RL1, RL2 and RL3, respectively. Non-angular MPMs, i.e., Planar and DC mode, always select RL0. At encoder, additional SATD checks are performed to evaluate the MPM modes using the reference samples from one of RLs.

Experimental Results

The proposed AMPM is implemented on top of VTM and BMS configurations under common test conditions [2]. Three tests were conducted:

Test 1: AMPM

Test 2: AMPM with ‘Intra65Ang = 1’ on top of original MPM list in VTM (3 MPMs)

Note that the original MPM list in VTM includes 3 MPMs, but the original MPM list in BMS includes 6 MPMs. Table 1 and Table 2 show the experimental results for Test 1 and Test 2 in VTM configuration. Table 3 and Table 4 show the experimental results for Test 1, Test 2 and Test 3 in BMS configuration.

Table 1. Summary of simulation results (Test 1 vs. VTM-1.0 Anchor)

Table 2. Summary of simulation results (Test 2 vs. VTM-1.0 Anchor)

Table 3. Summary of simulation results (Test 1 vs. BMS-1.0 Anchor)

Table 4. Summary of simulation results (Test 2 vs. BMS-1.0 Anchor)

Conclusion

This contribution proposes an advanced MPM (AMPM) based on an intra reference line selection scheme. Conceptually, it is related to CE3.5 by applying the multiple reference lines methods to MPM modes. It can be expected various methods tested in CE3.5 can further improve the proposed AMPM. Moreover, AMPM with ‘Intra65Ang = 1’ configuration provides a competitive performance in terms of bitrate reduction and coding runtime as compared with different MPM methods in CE3.3. Therefore, it is suggested to consider the proposed AMPM for the potential adoption into VTM.

Reference

- J. Chen and E. Alshina, “Algorithm description for Versatile Video Coding and Test Model 1 (VTM 1),” JVET-J1002, San Diego, USA, Apr. 2018.

- J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, USA, Apr. 2018.

Patent rights declaration(s)

Foxconn Technology Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).