| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0175-v3 |
| --- | --- |

| Title: | CE3-related: Advanced MPM based on intra reference line selection scheme |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hui-Yu Jiang Yao-Jen Chang | Email: | teresa.hy.jiang@foxconn.com morriss.yj.chang@foxconn.com |
| Source: | Foxconn |  |  |

_____________________________

- Abstract

An advanced MPM (AMPM) is proposed to signal the selected reference line with the best reference samples to the decoder for the CU coded by angular MPM modes. On top of VTM-1.0 AI configuration, the proposed AMPM reaches 0.5% luma coding gain with 102%/99% coding runtime as compared to VTM-1.0 anchor.

Introduction

In VTM [1], candidate list for 3 MPMs is composed of the intra modes from left and above neighboring blocks, derived angular modes and default modes, i.e., Planar, DC and mode 26. An MPM flag is signaled to indicate whether to enable the MPM for each CU. If the MPM is enabled for this CU, an MPM index flag is further signaled to indicate which MPM mode is selected. Conventional MPMs always use the reference samples out of the nearest reference line (RL) for prediction, i.e., RL0 in Fig. 1. This contribution proposes an advanced MPM (AMPM) based on intra reference line selection scheme to signal the reference line with the best intra reference samples among RL0, RL1, RL2 and RL3 for the CU coded by angular MPM. Note that RL0, RL1, RL2 and RL3 are as depicted in Fig. 1.

Fig. 1. Intra reference samples around the CU

Method Description

The proposed AMPM signals a RL index flag after the MPM flag and the MPM index flag to select the RL with the best reference samples for each angular MPM. Non-angular MPMs, i.e., Planar and DC mode, always select RL0. The RL index flag signals 0, 10, 110, and 111 for RL0, RL1, RL2 and RL3, respectively. At encoder, additional SATD checks are performed to evaluate the MPM modes using the reference samples from one of reference lines.

Experimental Results

The proposed AMPM is implemented on top of VTM-1.0 and BMS-1.0 configurations under common test conditions [2]. The following two tests are conducted:

Test 1: AMPM

Test 2: 6 MPMs method (6 MPMs of BMS implemented to VTM-1.0)

Note that the original MPM list in VTM includes 3 MPMs, but the original MPM list in BMS includes 6 MPMs. Table 1 and Table 2 show the experimental results for Test 1 and Test 2 in VTM configuration. Table 3 shows the experimental results for Test 1 in BMS configuration.

Table 1. Summary of simulation results (Test 1 vs. VTM-1.0 Anchor)

Table 2. Summary of simulation results (Test 2 vs. VTM-1.0 Anchor)

Table 3. Summary of simulation results (Test 1 vs. BMS-1.0 Anchor)

Conclusion

This contribution proposes an advanced MPM (AMPM) based on an intra reference line selection scheme. Conceptually, it is related to CE3.5 by applying the multiple reference lines methods to MPM modes only. It could be expected various methods tested in CE3.5 can further improve the proposed AMPM. Moreover, AMPM provides more competitive performance than 6 MPMs method (6 MPMs of BMS implemented to VTM-1.0) in terms of bitrate reduction and coding runtime. Therefore, it is suggested to consider the proposed AMPM for the potential adoption into VTM.

Reference

- J. Chen and E. Alshina, “Algorithm description for Versatile Video Coding and Test Model 1 (VTM 1),” JVET-J1002, San Diego, USA, Apr. 2018.

- J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, USA, Apr. 2018.

Patent rights declaration(s)

Foxconn may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).